| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0123 |
| --- | --- |

| Title: | Proposal to Consider A New Work Item and Its Use Case – REI : An Ultra-multiview 3D Display – |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehrdad Panahpour Tehrani Takanori Senoh Makoto Okui Kenji Yamamoto Naomi Inoue Toshiaki Fujii Hiroya Nakamura | Tel: Email: | mehrdad.ptehrani@nict.go.jp senoh@nict.go.jp okui.m@nict.go.jp k.yamamoto@nict.go.jp na-inoue@nict.go.jp fujii@nuee.nagoya-u.ac.jp nakamura.hiroya@jvckenwood.com |
| Source: | NICT, Nagoya University, JVCKENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution encourages the MPEG community to consider the use case that is introduced in this document. This contribution is proposing the MPEG-3DV group to move to a next phase that seeks standard for compression of contents of super/ultra-multiview 3D displays.

MPEG community is currently standardizing a new codec for autostereoscopic displays where the test input signal is 2view+2depth or 3view+3depth. In this scenario, at most about 30views can be generated and tested at display side. Recently, super/ultra-multiview 3D displays are emerging widely, anticipated as the next generation of auto-stereoscopic display. Since these super/ultra multiview 3D displays require a huge number of multi-view images in real-time, a new coding standard is essential to realize their services and products in the market.

# Background

The demanding requirements for 3D display are 3D glasses-free, a higher resolution (pixel assignment) per view, supports as many as viewers at the same time, enlarge screen size to support life size representation of environment. National Institute of Information and Communications Technology (NICT), Japan, balanced these requirements and has developed a 200-inch full HD super multiview 3D display, i.e. REI (Ray Emergent Imaging) [1][2] that consist of 170 views, fluid motion parallax in a horizontal direction. Many pioneering works [3]-[6] helped to find our ways for attaining a large imaging, higher definition, and fluid and dense motion parallax. REI provides 3D impression without especial glasses, and users have freedom to choose the view angle by moving horizontally in front of the display. It is the world largest 3D multi-view display (200-inch) and has been also demonstrated worldwide in NAB show April 2012, USA. Since April 26, 2013 it has been moved to a public area located in Grand Front Osaka Japan. Having this display available for public and referring to individual, industrial and content providers’ feedbacks, we faced a greater need from many users for commercialized version of REI and its contents.

Super/ultra-multiview 3D displays are emerging widely, anticipated as the next generation of auto-stereoscopic display, such as Holovizio by Holographica [7], SMV256 by Takaki [6], 360 degree LFD by USC [8], Holo Table by Holy Mine [9], f-vision by NICT [10], Integral Photography by NHK [30] and 3D VIVANT[33], 3D TV by MERL[31], Projection type IP by Seoul National University display [32], etc. Since these super/ultra multiview 3D displays require a huge number of multi-view images in real-time, a new coding standard is essential to realize their services and products in the market.

The use-cases of these super/ultra-multiview displays are in the 3D movie theatres, 3D exhibitions, medical fields, educations and trainings, by providing overwhelming reality and immersive presence.

# Motivation

Our motivation to input this technical document to MPEG is to indicate that the display technology is ahead of the progress in compression technology. Therefore, there is an immediate need for an efficient compression scheme of the contents for such a display.

Compression of MVV (Multi-View Video) is a challenging and an important issue that has already been investigated by MPEG (Motion Picture Expert Group). In order to achieve an efficient and standard method for coding of multiview images, MPEG started its standardization in 2004. MVC (Multiview Video Coding) [11][12] was standardized in 2007 and has been commercialized since 2009. MVC considers correlation in multiview video data. MVC applies motion compensation approach in H.264/MPEG-AVC (Advanced Video Coding) [13] to not only time and but also view direction. MVC outperforms H.264/MPEG-AVC with bit-rate saving about 20-30%.

Furthermore, MPEG defined a reference model, where MVD (Multiview Video plus Depth) data format is adopted [14]. In January 2008, MPEG targeted the standardization of 3DV (3D Video) coding based on MVD. This activity has requirement of applications such as auto-stereoscopic displays. MPEG is currently working on compression standard for MVD data format [15]. Using MVD requires less number of data for transmission, and the missing viewpoint can be easily synthesized and delivered to user right before the display.

For a large 3D display such as REI, NICT 200-inch 3D Display, MVD compression can be an initial candidate. However, due to large scale of the system, the data size for compression is huge. Other data format such as LDI (Layered Depth Image) [16], LDV (Layered Depth Video) [17], FDU (FTV Data Unit) [18], and GVD (Global View and Depth) [19]-[23] were proposed based on MVD. Currently, survey on alternative 3D format is being progressed by AhG on Alternative 3D Formats, indicating the possibility for new 3D video representation.

Figure 1.  (top) The 200-inch 3D display, projector array and screen. (bottom) Two views at a frame from two different angles.

# A Use Case – REI

REI (Ray Emergent Imaging) is fed by 170 full HD images per frame to compete with conventional HDTV or current 3D cinema [1][2]. Users have natural 3D depth impression without using special glasses. The viewpoint can be freely chosen by relocating in front of the display, as if we relocate in front of a real object. Such life-sized objects as humans and cars can be fit or rendered in the 16:9, 200-inch large screen display. REI provides dense and fluid horizontal motion parallax and limited flipping effect appears in the observed images.

The display consists of a screen and a projector array. The screen is the combination of a diffuser next to a condenser lens. The diffuser narrows the angle of the incident light from behind in horizontal direction to provide accurate motion parallax, while the vertical diffusion is widen. The condenser lens converge the light rays to the designated viewpoint.

There are 170 customized projectors in operation that are aligned slanted. The projectors are located behind the screen; see Figure 1(top). Each projector has significant image quality and a unique horizontal location and provides a single viewpoint through the screen optics, with 33mm pitch. Optical centers are aligned perpendicular to the screen. The viewing interval is 22.8mm. Figure 1(bottom) shows two screenshot views images of the display at the same time from two different locations. The display specifications are in Table 1. In future we are planning to increase the FoV (Field of View) by increasing the number of projectors.

Figure 2.  Top view of REI display core.

Our previous implementation [24] had a viewing zone angle up to 13-deg. that corresponded to a 1.5m wide viewing zone at 5.5m designated viewing distance. We improved our previous viewing zone angle to support many more viewers. Fig. 2 explains the core design of REI, our automultiscopic projection display. Limiting the size of the parallax interval is effective for natural motion parallax [25]. A projector array must be designed to improve image quality. The ideal alignment of the projection units is horizontally straight, but a projection unit cannot be small. Especially, a higher definition projector itself has certain physical width and height. Projection units in an array are aligned with a horizontal pitch p, and a vertical pitch h (Figure 3).

Table 1.  REI display Specifications

| Item | Description |
| --- | --- |
| Screen size | 16:9, 200 inch, 4.4m x 2.5m |
| Number of projection units | 170 |
| Projector pitch, p | 33mm |
| Projection distance, Dp | 8.0m |
| Viewing distance, Do | 5.517m |
| Viewing interval, d | 22.8mm |
| Viewing zone angle | 40o |

Figure 3.  Alignment of projection units for the array. (Front view)

## Viewing Zone

In the current prototype we have broaden the viewing zone angle without increasing the viewpoint interval. In order to achieve this goal, we have added more projection units to the projection array to expand the viewing zone. If a projection unit is placed in an angled position far outside a line perpendicular to the screen and through its centre, the micro device display’s surface is also placed at an angle against the screen surface (Figure 3). This modification changes the projection distances on both the left and right sides of the screen image. Projection lens must have enough depth of focus to project from an angled position.

The projector array consists of 170 projection units, each of which is equipped with RGB 1080/60p three-panel micro device displays. The projection units are grouped into 5 racks as depicted in Figure 4.The centre rack is placed parallel to the screen. Next two racks to the centre rack are rotated 8 deg. (θ1) against the centre rack. Outmost two racks are rotated 13 deg. (θ2) against the centre rack.

When the amount of lens shift exceeds the maximum of the projection lens, the projected image cannot cover the entire screen space. For viewers, the lack of lens shift appears as a missing black region on a side edge; angled projection is needed. Since the projection unit is placed in a slanted position along a line perpendicular to the screen, the keystone effect occurs in projected images. This effect has to be eliminated for precise overlaid projection. Lens shift is an optical correction process for non-angled projections, but universal keystone correction needs to electrically compensate the input signals. This loosens the resolution of the screen image, depending on the slanted angle of a projection unit.

The slanted angles can be defined as follows. First we determine the zoom value. Then each slanted angle is decided based on the projection unit’s locus in an array. This angle can be different in each projection unit, or the same angle can be assigned to plural units for easy array set-up. This angle should also be equivalent to the maximum slanted angle among the projection units.

Figure 4.  An example of an angled projection. (Top view)

## Projector Array

Before 3D image can be viewed, all parallax images for the array must be provided. Whether they are computer-generated imagery or actual camera footage, parallax images must be rendered with parallel (not converging) camera condition [26]. With this condition, we just replace the eye marks in Fig. 2 with cameras [27]. Then the source parallax images are processed with the following procedures.

## REI System

All parallax images are stored as uncompressed data in the backend systems as shown in Fig. 5 and sent to each projection unit in a form of baseband signal. 3G-SDI format was chosen since it has enough capabilities to transmit true high definition images at 60 Hz; moreover it is also de facto standard to record stereoscopic 3D signals among professional video communities. The multiple hardware based video servers can perform a playback with frame-accurate synchronization. For this synchronization control, the frame buffers of each video server use and share the SDI embedded control signal. An external reference should be provided only to the master server referred to as “video server #1” in Fig. 5.

- Video servers: variant of RAM based memory player. We employ PC workstations equipped with the two frame buffer cards that have both multi-channel baseband video signal I/O and network file transfer capability connected by a PCI Express bus.

(a) 3G-SDI video signal 8 in/8 out: SMPTE 424M format, 1080/59.94i, 1080/59.94p, 1080/60i, 1080/60p, YCbCr 4:2:2 8/10 bpp, RGB 4:4:4 10/12 bpp. uncompressed, baseband signal

(b) Ancillary timecode support: SMPTE RP188/ARIB STD-B4 timecode embedded on a 3G-SDI video signal, timecode embedded in a 3G-SDI output signal can be synchronize such external equipment as audio players and lighting controllers.

(c) Memory capacity: 8GB per channel - 1,550 frames at 1080/60p YCbCr 10 bpp uncompressed format

(d) Video payload identification: an embedded channel ID confirms that every single connection is firm and safe for large-scale installation.

(e) Sync: supports 3G-SDI as external references (future support: tri-level sync, black burst)

(f) Data storage capacity: 8 TB (RAID0) for every 8-ch

(g) Hardware playback: hardware takes full control at normal speed playback.

- Signal format converter: projector can receive HDMI signals, which are complex and difficult to extend more than 5m. We use a 3C-FB coaxial cable with BNC connectors since it is inexpensive and can transmit 1080/60p signal up to 40 m.

(a) Input: 24ch, SMPTE 424M standard 3G-SDI video signal, 1080/59.94p, 1080/60p, 4:2:2 10 bpp, 4:4:4 8/10 bpp

(b) Video payload identification: supported, channel ID can be verified

(c) Output: 24-ch HDMI format, RGB/YCbCr 8/10bpp

Figure 5.  Backend systems diagram (24-ch extraction)

# Introduction of SECOND-MVD: An example of 3D format for super/ultra-multiview 3D displays

SECOND-MVD (Synthesis Error COmpeNsateD Multiview Video plus Depth) is an example of codecs for such use-case of super/ultra-multiview 3D displays that we proposed for representation and compression of large multiview video. This type of real-time codec for the super/ultra-multiview 3D displays may be standardized as an alternative 3D format of currently on-going 3D video standardization work, or may become a new work item of the standardization. In any case, it will be worthwhile to explain its coding architecture. SECOND-MVD is derived from MVD. Figure 5 shows an example of view configuration and conversion process of SECOND-MVD. In this data format, images at some viewpoint remain original, and the others are converted to a novel format. Residual based representation, such as LDV, FDU and GVD were proposed and can be categorized as a type of SECOND-MVD. We proposed a hybrid format for converted image, which consists of residual and remainder pixels. Generation and reconstruction methods of a hybrid image uses virtual image synthesized by the images that remained original. Details algorithms for this data conversion can be found in [28][29].

We have investigated the compression performance of SECOND-MVD using hybrid image based on configuration depicted in Figure 6. Experiments demonstrated up to 5% reduction in bit rate using hybrid image against residual image in SECOND-MVD framework. Detailed report on our method may appear in future MPEG meetings.

Figure 6.  MVD and SECOND-MVD data format configurations.

Figure 7.  Configuration for Compression of SECOND-MVD

# Our Recommendation to MPEG

As discussed earlier in this document, super/ultra-multiview 3D displays are emerging widely, anticipated as the next generation of auto-stereoscopic display, such as Holovizio by Holographica [7], SMV256 by Takaki [6], 360 degree LFD by USC [8], Holo Table by Holy Mine [9], f-vision by NICT [10], Integral Photography by NHK [30] and 3D VIVANT[33], 3D TV by MERL[31], Projection type IP by Seoul National University display [32], etc. Since these super/ultra multiview 3D displays require a huge number of multi-view images in real-time, a new coding standard is essential to realize their services and products in the market.

Therefore, we highly recommend MPEG to consider this use case, and immediate start of the next phase of 3D-Video-Coding by investigating following issues.

- Consider the super/ultra multiview 3D displays, as a new use case

- Consider the codec standardization for super/ultra multiview 3D displays as a new work item.

- Consider the requirement on Input and Output Data for such a codec.

- Call for test sequences.

- Requirement on system configuration and specifications.

- Experimental Exploration on existing coding tools for very large multi-view video.

- Experimental Exploration on novel coding tools for very large multi-view video

# References

[1] S, Iwasawa, M. Kawakita, and N. Inoue, “REI: an automultiscopic projection display,” Proc. 3DSA 2013, Selected paper 1, Jun (2013).

[2] S. Iwasawa, et al., “Implementation of autostereoscopic HD projection display with dense horizontal parallax,” Proc. SPIE 7863, 78630T (2011)

[3] W. Matusik and H. Pfister, “3D TV: a scalable system for real-time acquisition, transmission, and autostereoscopic display of dynamic scenes,” ACM Trans. Graph. 23(3), 814-824 (2004)

[4] T. Agocs, T. Balogh, T. Forgacs, F. Bettio, E. Gobbetti, G. Zanetti, and E. Bouvier, “A large scale interactive holographic display,” Virtual Reality Conference, 2006, 311, 25-29 (2006)

[5] R. Yang, X. Huang, S. Li, and C. Jaynes, “Toward the light field display: Autostereoscopic rendering via a cluster of projectors,” IEEE Trans. Visualization and Computer Graphics 14, 84-96 (2008)

[6] Y. Takaki and N. Nago, "Multi-projection of lenticular displays to construct a 256-view super multi-view display," Opt. Express 18, 8824-8835 (2010).

[7] http://www.holografika.com/

[8] http://gl.ict.usc.edu/Research/3DDisplay/

[9] http://www.holymine3d.com/prod/prod03.html

[10] http://urv.nict.go.jp/member/shun/fVisiOn/fVisiOn.html

[11] “Introduction to Multi-view Video Coding”, ISO/IEC JTC 1/SC 29/WG11, N7328, Jul. 2005.

[12] P. Merkle, A. Smolic, K. Müller, and T. Wiegand, "Efficeint Compression of Multi-view Video Exploiting inter-view dependency based on H.264/MPEG-AVC," Proc. of ICME 2006, 1717-1720, (2006).

[13] T. Wiegand, G. J. Sullivan, G. Bjontegaard, and A. Luthra, "Overview of the H.264/AVC Video Coding Standard", IEEE Transactions on Circuits and Systems for Video Technology, 13- 7, 560-576, Jul (2003).

[14]ISO/IEC 14496-10:2012/FDAM2: MVC extensions for inclusion of depth maps, May, 2013

[15]ISO/IEC JTC1/SC29/WG11MPEG2011/N12036: Call for Proposals on 3D Video Coding Technology , March 2011

[16] J. Shade, S. J. Gortler, L. He, R. Szeliski, Layered Depth Images, Proc. SIGGRAPH 98, 231-242, Jul. 1998.

[17] K. Müller, A. Smolic, K. Dix, P. Merkle, P. Kauff T. Wie-gand, “Reliability-based generation and view synthesis in layered depth video“, Proc. of IEEE Intl Conf on MMSP, 34-39, Oct.2008.

[18] M. Tanimoto, M. Wildeboer, “Frameworks for FTV coding”, Proc. of PCS, 1-4, May 2009.

[19] T. Ishibashi, T. Yendo, M. Panahpour Tehrani, T. Fujii, M. Tanimoto, “3D space representation using epipolar plane depth image”, Proc. of PCS, 1-4, Dec 2010.

[20] T. Senoh, Y. Ichihashi, H. Sasaki, K. Yamamoto, “Simple multi-view coding with depth maps”, Springer, 3D Research, 04, 02(2013)2, 1-12, April 2013.

[21] T. Senoh, Y. Ichihashi, H. Sasaki and K. Yamamoto, “Multi-View Image and Depth Map Coding for Holographic reconstruction,” Proc. 3DSA 2013, S14-1, Osaka, Japan, Jun (2103).

[22] T. Senoh, Y. Ichihashi, H. Sasaki, .K. Yamamoto, M. Tanimoto, K. Suzuki, “AHG8 Revision on Depth View Information SEI message in 3D-AVC DT5,” ISO/IEC JTC1/SC29/WG11/ M28873r3, April 2013.

[23] T. Senoh, Y. Ichihashi, H. Sasaki, .K. Yamamoto, M. Tanimoto, K. Suzuki, “AHG8 Description of Global View and Depth Format for Depth View SEI Message,” ISO/IEC JTC1/SC29/WG11/ M28874r2, April 2013.

[24] S. Iwasawa, M. Kawakita, “Behind the Screen of NICT’s 200-in. Automultiscopic Display,” Proc. The International Display Workshops (IDW/AD '12), vol. 19, 3D9-1 (2012).

[25] Y. Kajiki, H. Yoshikawa, and T. Honda, “Hologram like Video Images by 45-view Stereoscopic Display,” Proc. SPIE vol. 3012, pp. 154-166 (1997).

[26] R. Lopez-Gulliver, S. Iwasawa, M. Makino, M. Kawakita, and H. Ando, “3D Computer Graphics Rendering for Multi-projector based Autostereoscopic Displays,” Proc. 3D Systems and Applications 2011, pp. 289-290 (2011).

[27] M. Kawakita, et al., “3D video capturing for multiprojection type 3D display,” Proc. SPIE vol. 8043, 804311 (2011).

[28] M. Panahpour Tehrani, A. Ishikawa, M. Kawakita, N. Inoue, T. Fujii, “A hybrid representation for multi-view image,” Proc. of 3DTV-CON 2012, Oct. (2012).

[29] M. Panahpour Tehrani, A. Ishikawa, M. Kawakita, N. Inoue, T. Fujii,” Compression of Multi-View Images Using Hybrid Representation for 200-Inch 3D Display,” Proc. IWAIT 2013, pp.498-503, Nagoya, Japan, Jan (2013).

[30] J. Arai, F. Okano, M. Kawakita, M. Okui, Y. Haino, M. Yoshimura, M. Furuya, and M. Sato, “Integral Three-Dimensional Television Using a 33-Megapixel Imaging System,” JOURNAL OF DISPLAY TECHNOLOGY, VOL. 6, NO. 10, pp.422-430, OCTOBER 2010.

[31] W. Matusik, H. Pfister, “3D TV: A Scalable System for Real-Time Acquisition, Transmission and Autostereoscopic Display of Dynamic Scenes”, ACM SIGGRAPH, ISSN: 0730-0301, Vol. 23, Issue 3, pp. 814-824, August 2004.

[32] Hong J, Kim Y, Park SG, Hong JH, Min SW, Lee SD, Lee B., "3D/2D convertible projection-type integral imaging using concave half mirror array.", Opt Express. 2010 Sep 27;18(20):20628-37. doi: 10.1364/OE.18.02062

[33] JCT3V-C0210:  3D Holoscopic Video Test Material, FP7 3D VIVANT Consortium, Jan. 2013

# Patent rights declaration(s)

NICT, Nagoya University, and JVCKENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).