| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AU, July 27– Aug. 2, 2013; Document: JCT3V-Exxxx _proposed text. | Document: JCT3V-E0125 _proposed text |
| --- | --- |

| Title: | Proposed text for JCT3V-E0125 based on 3D-HEVC Test Model 4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Liu He Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides text for Draft 4 of 3D-HEVC Test Model Description. All the changes are marked with red strikethrough, the newly added parts are highlighted in green.

# Proposed specification draft

- H.8.1.1	Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

  - The decoding of NAL units is specified in subclause 8.2.

  - The following steps specify the decoding processes using syntax elements in the slice segment layer and above:

    - Prior to decoding the first slice of the current picture, subclause G.8.1.1 is invoked.

    - At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause G.8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

    - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1 or iv_res_pred_flag[ nuh_layer_id ] is equal to 1, a motion vector variable mvDDV is initialized to be (0, 0) and decoding process for candidate picture list for disparity vector derivation in subclause Error: Reference source not found is invoked at the beginning of the decoding process for each P or B slice [Ed. (GT): VSP should be added here as condition.]

    - At the beginning of the decoding process for each P or B slice, the derivation process for the alternative target reference index for TMVP in merge mode as specified in subclause Error: Reference source not found is invoked.

    - When dlt_flag[ nuh_layer_id ] is equal to 1, the decoding process for the depth lookup table in subclause Error: Reference source not found is invoked at the beginning of the decoding process of first slice.

    - After the decoding of the last slice of the current picture, the marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause G.8.1.2 is invoked.

  - The following ordered steps specify the decoding processes using syntax elements in the coding tree unit layer and above.

    - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, the derivation process for disparity vectors as specified in subclause H.8.5.4 is invoked with the luma locations ( xC , yC ), the coding block size nCSL as the inputs.

    - The processes in subclauses Error: Reference source not found, , Error: Reference source not found, and Error: Reference source not found are invoked for the decoding processes using syntax elements in the coding tree unit layer and above.

- H.8.5	Decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause 8.6.1 is invoked with the luma location ( xC, yC ) as input.

The variable nCSL is set equal to 1 << log2CbSize and the variable nCSC is set equal to ( 1 << log2CbSize ) >> 1.

Decoding process for coding units coded in inter prediction mode consists of following ordered steps:

  - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, the derivation process for disparity vectors as specified in subclause  is invoked with the luma locations ( xC , yC ), the coding block size nCSL as the inputs.

  - The inter prediction process as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), and the luma coding block size log2CbSize as the inputs and the outputs are three arrays predSamplesL, predSamplesCb, predSamplesCr.

  - ..

- H.8.5.2.1	Derivation process for motion vector components and reference indices

…

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xP.. ( xP + nPbW − 1 ), y = yP..( yP + nPbH− 1 ) (with X being either 0 or 1):

  - IvpMvFlagLX[ x ][ y ] = ivpMvFlagLX		(Error: Reference source not found-1)
VspModeFlag[ x ][ y ] = vspModeFlag		(Error: Reference source not found-2)

- H.8.5.2.1.1	Derivation process for luma motion vectors for merge mode

...

  - The disparity availability flag ivpMvFlagLX is derived as follows (with X being replace by 0 or 1).

    - If all of the following conditions are true, ivpMvFlagLX is set equal to 1

- [Ed. (GT) There is some redundancy in draft and software since disparities equal for both lists.(#7) ]

      - availableFlagIvMC = = 1

      - merge_idx[ xP][ yP ] = = 0

      - predFlagLXIvMC = = 1

[Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

    - Otherwise, ivpMvFlagLX is set equal to 0.

- H.8.5.4	Derivation process for a disparity vector

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

When xC and yC % CtbSizeY are both equal to 0, mvDDV is set to 0.

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

The variable checkParallelMergeFlag is derived as follows:

- If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

  - PredMode[ xC ][ yC ] is equal to MODE_SKIP.

  - PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xC ][ yC ] is equal to 1.

- Otherwise, checkParallelMergeFlag is set equal to 0.

The derivation process for a disparity vector from temporal neighbour block as specified in Error: Reference source not found is invoked with the luma location ( xC, yC ), and the variable nCS as inputs, and the outputs are the flag availableDV, the disparity vector mvDisp and the reference view order index refViewIdx.

When availableDV is equal to 0, for each N being A1, B1, B0, A0, or B2, and ( xN, yN ) being ( xC − 1,  yC + nCS − 1 ), ( xC + nCS − 1,  yC − 1 ), ( xC + nCS,  yC − 1 ), ( xC − 1,  yC + nCS  ), or ( xC − 1,  yC − 1 ) the following ordered steps apply.

  - When yC − 1 is less than ( ( yC  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ), the following applies.

        - xB0 = ( ( xB0  >>  3 )  <<  3 ) + ( ( xB0  >>  3 ) & 1) * 7	(Error: Reference source not found-3)
xB1 = ( ( xB1  >>  3 )  <<  3 ) + ( ( xB1  >>  3 ) & 1) * 7	(Error: Reference source not found-4)
xB2 = ( ( xB2  >>  3 )  <<  3 ) + ( ( xB2  >>  3 ) & 1) * 7	(Error: Reference source not found-5)

  - The derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to the ( xC, yC ) and the luma location ( xN, yN ) as the input and the output assigned to availableN.

  - When availableN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_INTRA, availableN is set equal to 0. [Ed. (GT): 2+3 correspond to 6.4.2 for CU when ( xN, yN ) outside CU. Cross-check appreciated. ]

  - When all of the following conditions are true, availableN is set equal to 0.

    - checkParallelMergeFlag is equal to 1

    - ( xC  >>  ( log2_parallel_merge_level_minus2 + 2) ) is equal to
( xN  >>  ( log2_parallel_merge_level_minus2 + 2) )

    - ( yC  >>  ( log2_parallel_merge_level_minus2 + 2) ) is equal to ( yN  >>  ( log2_parallel_merge_level_minus2 + 2) ).

  - The flag availableIvpMvSearchFlagN is set equal to availableN.

  - When one of the following conditions is true, availableIvpMvSearchFlagN is set equal to 0.

    - N is equal to B0 and ( ( yN  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ) is less than ( ( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

    - N is equal to B1 and ( ( yN  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ) is less than ( ( yC >> Log2CtbSizeY )  <<  Log2CtbSizeY)

    - N is equal to B2 and ( ( yN  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ) is less than ( ( yC >> Log2CtbSizeY )  <<  Log2CtbSizeY )

  - The flag availableFlagIvpMvN is set equal to 0.

  - For each X from 0 to 1, the following applies:

    - When availableDV is equal to 0, availableN is equal to 1, RefIdxLX[ xN ][ yN ] is greater than or equal to 0, and PredFlagLX[ xN ][ yN ] is equal to 1, the following applies:

      - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture, the following applies:

          - refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(Error: Reference source not found-6)
mvDisp = MvLXN[ xN ][ yN ]		(Error: Reference source not found-7)
availableDV = 1		(Error: Reference source not found-8)

      - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies:

        - When availableIvpMvSearchFlagN is equal to 1, availableFlagIvpMvN is equal to 0, and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

            - ivpMvDispN = MvDispRefined[ xN ][ yN ]	(Error: Reference source not found-9)
refViewIdxN = RefViewIdx[ xN ][ yN ]	(Error: Reference source not found-10)
availableFlagIvpMvN = 1		(Error: Reference source not found-11)
[Ed. (GT). In HTM-7.0 refViewIdxN (related to #5) is set equal to 0 and ivpMvDispN[ 0 ] are set equal to 0. ]

When availableDV is equal to 0 for each N being A1, B1, B0, A0, and B2, the following applies.

  - When availableDV is equal to 0 and availableFlagIvpMvN is equal to 1, the following applies:

      - mvDisp = ivpMvDispN		(Error: Reference source not found-12)
refViewIdx = refViewIdxN		(Error: Reference source not found-13)
availableDV = 1		(Error: Reference source not found-14)

- When availableDV is equal to 1, mvDDV is set equal to mvDisp.

When availableDV is equal to 0 refViewIdx is set equal to 0, mvDisp is set equal to ( 0, 0 ) mvDDV, and availableDV is set equal to 1. The variable mvRefinedDisp is set equal to mvDisp.

When depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following ordered steps apply:

  - The derivation process for disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xC, yC, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCS, the variable nPSH equal to nCS, the variable nSubBlkW equal to nCS, and the variable nSubBlkH equal to nCS, as the inputs, and the output is the array disparitySamples of size (nCS)x(nCS).

  - The horizontal component of the disparity vector mvRefinedDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].

  - mvDDV is set equal to mvRefinedDisp.

[Ed. (GT): The disparity vector derivation has the problem related to refViewIdx as described in the editor's comment in subclause Error: Reference source not found and needs further discussion. (#5) ]

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xC.. ( xC + nCS − 1 ), y = yC..( yC + nCS− 1 ):

  - MvDisp[ x ][ y ] = mvDisp ivpMvFlagLX		(Error: Reference source not found-15)
MvRefinedDisp[ x ][ y ] = mvRefinedDisp		(Error: Reference source not found-16)
RefViewIdx[ x ][ y ] = refViewIdx		(Error: Reference source not found-17)