| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0125 |
| --- | --- |

| Title: | CE2.h: Derived disparity vector for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Liu He Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, the NBDV process checks the neighboring blocks for not only disparity motion vectors but also implicit disparity vectors (IDVs) which are generated and stored during the NBDV processes for the neighboring blocks. It is considered that the IDVs may slightly increase the storage. Therefore, in this proposal, IDV is simplified by using the so-called derived disparity vector (DDV). This is a follow-up proposal of JCT3V-D0194. In this proposal, the DDV is initially set to zero, at the beginning of each coding tree block (CTB) row and updated after a CU is coded. With the proposed method, the storage increase in the block-level is not needed anymore. Negligible coding changes are noticed (smaller than 0.1% bitrate reduction) for synthesized views for both CTC and BVSP off cases.

# Introduction

In 3D-HEVC, the NBDV process checks neighboring blocks to derive a disparity vector. There might be cases when neighboring blocks do not contain any motion vector that is a disparity motion vector. To still return a disparity vector in the above cases, each block can contain not only motion vectors, but also implicit disparity vectors stored if the block utilizes a disparity vector for certain conditions. For simplicity, such a disparity vector is named implicit disparity vector in this document.

Each PU may contain implicit disparity vectors (IDVs) corresponding to different prediction direction (RefPicList0 or RefPicList1).

Although it has been optimized such that IDVs are never required to be stored into DPB, they need to be stored for the view component being processed. The storage used to store IDVs can be as large as that used for a motion field.

# Simulations results

Two sets of results are provided:

- Proposed method (with IDV is turned off) compared with 3D-HEVC CTC.

- Proposed method (with IDV is turned off) with BVSP off compared with 3D-HEVC CTC BVSP off.

For the second test set, “BVSP off” indicates that no texture view is coded based on any depth view, i.e., both BVSP and Do-NBDV are both disabled. The platform 3D-HTM v7.0r1 [1] is utilized and the proposed method is implemented on it. Simulation results of the above two cases are described in the following sub-sections.

## Proposal vs 3D-HTM(CTC)

In this test, the anchor is generated under CTC with 3D-HTM v7.0r1 wherein IDV is enabled, and the tested method is the proposed DDV with IDV disabled. As shown in Table 1, the overall average bitrate increase is 0.05%, 0.05% and 0.02% for texture views, coded views, and synthesized views, respectively.

Table 1: Coding gain of DDV with respect to 3D-HTM anchor for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.22% | 0.29% | 0.11% | 0.10% | 0.05% | 101% | 108% | 102% |
| Kendo | 0.44% | 0.32% | 0.15% | 0.16% | 0.11% | 104% | 101% | 97% |
| Newspapercc | 0.21% | 0.12% | 0.06% | 0.06% | -0.23% | 104% | 103% | 100% |
| GhostTownFly | 0.07% | 0.14% | 0.02% | 0.02% | 0.03% | 100% | 97% | 99% |
| PoznanHall2 | -0.08% | 0.02% | -0.02% | -0.03% | 0.04% | 100% | 99% | 97% |
| PoznanStreet | 0.11% | 0.27% | 0.05% | 0.04% | 0.05% | 101% | 106% | 96% |
| UndoDancer | 0.12% | 0.03% | 0.02% | 0.01% | 0.09% | 111% | 106% | 100% |
| 1024x768 | 0.29% | 0.25% | 0.11% | 0.11% | -0.02% | 103% | 104% | 100% |
| 1920x1088 | 0.06% | 0.11% | 0.02% | 0.01% | 0.05% | 103% | 102% | 98% |
| average | 0.16% | 0.17% | 0.05% | 0.05% | 0.02% | 103% | 103% | 99% |

## Proposal vs 3D-HTM (BVSP off)

In this test, the anchor is generated with 3D-HTM v7.0r1 with BVSP disabled and IDV enabled, and the tested method is the proposed DDV, IDV and BVSP disabled. As shown in Table 2, the overall average bitrate reduction is 0.05%, 0.04% and 0.05% for texture views, coded views, and synthesized views, respectively.

Table 2: Coding gain of DDV with respect to 3D-HTM anchor for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.18% | -0.57% | -0.09% | -0.08% | -0.12% | 101% | 108% | 102% |
| Kendo | 0.08% | -0.11% | -0.01% | -0.02% | 0.01% | 104% | 101% | 97% |
| Newspapercc | -0.49% | -0.67% | -0.24% | -0.19% | -0.21% | 104% | 103% | 100% |
| GhostTownFly | -0.08% | 0.17% | 0.01% | 0.02% | -0.01% | 100% | 97% | 99% |
| PoznanHall2 | 0.29% | -0.46% | -0.02% | -0.01% | -0.10% | 100% | 99% | 97% |
| PoznanStreet | 0.02% | 0.18% | 0.02% | 0.02% | 0.07% | 101% | 106% | 96% |
| UndoDancer | 0.14% | -0.05% | 0.00% | 0.00% | 0.01% | 111% | 106% | 100% |
| 1024x768 | -0.08% | -0.45% | -0.11% | -0.10% | -0.11% | 103% | 104% | 100% |
| 1920x1088 | 0.09% | -0.04% | 0.00% | 0.01% | -0.01% | 103% | 102% | 98% |
| average | 0.02% | -0.22% | -0.05% | -0.04% | -0.05% | 103% | 103% | 99% |

## Proposal vs JCT3V-D0194

Note that for both cases, the results provided by the method in JCT3V-D0194 [3] are almost the same as those provided by the current proposal.

Table 3 and Table 4 list the coding gain of this proposal compared to JCT3V-D0194 wherein the DDV is not reset to (0, 0) at the beginning of a new CBT row under CTC and BVSP off, respectivley. As shown in Table 5 and Table 6, the overall average bitrate increase is almost the same, 0.00% for texture views, coded views, and synthesized views under both test conditions.

Table 7: Coding gain of proposal vs JCT3V-D0194 (CTC)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.05% | 0.05% | 0.02% | 0.01% | 0.00% |
| Kendo | 0.02% | 0.00% | 0.00% | -0.01% | 0.00% |
| Newspapercc | 0.01% | 0.01% | 0.00% | 0.01% | 0.01% |
| GhostTownFly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| PoznanHall2 | -0.05% | 0.04% | 0.00% | 0.00% | -0.01% |
| PoznanStreet | 0.04% | 0.01% | 0.01% | 0.01% | 0.01% |
| UndoDancer | -0.01% | 0.00% | 0.00% | 0.00% | 0.00% |
| 1024x768 | 0.02% | 0.02% | 0.01% | 0.00% | 0.00% |
| 1920x1088 | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% |
| average | 0.01% | 0.01% | 0.00% | 0.00% | 0.00% |

Table 8: Coding gain of proposal vs JCT3V-D0194 (BVSP off)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.01% | 0.00% | 0.01% | 0.01% |
| Kendo | -0.03% | -0.02% | -0.01% | -0.01% | 0.00% |
| Newspapercc | 0.07% | 0.01% | 0.01% | 0.00% | 0.00% |
| GhostTownFly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| PoznanHall2 | 0.02% | -0.03% | 0.00% | 0.00% | 0.01% |
| PoznanStreet | -0.02% | -0.02% | 0.00% | 0.00% | 0.00% |
| UndoDancer | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% |
| 1024x768 | 0.02% | 0.00% | 0.00% | 0.00% | 0.00% |
| 1920x1088 | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% |
| average | 0.01% | -0.01% | 0.00% | 0.00% | 0.00% |

# References

- 3DV-HTM version 7.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1100, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

- L. Zhang, Y. Chen, M. Karczewicz, “CE2.h related: Derived disparity vector for 3D-HEVC,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D0194, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).