| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AU, July 27– Aug. 2, 2013 | Document: JCT3V-E0126 _proposed text |
| --- | --- |

| Title: | Proposed text for JCT3V-E0126 based on 3D-HEVC Test Model 4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vijayaraghavan Thirumalai, Li Zhang, Ying Chen, Marta Karczewicz, 5775 Morehouse Drive San Diego, CA 92121, USA Christine Guillemot, Laurent Guillo, INRIA, France Jian-Liang Lin, Yi-Wen Chen, Yu-Lin Chang MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | +1-858-845-6589 vthiruma@qti.qualcomm.com lizhang@qti.qualcomm.com +33 299847100 christine.guillemot@inria.fr laurent.guillo@inria.fr +886-3-5670766 ext. 25555 jl.lin@mediatek.com yiwen.chen@mediatek.com |
| Source: | Qualcomm Incorporated, INRIA, and MediaTek Inc. |  |  |

_____________________________

All the changes are highlighted in green or red strikethrough.

- H.8.5.2.1.1 Derivation process for luma motion vectors for merge mode

…

  - Depending on iv_mv_pred_flag[ nuh_layer_id ], the following applies.

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0, the flags availableFlagIvMCA[ i ] with i equal to 0 to 1 and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause  is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, as the inputs and the output is assigned to the availability flags array availableFlagIvMCA[ i ] with i equal to 0 to 1, and the availability flag availableFlagIvDC, the reference indexices array refIdxLXIvMCA[ i ] with i equal to 0 to 1, and the reference index refIdxLXIvDC, the prediction list utilization flags array predFlagLXIvMCA[ i ] with i equal to 0 to 1, and the prediction list utilization flag predFlagLXIvDC, and the motion vectors array mvLXIvMCA[ i ] with i equal to 0 to 1 and the motion vector mvLXIvDC (with X being 0 or 1, respectively).

- …

  - The merge candidate lists mergeCandList and mergeCandIsVspFlag are constructed as specified by the following ordered steps:

  - The variable numMergeCand is set equal to 0.

  - When availableFlagT is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to T, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvMCA[ 0 ] is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMCA[ 0 ], the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1, the following applies:

      - When the following condition is true,

        - availableFlagT = = 0 && availableFlagIvMCA[ 0 ] = = 0,

        - or one or more of the following conditions are true, with N being replaced by T and IvMCA[ 0 ]:

        - availableFlagN = = 1 && predFlagLXN ! = predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && mvLXN ! = mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && refIdxLXN ! = refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, the following applies:

      - When the following condition is true ,

        - availableFlagT = = 0 && availableFlagIvMCA[ 0 ] = = 0,

or one or more of the following conditions is true, with N being replaced by T and IvMCA[ 0 ]:

        - availableFlagN = = 1 && predFlagLXN ! = predFlagLXB1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && mvLXN ! = mvLXB1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && refIdxLXN ! = refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP + nPbW − 1 ][ yP − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP + nPbW ][ yP − 1 ] and the variable numMergeCand is increased by 1.

- When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to B2, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP − 1 ][ yP − 1 ] and the variable numMergeCand is increased by 1.

- The DSMV is generated by the following steps in order.

      - A variable availableFlagDSMV is set to 0 and for each of the spatial candidates N being equal to A0, A1, B0, B1 or B2 the following applies till availableFlagDSMV is 1.

        - When availableFlagN is equal to 1, predFlagL0N is equal to 1, refViewIdxL0N is not equal to curViewIdx and VspModeFlag[ xN ][  yN ] is equal to 0, availableFlagDSMV is set equal to 1, and the following apply in order

          - predflagLXDSMV = predflagLXN, ( with X being replaced by 0 and 1 )

          - refIdxLXDSMV = refIdxLXN, ( with X being replaced by 0 and 1 )

          - mvL0DSMV[ 0 ] = mvL0N[ 0 ] + 4

          - mvL0DSMV[ 1 ] = ( view_synthesis_pred_flag[ nuh_layer_id ] ? 0 : mvL0N[ 1 ])

          - mvL1DSMV = mvL1N

      - When availableFlagDSMV is equal to 0 the following apply

        - DSMV is set equal to IvDC with mvLXDSMV[0] = mvLXIvDC[0] + 4, when predFlagLXIvDC = 1.

- When DepthFlag is equal to 0, iv_mv_pred_flag[ nuh_layer_id ] is  equal to 1, and numMergeCand is less than 6, the following apply:

      - If availableFlagIvMCA[ 1 ] is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMCA[ 1 ], and the variable numMergeCand is increased by 1 if one or more of the following conditions is true.

        - availableFlagIvMCA[ 0 ] = = 0

        - predFlagLXIvMCA[ 0 ] ! = predFlagLXIvMCA[ 1 ] (with X being replaced by 0 and 1)

        - mvLXIvMCA[ 0 ] ! = mvLXIvMCA[ 1 ] (with X being replaced by 0 and 1)

        - refIdxLXIvMCA[ 0 ] ! = refIdxLXIvMCA[ 1 ] (with X being replaced by 0 and 1)

      - Otherwise, (availableFlagIvMCA[ 1 ] is equal to 0), the entry mergeCandList[ numMergeCand ] is set equal to DSMV and the variable numMergeCand is increased by 1.

- When availableFlagCol is equal to 1 and numMergeCand is less than 5 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to Col, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

…

  - The disparity availability flag ivpMvFlagLX is derived as follows (with X being replace by 0 or 1).

    - If all one of the following conditions are is true, ivpMvFlagLX is set equal to 1

- [Ed. (GT) There is some redundancy in draft and software since disparities equal for both lists.(#7) ]

      - availableFlagIvMC  = = 1

      - merge_idx[ xP][ yP ] = = 0

      - predFlagLXIvMC  = = 1

      - predFlagLXIvMCA[ 0 ] = = 1 and merge_idx[ xP][ yP ]  = = IvMCA[ 0 ]

      - predFlagLXIvMCA[ 1 ] = = 1 and merge_idx[ xP][ yP ] = = IvMCA[ 1 ]

[Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

    - Otherwise, ivpMvFlagLX is set equal to 0.

- H.8.5.2.1.9 Derivation process for inter-view merge candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags array availableFlagIvMCA[ i ] with i equal to 0 to 1 and availability flag availableFlagIvDC specifying whether the inter-view merge candidates are available,

- the reference indices index array refIdxLXIvMCA[ i ] with i equal to 0 to 1 and reference index refIdxLXIvDC,

- the prediction list utilization flags array predFlagLXIvMCA[ i ] with i equal to 0 to 1 and prediction list utilization flag predFlagLXIvDC,

- the motion vectors array mvLXIvMCA[ i ] with i equal to 0 to 1 and motion vector mvLXIvDC,

The temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause  is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the prediction list indication X , the view order index RefViewIdx[ xP ][ yP ], the disparity vector MvRefinedDisp[ xP ][ yP ], the flag mergeFlag being equal to 1, and the reference index array refIdxLXA[ i ] being equal to −1, with i equal to 0 to 1, inclusive and as the inputs and the outputs are the flag array availableFlagLXIvMCA[ i ], the motion vector array mvLXIvMCA[ i ] and the reference index array refIdxLXIvMCA[ i ], with i equal to 0 to 1, inclusive.

  - The availability flag array availableFlagIvMCA[ i ], and the prediction utilization flags arrays predFlagL0IvMCA[ i ] and predFlagL1IvMCA[ i ] are derived by the following for each i from 0 to 1, inclusive.

      - availableFlagIvMCA[ i ] = availableFlagL0IvMCA[ i ] | | availableFlagL1IvMCA[ i ] 	(Error: Reference source not found-1)
predFlagL0IvMCA[ i ] = availableFlagL0IvMCA[ i ] 	(Error: Reference source not found-2)
predFlagL1IvMCA[ i ] = availableFlagL1IvMCA[ i ] 	(Error: Reference source not found-3)

The disparity inter-view motion vector merging candidate is derived by the following ordered steps.

- …

- H. 8.5.2.1.10 Derivation process for a temporal inter-view motion vector candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a reference view index refViewIdx.

- a disparity vector mvDisp,

- a flag mergeFlag specifying whether a merge candidate is derived,

- a reference index array refIdxLXA[ i ], with i equal to 0 to 1, inclusive, each specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

- a flag array availableFlagLXInterViewA[ i ], with i equal to 0 to 1, inclusive, each specifying whether the temporal inter-view motion vector candidate is available,

- a temporal inter-view motion vector candidate array mvLXInterViewA[ i ], with i equal to 0 to 1 (if availableFlagLXInterViewA[ i ] is equal to 1),

- a reference index array refIdxLXA[ i ], with i equal to 0 to 1, inclusive, each specifying a reference picture in the reference picture list RefPicListLX,

- The flag array availableFlagLXInterViewA[ i ], is set to 0 for i from 0 to 1, inclusive, both components of the array mvLXInterViewA[ i ], are set equal to 0, for i from 0 to 1, inclusive.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

Set sftValA[ 0 ] [ 0 ] equal to 0; sftValA[ 0 ] [ 1 ] equal to 0;

Set sftValA[ 1 ] [ 0 ] equal to (((nPSW/2)*4)+4) and sftValA[ 1 ] [ 1 ] equal to (((nPSH/2)*4)+4);.

The following applies for each i from 0 to 1, inclusive:

The reference layer luma location ( xRef, yRef ) is derived by

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 )  >>  1 ) + ( ( mvDisp[ 0 ] + sftValA[ i ][ 0 ] +  2 )  >>  2 ) )	(Error: Reference source not found-4)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 )  >>  1 ) + ( ( mvDisp[ 1 ] + sftValA[ i ][ 1 ] +  2 )  >>  2 ) )	(Error: Reference source not found-5)

Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx.

[Ed. (GT:) In software refCU is the coding unit that covers the luma location ( xRef, yRef ) in the view component with smallest ViewIdx and that is included in one of the reference lists of the current view component.(#6)]

[Ed. (CY): What is implemented in the software has no impact on the final results in common test condition, so it is preferred to have the current text and software changes may be desirable. ]

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply, for Y in the range of X to (1 – X), inclusive:

  - The variable refPredFlagLY is set equal to the prediction utilization flag predFlagLY of the prediction unit refPU.

  - The variable refRefIdxLY, is set equal to the reference index refIdxLY of  the prediction unit refPU.

  - The variable refMvLY is set equal to the motion vector mvLY of the prediction unit refPU.

  - The variable refRefPicListLY, is set equal to the reference picture list RefPicListLY of the prediction unit refCU.

  - When refPredFlagLY is equal to 1, the following applies for each i from ( mergeFlag ? 0 : refIdxLX ) to ( mergeFlag ? num_ref_idx_lX_active_minus1 : refIdxLX), inclusive:

    - When availableFlagLXInterViewA[ i ] is equal to 0, and the picture order count of the picture refRefPicListLY[ refRefIdxLY ] is equal to the picture order count of the picture RefPicListLX[ i ], the flag availableFlagLXInterViewA[ i ]  is set equal to 1 and the following applies.

      - The motion vector mvLXInterView is derived by:

        - mvLXInterViewA[ i ][ 0 ] = refMvLY[ 0 ]		(Error: Reference source not found-6)
mvLXInterViewA[ i ][ 1 ] = refMvLY[ 1 ]		(Error: Reference source not found-7)

      - When mergeFlag is equal to 1, the reference index refIdxLXA[ i ] is derived by:

        - refIdxLXA[ i ]  = i		(Error: Reference source not found-8)