| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AU, July 27– Aug. 2, 2013 | Document: JCT3V-E0126 |
| --- | --- |

| Title: | CE3.h: Merge candidates derivation from vector shifting |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vijayaraghavan Thirumalai, Li Zhang, Ying Chen, Marta Karczewicz, 5775 Morehouse Drive San Diego, CA 92121, USA Christine Guillemot, Laurent Guillo, INRIA, France Jian-Liang Lin, Yi-Wen Chen, Yu-Lin Chang MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | +1-858-845-6589 vthiruma@qti.qualcomm.com lizhang@qti.qualcomm.com +33 299847100 christine.guillemot@inria.fr laurent.guillo@inria.fr +886-3-5670766 ext. 25555 jl.lin@mediatek.com yiwen.chen@mediatek.com |
| Source: | Qualcomm Incorporated, INRIA, and MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal is a result of harmonization of techniques in JCT3V-D0178 and JCT3V-D0109. In JCT3V-D0178, up to two candidates may be inserted into a merge candidate list, wherein a candidate is derived by left/right shifting of the disparity vector to identify a candidate in a block of the reference view, if that candidate is unavailable, a disparity motion vector from the spatial neighboring blocks or the disparity vector is left/right shifted to create the candidate. In JCT3V-D0109, a candidate is purely derived by a shift considering the width and height of the current PU. In this joint contribution, it is proposed to add only one additional candidate in the merge candidate list. The additional candidate is firstly generated by shifting the disparity vector by considering the size of the current PU and utilizing the motion information of an identified block in the reference view, as it is done in JCT3V-D0109. Furthermore, similar to JCT3V-D0178, if the block doesn’t provide an available candidate, the candidate is derived by shifting horizontally a disparity motion vector. Compared to the current HTM design, the proposed method achieves a compression efficiency gain of 0.2% in terms of BD rate for both BVSP on and off cases, respectively.

# Introduction

In 3D-HEVC, up to six merging candidates are made available for selection when inter-view motion prediction is enabled. When inter-view motion prediction is enabled, the merge candidate list is constructed as follows:

- One candidate derived from inter-view motion prediction, is added to the merge candidate list if available.

As depicted in Figure 1, a corresponding block in the reference view is located based on the disparity vector Error: Reference source not found. The Inter-view Predicted Motion Candidate (IPMC) derived from the corresponding block, if available, is added. Note that IPMC is typically a candidate containing motion vectors referring to temporal reference pictures.

- Spatial merging candidates derived from the neighboring block positions A1, B1 and B0 are added to the merge candidate list. In addition, the spatial merging candidates derived from A1 and B1 are compared with IPMC to eliminate the redundancy.

- The disparity vector is converted to a disparity motion vector and is further added into the merge candidate list right after the spatial merging candidate from B0, if different from the candidates derived from A1 and B1.

- If BVSP is enabled for the current slice, the BVSP merging candidate is inserted to the merge candidate list.

- Spatial merging candidates derived from the neighboring block positions A0 and B2 are added to the merge candidate list.

- Other motion vector merging candidates (including temporal/combined bi-predictive/zero motion vector merging candidates) are added to the merge candidate list if the number of valid candidates is less than the maximum number.

Figure 1. Derivation of inter-view predicted motion candidate (IPMC) for the merge mode.

The current design may have the following problems that are listed below.

- The disparity vector (DV) may be not accurate enough which results in sub-optimal candidates.

- The disparity motion vector (DMV) of the spatial neighbouring candidates A0, A1, B0, B1 or B2 may be highly correlated with the disparity motion vector of the current coding PU; this is not well exploited in the current design.

Therefore, further coding efficiency improvement can be expected by including more candidates derived from the shifted DV and DMV.

# Proposed Scheme

In JCT3V-D0178 [2], it is proposed that up to two additional candidates with shifted DV may be added. To reduce the complexity, in this proposal, it is proposed to add only one candidate to the merge candidate list. Furthermore, it is considered to derive the additional inter-view motion vector candidate from the corresponding block located in the bottom right position shifted with the disparity vector, same as in proposal JCT3V-D0109 [3].

## Generation of an additional candidate derived from shifted DV and/or DMV

Only one additional candidate is generated by following the steps described below.

- The disparity vector DV is shifted by ( (PuWidth/2*4 + 4), (PuHeight/2*4 + 4) ) and it is used to derive an IPMC candidate in the reference view. Here, the size of the current PU is PuWidth x PuHeight.

- If the IPMC is unavailable from the above step 1, a candidate, denoted as Disparity Shifted Motion Vector (DSMV) is derived and set to be the additional candidate. DSMV is derived as follows.

  - Identify the first available disparity motion vector (DMV) corresponding to the RefPicList0 from the spatial neighboring blocks A1, B1, B0, A0 or B2.

  - If the DMV is available (that is denoted as ‘mv’)

    - DSMV[ 0 ] = mv[ 0 ], DSMV[ 1 ] = mv[ 1 ] and DSMV[ 0 ][ 0 ]+=4, reference indices are inherited from the first available candidate (which contains the DMV).

    - When BVSP is enabled, DSMV[ 0 ][ 1 ] is set to 0.

  - Otherwise, DSMV[ 0 ][ 0 ] = DV[ 0 ]+4, DSMV[ 1 ][ 0 ] = DV[ 0 ]+4; DSMV[ 0 ][ 1 ] = 0 and DSMV[ 1 ][ 1 ]= 0.

## Description of the whole process

To generate a merge candidate list, the following steps are performed in order: (newly introduced one step is highlighted in green)

- A disparity vector is derived and the corresponding block of current PU based on the disparity vector is identified. IPMC, if derived to be available, is added to the merge candidate list.

- Spatial merging candidates (SMC) A1, B1 and B0 are added to the merge candidate list as described in the current design.

  - In addition, during SMC generation process, if a SMC derived from A1 or B1 is the same as the IPMC in step 1 they are excluded from the merge candidate list.

- Disparity motion vector converted from the disparity vector is added to the merge candidate list, if they are different from the A1 and B1.

- The BVSP merging candidate is inserted to the merge candidate list, if BVSP is enabled for current slice.

- Spatial merging candidates (SMC) A0 and B2 are added to the merge candidate list as described in the current design.

- If IPMC was derived to be the additional candidate, it is compared with the IPMC in step 1. If it is not equal to the IPMC in step 1 or if the additional candidate is a DSMV (when the additional IPMC is unavailable), the generated additional candidate is inserted.

- Temporal merging candidate is added to the merge candidate list as the current design.

- Other motion vector merging candidates are added to the merge candidate list if the number of valid candidates is less than the maximum number.

It should be noted that the candidate list size is not changed (still six), once there are enough candidates in the candidate list, no more candidates will be further added.

# Experimental Results

This section provides simulation results of the proposed merge improvement in comparison with the 3DV-HTM anchor. The platform 3D-HTM 7.0r1 [4] is utilized and the proposed method is implemented on top of it. All the simulation tests are performed based on the common test conditions [5].

The coding performance of the proposed scheme in the BVSP on case is given in Table 1. As shown in Table 1, the overall average bitrate reduction is 0.1%, 0.1%, 0.2% for the decoded texture views, coded and synthesized views, respectively.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

Table 1: Coding gain with respect to anchor for 3-view case (proposed scheme vs. CTC)

The proposed scheme is also tested in the BVSP off case by setting the following in the HTM7.0r1 software.

UseVSPCompensation                                      0

UseDVPRefine                                                  0

For a fair comparison, the result of the proposed scheme with BVSP off is compared with the CTC in the BVSP off case. As shown in Table 2, the overall average bitrate reduction is 0.2%, 0.2%, 0.2% for decoded texture views, coded and synthesized views.

Table 2: Coding gain with respect to anchor for 3-view case in BVSP off case (proposed scheme vs. HTM 7.0r1 with BVSP off)

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Test Model 4,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1005, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

- V. Thirumalai, L. Zhang, Y. Chen, M. Karczewicz, T. Guionnet, C. Guillemot and L. Guillo, “CE5.h: 3D-CE5.h: Merge candidates derivation from vector shifting,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D0178, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang and S. Lei, “3D-CE5.h related: Additional inter-view merging candidate,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D0109, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

- 3DV-HTM version 7.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1100, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

INRIA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).