| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0128 |
| --- | --- |

| Title: | CE1.h: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Complexity Analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Fu Chen, Gwo Giun Lee, Bo-Syun Li, Chunhui Cui, Yan Huo No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU) and Hong Kong Applied Science and Technology Research Institute (ASTRI) |  |  |

_____________________________

# Abstract

Since the block-based B-VSP was adopted into JCT3V 3rd meeting [1], this proposal proposes the scheme that adaptively partitions the depth block instead of regular block partition to reduce data transfer rate and align the design to motion compensated prediction. This adaptively partition scheme is based on both global depth distribution (UseDLT flag) [2] and local depth variation. The supported partition block types in this proposal includes 16×16, 16×8, 8×16, 8×8, 8×4, and 4×8 blocks. The experimental results show proposed method 1 reports that no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average, with the 0.1% increased for decoding time; proposed method 2 reports also no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average, with the 0.1% increased for decoding time. Proposed method 1 and 2 with the 0.3% decreased for the encoding time. On the other hand, this proposal also reports the complexity analysis in comparison to current B-VSP in terms of the number of operations, data storage requirement, data transfer rate, and number of storage accessing. The number of operations could be reduced about 0.53%. The data transfer rate could be reduced 15.48% and 28.85% at average and best scenarios, respectively. In addition, the number of storage accessing also could be reduced 42.86%, 64%, and 93.63% at worst, average and best scenarios, respectively. On the other hand, there is only slight increasing 3.98% on data storage requirement and it is almost negligible. Hence, this proposal significantly reduces in data transfer rate and number of storage accessing with small coding loss and storage requirement increasing.

# Introduction and Problem Statement

According to the description of CE1 [3], the B-VSP would be tested at 8×8 or larger block sizes or adaptive block sizes and the block size would not be smaller than 4×8 or 8×4 block sizes to primarily align with motion compensation prediction. However, the large block size will result in the inaccurate disparity vectors for B-VSP. Thus, this proposal provides adaptively depth block partition strategy which is based on both global depth distribution and local depth variation. Consequently, this proposal also decrease the data transfer rate and number of storage accessing.

# Proposed Algorithm

## Algorithm description

This proposed algorithm uses both global and local depth information to content-adaptively partition the virtual depth block to derive the accurate disparity vectors for each subblock. The global depth information is determined by the UseDLT flag due to the fact that this flag presented the degree of depth homogenity. Thus, this proposal uses this flag to choose 8×8 or 16×16 subblock size in cooperation with PU size. Consequently, the flowchart of subblock size determination is shwon in Fig 1. Furthermore, 8×8 subblock size would be split into two 4×8, 8×4 blocks or remained one 8×8 block; the 16×16 subblock size would have the same partition manner by using the local depth variation. According to subblock size and the supported partition types, five representative depth candidates are extracted by using the maximum depth value out of four corners and the graphic representation of five representative depth candidates are illustrated in Fig 2.

| Fig 1. Subblock size determination | Fig 2. Five representative depth candidates |
| --- | --- |

The local depth variation is analyzed through five representative depth candidates and the calculated five representative depth candidates can be used to the representative depth of partitioned subblocks directly without re-computation. To determine appropriate partition type, this proposal uses five representative depth candidates to detect depth gradient in horizontal and vertical direction and the overall flowchart to determine appropriate partition type is illustrated in Fig 3.

Fig 3. Local depth variation analysis and partition flow of subblock

## Complexity analysis

Since the proposed algorithm also would like to achieve the complexity redection on B-VSP, the following sections provide the complexity analysis which is mainly composed of number of operations, data storage requirement, data transfer rate, and number of storage accessing to justifty the proposed B-VSP (method 1) is less complexity than current B-VSP and coding gain can be retained. The data granularity is one large coding unit (LCU), and LCU size is 64×64. The number of operations is accumulated within one LCU process. Data storage requirement only need to find out the worst case for one prediction unit (PU) process. Beacase buffer can be reused for the next PU process. The data transfer rate is estimated by the amount of input data  and output data for completing one LCU. The number of storage accessing is accumulated within one LCU process.

The proposed algorithm splits the PU into several 8×8 or 16×16 depth subblock and each subblock is independent; thus, the analyzed results of one PU would be multiples of the analyzed results of 8×8 or 16×16 subblocks. In the same way, the current B-VSP is 4×4 subblock based process, and hence the analyzed results of one PU would also be multiples of the analyzed results of 4×4 subblocks. On the other hand, one LCU analyzed results is the multiples of the one PU analyzed results.

The flowchart of the proposed algorithm are illustrated in Fig 4, and the difference between the proposed and current B-VSP are the blocks colored by blue. That is, there is no depth block analysis of current B-VSP. According to the flowchart, the complexity of proposal and current B-VSP is assessed and the measured metrics include number of operations, data storage requirement, data transfer rate, and number of storage accessing. The document shows overall summary only; for more information, please reference to the spreadsheet.

### Number of operations

Based on flowchart in Fig 4, the major processes of the proposed and current B-VSP are listed in the following entries. Moreover, the mathematical expression would be used to calculate number of operations with high accuracy. The analyzed number of operations is listed in Table 1 and the results show that there is 0.53% computation complexity decreasing as compared to current B-VSP at the worst case which is indicated by red color. The operations focus on addition, subtraction, and comparison. Other operations are recorded in the spreadsheet.

- Representation depth candidates generation (worst case): 15 comparison for one 8×8 or 16×16 subblock.

- Horizontal and vertical change detection: 2 subtractions for one 8×8 or 16×16 subblock.

- Determine appropriate block partition and corresponding representative depth: 2 comparisons for one 8×8 or 16×16 subblock.

- Reference pixel address calculation: 1 addition

- Interpolation (worst case): 7 additions/subtractions, 6 constant multiplications are needed for one sub-pixel of luminance component and 3 additions/subtractions and 4 constant multiplications are needed for one sub-pixel of chromiance component. (W×H times operations for Luma. And 2×(W/2)×(H/2) times operations for Chroma. are needed.)

Table 1 Number of operations of proposal (method 1) and current B-VSP per one LCU

|  | Proposal (Method 1) |  |  |  |  |  | Anchor | Ratio (to anchor) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | 16×16 | 16×8 | 8×16 | 8×8 | 8×4 | 4×8 | 4×4 |  |
| 64×64 | 35169 | 35169 | 35169 | 36225 | 36225 | 36225 | 36353 |  |
| 64×32 | 35170 | 35170 | 35170 | 36226 | 36226 | 36226 | 36354 |  |
| 32×64 | 35170 | 35170 | 35170 | 36226 | 36226 | 36226 | 36354 |  |
| 32×32 | N/A | N/A | N/A | 36228 | 36228 | 36228 | 36356 |  |
| 32×16 | N/A | N/A | N/A | 36232 | 36232 | 36232 | 36360 |  |
| 16×32 | N/A | N/A | N/A | 36232 | 36232 | 36232 | 36360 |  |
| 16×16 | N/A | N/A | N/A | 36240 | 36240 | 36240 | 36368 |  |
| 16×8 | N/A | N/A | N/A | 36256 | 36256 | 36256 | 36384 |  |
| 8×16 | N/A | N/A | N/A | 36256 | 36256 | 36256 | 36384 |  |
| 8×8 | N/A | N/A | N/A | 36288 | 36288 | 36288 | 36416 |  |
| 8×4 | N/A | N/A | N/A | N/A | 35712 | N/A | 36480 |  |
| 4×8 | N/A | N/A | N/A | N/A | N/A | 35712 | 36480 |  |
|  |  |  |  |  |  |  | Worst | 99.47% |

### Data storage requirement

The data storage requirement in algebric manner are listed in Table 2. For the current B-VSP, the major data storage are the reference virtual depth block, one disparity vector, reference texture block with extended taps and compensated block. On the other hand, the additional data storage required by the proposed B-VSP are the representative depth candidates and disparity vector information. W is the block width and H is the block height for one PU; M is the block width and height for the subblock; the one pixel of texture or depth is 8 bits and one disparity vector is 12 bits. The data storage requirement in numeric manner is illustrated in Table 3. Other detail data are recorded in the spreadsheet.

Because the proposed B-VSP subblock size is larger than the current B-VSP, the buffer size for storing reference texture block with extended taps data is increased. Hence, data storage requirement of the proposed B-VSP is higher than the current B-VSP. In the worst case, proposed B-VSP only slightly increases 3.98% data storage requirement than current B-VSP.

Table 2 Data storage requirement (bits)

|  | Proposal | Anchor |
| --- | --- | --- |
| Reference virtual depth block | W×H×8 | W×H×8 |
| Representative depth candidates | 4×8 | N/A |
| Disparity vector | 2×12 | 1×12 |
| Reference texture block with extended taps | (3+M+4)×M×8 or (3+(M/2)+4)×M×8 or (3+M+4)×(M/2)×8 | (3+4+4)×4×8 |
| Compensated block | W×H×8 | W×H×8 |

Table 3 Data storage requirement of proposal (method 1) and current B-VSP (Bytes)

|  | Proposal (Method 1) |  |  |  |  |  | Anchor | Ratio (to anchor) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | 16×16 | 16×8 | 8×16 | 8×8 | 8×4 | 4×8 | 4×4 |  |
| 64×64 | 8565.5 | 8383 | 8439 | 8317.5 | 8259 | 8287 | 8237.5 |  |
| 64×32 | 4469.5 | 4287 | 4343 | 4221.5 | 4163 | 4191 | 4141.5 |  |
| 32×64 | 4469.5 | 4287 | 4343 | 4221.5 | 4163 | 4191 | 4141.5 |  |
| 32×32 | N/A | N/A | N/A | 2173.5 | 2115 | 2143 | 2093.5 |  |
| 32×16 | N/A | N/A | N/A | 1149.5 | 1091 | 1119 | 1069.5 |  |
| 16×32 | N/A | N/A | N/A | 1149.5 | 1091 | 1119 | 1069.5 |  |
| 16×16 | N/A | N/A | N/A | 637.5 | 579 | 607 | 557.5 |  |
| 16×8 | N/A | N/A | N/A | 381.5 | 323 | 351 | 301.5 |  |
| 8×16 | N/A | N/A | N/A | 381.5 | 323 | 351 | 301.5 |  |
| 8×8 | N/A | N/A | N/A | 253.5 | 195 | 223 | 173.5 |  |
| 8×4 | N/A | N/A | N/A | N/A | 126.5 | N/A | 109.5 |  |
| 4×8 | N/A | N/A | N/A | N/A | N/A | 154.5 | 109.5 |  |
|  |  |  |  |  |  |  | Worst | 103.98% |

### Data transfer rate

The data transfer rate for one PU in algebraic manner is listed in Table 4 and input data and output data transfer rate would be discussed. For the proposed B-VSP and current B-VSP, the major input data would be transferred include reference virtual depth block and reference texture block with extended taps. The major output data would be transferred is the compensated block. W is the block width and H is the block height for the PU; M is the block width and height for the subblock; the one pixel of texture or depth is 8 bits and one disparity vector is 12 bits. The data transfer rate for one LCU in numeric manner is illustrated in Table 5. Other detail data are recorded in the spreadsheet.

The data transfer rate of the proposed B-VSP is less than the current B-VSP. For method 1, in the best case colored by green, the proposed B-VSP can reduce 28.85%. In the average case, the proposed algorithm also can reduce 15.48% than current B-VSP.

Table 4 Data transfer rate for one PU (bits per PU)

|  | Proposal | Anchor |
| --- | --- | --- |
| Reference virtual depth block | W×H×8 | W×H×8 |
| Reference texture block with extended taps | (3+M+4)×M×(W/8)×(H/8)×8 + 2× [1+(M/2)+2]× (M/2)×(W/4)×(H/4)×8 or (3+M+4)×M×(W/16)×(H/16)×8 + 2× [1+(M/2)+2]×(M/2)×(W/8)×(H/8)×8 | (3+4+4)×4×(W/4)×(H/4)×8+ 2× [1+2+2]×2×(W/2)×(H/2)×8 |
| Compensated block | W×H×8 + 2×(W/2)×(H/2)×8 | W×H×8 + 2×(W/2)×(H/2)×8 |

Table 5 Data transfer rate of proposal (method 1) and current B-VSP per one LCU (Bytes)

|  | Proposal (Method1 ) |  |  |  |  |  | Anchor | Ratio (to anchor) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | 16×16 | 16×8 | 8×16 | 8×8 | 8×4 | 4×8 | 4×4 |  |
| 64×64 | 18944 | 18944 | 21504 | 21504 | 21504 | 26624 | 26624 |  |
| 64×32 | 18944 | 18944 | 21504 | 21504 | 21504 | 26624 | 26624 |  |
| 32×64 | 18944 | 18944 | 21504 | 21504 | 21504 | 26624 | 26624 |  |
| 32×32 | N/A | N/A | N/A | 21504 | 21504 | 26624 | 26624 |  |
| 32×16 | N/A | N/A | N/A | 21504 | 21504 | 26624 | 26624 |  |
| 16×32 | N/A | N/A | N/A | 21504 | 21504 | 26624 | 26624 |  |
| 16×16 | N/A | N/A | N/A | 21504 | 21504 | 26624 | 26624 |  |
| 16×8 | N/A | N/A | N/A | 21504 | 21504 | 26624 | 26624 |  |
| 8×16 | N/A | N/A | N/A | 21504 | 21504 | 26624 | 26624 |  |
| 8×8 | N/A | N/A | N/A | 21504 | 21504 | 26624 | 26624 |  |
| 8×4 | N/A | N/A | N/A | N/A | 21504 | N/A | 26624 |  |
| 4×8 | N/A | N/A | N/A | N/A | N/A | 26624 | 26624 |  |
|  |  |  |  |  |  |  | Worst | 100.00% |
|  |  |  |  |  |  |  | Average | 84.52% |
|  |  |  |  |  |  |  | Best | 71.15% |

### Number of storage accessing

The number of storage accessing is listed in Table 6 and this calculation is according to the number of memory address calculation. Due to the fact that fetching data from the external storage would cost some time depended on the characteristic of platform. Therefore, in addition to amount of data should be transferred; the number of storage accessing should be took into consideration concurrently. In the best case colored by green, the proposed B-VSP can reduce 93.63% as compared to current B-VSP. In the average, this proposed B-VSP also can reduce 64.01%. In the worst case colored by red, this proposal still can reduce 43.86%. Other detail data are recorded in the spreadsheet. This proposed B-VSP can greatly reduce the number of times of storage accessing than current B-VSP.

Table 6 Number of storage accessing (method 1) and current B-VSP per one LCU

|  | Proposal (Method 1) |  |  |  |  |  | Anchor | Ratio (to anchor) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | 16×16 | 16×8 | 8×16 | 8×8 | 8×4 | 4×8 | 4×4 |  |
| 64x64 | 49 | 97 | 97 | 193 | 385 | 385 | 769 |  |
| 64x32 | 50 | 98 | 98 | 194 | 386 | 386 | 770 |  |
| 32x64 | 50 | 98 | 98 | 194 | 386 | 386 | 770 |  |
| 32x32 | N/A | N/A | N/A | 196 | 388 | 388 | 772 |  |
| 32x16 | N/A | N/A | N/A | 200 | 392 | 392 | 776 |  |
| 16x32 | N/A | N/A | N/A | 200 | 392 | 392 | 776 |  |
| 16x16 | N/A | N/A | N/A | 208 | 400 | 400 | 784 |  |
| 16x8 | N/A | N/A | N/A | 224 | 416 | 416 | 800 |  |
| 8x16 | N/A | N/A | N/A | 224 | 416 | 416 | 800 |  |
| 8x8 | N/A | N/A | N/A | 256 | 448 | 448 | 832 |  |
| 8x4 | N/A | N/A | N/A | N/A | 512 | N/A | 896 |  |
| 4x8 | N/A | N/A | N/A | N/A | N/A | 512 | 896 |  |
|  |  |  |  |  |  |  | Worst | 57.14% |
|  |  |  |  |  |  |  | Average | 35.99% |
|  |  |  |  |  |  |  | Best | 6.37% |

### Summary report

The summary the complexity analysis report is listed in Table 7. The number of operations could be reduced about 0.53%. The data transfer rate could be reduced 15.48% to 28.85% at average and best scenarios. The number of storage accessing also could be reduced 42.86%, 64%, and 93.63% at worst, average and best scenarios, respectively. They are significant reduction in the proposed algorithm. In the data storage requirement only slight increasing 3.98%, this increase is almost negligible.

Table 7 Summary report of complexity analysis (method 1 vs. anchor, ratio to anchor)

|  | # of operations Add/Sub/Comp | Data storage | Data transfer | # of times of storage accessing |
| --- | --- | --- | --- | --- |
| Worst | 99.47% | 103.98% | 100.00% | 57.14% |
| Average | - | - | 84.52% | 35.99% |
| Best | - | - | 71.15% | 6.37% |

Fig 4. Flowchart of the proposed adaptive depth block partition B-VSP

# Experimental Results

The anchor software is HTM-7.0r1 as demanded in CTC [4]. The experimental results show proposed method 1 reports that no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average, with the 0.1% increased for decoding time; proposed method 2 reports also no bitrate increasing for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average, with the 0.1% increased for decoding time. Proposed method 1 and 2 with the 0.3% decreased for the encoding time.

Table 8 Experimental results of method 1 (Global and local depth information are considered)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.0% | 100.1% | 98.4% |
| Kendo | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 98.9% | 100.0% | 97.5% |
| Newspaper_CC | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 99.1% | 99.9% | 98.0% |
| GT_Fly | 0.0% | 0.5% | 0.3% | 0.1% | 0.1% | 0.1% | 100.1% | 100.6% | 97.4% |
| Poznan_Hall2 | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.1% | 100.5% | 100.0% | 100.1% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 100.0% | 97.3% |
| Undo_Dancer | 0.0% | 0.8% | 0.6% | 0.2% | 0.2% | 0.0% | 100.4% | 100.2% | 100.1% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 99.0% | 100.0% | 97.9% |
| 1920x1088 | 0.0% | 0.3% | 0.3% | 0.1% | 0.1% | 0.0% | 100.3% | 100.2% | 98.7% |
| average | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 99.7% | 100.1% | 98.4% |

Table 9 Experimental results of method 2 (Only local depth information is considered)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 100.5% | 100.1% | 99.5% |
| Kendo | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 100.2% | 99.9% | 98.4% |
| Newspaper_CC | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 100.3% | 100.0% | 98.7% |
| GT_Fly | 0.0% | 0.5% | 0.3% | 0.1% | 0.1% | 0.1% | 98.0% | 100.8% | 96.9% |
| Poznan_Hall2 | 0.0% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 100.4% | 100.0% | 100.1% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.5% | 100.0% | 99.5% |
| Undo_Dancer | 0.0% | 0.8% | 0.6% | 0.2% | 0.2% | 0.0% | 98.2% | 100.1% | 95.4% |
| 1024x768 | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 100.0% | 98.9% |
| 1920x1088 | 0.0% | 0.3% | 0.2% | 0.1% | 0.1% | 0.0% | 99.3% | 100.2% | 98.0% |
| average | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 99.7% | 100.1% | 98.4% |

# Conclusion

This proposed algorithm an adaptive depth block partition for B-VSP. According to global and local depth information, UseDLT flag and gradient, to choose the appropriate subblock sizes (8×8 or 16×16) and supported the partition types including the 16×16, 16×8, 8×16, 8×8, 8×4, and 4×8 blocks. With these partitioned block to derive the representative depth. That is, the proposed B-VSP can be aligned to MC to share the identical module via time-multiplexing method. According to the experiment results, method 1 reports no bitrate increasing for the video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate in average, with 0.1% increasing in decoding time and 0.3% decreasing in  encoding time. In complexity analysis, the number of operations could be reduced about 0.53%. The data transfer rate could be reduced 15.48% and 28.85% at average and best scenarios, respectively. In addition, the number of storage accessing also could be reduced 42.86%, 64%, and 93.63% at worst, average and best scenarios, respectively. On the other hand, there is only slight increasing 3.98% on data storage requirement and it is almost negligible. Hence, this proposal significantly reduces in data transfer rate and number of storage accessing with small coding loss and storage requirement increasing. According to coding performance and complexity analysis, method 1 is appropriate manner to reduce the data transfer rate and number of storage accessing with negligible coding loss.

# Acknowledge

Thanks for the cross-check by and NTT (JCT3V-E0215) and the help of high performance computing cluster system from NCKU computer and network center.

# Reference

- JCT3V-C0152, “CE1.h: Backward View Synthesis Prediction using Neighbouring Blocks, “Geneva, CH, 17-23 January 2013.

- JCT3V-B0036, “3D-CE6.h: Simplified Depth Coding with an optional Depth Lookup Table, “Shanghai, CN, 13–19 Oct. 2012.

- JCT3V-D1101, “Description of Core Experiment 1 (CE1) on View Synthesis Prediction, “Incheon, KR, 20–26 Apr. 2013.

- JCT3V-D1100, “Common Test Conditions of 3DV Core Experiments, “Incheon, KR, 20–26 Apr. 2013.

- JCT3V-D0089, “3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Corresponding Complexity Analysis, “Incheon, KR, 20-26 April 2013.

# Patent rights declaration(s)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ASTRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).