| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0132 |
| --- | --- |

| Title: | AHG 10: Complexity Assessment on Simplified Depth Coding (SDC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chun-Fu Chen, Gwo Giun Lee, Bo-Syun Li No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU) |  |  |

_____________________________

# Abstract

This proposal provides a case study of complexity assessment on simplified depth coding (SDC) in 3D-HEVC and the assessed complexity included number of operations, data storage requirement, and data transfer rate. The results in this proposal can be considered as anchor complexity to assist JCT3V to evaluate the proposals coming from several proponents in a fair manner.

# Introduction

The complexity issue has been studied for several decades; however, these studies may analyze the complexity based on the specific platforms. Thus, the assessed complexity would be the platform-dependent and it would be varied on different platform since the analyzed results cannot be transparent to another platform. The proposals in JCT3V 4th meeting [1][2][3], provide a guideline to evaluate the intrinsic complexity of an algorithm. Thus, this proposal follow the analysis manner to extract the intrinsic algorithmic complexity which is platform-independent and transparent to each platform directly. The coding tool, simplified depth coding (SDC), would be considered as an exercise to assess its complexity in this proposal.

# Method of Complexity Assessment

To assess the complexity, the flowchart for completing one data granularity should be established first to determine data causality. One important issue should be concerned during the flowchart construction, the major processing or key modules should be identified in this stage in order to achieve the further analysis work. However, the major processing identification would be affected by the analyzer; thus, this proposal also provide a general concept to construct the flowchart. Subsequently, the complexity metrics, such as data storage requirement, and data transfer rate, can be assessed in accordance with the constructed flowchart and the identified major processes or key modules at various data granularities. The data granularity means that amount of data should be performed or produced within one process.

- Number of operations

To estimate the number of operations of an algorithm, the starting point would be the mathematical expression due to the fact that the mathematical expression is more intrinsic to the algorithm and the mathematical expression would not be affected by platform. When encountering some if-else conditions, the worst case of conditions would be considered. Some basic operation types are considered only, such as addition, multiplication, and division, due to the fact that some equivalent operations would be treated as identical complexity. E.g. the subtraction can be easily implementation by the addition with 2’s complement representation plus a NOT logic and an incermentor and the shift operations can be implemented with the wire reconnection.

- Data storage requirement

The data storage requirement was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and generated output data should be stored to complete one data granularity and some intermediate data would be stored according to the algorithm. Since the assessed results would like to be transparent to different platforms, the data storage configuration shall not be considered at the initial stage.

- Data storage requirement

The data transfer rate was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and produced output data should be transferred during the process of one data granularity. The intermediate data would not be took into consideration since these data would be disappeared after finishing the process. In addition, transfer rate of input data and output data shall be discussed individually since the characteristic of sending data and fetching data are different.

In this proposal, the flowchart is constructed according to the specification and the test model [4] and the major process is defined according to that is there any data to be processed or not. The data granularities studied in this proposal is one LCU with all supported PU sizes.

# Complexity Assessment

## Algorithm description

According to the spec text and test model [4], there are three modes, including DC, Planar, and DMM 1, in SDC and flowchart of SDC is shown in Fig. 1. The major processing is predicted block generation, segment DC generation and current block reconstruction but DC mode has an additional processing, boundary filtering on predicted block. According to the flowchart, the complexity of SDC is assessed at various data granularities and the details are described in the following sections.

Fig. 1 Flowchart of current design (HTM-7.0r1)

## Complexity assessment

To compare the complexity, this proposal uses three metrics, including number of operations, data storage requirement, data transfer rate, to evaluate the anchor and the proposed algorithm. The data granularity to-be-analyzed is one largest coding unit (LCU) which is composed of various prediction unit (PU) size. In the following sections, the PU size is defined as N×N to provide the algebraic expression clearly. After obtaining the complexity of PU level, the results can be directly scaled to LCU level since the operation of PUs are independent.

For number of operations, the required computations of each mode are counted individually and then the worst case is found among all modes. On the other hand, data storage requirement and data transfer rate are analyzed for all modes directly since the required and transferred data for three modes are identical.

### Number of operations

#### DC mode

- Predicted block generation: (N-1)+(N-1) additions, 1 shift

- Boundary filter: 2+2(N-1)+2(N-1) additions, 2+2(N-1)+2(N-1) shifts

- Segment DC generation: N2 additions and 1 shift (0.25N2 additions and 1 shift)

- Current block reconstruction: 1+1+(N-1)+(N-1)+1 additions, 1 subtraction, and 2 LUT access

Table 1 Algebraic expression of number of operations for one PU (DC mode)

| Type of operations | Number of operations |
| --- | --- |
| Mul | 0 |
| Add/Sub/Comp | N2+8N-1 or 0.25N2+8N-1 |
| Shift | 4N |
| LUT access | 2 |

#### Planar mode

- Predicted block generation: 4N2 multiplications, 3N2 additions, and 1 shift (In software, 3N2+N additions, 2N subtractions, N2+2N shift)

- Segment DC generation: N2 additions and 1 shift (0.25N2 additions and 1 shift)

- Current block reconstruction: 1+N2 additions, 1 subtraction, and 2 LUT access

Table 2 Algebraic expression of number of operations for one PU (Planar mode)

| Type of operations | Number of operations |  |
| --- | --- | --- |
| Mul | 0 | 4N2 |
| Add/Sub/Comp | 5N2+3N+2 or 4.25N2+3N+2 | 5N2+2 or 4.25N2+2 |
| Shift | N2+2N+1 | N2+1 |
| LUT access | 2 | 2 |

#### DMM 1 mode

- Predicted block generation: 2 additions, 4 comparisons, and 2 shifts

- Segment DC generation: Actually, it did not need any computation to obtain segment DCs.

- Current block reconstruction: 2+2 additions, 2 subtractions, and 4 LUT access

Table 3 Algebraic expression of number of operations for one PU (DMM 1 mode)

| Type of operations | Number of operations |
| --- | --- |
| Mul | 0 |
| Add/Sub/Comp | 12 |
| Shift | 2 |
| LUT access | 4 |

### Data storage requirement

Input buffer of SDC is the reference neighboring pixels, and output buffer is the reconstructed block. There are some intermediate buffer for storing segment DC. The algebraic expression established according to PU size is list in Table 4.

Table 4 Algebraic expression of data storage requirement (worst case among three modes)

| Data | Required size |
| --- | --- |
| Reference pixels | 2(N+1) |
| Segment DC | 2 |
| Reconstructed block | N2 |

### Data transfer rate

Input data and output data should be transferred of SDC is the reference neighboring pixels and the reconstructed block, respectively. The algebraic expression established according to PU size is listed in Table 5.

Table 5 Algebraic representation of data transfer rate (worst case among three modes)

| Data | Amount of transferred data per PU |
| --- | --- |
| Reference pixels | 2(N+1) |
| Reconstructed block | N2 |

## Overall report

According to the algebraic expression on number of operations, data storage, and data transfer rate. Table 6, Table 7 and Table 8 show the numeric representation of required resource for one LCU with various PU size. Therefore, Table 9 provide an overall report on SDC’s complexity at LCU level.

Table 6 Number of operations for one LCU with various PU sizes

|  | Number of operations (Add/Sub/Comp) |  |  |
| --- | --- | --- | --- |
| PU_size | DC mode | Planar mode | DMM 1 mode |
| 64x64 | 1,535 | 17,602 | - |
| 32x32 | 2,044 | 17,800 | 48 |
| 16x16 | 6,128 | 21,280 | 192 |
| 8x8 | 8,128 | 22,144 | 768 |

Table 7 Data storage requirement for various PU sizes

|  | Data storage (bits) |  |  |
| --- | --- | --- | --- |
| PU_size | Reference pixels | Segment DC | Reconstructed block |
| 64x64 | 1,040 | 16 | 32,768 |
| 32x32 | 528 | 16 | 8,192 |
| 16x16 | 272 | 16 | 2,048 |
| 8x8 | 144 | 16 | 512 |

Table 8 Data transfer rate for one LCU with various PU sizes

|  | Data transfer rate (bits per LCU) |  |
| --- | --- | --- |
| PU_size | Reference pixels | Reconstructed block |
| 64x64 | 1,040 | 32,768 |
| 32x32 | 2,112 | 32,768 |
| 16x16 | 4,352 | 32,768 |
| 8x8 | 9,216 | 32,768 |

Table 9 Overall complexity of SDC at LCU level

| Complexity Metric | Worst case |
| --- | --- |
| Number of operations (Add/Sub/Comp) | 22,144 |
| Data storage requirement (bits) | 33,824 |
| Data transfer rate (bits per LCU) | 41,984 |

## Experimental results

This proposal also evaluated coding performance of SDC in CTC and AI configuration and results showed in Table 10 and Table 11. The coding loss is 0.29% and 0.28% for coded and synthesis BD-rate at CTC configuration and the coding loss is 0.96% and 0.84% for coded and synthesis BD-rate at AI configuration. Moreover, the decoding time is increasing significantly at all intra configuration.

Table 10 Experimental results with SDC OFF (CTC configuration)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.10% | -0.12% | -0.05% | 0.35% | 0.35% | 99.5% | 100.8% | 99.2% |
| Kendo | 0.00% | 0.04% | -0.19% | -0.04% | 0.77% | 0.67% | 99.1% | 101.6% | 99.2% |
| Newspaper_CC | 0.00% | 0.02% | -0.09% | 0.01% | 0.55% | 0.16% | 99.1% | 101.1% | 99.0% |
| GT_Fly | 0.00% | -0.39% | -0.53% | -0.11% | 0.17% | 0.31% | 96.9% | 101.2% | 96.7% |
| Poznan_Hall2 | 0.00% | -0.11% | -0.02% | -0.05% | 0.07% | 0.14% | 99.6% | 100.9% | 100.2% |
| Poznan_Street | 0.00% | -0.17% | -0.24% | -0.07% | 0.06% | 0.07% | 99.2% | 100.3% | 99.5% |
| Undo_Dancer | 0.00% | 0.42% | 0.46% | 0.12% | 0.07% | 0.24% | 97.3% | 101.0% | 95.4% |
| 1024x768 | 0.00% | -0.01% | -0.13% | -0.03% | 0.56% | 0.39% | 99.2% | 101.1% | 99.1% |
| 1920x1088 | 0.00% | -0.06% | -0.08% | -0.03% | 0.09% | 0.19% | 98.2% | 100.8% | 97.9% |
| average | 0.00% | -0.04% | -0.10% | -0.03% | 0.29% | 0.28% | 98.7% | 101.0% | 98.5% |

Table 11 Experimental results with SDC OFF (AI configuration)

|  | video 0 | video 1 | video 2 | video PSNR/ video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 1.71% | 1.23% | 97.5% | 121.7% | 105.3% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 2.45% | 1.74% | 95.4% | 121.3% | 105.2% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 1.06% | 0.91% | 96.3% | 118.3% | 110.9% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.85% | 1.01% | 96.6% | 118.6% | 99.2% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.50% | 0.69% | 99.5% | 121.3% | 130.0% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.28% | 0.18% | 96.7% | 117.6% | 100.3% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | -0.11% | 0.13% | 98.7% | 113.3% | 99.9% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 1.74% | 1.29% | 96.4% | 120.4% | 107.1% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.38% | 0.50% | 97.9% | 117.7% | 107.3% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.96% | 0.84% | 97.2% | 118.9% | 107.3% |

# Conclusion

From Table 1 to Table 9, the complexity of SDC are assessed to assist the JCT3V team to evaluate the proposals related to SDC. The results can be considered as anchor complexity. Thus, with the help of the intrinsic complexity analysis, this proposal would assist the designer to evaluate the developed coding tools. This proposal provides not only an evaluated complexity of the coding tools but also an instruction on that how to assess the intrinsic algorithmic complexity. Due to the fact that the assessed complexity is independent from any platform or architecture, these numbers provide a fair complexity comparison. Hence, this proposal recommends that this analysis method would be took into consideration for the further coding tool development.

# Reference

- JCT3V-D0010, “JCT-3V AHG Report: Complexity assessment (AHG10),“ Incheon, KR, 20-26 April 2013.

- JCT3V-D0301, “AHG 10 Overview of Algorithmic Intrinsic Complexity Measures,“ Incheon, KR, 20-26 April 2013.

- JCT3V-D0301, “AhG 10: Complexity Assessment on Adaptive Luminance Compensation and Plane Segmentation based Intra Prediction in 3D-AVC,“ Incheon, KR, 20-26 April 2013.

- JCT3V-D1005, “3D-HEVC Test Model 4,“ Incheon, KR, 20-26 April 2013.