| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013 | Document: JCT3V-E0133 |
| --- | --- |

| Title: | Inter-view motion vector prediction for depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vijayaraghavan Thirumalai Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-5663 vthiruma@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal extends the inter-view motion prediction scheme currently used for texture coding to depth coding. In order to improve the accuracy of the additional merge candidates for the dependent depth views two aspects are added. In the first aspect, a disparity vector is derived from the neighboring reconstructed depth pixels and more merge candidates are derived for a depth block by utilizing the motion information from the reference depth views. In the second aspect, the center pixel, with which a reference block is identified, has been shifted by (1, 1) to provide more efficient merge candidates. Experimental results show that the proposed scheme gives an overall synthesized coding gain of 0.2% when compared to the CTC results.

# Introduction

In 3D-HEVC, up to six merging candidates are made available for depth view coding that is constructed as follows:

- Motion parameter Inheritance (MPI) candidate is derived from the co-located texture block and it added to the merge candidate list, if available.

Error: Reference source not found1 shows an example of the derivation process of the MPI candidate where the corresponding texture block is selected as the 4x4 block located to the right bottom of the center of the current PU.

- Spatial merging candidates derived from neighboring prediction unit (PU) partitions are added to the merge candidate list. This process is similar to that in HEVC, where redundant candidates among the initial available spatial candidates are eliminated from the merging list by comparing motion information of the candidates. In addition, the spatial merging candidates derived from A1 and B1 are compared with MPI to eliminate the redundancy. Figure 2 depicts the spatial neighbours (A1, B1, B0, A0, B2) which are the potential candidates for the merging list.

- Other motion vector merging candidates (including temporal/combined bi-predictive/zero motion vector merging candidates) are added to the merge candidate list if the number of valid candidates is less than the maximum number.

Figure 1. Derivation of the MPI candidate for depth coding.

Figure 2. Potential spatial neighbouring candidates for the merge candidate list. Arrows indicate which spatial candidate(s) are to be compared.

The current design of motion related technologies for the depth coding in 3D-HEVC may have the following problem:

- The motion vector predictive coding of the non-base depth views fails to consider the motion information of the already coded reference (base) depth views for the merge/skip modes. In other words, the correlation between the motion information (including reference index and motion vectors) of the block in one non-base depth view and its corresponding block in its reference depth view is not exploited.

Further coding efficiency improvement may be expected by including inter-view motion vector prediction candidates in the merge candidate list.

# Proposed scheme

This proposal proposes to improve the motion prediction of the dependent depth views by deriving more candidates from the already coded motion vectors of the reference depth views. To derive the merge candidates, a disparity vector is firstly derived from the neighbouring reconstructed depth values. Then, additional merge candidates are generated based on this derived disparity vector and added into the merge candidate list.

## Generation of the disparity vector

A single disparity vector is derived for each CU from a single depth value. The depth value is calculated from the three neighboring samples that are adjacent to the corners of the current block. As shown in Figure 2, three neighboring sample positions namely above-left, above-right and bottom-left are used.

A single depth value from the three neighboring samples is calculated as

Depth = ( 5*D[ xC-1 ][  yC-1 ] + 5*D[ xC-1 ][  yC+ 2N -1 ] +6*D[ xC+ 2N -1 ][  yC-1 ] + 8 )>>4

Here D[x][y] represents the reconstructed depth value at location (x,y) and (xC, yC) represents the top-left corner of the current CU with a size of 2Nx2N.

Note that the weighted average as shown above for the three pixels are utilized maybe to avoid the division by 3.

The weighted average depth value is directly converted to a disparity vector that is denoted as DV.

The derived disparity vector DV is applied to all the PU’s therein.

Figure 2: Derivation of a disparity vector from the three (red) neighboring reconstructed samples.

## Generation of the inter-view motion vector candidates

Similar to texture coding, two more merge candidates are derived. One is derived from a block in the reference view, identified by the calculated disparity vector (DV), namely the IPMC candidate, and the other is the inter-view disparity motion vector converted from DV.

For depth coding, the disparity vector DV can be converted into an inter-view disparity motion vector and it may be added to the merge candidate list. The disparity vector DV = (mvX, 0) is converted into a disparity motion vector as ((mvX+2) >> 2, 0).

In addition, instead of always using the center position namely C0 with a relative coordination of (xP + ( ( nPSW – 1 )  >>  1 ) + ( ( DV[ 0 ] + 2 )  >>  2), yP + ( ( nPSH – 1 )  >>  1 ) + ( ( DV[1] + 2 )  >>  2 ) ), the C0 position is shifted by (1,1) for the identification of a reference block. More precisely, the location of the reference pixel position (xRef, yRef) in the reference base view is calculated as

xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( (nPSW-1)  >>  1 ) + ( (DV[ 0 ] + 2 )  >>  2 ) )
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( (nPSH-1)  >>  1 ) + ( (DV[ 1 ] + 2 )  >>  2 ) )

Here (xP, yP) represents the top-left corner of the current PU with a size of nPSW x nPSH.

Note that the change of the reference pixel helps for the texture coding as well.

## Merge candidate list construction with additional candidates

The merge candidate list for the depth views are generated using the following steps by considering the two more additional candidates. The newly introduced candidates are highlighted in green color.

- MPI is derived by the procedure described above. If it is available, it is inserted to the merge list.

- If the IPMC derived from the base view is available, and it is different from the MPI candidate it is inserted into the merge candidate list; otherwise IPMC is not inserted in the list.

- Spatial merging candidates (SMC) A1 and B1 are added to the merge candidate list as described in the current design.

- The disparity vector is converted into a disparity motion vector and it is inserted in the list if it is different from the spatial candidates A1 and B1; otherwise it is not inserted in the list.

- Spatial merging candidates (SMC) B0, A0 and A2 are added to the merge candidate list as described in the current design.

- Other motion vector merging candidates (including temporal/combined bi-predictive/zero motion vector merging candidates) are added to the merge candidate list if the number of valid candidates is less than the maximum number.

It should be noted that the candidate list size is not changed (still six), once there are enough candidates in the candidate list, no more candidates will be further added.

# Experimental Results

The proposed method is integrated into current 3D-HTM 7.0r1 software [2] and the simulation tests are performed based on the common test conditions [3].

We first provide results of the proposed method under CTC.

Additional results are also provided to show that the proposed method is fully additive to the major depth coding technique as in JCT3V-E0156 and JCT3V-E0157.

There might be scenarios where the associated texture picture of the current depth picture is unavailable or it may not be desirable to access the texture pictures for the coding of depth. Therefore, we disable the MPI candidate and provide additional results.

## CTC Results

The coding performance of the proposed scheme in comparison with the CTC results is given in Table 1. As shown in Table 1, the overall average bitrate reduction is 0.2% for the synthesized views.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

Table 1: Coding gain with respect to anchor for 3-view case (proposed scheme vs. CTC)

## Proposed method integrated in the SDC-based depth coding

The proposed method can also be integrated on top of the SDC-based depth coding methods, as described in JCT3V-E0156 and JCT3V-E0157.

The overall coding performance is given in Table 2.

Table 2: Results of the proposed method integrated on top of the SDC-based depth coding methods

Table 2 results to be provided

## Coding gain of our method by replacing the MPI candidate

To demonstrate the potentials of the proposed scheme, the existing MPI candidate is replaced by the proposed depth IPMC candidate.

The following two configurations are compared:

- Cfg #1: MPI is disabled, i.e., MPI candidate is not inserted in the candidate list, by disabling the macro MTK_DEPTH_MERGE_TEXTURE_CANDIDATE_C0137.

- Cfg #2: MPI is disabled and inter-view motion prediction for depth, as proposed in this document, is enabled.

Table 3 summarizes the coding gain of the proposed method when MPI is disabled.

Table 3: Proposed method with MPI disabled versus CTC with MPI disabled

Table 3 results to be provided

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Test Model 4,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1005, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

- 3DV-HTM version 7.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1100, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).