| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0134 |
| --- | --- |

| Title: | Signalling of camera parameters in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Currently camera parameters are signalled in sequence parameter set 3DV extension (namely sequence parameter set extension 2) in the draft text of 3D-HEVC. The signaling of such parameters may rely on the information in the video parameter set, such as the nuh_layer_id of a view or whether certain layer is a depth layer or a depth layer. Therefore exists a parsing dependency of the SPS on the VPS. It is proposed that the camera parameters are moved from the SPS to the VPS to resolve the parsing dependency issue as well as to avoid the need of the syntax structure of SPS extension 2.

# Proposal

## Syntax

### Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| view_id[ layerId ] | u(8) |
| if ( layerId ! = 0 ) { |  |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| } else { |  |
| vps_depth_modes_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| if( vps_depth_modes_flag[ layerId ] |  |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| cp_precision | ue(v) |
| cp_in_slice_header_flag | u(1) |
| if( !cp_in_slice_header_flag ) |  |
| for ( i = 0; i < = vps_max_layers_minus1; i++) [Ed. (GT): Parsing dependency from VPS] |  |
| if ( !VpsDepthFlag[ layer_id_in_nuh[ i ]] ) { |  |
| j = VpsViewIdx[ layer_id_in_nuh[ i ] ] |  |
| cp_scale[ j ] | se(v) |
| cp_off[ j ] | se(v) |
| cp_inv_scale_plus_scale[ j ] | se(v) |
| cp_inv_off_plus_off[ j ] | se(v) |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

### Sequence parameter set extension 2 syntax

The relevant sub-clause is avoided.

- H.7.3.2.2.2	Sequence parameter set extension 2 syntax

[ Ed. (GT). ViewIdx[ nuh_layer_id ] and potentially also VpsDepthFlag[ nuh_layer_id ] are derived from VPS extension hence a parsing dependency from VPS occurs here. This needs further discussion. ]

| sps_extension2( ) { | Descriptor |
| --- | --- |
| if( !VpsDepthFlag[ nuh_layer_id ] ) { [Ed. (GT): Parsing dependency from VPS] |  |
| cp_precision | ue(v) |
| cp_in_slice_header_flag | u(1) |
| if( !cp_in_slice_header_flag ) { |  |
| for ( i = 0; i < ViewId[ nuh_layer_id ]; i++ ) { [Ed. (GT): Parsing dependency from VPS] |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics

### Video parameter set extension 2 semantics

…

cp_precision specifies the precision of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ]. The value of cp_precision shall be in the range of 0 to 5, inclusive.

cp_in_slice_header_flag equal to 0 specifies that the syntax elements of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] are present in the sequence parameter set. A value of camera_parameter_in_slice_header_flag equal to 1 specifies that the cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] syntax elements are present in the slice segment header. When not present, cp_in_slice_header_flag is inferred to be equal to 0.

cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] specify conversion parameters for converting a depth value to a disparity value.

### Sequence parameter set extension 2 semantics

The relevant sub-clause is avoided.

- H.7.4.2.2.1	Sequence parameter set extension 2 semantics

cp_precision specifies the precision of cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ]. The value of cp_precision shall be in the range of 0 to 5, inclusive.

cp_in_slice_header_flag equal to 0 specifies that the syntax elements of cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] are present in the sequence parameter set. A value of camera_parameter_in_slice_header_flag equal to 1 specifies that the cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] syntax elements are present in the slice segment header. When not present, cp_in_slice_header_flag is inferred to be equal to 0.

cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] specify conversion parameters for converting a depth value to a disparity value.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.