| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AU, July 27– Aug. 2, 2013; Document: JCT3V-Exxxx _proposed text. | Document: JCT3V-E0135 |
| --- | --- |

| Title: | CE4.h related: Coding of illumination compensation flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jewon Kang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8457 jewonk@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, a CU-level flag for illumination compensation (IC) (i.e., ic_flag) may or may not be signaled in a different condition depending on a slice-level flag and a prediction mode without parsing dependency problem [1]. This proposal presents modifications on the ic_flag signaling including the following aspects. First, a slice-level flag is removed and replaced by checking whether the current slice belongs to an IRAP picture. Second the derivation of the ic_flag signaling condition is simplified by skipping to check the reference picture indices due to disabling AMVP in the IC mode. Third, the CABAC context of the ic_flag is modified. Experimental results show that the two modifications provide the only minor change (0.1% gain) in coding gain in the common test condition (CTC).

# Introduction

In the current 3D-HEVC [2], a CU-level flag (i.e. ic_flag) for illumination compensation (IC) is adaptively signaled for a block predicted from an adjacent view to compensate the luminance discrepancy among views. Since the IC is applied with the inter-view reference picture, the ic_ flag may be signaled when the PU in the CU is coded with inter-view prediction. However, in general, the signaling condition should be made with no delay to the decoding process of the flag. For example, the completion of the merge candidate list construction of each PU to know reference indices may pause the decoding process of the flag.

The current ic_flag signaling condition was developed in [1] to remove the parsing dependency problem. The modifications consist of two parts, i.e., the disable of the IC mode for a CU coded with a partition size other than 2Nx2N and an additional slice-level flag (i.e., slice_ic_disable_merge_zero_idx) controlling whether the flag is signaled or not in special cases concerning the merge mode. In the HTM7.0rev1, the flag is turned off in every intra period, and turned on in the other pictures. If the slice-level flag is turned on, a merge index is further checked in the ic_flag signaling: if the merge index is equal to 0, the ic_flag is not signaled in the merge mode but is inferred to be 0. The reason behind this scheme is that the merge index 0 in non-IRAP pictures is largely inter-view motion vector prediction where the reference pictures are from the temporal, and, thus, the ic_flag is no use in this situation.

# Proposal

## Removal of slice_ic_disable_merge_zero_idx

The parsing dependency of the ic_flag is mainly introduced by checking whether the current CU is (partially) inter-view predicted (using an inter-view reference picture for prediction) or not. Since such information may require the parsing of the reference index, the condition can only be checked after a merge candidate list is constructed for PUs coded with merge mode.

Therefore, an additional slice-level flag (slice_ic_disable_merge_zero_idx) to control the presence of the ic_flag is added. The purpose of this flag was mainly cover the case when all (majority) of the coding units in a slice are using inter-view prediction. Therefore, at the current HTM encoder, this flag is always set to 0 when the current slices belongs to a random access point, since in this case any reference picture list of the current slice consists of inter-view reference pictures solely. So in the current design, in such a slice, the ic_flag is always signalled for merge or AMVP mode in such a slice because each inter-coded PU is predicted from an inter-view reference picture.

Therefore, the functionality of the flag slice_ic_disable_merge_zero_idx, is fully achievable by checking whether there is any temporal reference picture in a reference picture list. In the CTC, the flag can be replaced with checking whether the current picture is an IRAP or not.

With the above analysis, we propose that in an IRAP picture, all the CU therein has the ic_flag present and the slice_ic_disable_merge_zero_idx can be removed.

The syntax changes of the proposed method are shown below.

- Slice header syntax

- H.7.3.5 .1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if( layer_id ) { |  |
| slice_ic_enable_flag | u(1) |
| slice_ic_disable_merge_zero_idx_flag | u(1) |
| } |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| slice_header_extension |  |
| slice_header_extension2_flag | u(1) |
| if ( slice_header_extension2_flag ) { |  |
| slice_header_extension2_length | ue(v) |
| for( i = 0; i < slice_header_extension2_length; i++) |  |
| slice_header_extension2_data_byte[ i ] | u(8) |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

- H.7.4.5.1 General slice header semantics

...

slice_ic_disable_merge_zero_idx_flag equal to 1 specifies that ic_flag is not present in the coding units with partitioning mode equal to PART_2Nx2N of the current slice when merge_flag is equal to 1 and merge_idx of the prediction unit in the coding unit is equal to 0. slice_ic_disable_merge_zero_idx_flag equal to 0 specifies that ic_flag might be present in the coding units with partitioning mode equal to PART_2Nx2N of the current slice when merge_flag is equal to 1 and merge_idx of the prediction unit in the coding unit is equal to 0. When not present, slice_ic_disable_merge_zero_idx_flag is inferred to be equal to 0.

- H.7.4.9.1 General coding unit semantics

...

- The variable icEnableFlag is set equal to 0 and when slice_ic_enable_flag is equal to 1 and PartMode is equal to 2Nx2N and PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

  - icEnableFlag = ( merge_idx[ x0 ][ y0 ] ! = 0 ) | | !slice_ic_disable_merge_zero_idx_flag (nal_unit_type  >=  BLA_W_LP  &&  nal_unit_type  <=  RSV_IRAP_VCL23)	(Error: Reference source not found-1)

- Otherwise ( merge_flag[ x0 ][ y0 ] is equal to 0 ), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to the view order index of RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ].

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = 
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_L0 && refViewIdxL1[ x0 ][ y0 ] ! = ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_L1 && refViewIdxL0[ x0 ][ y0 ] ! = ViewIdx)	(Error: Reference source not found-2)

## Simplification of CU-level IC flag signaling condition

In addition, it is proposed that the AMVP mode for the IC is disabled in non-RAP picture, so the reference index does not need to be checked to derive the condition of the ic_flag signaling.

We propose that in a non-RAP picture, if and only if a CU coded with the merge mode having non-zero merge index has the ic_flag signalled.

The syntax changes on top of 2.1 are shown below, note that for sub-clause H.7.4.9.1., the changes in this sub-section replaces the changes in 2.1.

- H.7.4.9.1 General coding unit semantics

...

- The variable icEnableFlag is set equal to 0 and when slice_ic_enable_flag is equal to 1 and PartMode is equal to 2Nx2N and PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

  - icEnableFlag = ( merge_idx[ x0 ][ y0 ] ! = 0 ) | | !slice_ic_disable_merge_zero_idx_flag (nal_unit_type  >=  BLA_W_LP  &&  nal_unit_type  <=  RSV_IRAP_VCL23)	(Error: Reference source not found-3)

- Otherwise ( merge_flag[ x0 ][ y0 ] is equal to 0 ), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to the view order index of RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ].

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = (nal_unit_type  >=  BLA_W_LP  &&  nal_unit_type  <=  RSV_IRAP_VCL23)	
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_L0 && refViewIdxL1[ x0 ][ y0 ] ! = ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_L1 && refViewIdxL0[ x0 ][ y0 ] ! = ViewIdx)	(Error: Reference source not found-4)

Editorial improvements can be done in this sub-clause to derive the icEnableFlag as follows:

- If the current picture is an IRAP picture, icEnableFlag is set equal to 1.

- Otherwise (if the current picture is not an IRAP picture), the following applies:

- icEnableFlag = ( merge_flag[ x0 ][ y0 ] ) & & ( merge_idx[ x0 ][ y0 ] ! = 0 )

## CABAC context modification

It is proposed that three contexts are used for the CABAC coding of the ic_flag while there is only one context in the current design. The context increment is determined with the ic_flag in the above CU and the left CU as their sum. Specifically, if both of the left CU and the above CU have the ic_flags set equal to one, the context increment is two. If one of them has the ic_flag set equal to one, the context increment is one. Otherwise, it is zero. In addition, the context initialization table is also modified, accordingly.

The modifications are provided below.

- H.9.3.1.1 Initialization process for context variables

- ...

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) depth_mode_parameters( ) | depth_intra_mode | Error: Reference source not found | 0..7 | 8..15 | 16..23 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_predtex_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_delta_end_flag dmm_delta_end_abs_minus1 | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_dc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | edge_code | Error: Reference source not found | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | ic_flag | Table Error: Reference source not found-2 |  | 0 0..2 | 1 3..5 |
|  | edge_dc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | edge_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_residual_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | prefix: Error: Reference source not found suffix: na | prefix:0 suffix: na | prefix:1 suffix: na | prefix:0 suffix: na |

- ...

Table Error: Reference source not found-2 – Values of variable initValue for ic_flag ctxIdx

| Initialization variable | ic_flag |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 197 | 197 |

| Initialization variable | ic_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 107 | 139 | 126 | 107 | 139 | 126 |

- H.9.3.1.1 Derivation process of ctxIdxInc using left and above syntax elements

The specifications in subclause 9.3.3.1.1 apply with the following modifications.

–	Error: Reference source not found is appended to the end of Table 9-38.

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| iv_res_pred_weight_idx | iv_res_pred_weight_idx [ xL ][ yL ] | iv_res_pred_weight_idx [ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |
| ic_flag | ic_flag[ xL ][ yL ] | ic_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |

# Experimental results

Simulation results of the proposal are shown in Table 1-3. The implementation was based on HTM7.0 rev1, and simulations are done under common test conditions [3].

In summary, the simplification of the ic_flag signaling presented in Sec. 2.1 and Sec. 2.2 provides no coding loss as shown in Table 1, and the context modification presented in Sec. 2.3 provides 0.1% coding gain in Table 2. Combining two aspects provides 0.1% coding gain in Table 3.

Table 1: Simplification of the ic_flag signaling (Sec.2.1 & Sec 2.2) VS HTM7.0r1 (CTC).

Table 2: Modification of the ic_flag context derivation (Sec.2.3)VS HTM7.0r1 (CTC).

Table 3: The simplification + CABAC modification of the ic_flag VS HTM7.0r1 (CTC).

# References

- Tomohiro Ikai, “3D-CE5.h related: Removal of parsing dependency in illumination compensation,” JCT3V-D0060, Apr. 2013.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 4,” JCT3V-D1005, Apr. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-D1100, Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).