| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0136_r1 |
| --- | --- |

| Title: | CE7: MB-level NBDV for 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Ying Chen Li Zhang Jewon Kang Ye-Kui Wang Rajan Joshi Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 86-010-5776-0696 xinzhao@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-651-8457 jewonk@qti.qualcomm.com yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-AVC design, flexible configurations of the decoding order are supported for depth and texture. However, there is a big coding performance gap between the following 2 configurations:

- Configuration 1 (aka depth-first coding): texture followed by depth for the base view, depth followed by texture for other views.

- Configuration 2 (aka texture-first coding): texture followed by depth for all views.

Currently, configuration 1 outperforms configuration 2 by about 21%, and configuration 2 provides only negligible coding gain (about 1%) compared to MVC+D. Since configuration 2 meets the requirement of stereo/multiview compatibility while configuration 1 does not, it is important to improve the performance of configuration 2 to achieve significantly higher coding efficiency as compared to MVC+D.

The proposed method provides 18.3% BD-rate saving for texture coding compared to MVC+D, as evaluated by the bitrates and the average PSNR values of the texture views. When BVSP is further enabled, the average bitrate increase of the proposed method compared to the current 3D-AVC best performing mode is 0.4%.

# Introduction

This section introduces the flexible coding order including two configurations, and the coding performance under these two configurations is also studied.

## Flexible coding order in 3D-AVC

3D-AVC supports so-called “flexible coding order” in terms of the decoding order of the texture and depth view components within each access unit.

### Depth-first coding

In the current common test conditions (CTC), the depth-first coding order is used, wherein for each non-base view, the depth view component is followed by the texture view component, for the base view, texture view component is followed by the depth view component.

The depth-first coding is illustrated in Figure 1. View components of one access unit are shown in this figure with the numbers being the view order index values and T/D being the texture/depth view. In Figure 1, the dependency within one view is illustrated with arrows in different colors while the inter-view dependency is not shown in this figure for simplicity. As shown in Figure 1, the base view is AVC (i.e., high profile) compatible, and the depth view component can be coded using the texture view component of the base view. However, for each non-base view, the texture view component MUST depend on the associated depth view component for decoding. In addition, there are view dependency among the texture views and view dependency among the depth views.

Figure 1: Depth-first coding.

In such a configuration, to decode the texture view component of each non-base view, its associated depth view component (of the same view) must be decoded.

The main motivation of depth-first coding is to facilitate the disparity vector generation for each non-base view such that inter-view prediction coding tools can be made more efficient. However it is questionable such a configuration is really needed if the disparity vector generation can be achieved by just accessing the texture views.

### Texture-first coding

Texture-first coding is enabled in the current 3D-AVC codec to meet an MPEG requirement on stereo/multiview compatibility.

In the requirement document of the 3DV CfP (w12035) [1], it is stated that “The compressed data format shall include a mode that enables the simple extraction of bitstreams for stereo and mono output, and support high-fidelity reconstruction of samples from the left and right views of the stereo video.”

Also, it is desirable to have the multi-view compatibility. Therefore, the current 3D-AVC enables the texture-first coding to support the stereo/multiview compatibility. The texture-first coding is illustrated in Figure 2, wherein the decoding of the texture view component does not rely on a depth view component.

Figure 2: Texture-first coding.

In this configuration, it becomes feasible that only the texture views are decoded when decoding a texture-only operation point. Therefore, bandwidth required for transmitting such a texture only operation point may be significantly reduced, and the complexity of decoding depth view components could be saved.

## Coding performance for the two 3D-AVC configurations

We provide coding performance comparisons between texture-first coding 3D-AVC, depth-first coding 3D-AVC, and MVC+D based on the ATM 8.0 software.

### 3D-AVC depth-first coding vs MVC+D

The depth-first coding mode of 3D-AVC significantly outperforms MVC+D, as indicated by the synthesized results, by about 17.80%.

The texture coding results, however, are problematic since the bitrates used in the current template (provided by CTC) only considers the bitrates of the texture views, while a slimmest operation point providing only the texture views requires all the depth views and texture views. Therefore the bitrates of the depth views need to be considered for measuring the texture coding performance.

### 3D-AVC texture-first coding vs MVC+D

As shown in the above table, the texture-first coding mode of 3D-AVC provides almost the same (only 1.1% better) coding performance of MVC+D for the texture only operation point (evaluated by the PSNR and the bitrates of the texture views). Meanwhile, the texture-first coding mode of 3D-AVC outperforms MVC+D by only about 1.5% when evaluated by the synthesized PSNR and overall bitrates.

### 3D-AVC texture-first coding vs 3D-AVC depth-first coding

It is noticed that texture-first coding has a significant loss, by 20% for overall, compared to depth-first coding, as configured in the common test condition (CTC) of JCT-3V.

# Problems

The shortcomings of the current 3D-AVC design are summarized as follows:

- In the current best performing configuration (depth-first coding), it is necessary that the depth view component is decoded prior to the texture view component in the same non-base view. In other words, in applications where only a texture only operation point is desired, the transmission and decoding of the depth view components is necessary, and this increases the burden of bandwidth and decoding complexity. For example, when the bitrates of the depth views are taken into account, the depth-first coding configuration outperforms MVC+D for texture coding by around only 9.5%. Furthermore, it requires decoding of six view components for the 3-view case, instead of only three view components for MVC+D.

- When configured in a way that texture view component can be decoded without accessing its depth view component, i.e. texture-first decoding, majority of the coding tools cannot work properly due to the missing disparity vector used for these coding tools. In such a configuration, around 20% coding loss is observed when compared to the depth-first coding, and only around 1% coding gain is observed when compared to MVC+D, or MVC, as shown in section 1.2.2.

# Proposal

It is proposed that for texture-first coding in 3D-AVC, a disparity derivation method independent from depth map is proposed, such that all the existing coding tools are enabled. An additional tool called Neighboring Block based Disparity Vector (NBDV), which is a simplified version of a similar tool in 3D-HEVC is proposed. With NBDV, for disparity vector derivation, there is no need to access the depth view component of the same view for decoding any texture view component, such that the dependency on depth is completely removed.

The proposed method enables all the existing tools in the texture-first mode.

## Proposal description

We propose to introduce MB-level NBDV into 3D-AVC. Similar to 3D-HEVC, neighboring blocks are checked in order, once a neighboring block contains a disparity motion vector (i.e., it is inter-view predicted), the horizontal component of the motion vector is identified to be a disparity vector, and the whole process terminates. The identified disparity vector is the output of the NBDV process, and is  used to replace the disparity vector which is currently derived based on the depth view component of the same view under the current 3D-AVC depth-first configuration.

Detailed spec. text changes are included, and the NBDV process described below is mainly included in subclause J.8.2.1.8.

### Spatial neighboring blocks

The spatial neighboring blocks that will be checked in AVC motion prediction process are checked in the order of A (left), B (above), C (above-right), and D (above-left) in the proposed NBDV process. The spatial neighboring blocks are shown as in Figure 3.

Figure 3: Spatial neighboring blocks for NBDV.

### Temporal neighboring blocks

Blocks from up to two reference pictures in the same view of the current picture are checked in order, RefPicList1[ 0 ] and RefPicList0[ 0 ] for B slices and RefPicList0[ 0 ] for P slices.

Currently only one temporal block located at the bottom-right (BR) position of co-located MB is checked for each selected reference picture, as shown as in Figure 4.

Figure 4: Temporal neighboring blocks for NBDV.

### Derived disparity vector

A single derived disparity vector, namely Dynamic derived Disparity Vector (DDV), is maintained for the whole slice and updated to be the output of NBDV after each non-Intra MB is decoded. During the NBDV process, if none of the temporal or spatial neighboring blocks has an available disparity motion vector, the DDV is set as the output of NBDV process.

### Termination of the process

The above mentioned neighboring blocks and DDV are checked in order, as illustrated in Figure 5. Similar to 3D-HEVC, temporal neighboring blocks are checked first and the spatial neighboring blocks are checked afterwards. Once a block contains an available disparity motion vector, the derivation process terminates.

## Highlights of the other aspects of the proposed method

- When accessing base view depth is enabled (e.g., in BVSP mode), disparity vector is further refined (namely NBDV-R) by the base view depth similar to 3D-HEVC. Based on the NBDV-R result, BVSP can be enabled in a similar way in 3D-HEVC.

- The temporal neighboring block is checked before spatial blocks in the current implementation. Besides, if the order is changed (i.e., checking spatial neighboring blocks first), the coding performance of the spatial neighboring block being checked first becomes slightly worse than the temporal neighboring block being checked first.

- Only one neighboring block corresponding to RefPicList0 are checked during the NBDV process for P slices, and two neighboring blocks corresponding to RefPicList0 and RefPicList1 are checked for B slices. In addition, for each neighboring block, only the motion vector in one reference picture list is checked during the NBDV process.

- A single derived disparity vector, is set to (0, 0) at the beginning of each slice and updated to the NBDV results after each non-Intra MB is decoded. When NBDV doesn’t find a disparity motion vector in neighboring blocks, it returns the derived disparity vector.

# Simulations results

In this section, simulation results are provided with different configurations and compared with MVC+D and 3D-AVC CTC respectively Error: Reference source not found. The software implementation as provided in this document is based on ATM 8.0.

“BVSP off” indicates that no texture view is coded based on any depth view.

“BVSP on” indicates that no texture view is coded based on any depth view of a non-base view, but non-base texture views are coded with access to the depth view components of the base view.

## Comparison of Proposal (BVSP off) and MVC+D

Comparisons on the coding performance between proposed method and MVC+D are tabulated in Table 1.a and Table 1.b. In Table 1.a, the anchor is MVC+D and the tested is proposed method with BVSP off, while in Table 1.b, the anchor and tested are reversed.

Table 1.a: Proposal (BVSP off) vs. MVC+D.

Table 1.b: MVC+D vs. Proposal (BVSP off).

As shown in the above table, the proposed method, when BVSP is turned off, outperforms MVC+D by 18% for texture only coding and 15% for the overall (evaluated by the overall bitrates and PSNR of synthesized views).

## Comparison of Proposal (BVSP on) and MVC+D

Comparisons on the coding performance between proposed method and MVC+D are tabulated in Table 2.a and Table 2.b. In Table 2.a, the anchor is MVC+D and the tested is proposed method with BVSP on, while in Table 2.b, the anchor and tested are reversed.

Table 2.a: Proposal (BVSP on) vs. MVC+D.

Table 2.b: MVC+D vs. Proposal (BVSP on).

As shown in the above table, after enabling BVSP, the proposed method outperforms MVC+D for 21% for texture only coding and 17% for the overall (evaluated by the overall bitrates and PSNR of synthesized views).

## Comparison of Proposal (BVSP on) vs 3D-AVC CTC (depth-first)

Comparisons on the coding performance between proposed method and 3D-AVC CTC are tabulated in Table 3.a and Table 3.b. In Table 3.a, the anchor is 3D-AVC CTC and the tested is proposed method with BVSP on, while in Table 3.b, the anchor and tested are reversed.

Table 3.a: Proposal (BVSP on) vs. 3D-AVC CTC.

Table 3.b: 3D-AVC CTC vs. Proposal (BVSP on).

As shown in the above table, after enabling BVSP, the proposed provides similar performance as 3D-AVC best performing configuration, when evaluated by the overall bitrates and the synthesized views.

## Summary of coding performance

For multiview compatible configuration, where texture decoding does not rely on decoded depth map, the results of proposed method are summarized as below,

In the above table, “ATM_TFC” indicates 3D-AVC texture first coding with BVSP disabled, and “QC_NBDV” indicates the proposed method with BVSP disabled.

For best performing configuration, the results of proposed methods are also summarized as below,

In the above table, “CTC” indicates the current 3D-AVC best performing mode, “QC_NBDV” indicates the proposed method with BVSP enabled. From the above table, it is observed that, the proposed method achieves comparable coding performance of the current 3D-AVC best performing mode. Even in this case, with the proposed method, there is no need to decode each depth view component for a texture view component in the same non-base view.

## Complexity Analysis

In this section, the complexity aspects are analyzed with respect to the memory requirement and memory access, as compared to H.264/AVC and 3D-AVC. Detailed information could be found in the attached excel files.

For memory requirement, compared to 3D-AVC non-base view coding, the proposed method only needs 0.20% and 0.28% memory increase for 2-view and 3-view configuration, respectively.

For memory access, compared to 3D-AVC non-base view coding, when configured to be multiview compatible, the proposed NBDV increases the memory access by 0.1% for texture plus depth decoding. When configured as the best performing mode, i.e., access to depth is allowed for texture coding, the proposed method increases the memory access by 0.2% for texture plus depth decoding.

## Summary results

In this subsection, we summarize the comparisons between the current 3D-AVC and proposed NBDV method in terms of several aspects including coding performance and complexity.

As shown below in Table 4, comparisons between 3D-AVC and proposed NBDV, regarding to coding performance, memory storage, memory access, number of decoded views, are also tabulated. Please note that the coding gain of proposal in multiview compatible mode shown in the third row (i.e., -18.31%) is compared to MVC+D in terms of coded texture views.

Table 4: Comparison of memory requirement between 3D-AVC and Proposed

|  | Proposal compared to 3D-AVC CTC |  |  |  |
| --- | --- | --- | --- | --- |
|  | Coding gain | Memory increase | Memory access increase | Ratio of numbers of decoded views to decode all texture views |
| Proposal in multiview compatible mode | -18.31%* | 0.28% | 0.1% | Half (3 vs 6) |
| Proposal in best performing mode | 0.39% (for synth. views) | 0.28% | 0.2% | 2/3 (4 vs 6) |

# Conclusion

To remove dependency on depth view component when decoding each texture view component of the same non-base view, NBDV as utilized in 3D-HEVC is proposed in 3D-AVC with simplifications to derive the disparity vector required for coding tools.

The proposed method enables all the existing coding tools thus even provides slight gain compared with the best performing 3D-AVC configuration.

The proposed method provides 18% gain for the texture only coding compared to MVC+D, as evaluated by the bitrates and the PSNR values of the texture views, and 15% gain for the overall, as evaluated by the total bitrates and the PSNR of the synthesized views.

When BVSP is enabled, around 2% additional gain can be achieved to reach the best coding performance. Even in this case, there is no need to decode each depth view component for a texture view component in the same non-base view.

# References

- Video and Requirements Group, "Applications and Requirements on 3D Video Coding," ISO/IEC JTC1/SC29/WG11, w12035, Geneva, Switzerland, March 2011.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1100, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei, " 3D-CE3.a related: Unconstrained inside-view motion prediction in 3D video coding," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0136, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).