| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-D1002 |
| --- | --- |

| Title: | 3D-AVC Draft Text 6 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | Draft text |  |  |
| Editors: | Miska M. Hannuksela Nokia Corporation Ying Chen Qualcomm Teruhiko Suzuki Sony Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft | Email: | miska.hannuksela@nokia.com cheny@qti.qualcomm.com teruhikos@jp.sony.com ohm@ient.rwth-aachen.de garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

- Abstract

This document is the draft 6 of the 3D-AVC specification (“AVC compatible video-plus-depth extension”). The document is based on JCT3V-C1002 with adoptions of the JCT3V-D meeting integrated.

A list of adoptions in the JCT3V-D meeting:

- Missing adoptions from the JCT3V-C meeting and clarifications (as reported in JCT3V-D0002)

  - ALC

- Comments received in the ballot of ISO/IEC 14496-10:2012/PDAM 3

- Resolve editor’s comments. (ballot comments FI2, US16).

Editor’s comments have been resolved except for those that relate to new adoptions in the JCT3V-D meeting.

- Editorial improvements: US5, US9, US10, US11, US15

- Specify the allowable ranges for syntax elements in the depth parameter set (ballot comment US7)

- Specify the derivation process of subclause J.8.3.1 without reference to subclause 8.4.1. (ballot comment US8)

- Removal of the following depth coding tools (as per responding to ballot comments FI7, US13/14):

- depth intra coding and the related deblocking change

- IVMP

- JCT3V-D0162: 3D-CE1.a related: Harmonization of inter-view and view synthesis prediction in 3D-AVC.

- JCT3V-D0107: 3D-CE2.a: MB-level depth-to-DV conversion in ATM.

- JCT3V-D0115: Skip flag coding in 3D-AVC

- JCT3V-D0116: Simplification of adaptive luminance compensation in 3D-AVC

- JCT3V-D0161: 3D-AVC: Coding of unpaired MVD data

- JCT3V-D0070:AHG8 Revision on Depth View Information SEI message in 3D-AVC DT5

In 0.6, add the following paragraph after the paragraph that starts with “An extension of multiview video coding that additionally supports the inclusion of depth maps”:

A multiview video coding extension with depth information is specified in Annex J. Sub-bitstreams consisting of a texture base view conform to this Specification, sub-bitstreams consisting of multiple texture views may also conform to Annex H of this Specification, and sub-bitstreams consisting of one or more texture views and one or more depth views may also conform to Annex I of this Specification. Enhanced texture view coding that utilizes the associated depth views and decoding processes for depth views are specified for this extension.

In 0.7, add the following paragraph after the paragraph that starts with “Annex I specifies”:

Annex J specifies a multiview video coding extension with depth information (3D-AVC). The reader is referred to Annex J for the entire decoding process for 3D-AVC, which is specified there with references being made to clauses 2-9 and Annexes A-E and Annexes H-I. Subclause J.10 specifies one profile for 3D-AVC.

In 7.3.1 NAL unit syntax, replace the syntax table with:

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| NumBytesInRBSP = 0 |  |  |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 14 ¦ ¦ nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21 ) { |  |  |
| if( nal_unit_type ! = 21 ) |  |  |
| svc_extension_flag | All | u(1) |
| else |  |  |
| avc_3d_extension_flag | All | u(1) |
| if( svc_extension_flag ) { |  |  |
| nal_unit_header_svc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| nalUnitHeaderBytes += 3 |  |  |
| } |  |  |
| else if( avc_3d_extension_flag ) { |  |  |
| nal_unit_header_3davc_extension( ) /* specified in Annex J */ |  |  |
| nalUnitHeaderBytes += 2 |  |  |
| } |  |  |
| else { |  |  |
| nal_unit_header_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| nalUnitHeaderBytes += 3 |  |  |
| } |  |  |
| } |  |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| i += 2 |  |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | All | f(8) |
| } else |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| } |  |  |
| } |  |  |

In 7.3.2.1.3,Subset sequence parameter set data syntax, replace the syntax table with:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 138 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex I */ |  |  |
| } else if( profile_idc = = 139 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex I */ |  |  |
| seq_parameter_set_3davc_extension( ) /* specified in Annex J */ |  |  |
| } |  |  |
| additional_extension2_flag | 0 | u(1) |
| if( additional_extension2_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension2_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

In 7.3.12.13 Slice layer extension RBSP syntax, replace the syntax table with:

| slice_layer_extension_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| if( svc_extension_flag ) { |  |  |
| slice_header_in_scalable_extension( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| if( !slice_skip_flag ) |  |  |
| slice_data_in_scalable_extension( ) /* specified in Annex Error: Reference source not found */ | 2 ¦ 3 ¦ 4 |  |
| } else if ( avc_3d_extension_flag ) { |  |  |
| slice_header_in_3davc_extension( ) /* specified in Annex J */ |  |  |
| slice_data_in_3davc_extension( ) /* specified in Annex J */ |  |  |
| } else { |  |  |
| slice_header( ) | 2 |  |
| slice_data( ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| rbsp_slice_trailing_bits( ) | 2 |  |
| } |  |  |

Replace Table 7-1 with:

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C | Annex A NAL unit type class | Annex G and Annex H NAL unit type class | Annex I and Annex J NAL unit type class |
| --- | --- | --- | --- | --- | --- |
| 0 | Unspecified |  | non-VCL | non-VCL | non-VCL |
| 1 | Coded slice of a non-IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | VCL | VCL | VCL |
| 2 | Coded slice data partition A slice_data_partition_a_layer_rbsp( ) | 2 | VCL | not applicable | not applicable |
| 3 | Coded slice data partition B slice_data_partition_b_layer_rbsp( ) | 3 | VCL | not applicable | not applicable |
| 4 | Coded slice data partition C slice_data_partition_c_layer_rbsp( ) | 4 | VCL | not applicable | not applicable |
| 5 | Coded slice of an IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3 | VCL | VCL | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | 5 | non-VCL | non-VCL | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | 0 | non-VCL | non-VCL | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | 1 | non-VCL | non-VCL | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | 6 | non-VCL | non-VCL | non-VCL |
| 10 | End of sequence end_of_seq_rbsp( ) | 7 | non-VCL | non-VCL | non-VCL |
| 11 | End of stream end_of_stream_rbsp( ) | 8 | non-VCL | non-VCL | non-VCL |
| 12 | Filler data filler_data_rbsp( ) | 9 | non-VCL | non-VCL | non-VCL |
| 13 | Sequence parameter set extension seq_parameter_set_extension_rbsp( ) | 10 | non-VCL | non-VCL | non-VCL |
| 14 | Prefix NAL unit prefix_nal_unit_rbsp( ) /* specified in Annex G and Annex H */ | 2 | non-VCL | suffix dependent | suffix dependent |
| 15 | Subset sequence parameter set subset_seq_parameter_set_rbsp( ) /* specified in Annex G, Annex H and Annex I */ | 0 | non-VCL | non-VCL | non-VCL |
| 16 | Depth parameter set | 11 | non-VCL | non-VCL | non-VCL |
| 17..18 | Reserved |  | non-VCL | non-VCL | non-VCL |
| 19 | Coded slice of an auxiliary coded picture without partitioning slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | non-VCL | non-VCL | non-VCL |
| 20 | Coded slice extension slice_layer extension_rbsp( ) | 2, 3, 4 | non-VCL | VCL | VCL |
| 21 | Coded slice extension for a depth view component or a 3D-AVC texture view component slice_layer_extension_rbsp( ) | 2, 3, 4 | non-VCL | non-VCL | VCL |
| 22..23 | Reserved |  | non-VCL | non-VCL | non-VCL |
| 24..31 | Unspecified |  | non-VCL | non-VCL | non-VCL |

In 7.4.1, Remove the following sentence.

When nal_unit_type is equal to 21, svc_extension_flag shall be equal to 0 and the semantics of svc_extension_flag equal to 1 are reserved for future specification by ITU-T | ISO/IEC.

In 7.4.1, add the following paragraphs in the semantics of svc_extension_flag just before the semantics of rbsp_byte[ i ].

When svc_extension_flag is not present, the value of svc_extension_flag is inferred to be equal to 0.

The value of svc_extension_flag shall be equal to 0 for coded video sequences conforming to one or more profiles specified in Annex J. Decoders conforming to one or more profiles specified in Annex J shall ignore (remove from the bitstream and discard) NAL units for which nal_unit_type is equal to 14 or 20 and for which svc_extension_flag is equal to 1.

avc_3d_extension_flag indicates for NAL units having nal_unit_type equal to 21 whether a nal_unit_header_mvc_extension( ) or nal_unit_header_3davc_extension( ) will follow next in the syntax structure.

When avc_3d_extension_flag is not present, the value of avc_3d_extension_flag is inferred to be equal to 0.

The value of DepthFlag is specified as follows.

DepthFlag = ( nal_unit_type  ! =  21 ) ? 0 : ( avc_3d_extension_flag ? depth_flag : 1 )	(7-xx)

The value of avc_3d_extension_flag shall be equal to 0 for coded video sequences conforming to one or more profiles specified in Annex I. Decoders conforming to one or more profiles specified in Annex I shall ignore (remove from the bitstream and discard) NAL units for which nal_unit_type is equal to  21 and for which avc_3d_extension_flag is equal to 1.

In Table D.1, add the following after “depth_sampling_info( payloadSize )”:

| else if( payloadType = = 54 ) |  |  |
| --- | --- | --- |
| constrained_depth_parameter_set_identifier( payloadSize ) /* specified in Annex J */ | 5 |  |
| else if( payloadType = = 55 ) |  |  |
| depth_view_info ( payloadSize ) /* specified in Annex J */ | 5 |  |

In I.7.4.3 Slice header semantics, replace the semantics with:

The semantics specified in clause Error: Reference source not found apply with the substitution of texture view component (for nal_unit_type equal to 1, 5, and 20) or depth view component (for nal_unit_type equal to 21 and avc_3d_extension_flag equal to 0) for view component and with the following modifications.

When nal_unit_type is equal to 1, 5, or 20, all constraints specified in clause Error: Reference source not found apply only to the texture view components with the same value of VOIdx. When nal_unit_type is equal to 21 and avc_3d_extension_flag is equal to 0, all constraints specified in clause Error: Reference source not found apply only to the depth view components with the same value of VOIdx.

The value of the following MVCD sequence parameter set syntax elements shall be the same across all coded slice NAL units of nal_unit_type equal to 1, 5, and 20 of an access unit: chroma_format_idc.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type equal to 1, 5, and 20 of an access unit: field_pic_flag and bottom_field_flag.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type equal to 21 and avc_3d_extension_flag equal to 0 of an access unit: field_pic_flag and bottom_field_flag.

Annex J

Multiview and depth video coding

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies multiview and depth video coding, referred to as 3D-AVC.

  - Scope

Bitstreams and decoders conforming to the profile specified in this annex are completely specified in this annex with reference made to clauses Error: Reference source not found-Error: Reference source not found and Annexes Error: Reference source not found-I.

  - Normative references

The specifications in clause Error: Reference source not found apply.

  - Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause I.Error: Reference source not found. These definitions are either not present in clause I.Error: Reference source not found or replace definitions in clause I.Error: Reference source not found.

- MVC texture view component: A texture view component composed of coded slice NAL units of nal_unit_type not equal to 21.

- texture view component: A coded representation of the texture of a view in a single access unit. A texture view component may be a 3D-AVC texture view component or an MVC texture view component.

- view component pair: A texture view component and a depth view component of the same view within the same access unit.

- view synthesis prediction: A prediction derived from samples of inter-view reference components using motion vectors derived from a depth view component for decoding a texture view component.

- 3D-AVC sequence parameter set: A collective term for sequence parameter set or subset sequence parameter set.

- 3D-AVC texture view component: A texture view component composed of coded slice NAL units of nal_unit_type equal to 21.

- 3DV acquisition parameters: The closest and farthest depth values.

  - Abbreviations

The specifications in clause Error: Reference source not found apply with the following additions:

3D-AVC	Multiview Video Coding with Depth as specified in Annex J

VSP	View Synthesis Prediction

  - Conventions

The specifications in clause Error: Reference source not found apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause Error: Reference source not found apply with substitution of 3D-AVC sequence parameter set for sequence parameter set.

  - Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

    - Method of specifying syntax in tabular form

The specifications in subclause I.Error: Reference source not found apply.

    - Specification of syntax functions, categories, and descriptors

The specifications in subclause I.Error: Reference source not found apply.

    - Syntax in tabular form

      - NAL unit syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - NAL unit header 3D-AVC extension syntax

| nal_unit_header_3davc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| view_idx | All | u(8) |
| depth_flag | All | u(1) |
| non_idr_flag | All | u(1) |
| temporal_id | All | u(3) |
| anchor_pic_flag | All | u(1) |
| inter_view_flag | All | u(1) |
| } |  |  |

      - Raw byte sequence payloads and RBSP trailing bits syntax

        - Sequence parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

            - Scaling list syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set extension RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Subset sequence parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set MVC extension syntax

The syntax table is specified in subclause I.7.3.2.1.4.

          - Sequence parameter set 3D-AVC extension syntax

| seq_parameter_set_3davc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( NumDepthViews > 0 ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| for( i = 0; i < NumDepthViews; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc ) { |  |  |
| depth_ranges( NumDepthViews, 2, 0 ) |  |  |
| vsp_param( NumDepthViews, 2, 0 ) |  |  |
| } |  |  |
| reduced_resolution_flag | 0 | u(1) |
| slice_header_prediction_flag | 0 | u(1) |
| seq_view_synthesis_flag | 0 | u(1) |
| nonlinear_depth_representation_num | 0 | ue(v) |
| for( i = 1; i <= nonlinear_depth_representation_num; i++ ) |  |  |
| nonlinear_depth_representation_model[ i ] | 0 | ue(v) |
| } |  |  |
| alc_sps_enable_flag | 0 | u(1) |
| enable_rle_skip_flag | 0 | u(1) |
| } |  |  |

        - Picture parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Supplemental enhancement information RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Supplemental enhancement information message syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Access unit delimiter RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - End of sequence RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - End of stream RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Filler data RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Slice layer without partitioning RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Slice data partition RBSP syntax

Slice data partition syntax is not present in coded video sequences conforming to one or more of the profiles specified in this annex.

        - RBSP slice trailing bits syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - RBSP trailing bits syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Prefix NAL unit RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Depth parameter set RBSP syntax

| depth_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| depth_parameter_set_id | 11 | ue(v) |
| pred_direction | 11 | ue(v) |
| if( pred_direction = = 0 ¦ ¦ pred_direction = = 1 ) { |  |  |
| ref_dps_id0 | 11 | ue(v) |
| predWeight0 = 64 |  |  |
| } |  |  |
| if( pred_direction = = 0 ) { |  |  |
| ref_dps_id1 | 11 | ue(v) |
| pred_weight0 | 11 | u(6) |
| predWeight0 = pred_weight0 |  |  |
| } |  |  |
| num_depth_views_minus1 | 11 | ue(v) |
| depth_ranges( num_depth_views_minus1 + 1, pred_direction, depth_parameter_set_id ) |  |  |
| vsp_param_flag | 11 | u(1) |
| if( vsp_param_flag ) |  |  |
| vsp_param( num_depth_views_minus1 + 1, pred_direction, depth_parameter_set_id ) |  |  |
| depth_param_additional_extension_flag | 11 | u(1) |
| if(depth_param_additional_extension_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| depth_param_additional_extension_data_flag | 11 | u(1) |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

          - Depth ranges syntax

| depth_ranges( numViews, predDirection, index ) { | C | Descriptor |
| --- | --- | --- |
| z_near_flag | 11 | u(1) |
| z_far_flag | 11 | u(1) |
| if( z_near_flag ) |  |  |
| 3dv_acquisition_element( numViews, 0, predDirection, 7, 0, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| 3dv_acquisition_element( numViews, 0, predDirection, 7, 0, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| } |  |  |

          - 3DV acquisition element syntax

| 3dv_acquisition_element( numViews, deltaFlag, predDirection, precMode, expLen, OutSign, OutExp, OutMantissa, OutManLen ) { | C | Descriptor |
| --- | --- | --- |
| if( numViews – deltaFlag > 1 ) |  |  |
| element_equal_flag | 11 | u(1) |
| if( element_equal_flag = = 0 ) |  |  |
| numValues = numViews – deltaFlag |  |  |
| else |  |  |
| numValues = 1 |  |  |
| for( i = 0; i < numValues; i++ ) { |  |  |
| if( predDirection = = 2 && i = = 0 ) { |  |  |
| if( precMode = = 0 ) { |  |  |
| matissa_len_minus1 | 11 | u(5) |
| OutManLen[ index, i ] = manLen = mantissa_len_minus + 1 |  |  |
| } else |  |  |
| prec | 11 | u(5) |
| } |  |  |
| if( predDirection = = 2 ) { |  |  |
| sign0 | 11 | u(1) |
| OutSign[ index, i ] = sign0 |  |  |
| exponent0 | 11 | u(expLen) |
| OutExp[ index, i ] = exponent0 |  |  |
| if( precMode = = 1 ) { |  |  |
| if( exponent0 = = 0 ) |  |  |
| OutManLen[ index, i ] = manLen = Max( 0, prec – 30 ) |  |  |
| else |  |  |
| OutManLen[ index, i ] = manLen = Max( 0, exponent0 + prec – 31 ) |  |  |
| } |  |  |
| mantissa0 | 11 | u(manLen) |
| OutMantissa[ index, i ] = mantissa0 |  |  |
| } else { |  |  |
| skip_flag | 11 | u(1) |
| if( skip_flag = = 0 ) { |  |  |
| sign1 | 11 | u(1) |
| OutSign[ index, i ] = sign1 |  |  |
| exponent_skip_flag | 11 | u(1) |
| if( exponent_skip_flag = = 0 ) { |  |  |
| exponent1 | 11 | u(expLen) |
| OutExp[ index, i ] = exponent1 |  |  |
| } else |  |  |
| OutExp[ index, i ] = OutExp[ ref_dps_id0, i ] |  |  |
| mantissa_diff | 11 | se(v) |
| mantissaPred = (( OutMantissa[ ref_dps_id0, i ] * predWeight0 + OutMantissa[ ref_dps_id1, i ] * ( 64-predWeight0 ) + 32 ) >> 6 ) |  |  |
| OutMantissa[ index, i ] = mantissaPred + mantissa_diff |  |  |
| OutManLen[ index, i ] = OutManLen[ ref_dps_id0, i ] |  |  |
| } else { |  |  |
| OutSign[ index, i ] = OutSign[ ref_dps_id0, i ] |  |  |
| OutExp[ index, i ] = OutExp[ ref_dps_id0, i ] |  |  |
| OutMantissa[ index, i ] = OutMantissa[ ref_dps_id0, i ] |  |  |
| OutManLen[ index, i ] = OutManLen[ ref_dps_id0, i ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( element_equal_flag = = 1 ) { |  |  |
| for( i = 1; i < num_views_minus1 + 1 – deltaFlag; i++ ) { |  |  |
| OutSign[ index, i ] = OutSign[ index, 0 ] |  |  |
| OutExp[ index, i ] = OutExp[ index, 0 ] |  |  |
| OutMantissa[ index, i ] = OutMantissa[ index, 0 ] |  |  |
| OutManLen[ index, i ] = OutManLen[ index, 0 ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

          - View synthesis prediction parameters syntax

| vsp_param( numViews, predDirection, index ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i < numViews; i++ ) |  |  |
| for( j = 0; j < i; j++ ) { |  |  |
| disparity_diff_wji[ j ][ i ] | 0 | ue(v) |
| disparity_diff_oji[ j ][ i ] | 0 | ue(v) |
| disparity_diff_wij[ i ][ j ] | 0 | ue(v) |
| disparity_diff_oij[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |

        - Slice layer extension RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

      - Slice header syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Reference picture list modification syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Reference picture list MVC modification syntax

The syntax table is specified in subclause I.7.3.3.1.1

        - Prediction weight table syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Decoded reference picture marking syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Slice header in 3D-AVC extension syntax

| slice_header_in_3davc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( avc_3d_extension_flag && slice_header_prediction_flag != 0 ) { |  |  |
| pre_slice_header_src | 2 | u(2) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| pre_ref_lists_src | 2 | u(2) |
| if ( !pre_ref_lists_src ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| } |  |  |
| } |  |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) { |  |  |
| pre_pred_weight_table_src | 2 | u(2) |
| if( !pre_pred_weight_table_src ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) { |  |  |
| pre_dec_ref_pic_marking_src | 2 | u(2) |
| if( !pre_dec_ref_pic_marking_src ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| } |  |  |
| slice_qp_delta | 2 | se(v) |
| } else { |  |  |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id | 2 | u(2) |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21 ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else |  |  |
| ref_pic_list_modification( ) | 2 |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != I && slice_type != SI ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| if( slice_type = = SP ¦ ¦ slice_type = = SI ) { |  |  |
| if( slice_type = = SP ) |  |  |
| sp_for_switch_flag | 2 | u(1) |
| slice_qs_delta | 2 | se(v) |
| } |  |  |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| if ( nal_unit_type = = 21 && ( slice_type != I && slice_type != SI )) { |  |  |
| if( DepthFlag ) |  |  |
| depth_weighted_pred_flag | 2 | u(1) |
| else if( avc_3d_extension_flag && ViewCompOrder( 0, view_id ) > ViewCompOrder( 1, view_id ) ) { |  |  |
| dmvp_flag | 2 | u(1) |
| if( seq_view_synthesis_flag ) |  |  |
| slice_vsp_flag | 2 | u(1) |
| } |  |  |
| if ( 3dv_acquisition_idc != 1 && ( depth_weighted_pred_flag ¦ ¦ dmvp_flag ) ) |  |  |
| dps_id | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( DepthFlag && profile_idc = = 139 ) |  |  |
| nonlinear_depth_representation_enable_flag | 2 | u(1) |
| } |  |  |

      - Slice data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Slice data in 3D-AVC extension syntax

| slice_data_in_3davc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( entropy_coding_mode_flag ) |  |  |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| moreDataFlag = 1 |  |  |
| prevMbSkipped = 0 |  |  |
| RunLength = 0 |  |  |
| do { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| if( !entropy_coding_mode_flag ) { |  |  |
| mb_skip_run | 2 | ue(v) |
| prevMbSkipped = ( mb_skip_run > 0 ) |  |  |
| for( i=0; i<mb_skip_run; i++ ) |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && mb_skip_run > 0 && VspRefExist ) |  |  |
| mb_skip_type_flag | 2 | u(1) |
| if( mb_skip_run > 0 ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| } else { |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && leftMbVSSkipped && upMbVSSkipped ) { |  |  |
| mb_vsskip_flag | 2 | ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| if( !mb_vsskip_flag ) { |  |  |
| mb_skip_flag | 2 | ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| } |  |  |
| RunLength = 0 |  |  |
| } else { |  |  |
| rleCtx = RLESkipContext( ) |  |  |
| if( rleCtx && !RunLength ) { |  |  |
| mb_skip_run_type | 2 | ae(v) |
| RunLength = 16 |  |  |
| } else if( !rleCtx && RunLength ) |  |  |
| RunLength = 0 |  |  |
| if( rleCtx && mb_skip_run_type ) |  |  |
| RunLength -= 1 |  |  |
| else |  |  |
| mb_skip_flag | 2 | ae(v) |
| if( rleCtx && !mb_skip_flag ) |  |  |
| RunLength = 0 |  |  |
| moreDataFlag = !mb_skip_flag |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && !mb_skip_flag ) { |  |  |
| mb_vsskip_flag | 2 | ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| } |  |  |
| } |  |  |
| if(alc_sps_enable_flag && nal_unit_type = = 21 && slice_type = = P && !DepthFlag && !mb_vsskip_flag && mb_skip_flag = = 1 ) |  |  |
| mb_alc_skip_flag | 2 | ae(v) |
| } |  |  |
| if( moreDataFlag ) { |  |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ¦ ¦ ( CurrMbAddr % 2 = = 1 && prevMbSkipped ) ) ) |  |  |
| mb_field_decoding_flag | 2 | u(1) ¦ ae(v) |
| macroblock_layer_in_3davc_extension( ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| if( !entropy_coding_mode_flag ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| prevMbSkipped = mb_skip_flag ¦ ¦ mb_vsskip_flag |  |  |
| if( MbaffFrameFlag && CurrMbAddr % 2 = = 0 ) |  |  |
| moreDataFlag = 1 |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |  |
| } |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

      - Macroblock layer syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Macroblock prediction syntax

The syntax table is specified in subclause I.7.3.5.1.

        - Sub-macroblock prediction syntax

The syntax table is specified in subclause I.7.3.5.2.

        - Residual data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual luma syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual block CAVLC syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual block CABAC syntax

The syntax table is specified in subclause I.Error: Reference source not found.

      - Macroblock layer in 3D-AVC extension syntax

| macroblock_layer_in_3davc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| mb_type | 2 | ue(v) ¦ ae(v) |
| if( nal_unit_type = = 21 && !DepthFlag && slice_type = = B && direct_spatial_mv_pred_flag && VspRefExist && mb_type = = B_Direct_16x16 ) |  |  |
| mb_direct_type_flag | 2 | u(1) ¦ ae(v) |
| if( alc_sps_enable_flag && nal_unit_type = = 21 && slice_type = = P && !DepthFlag && ( mb_type = = P_L0_16x16 ¦¦ mb_type = = P_L0_L0_16x8 ¦¦ mb_type = = P_L0_L0_8x16 ) ) |  |  |
| mb_alc_flag | 2 | u(1) ¦ ae(v) |
| if( mb_type = = I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 3 | f(1) |
| for( i = 0; i < 256; i++ ) |  |  |
| pcm_sample_luma[ i ] | 3 | u(v) |
| for( i = 0; i < 2 * MbWidthC * MbHeightC; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 3 | u(v) |
| } else { |  |  |
| noSubMbPartSizeLessThan8x8Flag = 1 |  |  |
| if( mb_type != I_NxN && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) { |  |  |
| sub_mb_pred_in_3davc_extension( mb_type ) | 2 |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 ) { |  |  |
| if( NumSubMbPart( sub_mb_type[ mbPartIdx ] ) > 1 ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else if( !direct_8x8_inference_flag ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else { |  |  |
| if( transform_8x8_mode_flag && mb_type = = I_NxN ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| mb_pred_in_3davc_extension( mb_type ) | 2 |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) { |  |  |
| coded_block_pattern | 2 | me(v) ¦ ae(v) |
| if( ( CodedBlockPatternLuma > 0 ¦ ¦ mb_alc_flag = = 1 ) && transform_8x8_mode_flag && mb_type != I_NxN && noSubMbPartSizeLessThan8x8Flag && ( mb_type != B_Direct_16x16 ¦ ¦ direct_8x8_inference_flag ) ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | se(v) ¦ ae(v) |
| residual( 0, 15 ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |

        - Macroblock prediction in 3D-AVC extension syntax

| mb_pred_in_3davc_extension( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ) |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ) |  |  |
| for( luma8x8BlkIdx=0; luma8x8BlkIdx<4; luma8x8BlkIdx++ ) { |  |  |
| prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] ) |  |  |
| rem_intra8x8_pred_mode[ luma8x8BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| if( ChromaArrayType = = 1 ¦ ¦ ChromaArrayType = = 2 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 && mb_alc_flag = = 0 ) { |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| if( VspRefL0Flag[ mbPartIdx ] && slice_vsp_flag ) |  |  |
| bvsp_flag_l0[ mbPartIdx ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) { |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| if( VspRefL1Flag[ mbPartIdx ] && slice_vsp_flag ) |  |  |
| bvsp_flag_l1[ mbPartIdx ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 && ( !VspRefL0Flag[ mbPartIdx ] ¦ ¦ !bvsp_flag_l0[ mbPartIdx ] ) ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 && ( !VspRefL1Flag[ mbPartIdx ] ¦ ¦ !bvsp_flag_l1[ mbPartIdx ] ) ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

        - Sub-macroblock prediction syntax

| sub_mb_pred_in_3davc_extension( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| sub_mb_type[ mbPartIdx ] | 2 | ue(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && mb_type != P_8x8ref0 && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && mb_alc_flag = = 0 ) { |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| if( VspRefL0Flag[ mbPartIdx ] && slice_vsp_flag ) |  |  |
| bvsp_flag_l0[ mbPartIdx ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| if( VspRefL1Flag[ mbPartIdx ] && slice_vsp_flag ) |  |  |
| bvsp_flag_l1[ mbPartIdx ] | 2 | u(1) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && ( !VspRefL0Flag[ mbPartIdx ] ¦ ¦ !bvsp_flag_l0[ mbPartIdx ] ) ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 && ( !VspRefL1Flag[ mbPartIdx ] ¦ ¦ !bvsp_flag_l1[ mbPartIdx ] ) ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |

    - Semantics

Semantics associated with the syntax structures and syntax elements within these structures (in subclause J.7.3 and in subclause I.7.3 by reference in subclause J.7.3) are specified in this subclause and by reference to subclause I.7.4. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Recommendation | International Standard.

      - NAL unit semantics

The semantics for the syntax elements in subclause 7.3.1 are specified in subclause I.7.3.1.

        - NAL unit header 3D-AVC extension semantics

- view_idx specifies the view oder index for the NAL unit.

- view_id is inferred to be equal to view_id[ view_idx ], where view_id[ ] is present in the active sequence parameter set.

- The variable VOIdx, representing the view order index of the view identified by view_id[ i ], is set equal to view_idx.

- depth_flag equal to 1 indicates that the current NAL unit belongs to a depth view component, depth_flag equal to 0 indicates that the current NAL unit belongs to a texture view component.

- non_idr_flag, temporal_id, anchor_pic_flag, and inter_view_flag have the same semantics as those syntax elements with the same names in Annex H.

        - Order of NAL units and association to coded pictures, access units, and video sequences

The specification of subclause I.7.4.1.2 applies.

          - Order of 3D-AVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

- The specification of subclause I.7.4.1.2.1 applies.

- In addition, the following applies for the activation of depth parameter set.

- A depth parameter set includes parameters that can be referred to by the coded slice NAL units of one or more texture view or depth view components of one or more coded pictures. A depth parameter set associated with depth_parameter_set_id equal to 0 contains the depth ranges syntax structure and the view synthesis prediction parameter syntax structure included in the active sequence parameter set. A depth parameter set with depth_parameter_set_id greater than 0 is a depth parameter set RBSP with that depth_parameter_set_id value.

- Each depth parameter set is initially considered not active at the start of the operation of the decoding process. At most one depth parameter set is considered as the active depth parameter set at any given moment during the operation of the decoding process, and when any particular depth parameter set becomes the active depth parameter set, the previously-active depth parameter set (if any) is deactivated.

- When a depth parameter set (with a particular value of depth_parameter_set_id) is not the active depth parameter set and it is referred to by a coded slice NAL unit (when dps_id is present in a slice header), it is activated. This depth parameter set is called the active depth parameter set until it is deactivated when another depth parameter set becomes the active depth parameter set. A depth parameter set, with that particular value of depth_parameter_set_id, shall be available to the decoding process prior to its activation. When a depth parameter set is activated, the same depth parameter set shall remain active for subsequent coded slice NAL units of the same access unit.

- If any depth parameter set is present that is never activated in the bitstream (i.e., it never becomes the active depth parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

          - Order of access units and association to coded video sequences

The specification of subclause I.7.4.1.2.2 applies.

          - Order of NAL units and coded pictures and association to access units

The specification of subclause I.Error: Reference source not found applies with the following modifications.

The association of VCL NAL units to primary or redundant coded pictures is specified in subclause I.7.4.1.2.5.

          - Detection of the first VCL NAL unit of a primary coded picture

The specification of subclause I.7.4.1.2.4 applies.

          - Order of VCL NAL units and association to coded pictures

Each VCL NAL unit is part of a coded picture.

Let voIdx be the value of VOIdx of any particular VCL NAL unit. The order of the VCL NAL units within a coded picture is constrained as follows:

- For all VCL NAL units following this particular VCL NAL unit, the value of VOIdx shall be greater than or equal to voIdx.

- All VCL NAL units for a depth view component, if present, shall follow any VCL NAL unit of an MVC texture view component with a same value of VOIdx.

For each set of VCL NAL units within a texture or depth view component, the following applies:

–	If arbitrary slice order, as specified in Annex Error: Reference source not found, clause Error: Reference source not found, clause Error: Reference source not found or clause J.10, is allowed, coded slice NAL units of a view component may have any order relative to each other.

- Otherwise (arbitrary slice order is not allowed), coded slice NAL units of a slice group shall not be interleaved with coded slice NAL units of another slice group and the order of coded slice NAL units within a slice group shall be in the order of increasing macroblock address for the first macroblock of each coded slice NAL unit of the same slice group.

The following applies:

–	If a coded texture view component with a particular view_id is the first field view component of a complementary field pair, the depth view component with the same view_id value, if present in the access unit, shall be a coded frame view component or the first field view component of a complementary field pair.

–	Otherwise, if a coded texture view component with a particular view_id is the second field view component of a complementary field pair, the depth view component with the same view_id value, if present in the access unit, shall be the second field view component of a complementary field pair.

–	Otherwise, if a coded texture view component with a particular view_id is a non-paired field, the depth view component with the same view_id value, if present in the access unit, shall be a coded frame view component or a non-paired field.

–	Otherwise (a coded texture view component with a particular view_id is a coded frame), the depth view component with the same view_id value, if present in the access unit, shall be a coded frame view component.

NAL units having nal_unit_type equal to 12 may be present in the access unit but shall not precede the first VCL NAL unit of the primary coded picture within the access unit.

NAL units having nal_unit_type equal to 0 or in the range of 24 to 31, inclusive, which are unspecified, may be present in the access unit but shall not precede the first VCL NAL unit of the primary coded picture within the access unit.

NAL units having nal_unit_type in the range of 22 to 23, inclusive, which are reserved, shall not precede the first VCL NAL unit of the primary coded picture within the access unit (when specified in the future by ITU-T | ISO/IEC).

      - Raw byte sequence payloads and RBSP trailing bits semantics

        - Sequence parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

            - Scaling list semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set extension RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Subset sequence parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set MVC extension semantics

The semantics specified in subclause I.7.4.2.1.4 apply with the substitution of texture view component or depth view component for view component

          - Sequence parameter set 3D-AVC extension semantics

The function ViewCompOrder( depthFlag, viewId ) is specified to return the value of viewCompOrder derived as follows:

i = 0
do {
while ( i <= num_views_minus1  &&  ( view_id[ i ]  !=  viewId  | |  
	( depthFlag  &&  !depth_view_present_flag[ i ] )  | | 
	( !depthFlag  &&  !texture_view_present_flag[ i ] ) )	(J-1)
	i++
if( i > num_views_minus1 )
	viewCompOrder = MAX_INT
else
	viewCompOrder = 2 * i + depthFlag

3dv_acquisition_idc equal to 0 indicates that no depth ranges or view synthesis prediction parameters syntax structures are present in the sequence parameter set. 3dv_acquisition_idc equal to 1 indicates that depth ranges and view synthesis prediction parameters syntax structures are present in the sequence parameter set and valid for the entire coded video sequence. 3dv_acquisition_idc equal to 2 indicates that depth ranges and view synthesis prediction parameters syntax structures are present in the sequence parameter set and depth parameter sets with depth_parameter_set_id greater than 0 may be activated. 3dv_acquisition_idc values greater than 2 are reserved.

The function ViewIdTo3DVAcquisitionParamIndex( viewId ) is specified to return the value of i for which view_id_3dv[ i ] is equal to viewId in the active sequence parameter set.

reduced_resolution_flag equal to 1 specifies that the depth view components of a view component pair have a lower spatial resolution than the luma component of the texture view component of the same view component pair, and the width and height (as represented by pic_width_in_mbs_minus1+1 and pic_height_in_map_units_minus1+1 in the referred subset sequence parameter set) of the depth view components are both half of the width and height of all the texture view components. reduced_resolution_flag equal to 0 specifies that when both depth view components and texture view components are present, they have the same spatial resolution.

slice_header_prediction_flag equal to 0 indicates that slice header prediction from texture view component to depth view component or vice versa is disallowed. slice_header_prediction_flag equal to 1 indicates that the prediction is used.

seq_view_synthesis_flag equal to 1 indicates view synthesis prediction is enabled. seq_view_synthesis_flag equal to 0 indicates that view synthesis prediction is disabled for all view components referring the current sequence parameter set.

nonlinear_depth_representation_num + 1 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

nonlinear_depth_representation_model[ i ] specifies the piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

NOTE – When nonlinear_depth_representation_num is equal to 0, a depth sample represents a disparity normalized to the range of 0 to 255, inclusive, so that 0 corresponds to ZFar value and 255 corresponds to ZNear value. Depending on the value of nonlinear_depth_representation_num, a depth view component is composed of either depth samples that can be converted to disparity using a linear equation or nonlinearly transformed depth samples. If nonlinear_depth_representation_num is equal to 0, depth view component contains directly depth samples that are uniformly quantized in terms of disparity, i.e. depth samples that can be transformed to disparity using a linear equation. If nonlinear_depth_representation_num is greater than 0, depth view component contains nonlinearly transformed depth samples. The use of nonlinear depth representation is further controlled by the nonlinear_depth_representation_enable_flag syntax element in the slice header.

alc_sps_enable_flag equal to 0 specifies that mb_alc_skip_flag and mb_alc_flag are not present. alc_sps_enable_flag equal to 1 specifies that specifies that mb_alc_skip_flag and mb_alc_flag may be present.

enable_rle_skip_flag equal to 0 specifies that mb_skip_run_type are not present. enable_rle_skip_flag equal to 1 specifies that mb_skip_run_type may be present. When enable_rle_skip_flag is not present, it is inferred to be equal to 0.

        - Picture parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Supplemental enhancement information RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Supplemental enhancement information message semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Access unit delimiter RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - End of sequence RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - End of stream RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Filler data RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Slice layer without partitioning RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Slice data partition RBSP semantics

Slice data partition syntax is not present in bitstreams conforming to one or more of the profiles specified in Annex J.

        - RBSP slice trailing bits semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - RBSP trailing bits semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Prefix NAL unit RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Depth parameter set RBSP semantics

depth_parameter_set_id identifies the depth parameter set that is referred to in the slice header. The value of depth_parameter_set_id shall be in the range of 1 to 63, inclusive.

pred_direction equal to 0 specifies that the closest and farthest depth values for the base view may be predicted from the respective variables of two pictures. pred_direction equal to 1 specifies that the closest and farthest depth values for the base view may be predicted from the respective variables of one picture. pred_direction equal to 2 specifies that the closest and farthest depth values for the base view are not predicted. The value of pred_direction shall be in the range of 0 to 2, inclusive.

ref_dps_id0 specifies a first reference depth parameter set to be used in prediction of the closest and farthest depth value. The value of ref_dps_id0 shall be in the range of 0 to 63, inclusive.

ref_dps_id1 specifies a second reference depth parameter set to be used in prediction of the closest and farthest depth value. The value of ref_dps_id1 shall be in the range of 0 to 63, inclusive.

pred_weight0 specifies a weight associated with the a first reference depth parameter set derived from ref_dps_id0. The value of pred_weight0 shall be in the range of 0 to 63, inclusive.

num_views_minus1 plus 1 specifies the number of views for which depth parameters are specified in the included 3dv_acquisition_element ( ) structure. num_view_minus1 shall be in the range of 0 to 1023, inclusive.

vsp_param_flag equal to 1 indicates the presence of the vsp_param( ) syntax structure. vsp_param_flag equal to 0 indicates the absence of the vsp_param( ) syntax structure.

depth_param_additional_extension_flag equal to 0 indicates that no additional data follows within the depth parameter set RBSP prior to the RBSP trailing bits. The value of depth_param_additional_extension_flag shall be equal to 0. The value of 1 for depth_param_additional_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for depth_param_additional_extension_flag in a depth parameter set RBSP.

          - Depth ranges semantics

The contents of the syntax structure are controlled through input variables predDirection and index the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	index may be equal to the depth_parameter_set_id of the depth parameter set wherein the parameters are present.

z_near_flag equal to 0 specifies that the syntax elements specifying the closest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the closest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

          - 3DV acquisition element semantics

The syntax structure specifies the value of an element in the depth ranges syntax structure. The element may contain one or more loop entries i of the order specified by view_id_3dv syntax elements.

The contents of the syntax structure are controlled through input variables predDirection, precMode, and expLen the semantics of which are as follows.

–	deltaFlag equal to 0 specifies that the each loop entry corresponds to a value specific to the view indicated by i. deltaFlag equal to 1 specifies that each loop entry corresponds to a difference of values between the views indicated by i and i + 1.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	precMode equal to 0 specifies that the number of bits in the mantissa syntax element.

–	expLen specifies the number of bits in the exponent syntax element.

The syntax structure uses OutSign, OutExp, and OutMantissa variables for both input and output, where each variable is indexed by [ index, viewIdc ], index being an identifier (equal to either 0 when decoding depth ranges in sequence parameter set or depth_parameter_set_id value when decoding depth range parameter set) to a depth parameter set and viewIdc being a view indicator (in the order of views for 3DV acquisition parameters).

element_equal_flag equal to 0 specifies that the sign, exponent, and mantissa may not be identical to respective values for any two loop entries i and j. element_equal_flag equal to 1 specifies that the sign, exponent, and mantissa are identical to respective values for any two loop entries i and j.

mantissa_len_minus1 + 1 specifies the number of bits in the mantissa syntax element. The value of mantissa_len_minus1 shall be in the range of 0 to 31, inclusive.

prec specifies the exponent of the maximum allowable truncation error for the value represented by the sign, exponent, and mantissa given by 2−prec. The value of prec shall be in the range of 0 to 31, inclusive.

sign0 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign0 equal to 1 indicates that the sign is negative.

exponent0 specifies the exponent of the value provided by the loop entry. The value of exponent0 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa0 specifies the mantissa of the value provided by the loop entry.

skip_flag equal to 0 specifies that syntax elements sign1, exponent_skip_flag and mantissa_diff are present for the loop entry. skip_flag equal to 1 specifies that elements sign1, exponent_skip_flag and mantissa_diff are not present for the loop entry.

sign1 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign1 equal to 1 indicates that the sign is negative.

exponent1, if present, specifies the exponent of the value provided by the loop entry. The value of exponent1 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa_diff specifies the difference of the mantissa of the value provided by the loop entry relative to its prediction value.

          - View synthesis prediction parameters semantics

The contents of the syntax structure are controlled through input variables predDirection and index the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	index identifies a depth parameter set.

disparity_diff_wji[ j ][ i ], disparity_diff_oji[ j ][ i ], disparity_diff_wij[ i ][ j ] and disparity_diff_oij[ i ][ j ] specify the variables DisparityScale and DisparityOffset as follows.

if( predDirection  = =  2 ) {
	DisparityScale[ index ][ j ][ i ] = disparity_diff_wji[ j ][ i ]
	DisparityOffset[ index ][ j ][ i ] = disparity_ diff_oji[ j ][ i ]
	DisparityScale[ index ][ i ][ j ] = disparity_diff_wij[ i ][ j ] – disparity_diff_wji[ j ][ i ]
	DisparityOffset[ index ][ i ][ j ] = disparity_diff_oij[ i ][ j ] – disparity_diff_oji[ j ][ i ]
} else {					(J-2)
	DisparityScale[ index ][ j ][ i ] = disparity_diff_wji[ j ][ i ] + ( DisparityScale[ ref_dps_id0 ][ j ][ i ] *
		predWeight0 + DisparityScale[ ref_dps_id1 ][ j ][ i ] * ( 64 – predWeight0 ) + 32 ) >> 6
	DisparityOffset[ index ][ j ][ i ] = disparity_diff_oji[ j ][ i ] + ( DisparityOffset[ ref_dps_id0 ][ j ][ i ] *
		predWeight0 + DisparityOffset[ ref_dps_id1][ j ][ i ] * ( 64 – predWeight0 ) + 32 ) >> 6
	DisparityScale[ index ][ i ][ j ] = disparity_diff_wij[ i ][ j ] + ( DisparityScale[ ref_dps_id0 ] i ][ j ] * 
		predWeight0 + DisparityScale[ ref_dps_id1 ][ i ][ j ] * ( 64 – predWeight0 ) + 32 ) >> 6
	DisparityOffset[ index ][ i ][ j ] = disparity_diff_oij[ i ][ j ] + ( DisparityOffset[ ref_dps_id0 ][ i ][ j ] * 
		predWeight0 + DisparityOffset[ ref_dps_id1 ][ i ][ j ] * ( 64 – predWeight0 ) + 32 ) >> 6
}

        - Slice layer extension RBSP semantics

The semantics specified in subclause I.7.4.2.14 apply.

      - Slice header semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Reference picture list modification semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Reference picture list MVC modification semantics

The semantics specified in subclause I.7.4.3.1.1 apply.

        - Prediction weight table semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Decoded reference picture marking semantics

The semantics specified in subclause I.Error: Reference source not found apply to each view independently, with "sequence parameter set" being replaced by "3D-AVC sequence parameter set", with "primary coded picture" being replaced by "texture view component” for nal_unit_type 1, 5, and 20 as well as nal_unit_type 21 when DepthFlag is equal to 1, and by “depth view component” for nal_unit_type 21 when DepthFlag is equal to 0.

        - Slice header in 3D-AVC semantics

The semantics specified in subclause H.Error: Reference source not found apply with the substitution of texture view component or depth view component for view component and with the following modifications.

When nal_unit_type is equal to 1, 5, 20, or 21 with DepthFlag equal to 0, all constraints specified in subclause H.Error: Reference source not found apply only to the texture view components with the same value of VOIdx. When nal_unit_type is equal to 21 and DepthFlag is equal to 1, all constraints specified in subclause H.Error: Reference source not found apply only to the depth view components with the same value of VOIdx.

The value of the following 3D-AVC sequence parameter set syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, 20 and 21 with DepthFlag equal to 0 of an access unit: chroma_format_idc.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, 20 and 21 with DepthFlag equal to 0 of an access unit: field_pic_flag and bottom_field_flag.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type equal to 21 and DepthFlag equal to 1 of an access unit: field_pic_flag and bottom_field_flag.

pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src specify if the respective syntax elements are present in the slice header, and, if not, the slice header from which the values of the respective syntax elements are taken as specified in Table J-1 and Table J-2.

When a syntax element has an inferred value in the slice header from which its value is taken according to Table J-1 and Table J-2, the syntax element value in the current slice header is equal to this inferred value. When a syntax element is not present and has no inferred value in the slice header from which its value is taken according to Table J-1 and Table J-2, the syntax element is inferred to be absent in the current slice header.

pre_slice_header_src shall not be equal to 0.

When ViewCompOrder( DepthFlag, view_id ) is smaller than ViewCompOrder( !DepthFlag, view_id ), pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src shall not be equal to 2

- Table J-1 – Respective syntax elements for pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src

| Prediction indication syntax element | Respective syntax elements |
| --- | --- |
| pre_slice_header_src | colour_plane_id, frame_num, field_pic_flag, bottom_field_flag, idr_pic_id, pic_order_cnt_lsb, delta_pic_order_cnt_bottom, delta_pic_order_cnt[ 0 ], delta_pic_order_cnt[ 1 ], redundant_pic_cnt, direct_spatial_mv_pred_flag, cabac_init_idc, sp_for_switch_flag, slice_qs_delta, disable_deblocking_filter_idc, slice_alpha_c0_offset_div2, slice_beta_offset_div2, slice_group_change_cycle, depth_weighted_pred_flag, dmvp_flag, slice_vsp_flag, dps_id |
| pre_ref_lists_src | num_ref_idx_active_override_flag, num_ref_idx_l0_active_minus1, num_ref_idx_l1_active_minus1 and reference picture list modification syntax table |
| pre_pred_weight_table_src | pred_weight_table( ) syntax structure |
| pre_dec_ref_pic_marking_src | dec_ref_pic_marking( ) syntax structure |

- Table J-2 – Semantics of the values of pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src

| Value of pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src or pre_dec_ref_pic_marking_src | Semantics |
| --- | --- |
| 0 | The respective syntax elements are not predicted but included in the slice header. |
| 1 | The values of the respective syntax elements are taken from the slice header of the first slice of the previous view component in decoding order having the same value of DepthFlag as the current slice, belonging to a dependent view of the current view, and residing in the same access unit. |
| 2 | The values of the respective syntax elements are taken from the first slice header of the first slice of the view component having the same view_id as the current slice and a different value of DepthFlag. |
| 3 | The values of the respective syntax elements are taken from the first slice header of the first slice of the view component in the same access unit having view order index equal to 0 and the same value of DepthFlag as the current slice. |

depth_weighted_pred_flag equal to 0 specifies that no depth-range-based weighted prediction is used for corresponding slice RBSP. depth_weighted_pred_flag equal to 1 specifies that depth-range-based weighted prediction is used for corresponding slice RBSP. When not present, depth_weighted_pred_flag is inferred to be equal to 0. When depth_weighted_pred_flag is equal to 1, the process of derivation of prediction weights specified in  applies.

dmvp_flag is used in the decoding process for inter prediction, inter-view prediction, view synthesis prediction and adaptive luminance compensation as specified in subclause J.8.2.

slice_vsp_flag together with bvsp_flag[ mbPartIdx ] specify, when ref_idx_lX[ mbPartIdx ] (with X equal to 0 or 1) refers to an inter-view reference picture, which motion vector derivation process specified in subclause 8.2.1 is in use.

dps_id specifies the depth parameter set in use. The value of dps_id shall be in the range of 0 to 63, inclusive. When dps_id is equal to 0, depth parameters are set according to syntax elements in seq_parameter_set_3davc_extension( ) of the active sequence parameter set. When present, the value of dps_id shall be the same in all slice headers within an access unit.

nonlinear_depth_representation_enable_flag equal to 0 specifies that the current slice is represented in linear form. nonlinear_depth_representation_flag equal to 1 specifies that the current slice is represented in nonlinear form.

NOTE – The derivation of the variable NdrInverse[ i ], as specified below, depends on the value of nonlinear_depth_representation_enable_flag. When nonlinear_depth_representation_enable_flag in the current slice is equal to 0, NdrInverse[ i ] indicates a linear mapping and, in practice, the linear mapping indicates that input is transferred to the ouput without any change(e.g. scaling and offset).

When nonlinear_depth_representation_enable_flag in the current slice is equal to 1, NdrInverse[ i ], as specified below, is used to transform depth sample values from nonlinear representation to the linear representation. The shape of this transform is defined by means of line-segment-approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first (0, 0) and the last (255, 255) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (nonlinear_depth_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to 255, inclusive, with spacing depending on the value of nonlinear_depth_representation_num.

When nonlinear_depth_representation_enable_flag in the current slice is equal to 0, Variable NdrInverse[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

nonlinear_depth_representation_model[ 0 ] = 0
nonlinear_depth_representation_model[ nonlinear_depth_representation_num + 1 ] = 0
for( k=0; k<= nonlinear_depth_representation_num; ++k )
{
	pos1 = ( 255 * k ) / ( nonlinear_depth_representation_num + 1 )
	pos2 = ( 255 * ( k+1 ) ) / ( nonlinear_depth_representation_num + 1 ) )

	x1 = pos1			(J-3)
	y1 = pos1
	x2 = pos2
	y2 = pos2

	for ( x = Max( x1, 0 ); x <= Min( x2, 255 ); ++x )
		NdrInverse[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

When nonlinear_depth_representation_enable_flag in the current slice is equal to 1, Variable NdrInverse[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

nonlinear_depth_representation_model[ 0 ] = 0
nonlinear_depth_representation_model[ nonlinear_depth_representation_num + 1 ] = 0
for( k=0; k<= nonlinear_depth_representation_num; ++k )
{
	pos1 = ( 255 * k ) / ( nonlinear_depth_representation_num + 1 )
	dev1 = nonlinear_depth_representation_model[ k ]
	pos2 = ( 255 * ( k+1 ) ) / ( nonlinear_depth_representation_num + 1 ) )
	dev2 = nonlinear_depth_representation_model[ k+1 ]

	x1 = pos1 - dev1		(J-4)
	y1 = pos1 + dev1
	x2 = pos2 - dev2
	y2 = pos2 + dev2

	for ( x = Max( x1, 0 ); x <= Min( x2, 255 ); ++x )
		NdrInverse[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

      - Slice data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Slice data in 3D-AVC extension semantics

The semantics specified in subclause I.Error: Reference source not found apply with the following additions.

When mb_skip_flag is not present, it is inferred to be equal to 0.

mb_skip_type_flag is used to derive the variable MbVSSkipFlag. When mb_skip_type_flag is present, the variable MbVSSkipFlag is set equal to mb_skip_type_flag.

mb_vsskip_flag is used to derive the variable MbVSSkipFlag. When mb_vsskip_flag is present, the variable MbVSSkipFlag is set equal to mb_vsskip_flag.

NOTE – MbVSSkipFlag controls whether subclause 8.2.1.1 or 8.2.1.2 is used for deriving motion vectors for P_Skip macroblocks and whether subclause 8.2.1.3 or 8.2.1.6 is used for deriving motion vectors and reference indices for B_Skip macroblocks.

leftMbVSSkipped is derived to be 1 if the left macroblock adjacent to the current macroblock is available and the MbVSSkipFlag of the left macroblock is equal to 1, leftMbVSSkipped is derived to be 0 otherwise.

upMbVSSkipped is derived to be 1 if the upper macroblock adjacent to the current macroblock is available and the MbVSSkipFlag of the upper macrolbock is equal to 1, upMbVSSkipped is derived to be 0 otherwise.

The function RLESkipContext( ) is specified as follows:

–	The derivation process for neighbouring macroblocks specified in subclause 6.4.11.1 is invoked and the output is assigned to mbAddrA and mbAddrB;

–	If all of the following conditions are true, the return value of RLESkipContext( ) is equal to TRUE.

–	enable_rle_skip_flag is equal to 1

–	nal_unit_type is equal to 21

–	slice_type is equal to B

–	mbAddrA is unavailable or mb_skip_flag for the macroblock mbAddrA is equal to 1

–	mbAddrB is unavailable or mb_skip_flag for the macroblock mbAddrB is equal to 1

–	Otherwise, the return value of RLESkipContext( ) is equal to FALSE.

mb_skip_run_type equal to 0 specifies that mb_skip_run_type is not present in the slice_data( ) until mb_skip_flag is equal 0. When RunLength is greater than 0 and mb_skip_run_type equal to 1, mb_skip_flag is inferred to be equal to 1. mb_skip_run_type shall be equal to 0 or 1.

mb_alc_skip_flag equal to 1 specifies that the adaptive luminance compensation is applied for the current macroblock. mb_alc_skip_flag equal to 0 specifies that the adaptive luminance compensation is not applied for the current macroblock. When not present, mb_alc_skip_flag is inferred to be equal to 0. When mb_alc_skip_flag is equal to 1, the current macroblock shall be coded as P_Skip.

      - Macroblock layer semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Macroblock prediction semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Sub-macroblock prediction semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Residual data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual luma semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual block CAVLC semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual block CABAC semantics

The semantics specified in subclause I.Error: Reference source not found apply.

      - Macroblock layer in 3D-AVC extension semantics

The semantics specified in subclause I.Error: Reference source not found apply with the following additions.

mb_direct_type_flag is used to determine the derivation process for motion vectors and reference indices for B_Direct_16x16 macroblocks and B_Direct_8x8 sub-macroblocks as specified in subclause 8.2.1.

mb_alc_flag equal to 1 specifies that the adaptive luminance compensation mode is in use for the current macroblock. mb_alc_flag equal to 0 specifies that the adaptive luminance compensation mode is not in use for the current macroblock. When mb_alc_flag is not present, it is inferred to be equal to 0. When mb_alc_flag is equal to 1, the current macroblock shall be coded as P_L0_16x16, P_L0_L0_16x8, or P_L0_L0_8x16.

        - Macroblock prediction in 3D-AVC extension semantics

The semantics specified in subclause I.Error: Reference source not found apply with the following additions.

bvsp_flag_lX[ mbPartIdx ] (X being equal to 0 or 1) equal to 0 specifies, when ref_idx_lX[ mbPartIdx ] (with X equal to 0 or 1) refers to an inter-view reference picture, which motion vector derivation process specified in subclause 8.2.1 is in use. When bvsp_flag_lX[ mbPartIdx ] is not present and VspRefLXFlag[ mbPartIdx ] is equal to 0, bvsp_flag_lX[ mbPartIdx ] is inferred to be equal to 0. When bvsp_flag_lX[ mbPartIdx ] is not present and VspRefLXFlag[ mbPartIdx ] is equal to 1, bvsp_flag_lX[ mbPartIdx ] is inferred to be equal to slice_vsp_flag.

        - Sub-macroblock prediction in 3D-AVC semantics

The semantics specified in subclause I.Error: Reference source not found apply with the following additions.

bvsp_flag_lX[ mbPartIdx ] has the same semantics as bvsp_flag_lX[ mbPartIdx ] in subclause 7.4.5.1.

  - 3D-AVC decoding process

This subclause specifies the decoding process for an access unit of a coded video sequence conforming to one or more of the profiles specified in Annex J. Specifically, this subclause specifies how the decoded picture with multiple texture view components and multiple depth view components is derived from syntax elements and global variables that are derived from NAL units in an access unit when the decoder is decoding the operation point identified by the target temporal level and the target output views.

The decoding process is specified such that all decoders shall produce numerically identical results for the target output views. Any decoding process that produces identical results for the target output views to the process described here conforms to the decoding process requirements of this Recommendation | International Standard.

Unless stated otherwise, the syntax elements and derived upper-case variables that are referred to by the decoding process specified in this subclause and all child processes invoked from the process specified in this subclause are the syntax elements and derived upper-case variables for the current access unit.

The target output texture and depth views are either specified by external means not specified in this Specification, or, when not specified by external means, there shall be one target output texture view which is the base texture view.

NOTE – The association of VOIdx values to view_id values according to the decoding process of clause Error: Reference source not found may differ from that of the decoding process of clause 7.4.6.

A target output view may include only a texture view, only a depth view, or both the texture view and the depth view, which have the same view_id value.

All sub-bitstreams that can be derived using the sub-bitstream extraction process with pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more viewIdTarget's identifying the views in the bitstream as inputs as specified in subclause J.8.3 shall result in a set of coded video sequences, with each coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex I and Annex J.

Let the variable textureFirstFlag be 1 when NumDepthViews is equal to 0 or ViewCompOrder( 0, view_idx ) < ViewCompOrder( 1, view_idx ), and let it be 0 when ViewCompOrder( 0, view_idx ) > ViewCompOrder( 1, view_idx ).

Let vOIdxList be a list of integer values specifying the VOIdx values of the view components of the access unit. The variable VOIdxMax is set equal to the maximum value of the entries in the list vOIdxList, and the variable vOIdxMin is set to the minimum value of the entries in the list vOIdxList. VOIdxMax shall be the same for all access units within a coded video sequence. vOIdxMin shall be the same for all anchor access units within a coded video sequence. When the current access unit is an anchor access unit, the variable VOIdxMin is set to vOIdxMin.

The 3D-AVC decoding process specified in this subclause is repeatedly invoked for each texture and depth view component with VOIdx from vOIdxMin to VOIdxMax, inclusive, which is present in the list vOIdxList, in increasing order of VOIdx and in decoding order of texture or depth view components as specified in subclause 7.4.1.2.5.

Outputs of the multiview video decoding process are decoded samples of the current primary coded picture including all decoded texture and depth view components of the target output texture and depth views.

For each texture view component and each depth view component, the specifications in clause I.Error: Reference source not found apply, with the decoding process for reference picture lists construction being modified in subclauses J.8.1. The 3D-AVC inter prediction, inter-view prediction, view synthesis prediction and inter prediction with adaptive luminance componesnation processes are specified in subclause J.8.3. The decoding process for depth range parameters is specified in subclause J.8.4. Additionally, the specification of bitstream subsets is specified in subclause J.8.3.

    - 3D-AVC decoding process for reference picture lists construction

The specifications of subclause I.8.1 apply with the following additions:

–	A horizontal disparity vector dvMBX is set to 0.

–	When DepthFlag is equal to 0, the variable VspRefExist is specified after applying subclause H.8.2 as follows.

–	If seq_view_synthesis_flag is equal to 0 (view synthesis prediction is disabled), VspRefExist is set to 0.

–	Otherwise, if the current slice is a P or SP slice and there exists at least one inter-view reference component in RefPicList0, or if  the current slice is a B slice and there exists at least one inter-view reference component in either RefPicList0 or RefPicList1, VspRefExist is set to 1;

–	Otherwise, VspRefExist is set to 0.

–	The variable VspRefL0Flag[ mbPartIdx ] is specified as follows.

–	If VspRefExist is equal to 1, ref_idx_l0[ mbPartIdx ] is present, and ref_idx_l0[ mbPartIdx ] indicates an inter-view reference component, the variable VspRefL0Flag[ mbPartIdx ] is set to 1.

–	Otherwise, the variable VspRefL0Flag[ mbPartIdx ] is set to 0.

–	The variable VspRefL1Flag[ mbPartIdx ] is specified as follows.

–	If VspRefExist is equal to 1, ref_idx_l1[ mbPartIdx ] is present, and ref_idx_l1[ mbPartIdx ] indicates an inter-view reference component, the variable VspRefL1Flag[ mbPartIdx ] is set to 1.

–	Otherwise, the variable VspRefL1Flag[ mbPartIdx ] is set to 0.

    - 3D-AVC inter prediction, inter-view prediction, view synthesis prediction and adaptive luminance compensation

This process is invoked when decoding P and B macroblock types and when nal_unit_type is equal to 21.

Outputs of this process are Inter prediction samples for the current macroblock that are a 16x16 array predL of luma samples and when ChromaArrayType is not equal to 0 two (MbWidthC)x(MbHeightC) arrays predCb and predCr of chroma samples, one for each of the chroma components Cb and Cr.

The variable log2Div is specified to be equal to BitDepthY + 6.

The function Disparity( depthSample, srcIndex, refIndex ) is specified to return disparityValue specified as follows.

disparityValue = ( NdrInverse[ depthSample ] * DisparityScale[ dps_id ][ srcIndex ][ refIndex ] + 
	( DisparityOffset[ dps_id ][ srcIndex ][ refIndex ] << BitDepthY ) + 	(J-5)
	( 1 << ( log2Div – 1 ) ) ) >> log2Div

The function DisparityForBlock( depthPic, x1, y1, blWidth, blHeight, srcViewId, refViewId ) is specified to return maxDisp specified with the following ordered steps.

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < blHeight; j += ( blHeight – 1 ) )
	for( i = 0; i < blWidth; i += ( blWidth – 1 ) )		(J-6)
		dX = Clip3(0, PicWidthInSamplesL, (x1+i) >> reduced_resolution_flag) 
		dY = Clip3(0, PicHeightInSamplesL, (y1+j) >> reduced_resolution_flag)	
		if( depthPic[ dX, dY ] > maxDepth ) 
			maxDepth = depthPic[ dX, dY ]

- The variable maxDisp is specified as follows:

srcIndex = ViewIdTo3DVAcquisitionParamIndex( srcViewId )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )	(J-7)
maxDisp = Disparity( maxDepth, srcIndex, refIndex)

When DepthFlag is equal to 0 and dmvp_flag is equal to 1, the variables DepthRefPicList0, DepthRefPicList1 for B slices, and DepthCurrPic are specified as follows. The variable DepthRefPicList0 is specified to consist of the depth view components of the view component pairs for which the texture view components are in RefPicList0 in the order that RefPicList0[ i ] and DepthRefPicList0[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l0_active_minus1. The variable DepthRefPicList1 is specified for B slices to consist of the depth view components of the view component pairs for which the texture view components are in RefPicList1 in the order that RefPicList1[ i ] and DepthRefPicList1[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l1_active_minus1. The variable DepthCurrPic is specified to be the upsampled decoded sample array of the depth view component of the view component pair for which the texture view component is the current texture view component when textureFirstFlag is equal to 0 and it is specified to be the upsampled decoded sample array of the depth view component of the view component pair for which the texture view component is the texture view component of base view when textureFirstFlag is equal to 1.

The partitioning of a macroblock is specified by mb_type. Each macroblock partition is referred to by mbPartIdx. When the macroblock partitioning consists of partitions that are equal to sub-macroblocks, each sub-macroblock can be further partitioned into sub-macroblock partitions as specified by sub_mb_type[ mbPartIdx ]. Each sub-macroblock partition is referred to by subMbPartIdx. When the macroblock partitioning does not consist of sub-macroblocks, subMbPartIdx is set equal to 0.

The following steps are specified for each macroblock partition or for each sub-macroblock partition.

The functions MbPartWidth( ), MbPartHeight( ), SubMbPartWidth( ), and SubMbPartHeight( ) describing the width and height of macroblock partitions and sub-macroblock partitions are specified in Tables Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found.

When nal_unit_type is equal to 21, DepthFlag is equal to 0, textureFirstFlag is equal to 1 and either dmvp_flag or seq_view_synthesis_flag is equal to 1, dvMBX is set equal to zero when CurrMbAddr is equal to first_mb_in_slice, and subclause 8.2.1.8 is invoked.

The range of the macroblock partition index mbPartIdx is derived as follows:

–	If mb_type is equal to B_Skip or B_Direct_16x16 or if both mb_type is equal to P_Skip and MbVSSkipFlag is equal to 1, mbPartIdx proceeds over values 0..3.

–	Otherwise (mb_type is not equal to B_Skip or B_Direct_16x16), mbPartIdx proceeds over values 0..NumMbPart( mb_type ) − 1.

For each value of mbPartIdx, the variables partWidth and partHeight for each macroblock partition or sub-macroblock partition in the macroblock are derived as follows:

–	If mb_type is not equal to P_8x8, P_8x8ref0, B_Skip, B_Direct_16x16, or B_8x8, subMbPartIdx is set equal to 0 and the following applies:

–	If mb_type is equal to P_Skip and MbVSSkipFlag is equal to 1, partWidth and partHeight are derived as:

partWidth = 8			(J-8)

partHeight = 8			(J-9)

–	Otherwise, partWidth and partHeight are derived as:

partWidth = MbPartWidth( mb_type ) 		(J-10)

partHeight = MbPartHeight( mb_type ) 		(J-11)

–	Otherwise, if mb_type is equal to P_8x8 or P_8x8ref0, or mb_type is equal to B_8x8 and sub_mb_type[ mbPartIdx ] is not equal to B_Direct_8x8, subMbPartIdx proceeds over values 0..NumSubMbPart( sub_mb_type[ mbPartIdx ] ) − 1, and partWidth and partHeight are derived as:

partWidth = SubMbPartWidth( sub_mb_type[ mbPartIdx ] ) 	(J-12)

partHeight = SubMbPartHeight( sub_mb_type[ mbPartIdx ] ). 	(J-13)

–	Otherwise (mb_type is equal to B_Skip or B_Direct_16x16, or mb_type is equal to B_8x8 and sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8), subMbPartIdx proceeds over values 0..3, and partWidth and partHeight are derived as:

partWidth = 4 		(J-14)

partHeight = 4 		(J-15)

When ChromaArrayType is not equal to 0, the variables partWidthC and partHeightC are derived as:

partWidthC = partWidth / SubWidthC		(J-16)
partHeightC = partHeight / SubHeightC		(J-17)

Let the variable MvCnt be initially set equal to 0 before any invocation of clause 8.2.1, 8.2.2 or 8.2.1 for the macroblock.

The Inter prediction process for a macroblock partition mbPartIdx and a sub-macroblock partition subMbPartIdx consists of the following ordered steps:

- The following applies:

–	If nal_unit_type is equal to 21 and DepthFlag is equal to 0, the following applies:

–	If mb_alc_skip_flag is equal to 1 or mb_alc_flag is equal to 1, subclause 8.2.2 is invoked.

–	Otherwise, if dmvp_flag is equal to 1, subclause 8.2.1 is invoked.

–	Otherwise, subclause 8.4.1 is invoked.

–	Otherwise, the derivation process for motion vector components and reference indices as specified in clause 8.2.1 is invoked.

Inputs to the processes in subclauses 8.2.1, 8.2.2 and 8.2.1 are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx.

Outputs of this processes in subclauses 8.2.1, 8.2.2 and 8.2.1 are:

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1

–	reference indices refIdxL0 and refIdxL1

–	prediction list utilization flags predFlagL0 and predFlagL1

–	the sub-macroblock partition motion vector count subMvCnt.

- The variable MvCnt is incremented by subMvCnt.

- When (weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (weighted_bipred_idc is greater than 0 and (slice_type % 5) is equal to 1), the following applies:

–	If mb_alc_skip_flag is equal to 1 or mb_alc_flag is equal to 1, subclause 8.2.3 is invoked.

–	Otherwise, the derivation process for prediction weights as specified in clause Error: Reference source not found is invoked.

Inputs to this processes in subclauses 8.4.3 and 8.2.3 are:

–	reference indices refIdxL0 and refIdxL1

–	prediction list utilization flags predFlagL0 and predFlagL1

Outputs of this processes in subclauses 8.4.3 and 8.2.3 are variables for weighted prediction logWDC, w0C, w1C, o0C, o1C with C being replaced by L and, when ChromaArrayType is not equal to 0, Cb and Cr.

- When (nal_unit_type is equal to 21 and depth_weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (depth_weighted_bipred_flag is equal to 1 and (slice_type % 5) is equal to 1), the derivation process for prediction weights in depth-range-based weighted prediction in subclause  is invoked.

- The decoding process for Inter prediction samples as specified in clause Error: Reference source not found is invoked.

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx,

–	variables specifying partition width and height for luma and chroma (if available), partWidth, partHeight, partWidthC (if available), and partHeightC (if available),

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1,

–	variables for weighted prediction logWDC, w0C, w1C, o0C, o1C with C being replaced by L and, when ChromaArrayType is not equal to 0, Cb and Cr.

Outputs of this process are inter prediction samples (pred); which are a (partWidth)x(partHeight) array predPartL of prediction luma samples and when ChromaArrayType is not equal to 0 two (partWidthC)x(partHeightC) arrays predPartCr, and predPartCb of prediction chroma samples, one for each of the chroma components Cb and Cr.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made:

MvL0[ mbPartIdx ][ subMbPartIdx ] = mvL0		(J-18)

MvL1[ mbPartIdx ][ subMbPartIdx ] = mvL1		(J-19)

RefIdxL0[ mbPartIdx ] = refIdxL0		(J-20)

RefIdxL1[ mbPartIdx ] = refIdxL1		(J-21)

PredFlagL0[ mbPartIdx ] = predFlagL0		(J-22)

PredFlagL1[ mbPartIdx ] = predFlagL1		(J-23)

The location of the upper-left sample of the macroblock partition relative to the upper-left sample of the macroblock is derived by invoking the inverse macroblock partition scanning process as described in clause Error: Reference source not found with mbPartIdx as the input and ( xP, yP ) as the output.

The location of the upper-left sample of the sub-macroblock partition relative to the upper-left sample of the macroblock partition is derived by invoking the inverse sub-macroblock partition scanning process as described in clause Error: Reference source not found with subMbPartIdx as the input and ( xS, yS ) as the output.

The macroblock prediction is formed by placing the macroblock or sub-macroblock partition prediction samples in their correct relative positions in the macroblock, as follows.

The variable predL[ xP + xS + x, yP + yS + y ] with x = 0..partWidth − 1, y = 0..partHeight − 1 is derived by:

predL[ xP + xS + x, yP + yS + y ] = predPartL[ x, y ] 		(J-24)

When ChromaArrayType is not equal to 0, the variable predC with x = 0..partWidthC − 1, y = 0..partHeightC − 1, and C in predC and predPartC being replaced by Cb or Cr is derived by:

predC[ xP / SubWidthC + xS / SubWidthC + x, yP / SubHeightC + yS / SubHeightC + y ] = predPartC[ x, y ]
						(J-25)

      - Derivation process for motion vector components and reference indices

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx.

Outputs of this process are:

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1,

–	a motion vector count variable subMvCnt.

For the derivation of the variables mvL0 and mvL1 as well as refIdxL0 and refIdxL1, the following applies:

–	If mb_type is equal to P_Skip, the following applies:

–	If MbVSSkipFlag is equal to 0, the following applies:

–	If nal_unit_type is equal to 21 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the depth-based derivation process for luma motion vectors for skipped macroblock in P and SP slices in subclause 8.2.1.1 is invoked with the output being the luma motion vectors mvL0 and refernece indices refIdxL0, and predFlagL0 is set equal to 1.

–	Otherwise (nal_unit_type is not equal to 21 or DepthFlag is equal to 1 or dmvp_flag is equal to 0), the derivation process for luma motion vectors for skipped macroblock in P and SP slices in subclause 8.4.1.1 is invoked with the output being the luma motion vectors mvL0 and reference indices refIdxL0, and predFlagL0 is set equal to 1.

–	Otherwise (MbVSSkipFlag is equal to 1), the derivation process for luma motion vectors for VSP skipped macroblock in P and SP slices in subclause 8.2.1.2 is invoked with mbPartIdx as input and with the output being the luma motion vectors mvL0 and reference indices refIdxL0, and predFlagL0 is set equal to 1.

–	mvL1 and refIdxL1 are marked as not available and predFlagL1 is set equal to 0. The motion vector count variable subMvCnt is set equal to 1.

–	Otherwise, if mb_type is equal to B_Skip or B_Direct_16x16 or sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, the following applies.

–	The variable vspFlag is specified as follows:

vspFlag = !( ( mb_type  = =  B_Skip  &&  MbVSSkipFlag  = =  0 )  | |  
	( ( mb_type  = =  B_Direct_16x16  | |  sub_type[ mbPartIdx ]  = =  B_Direct_8x8 )  &&  
	!mb_direct_type_flag ) ) 		(J-26)

–	If vspFlag is equal to 0 and nal_unit_type is equal to 21 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the depth-based derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.2.1.3 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

–	Otherwise, if vspFlag is equal to 0 either nal_unit_type is not equal to 21 or DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.4.1.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

–	Otherwise (vspFlag is equal to 1), the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.2.1.6 is invoked.

–	Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX, and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, the following applies:

- The variables refIdxLX and predFlagLX are derived as follows:

–	If MbPartPredMode( mb_type, mbPartIdx ) or SubMbPredMode( sub_mb_type[ mbPartIdx ] ) is equal to Pred_LX or to BiPred,

refIdxLX = ref_idx_lX[ mbPartIdx ]		(J-27)

predFlagLX = 1		(J-28)

–	Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = −1		(J-29)

predFlagLX = 0		(J-30)

- The motion vector count variable subMvCnt is set equal to predFlagL0 + predFlagL1.

- The variable currSubMbType is derived as follows:

–	If the macroblock type is equal to B_8x8, currSubMbType is set equal to sub_mb_type[ mbPartIdx ].

–	Otherwise (the macroblock type is not equal to B_8x8), currSubMbType is set equal to "na".

- The following applies:

–	If VspRefLXFlag[ mbPartIdx ] is equal to 0 or both VspRefLXFlag[ mbPartIdx ] is equal to 1 and bvsp_flag_lX[ mbPartIdx ] is equal to 0, the following applies:

–	When predFlagLX is equal to 1 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the derivation process for luma motion vector prediction in subclause 8.2.1.7 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

–	When predFlagLX is equal to 1 and either DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

–	The luma motion vectors are derived by

mvLX[ 0 ] = mvpLX[ 0 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 0 ]	(J-31)

mvLX[ 1 ] = mvpLX[ 1 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 1 ]	(J-32)

–	Otherwise (VspRefLXFlag[ mbPartIdx ] is equal to 1 and bvsp_flag_lX[ mbPartIdx ] is equal to 1), the following apply:

–	if textureFirstFlag is equal to 0, the function DisparityForBlock is invoked with depthPic equal to DepthCurrPic, ( x1, y1 ) equal to the location of the top-left sample of macroblock partition mbPartIdx, and the width and height of macroblock partition mbPartIdx (assigned to blWidth and blHeight, respectively), srcViewId equal to view_id and refViewId equal to the view_id of refIdxLX as inputs and the output assigned to mvLX[ 0 ] and mvLX[ 1 ] is set equal to 0

–	otherwise (textureFirstFlag is equal to 1), the function DisparityForBlock is invoked with depthPic equal to DepthCurrPic, ( x1 , y1 ) equal to sum of ( dvMBX >> 2 , 0 ) and the location of the top-left sample of macroblock partition mbPartIdx, and the width and height of macroblock partition mbPartIdx (assigned to blWidth and blHeight, respectively), srcViewId equal to view_id and refViewId equal to 0 as inputs and the output assigned to mvLX[ 0 ] and mvLX[ 1 ] is set equal to 0.

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in clause Error: Reference source not found is invoked with mvLX and refIdxLX as input and the output being mvCLX.

        - Depth-based derivation process for luma motion vectors for skipped macroblocks in P and SP slices

This process is invoked when mb_type is equal to P_Skip, nal_unit_type is equal to 21, DepthFlag is equal to 0, dmvp_flag is equal to 1 and MbVSSkipFlag is equal to 0.

Outputs of this process are:

–	the motion vector mvL0,

–	the reference index refIdxL0.

For the derivation of the motion vector mvL0 and refIdxL0 of a P_Skip macroblock type, the following ordered steps are specified:

- The process specified in subclause 8.2.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag equal to 0 as input and the output is assigned to the motion vector mvL0 and the reference index refIdxL0.

- When refIdxL0 is equal to -1, the following applies:

–	The reference index refIdxL0 is set to 0.

–	The derivation process for luma motion vector prediction in subclause 8.2.1.7 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, refIdxL0, and currSubMbType = “na” as the inputs and the output being mvL0.

        - Derivation process for luma motion vectors for VSP skipped macroblocks in P and SP slices

This process is invoked when mb_type is equal to P_Skip, nal_unit_type is equal to 21, DepthFlag is equal to 0, and MbVSSkipFlag is equal to 1.

Inputs to this process are current macroblock partition index mbPartIdx.

Outputs of this process are the motion vector mvL0 and the reference index refIdxL0.

The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

The reference index refIdxL0 for a VSP skipped macroblock is derived as the inter-view picture that appears first in RefPicList0.

The variable refViewId is set equal to the view_id of the inter-view picture refIdxL0 when textureFirstFlag is equal to 0 and is set equal to 0 when textureFirstFlag is equal to 1.

The variable shiftedX is set to (textureFirstFlag ? (dvMBX >> 2) : 0 ).

The motion vector mvL0 is derived as follows:

mvL0[ 0 ] = DisparityForBlock( DepthCurrPic, ( x1 + dbx1 + shiftedX), ( y1 + dby1 ), 8, 8, view_id, refViewId ) 	(J-33)
mvL0[ 1 ] = 0

        - Derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8

Inputs to this process are current macroblock partition index mbPartIdx and subMbPartIdx.

Outputs of this process are the reference indices refIdxL0, refIdxL1, the motion vectors mvL0 and mvL1, the motion vector count variable subMvCnt, and the prediction list utilization flags, predFlagL0 and predFlagL1.

For the derivation of output, the following ordered steps are specified:

- Let the variable currSubMbType be set equal to sub_mb_type[ mbPartIdx ].

- The process specified in subclause 8.2.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag set equal to 0 as input and the output is assigned to the motion vector mvL0 and the reference index refIdxL0.

- The process specified in subclause 8.2.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag set equal to 1 as input and the output is assigned to the motion vector mvL1 and the reference index refIdxL1.

- When both reference indices refIdxL0 and refIdxL1 are equal to -1, the following applies:

–	The reference index refIdxL0 is set equal to 0.

–	The derivation process for luma motion vector prediction in subclause 8.2.1.7 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, refIdxLX (with X being 0 or 1), and currSubMbType as the inputs and the output being mvLX.

        - Derivation process for the disparity vector and the inter-view reference

Inputs to this process are the listSuffixFlag and depth reference view component depthPic, the location of a top-left sample ( dbx1, dby1 ) of a partition if textureFirstFlag is equal to 0.

Outputs of this process are a picture InterViewPic, an offset vector dV and a variable interViewAvailable.

Set interViewAvailable equal to 0.

The following applies to derive an inter-view reference picture or inter-view only reference picture InterViewPic:

for( cIdx = 0;cIdx<=num_ref_idx_l0_active_minus1  && !interViewAvailable; cIdx ++)
	if ( view order index of RefPicList0[ cIdx ] is equal to 0) {
		InterViewPic = RefPicList0[ cIdx ]		(J-34)
		interViewAvailable = 1
	}

When interViewAvailable is equal to 1, the following steps apply in order.

–	The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 0 as input and with reference indices refIdxCandL0[ i ] and the motion vectors mvCandL0[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

–	The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 1 as input and with reference indices refIdxCandL1[ i ] and the motion vectors mvCandL1[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

–	The variable dV is derived as specified by the following ordered steps:

–	Set DvAvailable[ i ] and mvCand[ i ] with i equal to 0, 1, and 2 corresponding to neighbouring partitions A, B, and C, respectively, as follows

for( i = 0; i < 3; i++ )
	if( view order index of RefPicList0[ refIdxCandL0[ i ] ] is equal to 0 ) {
		DvAvailable[ i ] = 1
		mvCand[ i ] = mvCandL0[ i ]
	} else if( view order index of RefPicList1[ refIdxCandL1[ i ] ] is equal to 0 ) {	(J-35)
		DvAvailable[ i ] = 1
		mvCand[ i ] = mvCandL1[ i ]
	} else
		DvAvailable[ i ] = 0

–	If DvAvailable[ 0 ] + DvAvailable[ 1 ] + DvAvailable[ 2 ] is equal to 1, the following applies:

dV[ 0 ] = mvCand[ i ][ 0 ]
dV[ 1 ] = mvCand[ i ][ 1 ]		(J-36)

–	Otherwise, the following steps apply in order:

–	If textureFirstFlag is equal to 0, the variable maxDepth is specified as follows:

maxDepth = INT_MIN 
for( j = 0; j < partHeight; j += ( partHeight – 1 ) )
	for( i = 0; i < partWidth; i += ( partWidth – 1 ) )	(J-37)
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth )
			maxDepth = depthPic[ dbx1 + i, dby1 + j ]

–	If textureFirstFlag is equal to 0, the variable dispVector is specified as follows:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the InterViewPic )

dispVector[ 0 ] = Disparity( maxDepth, currIndex, refIndex )	(J-38)
dispVector[ 1 ] = 0

–	For each value of i equal to 0, 1, and 2, when DvAvailable[ i ] is equal to 0, mvCand[ i ] is set to (textureFirstFlag ? (dvMBX, 0) : dispVector).

–	Each component of the variable dV is derived as follows:

dV[ 0 ] = Median( mvCand[ 0 ][ 0 ], mvCand[ 1 ][ 0 ], mvCand[ 2 ][ 0 ] )
dV[ 1 ] = Median( mvCand[ 0 ][ 1 ], mvCand[ 1 ][ 1 ], mvCand[ 2 ][ 1 ] )	(J-39)

        - Derivation process for the inter-view motion vector in inter-view reference

Inputs to this process are mbPartIdx, subMbPartIdx, and listSuffixFlag.

Outputs of this process are the motion vector mvCorrespond and the reference index refIdxCorrespond.

Inter-view reference picture InterViewPic and an offset vector dV are derived as specified by the following ordered steps:

–	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

–	When textureFirstFlag is equal to 0, the inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

–	When textureFirstFlag is equal to 0, the inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

–	The process specified in subclause 8.2.1.4 is invoked with listSuffixFlag and when textureFirstFlag is equal to 0, depthPic set to DepthCurrPic, dbx1 set to x1 + dx1 + dx2, dby1 set to y1 + dy1 + dy2 as inputs and InterViewPic, an offset vector dV and an variable interViewAvailable as outputs.

The refIdxCorrespond and mvCorrespond are set as follows.

–	If interViewAvailable is equal to 0, refIdxCorrespond is set to -1, and mvCorrespond [ 0 ] and mvCorrespond [ 1 ] are both set to 0.

–	Otherwise, the following step applies in order.

–	The variable luma4x4BlkIdx is derived as  (4 * mbPartIdx + subMbPartIdx).

–	The inverse 4x4 luma block scanning process as specified in subclause Error: Reference source not found is invoked with luma4x4BlkIdx as the input and ( x, y ) as the output. In addition, ( xCorrespond, yCorrespond ) is set equal to ( x + ( dV[ 0 ] >> 4 ), y + ( dV[ 1 ] >> 4 ) ) and mbAddrCorrespond is set equal to ( ( CurrMbAddr / PicWidthInMbs ) + ( dV[ 1 ] >> 6 ) ) * PicWidthInMbs + ( CurrMbAddr % PicWidthInMbs ) + ( dV[ 0 ] >> 6 ).

–	Set mbTypeCorrespond to the syntax element mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic. When mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, subMbTypeCorrespond is set to be the syntax element sub_mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic.

–	Set mbPartIdxCorrespond to the macroblock partition index of the corresponding partition and subMbPartIdxCorrespond to the sub-macroblock partition index of the corresponding sub-macroblock partition. The derivation process for macroblock and sub-macroblock partition indices as specified in subclause Error: Reference source not found is invoked with the luma location equal to ( xCorrespond, yCorrespond ), the macroblock type equal to mbTypeCorrespond, and when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, the list of sub-macroblock types subMbTypeCorrespond as the inputs and the outputs are the macroblock partition index mbPartIdxCorrespond and the sub-macroblock partition index subMbPartIdxCorrespond.

–	The motion vector mvCorrespond and the reference index refIdxCorrespond are derived as follows.

–	If the macroblock mbAddrCorrespond is coded in as Intra prediction mode, both components of mvCorrespond are set equal to 0 and refIdxCorrespond is set equal to –1.

–	Otherwise (the macroblock mbAddrCorrespond is not coded as Intra prediction mode), the prediction utilization flags predFlagLXCorrespond is set equal to PredFlagLX[ mbPartIdxCorrespond ], the prediction utilization flag of the macroblock partition mbAddrCorrespond\mbPartIdxCorrespond of the picture InterViewPic. In addition, the following applies.

–	When predFlagLXCorrespond is equal to 1, the mvCorrespond and the reference index refIdxCorrespond are set equal to MvLX[ mbPartIdxCorrespond ][ subMbPartIdxCorrespond ] and RefIdxLX[ mbPartIdxCorrespond ], respectively, which are the motion vector mvLX and the reference index refIdxLX that have been assigned to the (sub-)macroblock partition mbAddrCorrespond\mbPartIdxCorrespond\subMbPartIdxCorrespond inside the picture InterViewPic.

        - Derivation process for luma motion vectors for VSP skipped/direct macroblocks in B slices

Inputs to this process are current macroblock partition index mbPartIdx.

Outputs of this process are the motion vector mvL0, mvL1 and the reference index refIdxL0, refIdxL1.

The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

The reference index refIdxLX for a VSP skipped/direct macroblock is derived as the inter-view reference component that appears first in the reference picture list X, with X being replaced by 0 or 1. When there is no inter-view picture in the reference picture list X, refIdxLX is set equal to 0.

The variable refViewIdX is set equal to the view_id of the inter-view reference component refIdxLX if textureFirstFlag is equal to 0 and is set equal to 0 if textureFirstFlag is equal to 1.

The variable shiftedX is set to (textureFirstFlag ? (dvMBX >> 2) : 0).

The motion vector mvLX, with X being replaced by 0 or 1, is derived as follows.

mvLX[ 0 ] = DisparityForBlock( DepthCurrPic, ( x1 + dbx1 + shiftedX ), ( y1 + dby1 ), 8, 8, view_id, refViewIdX ) 	(J-40)
mvLX[ 1 ] = 0

        - Derivation process for luma motion vector prediction

Inputs to this process are:

–	the macroblock partition index mbPartIdx,

–	the sub-macroblock partition index subMbPartIdx,

–	the reference index of the current partition refIdxLX (with X being 0 or 1),

–	the variable currSubMbType.

Output of this process is the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The specifications of subclause 8.4.1.3 apply with the following changes.

–	The following additional sentence is applied

If refIdxLX is not equal to refIdxLXN for any N = A, B, or C and X equal to 0 or 1, the following applies:

mbAddrN\mbPartIdxN\subMbPartIdxN is marked as not available
refIdxLXN = -1
mvLXN 0 ] = 0		(J-41)
mvLXN[ 1 ] = 0

after the following paragraph in subclause 8.4.1.3:

The derivation process for the neighbouring blocks for motion data in subclause Error: Reference source not found is invoked with mbPartIdx, subMbPartIdx, currSubMbType, and listSuffixFlag = X (with X being 0 or 1 for refIdxLX being refIdxL0 or refIdxL1, respectively) as the input and with mbAddrN\mbPartIdxN\subMbPartIdxN, reference indices refIdxLXN and the motion vectors mvLXN with N being replaced by A, B, or C as the output.

–	The following additional sentence is applied

–	Otherwise, if refIdxLX is a reference index to an inter-view reference component or an inter-view only reference component, the depth-based derivation process for median luma motion vector prediction in subclause 8.2.1.7.1 is invoked with mbAddrN\mbPartIdxN\subMbPartIdxN, mvLXN, refIdxLXN with N being replaced by A, B, or C, and refIdxLX as the inputs and the output is assigned to the motion vector predictor mvpLX.

–	Otherwise, if refIdxLX is a reference index to a reference picture which is not an inter-view reference component or an inter-view only reference component, the depth-based derivation process for median luma temporal motion vector prediction in subclause 8.2.1.7.2 is invoked with mbAddrN\mbPartIdxN\subMbPartIdxN, mvLXN, refIdxLXN with N being replaced by A, B, or C, and refIdxLX as the inputs and the output is assigned to the motion vector predictor mvpLX.

after the following paragraph in subclause 8.4.1.3

–	Otherwise, if MbPartWidth( mb_type ) is equal to 8, MbPartHeight( mb_type ) is equal to 16, mbPartIdx is equal  mvpLX = mvLXCto 1, and refIdxLXC is equal to refIdxLX, the motion vector predictor mvpLX is derived by:

mvpLX = mvLXC

          - Depth-based derivation process for median luma motion vector prediction

Inputs to this process are:

–	the neighbouring partitions mbAddrN\mbPartIdxN\subMbPartIdxN (with N being replaced by A, B, or C),

–	the motion vectors mvLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference indices refIdxLXN (with N being replaced by A, B, or C) of the neighbouring partitions,,

–	the reference index refIdxLX of the current partition.

Output of this process is the motion vector prediction mvpLX.

When either partition mbAddrN\mbPartIdxN\subMbPartIdxN is not available or refIdxLXN is not equal to refIdxLX, mvLXN is derived as specified by the following:

If textureFirstFlag is equal to 0, the following steps apply in order:

1.	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

2.	The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

3.	The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

4.	The modification process of inter-view motion vector in median luma motion vector prediction as specified in subclause 8.2.1.7.1.1 is invoked with depthPic being equal to DepthRefPicList0[ refIdxL0 ], mbx1 being equal to x1 and mby1 being equal to y1 as inputs and the output is assigned to the motion vector mvLXN.

Otherwise, mvLXN is set equal to ( dvMBX , 0 ).

Each component of the motion vector prediction mvpLX is given by the median of the corresponding vector components of the motion vector mvLXA, mvLXB, and mvLXC:

mvpLX[ 0 ] = Median( mvLXA[ 0 ], mvLXB[ 0 ], mvLXC[ 0 ] ) 	(J-42)
mvpLX[ 1 ] = Median( mvLXA[ 1 ], mvLXB[ 1 ], mvLXC[ 1 ] )	(J-43)

            - Modification process for inter view motion vector in median luma motion vector prediction

Inputs to this process are

–	depth reference view component depthPic,

–	the location of a top-left sample ( mbx1, mby1 ) of the current macroblock.

Outputs of this process are:

–	the motion vector mv.

Let refViewId be the view_id value of depthPic.

The variables partHeight and partWidth are derived as follows:

partHeight = 16 >> reduced_resolution_flag

partWidth = 16 >> reduced_resolution_flag

The variable mv is derived as follows:

mv[ 0 ] = DisparityForBlock( depthPic, ( mbx1 >> reduced_resolution_flag ), 
	( mby1 >> 	reduced_resolution_flag ), partWidth, partHeight, view_id, refViewId ) 	(J-44)
mv[ 1 ] = 0

          - Depth-based derivation process for median luma temporal motion vector prediction

Inputs to this process are:

–	the neighbouring partitions mbAddrN\mbPartIdxN\subMbPartIdxN (with N being replaced by A, B, or C),

–	the motion vectors mvLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference indices refIdxLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference index refIdxLX of the current partition.

Output of this process is the motion vector prediction mvpLX.

When either partition mbAddrN\mbPartIdxN\subMbPartIdxN is not available or refIdxLXN is not equal to refIdxLX, mvLXN is derived as specified by the following ordered steps:

1.	When textureFirstFlag is equal to 0, the inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

2.	When textureFirstFlag is equal to 0, the inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

3.	When textureFirstFlag is equal to 0, the inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

4.	When textureFirstFlag is equal to 0, the process specified in subclause 8.2.1.7.2.1 is invoked with depthPic set to DepthCurrPic, mbx1 set to x1, mby1 set to y1 and listSuffixFlag as input and InterViewPic, an offset vector dV and an variable interViewAvailable as outputs.

5.	When textureFirstFlag is equal to 1, dv is set equal to ( dvMBX , 0 ) and an variable InterViewAvailable equal to IntViewAvaMB.

6.	The refIdxCorrespond and mvCorrespond are set as follows.

–	If interViewAvailable is equal to 0, refIdxCorrespond is set to -1, and mvCorrespond[ 0 ] and mvCorrespond[ 1 ] are both set to 0.

–	Otherwise, the following steps apply in order.

–	The variable luma4x4BlkIdx is set equal to ( 4 * mbPartIdx + subMbPartIdx ).

–	The inverse 4x4 luma block scanning process as specified in subclause Error: Reference source not found is invoked with luma4x4BlkIdx as the input and ( x, y ) as the output. In addition, ( xCorrespond, yCorrespond ) is set equal to ( x + ( dV[ 0 ] >> 4 ), y + ( dV[ 1 ] >> 4 ) ) and mbAddrCorrespond is set equal to ( ( CurrMbAddr / PicWidthInMbs ) + ( dV[ 1 ] >> 6 ) ) * PicWidthInMbs + ( CurrMbAddr % PicWidthInMbs ) + ( dV[ 0 ] >> 6 ).

–	Set mbTypeCorrespond to the syntax element mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic. When mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, subMbTypeCorrespond is set to be the syntax element sub_mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic.

–	Set mbPartIdxCorrespond to the macroblock partition index of the corresponding partition and subMbPartIdxCorrespond to the sub-macroblock partition index of the corresponding sub-macroblock partition. The derivation process for macroblock and sub-macroblock partition indices as specified in subclause Error: Reference source not found is invoked with the luma location equal to ( xCorrespond, yCorrespond ), the macroblock type equal to mbTypeCorrespond, and when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, the list of sub-macroblock types subMbTypeCorrespond as the inputs and the outputs are the macroblock partition index mbPartIdxCorrespond and the sub-macroblock partition index subMbPartIdxCorrespond.

–	The motion vector mvCorrespond and the reference index refIdxCorrespond are derived as follows.

–	If the macroblock mbAddrCorrespond is coded in as Intra prediction mode, both components of mvCorrespond are set equal to 0 and refIdxCorrespond is set equal to –1.

–	Otherwise (the macroblock mbAddrCorrespond is not coded as Intra prediction mode), the prediction utilization flags predFlagLXCorrespond is set equal to PredFlagLX[ mbPartIdxCorrespond ], the prediction utilization flag of the macroblock partition mbAddrCorrespond\mbPartIdxCorrespond of the picture InterViewPic. In addition, the following applies.

–	When predFlagLXCorrespond is equal to 1, the mvCorrespond and the reference index refIdxCorrespond are set equal to MvLX[ mbPartIdxCorrespond ][ subMbPartIdxCorrespond ] and RefIdxLX[ mbPartIdxCorrespond ], respectively, which are the motion vector mvLX and the reference index refIdxLX that have been assigned to the (sub-)macroblock partition mbAddrCorrespond\mbPartIdxCorrespond\subMbPartIdxCorrespond inside the picture InterViewPic.

7.	The motion vectors mvLXN is derived as follows.

–	If refIdxCorrespond is equal to refIdxLX, the following applies:

mvLXN[ 0 ] = mvCorrespond[ 0 ]
mvLXN[ 1 ] = mvCorrespond[ 1 ]		(J-45)

–	Otherwise, the following applies:

mvLXN[ 0 ] = 0
mvLXN[ 1 ] = 0

8.	The following applies for the derivation of mvpLX[ 0 ] and mvpLX[ 1 ]:

mvpLX[ 0 ] = Median( mvLXA[ 0 ], mvLXB[ 0 ], mvLXC[ 0 ] )	(J-46)
mvpLX[ 1 ] = Median( mvLXA[ 1 ], mvLXB[ 1 ], mvLXC[ 1 ] )	(J-47)

            - Derivation process for the disparity vector and the inter-view reference

Inputs to this process are depth reference view component depthPic, the location of a top-left sample ( mbx1, mby1 ) of the current macroblock and the listSuffixFlag.

Outputs of this process are a picture InterViewPic, an offset vector dV and a variable interViewAvailable

The variable interViewAvailable is set equal to 0.

The variables partHeight and partWidth are derived as follows:

partHeight = 16 >> reduced_resolution_flag		(J-48)
partWidth = 16 >> reduced_resolution_flag		(J-49)

The following applies to derive an inter-view reference picture or inter-view only reference picture, InterViewPic, with X set to 1 when listSuffixFlag is 1 or 0 otherwise:

for( cIdx = 0;cIdx<num_ref_idx_l0_active_minus1 + 1 && !interViewAvailable; cIdx ++)
	if ( view order index of RefPicList0[ cIdx ] is equal to 0) {
		InterViewPic = RefPicList0[ cIdx ]		(J-50)
		interViewAvailable = 1
	}

When interViewAvailable is equal to 1, the variable refViewId is set equal to view_id of InterViewPic and the variable dV is set equal to the output of function DisparityForBlock( depthPic, ( mbx1 >> reduced_resolution_flag ), ( mby1 >> reduced_resolution_flag ), partWidth, partHeight, view_id, refViewId ).

        - Macroblock-level neighbouring block based disparity vector derivation process

Input to this process is a macroblock currMB.

Let the variable availableDvFlag equal to 0, ( xP, yP ) be equal to the output of the subclause 6.4.2.1 (the location of upper-left luma sample for currMB partition 0).

- The variable dvMBCur is derived as specified by the following ordered steps:

- For X from 0 to 1, the following ordered steps apply:

      - When availableDvFlag is equal to 0 and RefPicListY[ 0 ] is available (with Y equal to 1-X), the following applies:

        - Set RefPicListCol0 is the reference picture list 0 of RefPicListY[ 0 ].

        - The reference index and the motion vector corresponding to RefPicListCol0 of the block covering ( xP + 16,  yP + 16 ) is set to MvL0N and RefIdxL0N respectively.

        - When the view order index of RefPicListCol0[ RefIdxL0N ] is unequal to view_idx or RefPicListCol0[ RefIdxL0N ] is a view synthetic reference component, and MvL0N[ 0 ] is unequal to 0, dvMBCur is set to MvL0N[ 0 ] and availableDvFlag is set to 1.

- For each N being A, B, C and D, as specified in subclause Error: Reference source not found with the refIdxL0N and mvL0N being the reference index and motion vector corrponding to reference picture list 0, the following applies:

      - When availableDvFlag is equal to 0, the following applies

        - RefPicList0[ refIdxL0N ] is available and its the view order index is unequal to the view_idx or RefPicList0[ refIdxL0N ] is the view synthetic reference component, and MvL0N[ 0 ] is unequal to 0, dvMBCur is set to mvL0N[ 0 ] and availableDvFlag is set to 1.

- When availableDvFlag is equal to 0, dvMBCur is set to dvMBX.

- dvMBCur is further refined by the following steps if seq_view_synthesis_flag is equal to 1:

  - Let refDepthPic being the decoded sample array of the depth view component of the view component pair for which the texture view component is the one of base view, PicWidthInSamplesL and PicHeightInSamplesL be the picture width and height for the luma component of refDepthPic.

  - The variables currIndex and refIndex are derived by:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of refDepthPic)

  - The variables dX, dY, and maxDepth are derived by:

maxDepth = INT_MIN
for( j = 0; j < 16; j+=15 )
	for( i = 0; i < 16; i+=15)
	{

dX = Clip3(0, PicWidthInSamplesL, (xP+i)+ (dvMBCur[ 0 ] >> 2)) >> reduced_resolution_flag)

dY = Clip3(0, PicHeightInSamplesL, (yP+j)+( dvMBCur[ 1 ] >> 2)) >> reduced_resolution_flag)
		if( refDepthPic[ dX, dY ] > maxDepth ) 
			maxDepth = refDepthPic[ dX, dY ]

}

  - dvMBCur is set to Disparity( maxDepth, currIndex, refIndex ).

- dvMBX is set to dvMBCur.

- Derivation of prediction weights in depth-range-based weighted prediction

For (nal_unit_type is equal to 21 and depth_weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (depth_weighted_bipred_flag is equal to 1 and (slice_type % 5) is equal to 1), the derivation process for prediction weights in depth-range-based weighted prediction described in this subclause is invoked.

Inputs to this process are

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction utilization flags predFlagL0 and predFlagL1.

Outputs of this process are variables for weighted prediction logWDC, w0C, o0C, w1C, o1C, where C being replaced by L (luma).

The variables index, refIndex, dpsId1 and dpsId2 are derived as follows:

index = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )	(J-51)
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the view of the reference picture)	(J-52)
dpsId1 = dps_id of the current picture		(J-53)
dpsId2 = dps_id of the reference picture		(J-54)

When predFlagL0 is equal to 1, the following is applied.

–	The derivation process for a single prediction weight in depth-range-based weighted prediction specified in subclause 8.2.1.9 is invoked with zN1 equal to ZNear[ dpsId1, index ], zF1 equal to ZFar[ dpsId1, index ], zN2 equal to ZNear[ dpsId2, refIndex ], zF2 equal to ZFar[ dpsId2, refIndex ], and X equal to 0.

When predFlagL1 is equal to 1 and depth_weighted_bipred_flag is equal to 1, the following is applied.

–	The derivation process for a single prediction weight in depth-range-based weighted prediction specified in sub-clause 8.2.1.9 is invoked with zN1 equal to ZNear[ dpsId1, index], zF1 equal to ZFar[ dpsId1, index  ], zN2 equal to ZNear[ dpsId2, refIndex ], zF2 equal to ZFar[ dpsId2, refIndex ], and X equal to 1.

        - Derivation of weight and offset parameteres

–	The variable wXC is calculated through the following steps.

- scale_w is set to 8;

- Calculate variable W_factora as follows:

temp1 = ZF2 - ZN2 
temp2 = ZF2
x = ( temp1 + ( temp2 >> 1 ) ) / temp2
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1		(J-55)
W_factora = (x<<scale_w)
W_factora += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2

- Calculate variable W_factorb as follows:

temp1 = ZF1
temp2 = ZF1 – ZN1
x = ( temp1 + ( temp2 >> 1 ) ) / temp2
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1		(J-56)
W_factorb = (x<<scale_w)
W_factorb += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2

- Calculate variable W_factorc as follows:

temp1 = ZN1
temp2 = ZN2
x = ( temp1 + ( temp2 >> 1 ) ) / temp2		(J-57)
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1
W_factorc = (x<<scale_w)
W_factorc += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2

- Calculate variable wX:

wX= (W_factora*W_factorb*W_factorc + (1<<(scale_w*3- logWD-1)) ) >> (scale_w*3- logWD)
wX=Clip(-127,128,wX)		(J-58)

–	The variable oX is calculated through the following derivation process.

- scale_o is set to 8

- Calculate variable O_factora as follows:

O_factora=( (ZN1<<(scale_o))+( ZF2>>1) ) / ZF2;	(J-59)

- Calculate variable O_factorb:

temp1 = ZF1- ZF2
temp2 = ZF1- ZN1
sign= (temp1 < 0) ? -1 : 1
x = ( temp1 + sign * ( temp2 >> 1 ) ) / temp2		(J-60)
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1
O_factorb = (x<<scale_o)
O_factorb += ( ((temp1 - x * temp2) << scale_o) + sign * (temp2 >> 1) ) / temp2

- Calculate variable o0:

oX=( O_factora*O_factorb + (1<<(scale_o*2-8-1)) ) >> (scale_o*2-8)	(J-61)
oX =Clip(-127,128, oX)

      - Derivation process for motion vector components and reference indices for inter prediction for adaptive luminance compensation

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx.

Outputs of this process are:

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1,

–	a motion vector count variable subMvCnt.

The motion vector count variable subMvCnt is set equal to 1.

Set interViewAvailable equal to 0.

The reference index refIdxL0 for a skipped macroblock is derived as:

for( cIdx = 0;cIdx<=num_ref_idx_l0_active_minus1 && !interViewAvailable; cIdx ++)
	if (RefPicList0[ cIdx ] and the current view component have different values of view oder index) {
		refIdxL0 = cIdx		(J-62)
		interViewAvailable = 1
	}

If dmvp_flag is equal to 1 and mb_alc_skip_flag is equal to 1, subclause 8.2.1.7 is invoked and mvL0 is set equal to mvpL0, the output of subclause 8.2.1.7.

Otherwise, if dmvp_flag is equal to 1 and mb_alc_ flag is equal to 1, subclause 8.2.1 is invoked for derivation of mvL0.

Otherwise, subclause 8.4.1 is invoked and mvL0 is set equal to mvpL0, the output of subclause 8.4.1.

      - Derivation process for prediction weights in adaptive luminance compensation

Inputs to this process are:

–	reference index refIdxL0

–	the luma sample array of the selected reference picture refPicL0L.

–	the current partition given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx

–	Luma4x4BlkIdx

–	the width and height partWidth, partHeight of this partition in luma-sample units

–	a luma motion vector mvL0 given in quarter-luma-sample units

–	array cSL containing already constructed luma samples prior to deblocking filter process.

Outputs of this process are:

–	variables for weighted prediction of the current partition logWDC, W0C, W1C,O0C, O1C, with C being replaced by L and, when ChromaArrayType is not equal to 0, Cb and Cr.

The variables W1C , O1C are derived as follows for C equal to L, Cb or Cr:

logWDC = 0			(J-63)
W1C = 0				(J-64)
O1C = 0 				(J-65)

The variables W0C, O0C are derived as follows for C if equal to Cb or Cr:

logWDC = 0			(J-66)
W0C = 1				(J-67)
O0C=0				(J-68)

When C is equal to L for luma samples, subclauses 8.2.3.1 through 8.2.3.5 are invoked sequentially to derive LogWDL, W0C, and O0C.

        - Defining of coordinates and sizes of a luma block to be predicted

Let ( xM, yM ) be equal to the output of the subclause 6.4.1 (the location of upper-left luma sample for the current macroblock with address mbAddr relative to the upper-left sample of the picture).

Let ( xP, yP ) be equal to the output of the subclause 6.4.2.1 (the location of upper-left luma sample for the macroblock partition mbPartIdx).

Let ( xB, yB ) be equal to the output of the subclause 6.4.2.2 (the location of upper-left luma sample for the 4x4 luma block defined by Luma4x4BlkIdx that can be 0...15) relative to the top-left sample of the sub-macroblock.

The variables xT, yT, xBlockWidth, yBlockHeight are set as follows:

–	xT is set equal to xM + xP;

–	yT is set equal to yM +yP;

–	xBlockWidth is set equal to MbPartWidth( mb_type );

–	yBlockHeight is set equal to MbPartHeight( mb_type );

If one or more of the following conditions are true, W0C is set to 1 and logWDC is set to 15.

–	( mvL0[ 0 ] + ( ( xT– 1 ) << 2 ) ) is smaller than 0;

–	( mvL0[ 1 ] + ( ( yT – 1 ) << 2 ) ) is smaller than 0;

–	( mvL0[ 0 ] + (( xT + xBlockWidth ) << 2 ) ) is greater than or equal to ( PicWidthInSamplesL << 2 );

–	( mvL0[ 1 ] + ( ( yT + yBlockHeight) << 2 ) ) is greater than or equal to ( PicHeightInSamplesL << 2 ).

Otherwise LRef, URef, LRec, URec sample values are derived as it is specified in 8.2.3.2 and 8.2.3.3 followed by calculation of variables NeighborRefSum, NeighborSum and W0L, O0L specified in the subclause 8.2.3.4 and 8.2.3.5 correspondently.

        - Deriving of left and up reference samples of the current block

LRec and URec blocks belong to an (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process.

Each luma sample LRec[ 0, yL ]  (0 <= yL < yBlockHeight) is specified as follows:

LRec[ 0, yL ] = cSL[ xT – 1, yT + yL ]		(J-69)

Each luma sample URec[ xL, 0 ]  (0 <= xL < xBlockWidth) is specified as follows:

URec[ xL, yL ] = cSL[ xT + xL, yT – 1]		(J-70)

        - Deriving of left and up reference samples of the reference block

For each luma sample location (0, yL) such as: 0<=yL< yBlockHeight inside LRef block, sample value LRef[0, yL] is derived by the following ordered steps:

1.	The variables xIntL, yIntL, xFracL, and yFracL are derived by:

xIntL = xT + ( mvL0[ 0 ] >> 2 ) - 1		(J-71)
yIntL = yT + ( mvL0[ 1 ] >> 2 ) + yL		(J-72)
xFracL= mvL0[ 0 ] & 3		(J-73)
yFracL= mvL0[ 1 ] & 3		(J-74)

2.	LRef[ 0, yL ] sample is derived as an output of the process specified in the 8.4.2.2.1 sub-clause with ( xIntL, yIntL), ( xFracL, yFracL ) and refPicL0L given as input.

For each luma sample location ( xL, 0 ) such as: 0<=xL< xBlockWidth inside URef block sample value URef[ xL, 0 ] is derived by the following ordered steps:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by:

xIntL = xT + ( mvL0[ 0 ] >> 2 ) + xL		(J-75)
yIntL = yT + ( mvL0[ 1 ] >> 2 ) - 1		(J-76)
xFracL = mvL0[ 0 ] & 3		(J-77)
yFracL= mvL0[ 1 ] & 3		(J-78)

- URef[ xL, yL ] sample is derived as an output of the process specified in the 8.4.2.2.1 sub-clause with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicL0L given as input.

        - Deriviation of NeighborRefSum and NeighborSum

Both NeighborRefSum and NeighborSum are set to 1 and further calculated as follows:

for( j=0; j< yBlockHeight;j++ )
	if ( Abs( LRec[ 0, j ] - LRef[ 0,j ] ) < 31 ) {
		NeighborRefSum = NeighborRefSum + LRef[ 0, j ]	(J-79)
		NeighborSum = NeighborSum + LRec[ 0, j ]
	}

for( i=0; i< xBlockWidth; i++ )
	if ( Abs( URec[ i, 0 ] - URef[ i ,0 ] ) < 31 ) {
		NeighborRefSum = NeighborRefSum + URef[ i, 0 ]	(J-80)
		NeighborSum = NeighborSum + URec[ i, 0 ]
	}

        - Deriviation of prediction weights

O0L is set to 0. W0L and LogWDL are derived as follows:

If (NeighborSum>>4) is equal to (NeighborRefSum >>4), W0L is set to 1 and LogWDL is set to 0.

Otherwise, LogWDL is set to 15 and W0L is equal to ( ( 1<< LogWDL ) * NeighborSum + ( NeighborRefSum >> 1 ) ) / NeighborRefSum.

    - Specification of bitstream subsets

The specifications of subclause I.8.5 apply.

    - Decoding process for depth range parameters

This process is invoked for decoding of the depth_ranges( ) syntax structure in sequence parameter set 3D-AVC extension and for decoding of a depth parameter set RBSP.

Inputs of this process are the variables obtained from parsing the active depth range parameter set RBSP:

–	the variables relative to the closest depth: ZNearSign[dps_id, i ], ZNearExp[dps_id, i ], ZNearMantissa[dps_id, i ], ZNearManLen[dps_id, i ];

–	the variables relative to the farthest depth: ZFarSign[dps_id, i ], ZFarExp[dps_id, i ], ZNearMantissa[dps_id, i ], ZNearManLen[dps_id, i ]

where i is the index to the order of views for 3DV acquisition parameters.

Outputs of this process are

–	the closest depth values ZNear[ dps_id, i ] and the respective variables respective variables ZNearSign[ dps_id, i ], ZNearExponent[ dps_id, i ], ZNearMantissa[ dps_id, i ], ZNearManLen[ dps_id, i ];

–	the farthest depth values ZFar[ dps_id, i ] and the respective variables respective variables ZFarSign[ dps_id, i ], ZFarExponent[ dps_id, i ], ZFarMantissa[ dps_id, i ], ZFarManLen[ dps_id, i ].

Some of the views for which the 3DV acquisition parameters are specified may not be present in the coded video sequence.

The output variables x in Table J-3 are derived as follows from the respective variables f, s, e, n, and v indicated in Table J-3.

–	If f is equal to 0 in the depth parameter set RBSP, f in the active sequence parameter set 3D-AVC extension shall be equal to 1 and x[ dps_id, i ] = x[ 0, i ], s[ dps_id, i ] = s[ 0, i ], e[ dps_id, i ] = e[ 0, i ], and n[ dps_id, i ] = n[ 0, i ] for all values of i.

–	Otherwise (f is equal to 1 in the depth parameter set RBSP), the variable x computed as follows for [ dps_id, i ] where i is index to the order of views for 3DV acquisition parameters:

–	If 0 < e < 127, x = (−1)s * 2e−31 * (1 + n ÷ 2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

- Table J-3 – Association between depth parameter variables and syntax elements.

| x | f | s | E | n | v |
| --- | --- | --- | --- | --- | --- |
| ZNear | z_near_flag | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | z_far_flag | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |

  - Parsing process

The specifications in clause Error: Reference source not found apply. Additionally, the following modifications are specified and added.

    - Alternative CABAC parsing process for slice data and macroblock layer in depth extension

Subclause 9.1.1specifies the initialisation process for the alternative CABAC parsing process for slice data and macroblock layer when nal_unit_type is equal to 21 and avc_3d_extension_flag is equal to 1.

Subclause 9.1.2 specifies the binarization process for the alternative CABAC parsing process for slice data and macroblock layer when nal_unit_type is equal to 21 and avc_3d_extension_flag is equal to 1.

Subclause 9.1.3 specifies the decoding process flow for the alternative CABAC parsing process for slice data and macroblock layer when nal_unit_type is equal to 21 and avc_3d_extension_flag is equal to 1.

      - Initialisation process

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Table J-5 contains the values of the variables n and m used in the initialisation of context variables that are assigned to syntax elements mb_vsskip_flag, and mb_direct_type_flag. Table J-6 contains the values of the variables n and m used in the initialisation of context variables that are assigned to syntax element mb_skip_run_type, mb_alc_skip_flag, mb_alc_flag and mb_vsp_flag. The initialization proces for two variables pStateIdx and valMPS is the same as other syntax elements, as defined in Equation 9-81. For all other syntax elements in subclause 7.3.5 the initialisation process of context variables as specified in subclause Error: Reference source not found applies.

In Table J-4, the ctxIdx for which initialisation is needed for I, P and B slices are listed. Also listed is the table number that includes the values of m and n needed for the initialisation.

- Table J-4 – Association of ctxIdx and syntax elements in the initialisation process

| Syntax element |  | Table |  | Slice type |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | I |  | P |  | B |
| mb_vsskip_flag | Table J-5 |  |  |  | 1031..1033 |  | 1034..1036 |  |
| mb_direct_type_flag | Table J-5 |  |  |  |  |  | 1037..1039 |  |
| mb_skip_run_type | Table J-6 |  |  |  |  |  | 1040 |  |
| mb_alc_skip_flag | Table J-6 |  |  |  | 1041 .. 1043 |  |  |  |
| mb_alc_flag | Table J-6 |  |  |  | 1044 .. 1046 |  |  |  |
| mb_vsp_flag | Table J-6 |  |  |  | 1047..1049 |  | 1050..1052 |  |

- Table J-5 – Values of variables m and n for ctxIdx from 1031 to 1039

| Value of cabac_init_idc | Initialisation variables | ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1031 | 1032 | 1033 | 1034 | 1035 | 1036 | 1037 | 1038 | 1039 |
| 0 | m | 23 | 23 | 21 | 18 | 9 | 29 | -46 | -20 | 1 |
|  | n | 33 | 2 | 0 | 64 | 43 | 0 | 127 | 104 | 67 |
| 1 | m | 22 | 34 | 16 | 26 | 19 | 40 | -45 | -15 | -4 |
|  | n | 25 | 0 | 0 | 34 | 22 | 0 | 127 | 101 | 76 |
| 2 | m | 29 | 25 | 14 | 20 | 20 | 29 | -32 | -22 | -2 |
|  | n | 16 | 0 | 0 | 40 | 10 | 0 | 127 | -117 | 74 |

- Table J-6 – Values of variables m and n for ctxIdx from 1040 to 1052

| Initialisation variables | ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1040 | 1041 | 1042 | 1043 | 1044 | 1045 | 1046 | 1047 | 1048 | 1049 | 1050 | 1051 | 1052 |
| m | 18 | 14 | 14 | 10 | 14 | 14 | 10 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 4 | 27 | 52 | 4 | 27 | 52 | 64 | 64 | 64 | 64 | 64 | 64 |

      - Binarization process

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element, maxBinIdxCtx, ctxIdxOffset, and bypassFlag.

Associated with each binarization or binarization part of a syntax element is a specific value of the context index offset (ctxIdxOffset) variable and a specific value of the maxBinIdxCtx variable as given in Table J-7.

The use of the DecodeBypass process and the variable bypassFlag is derived as follows:

- If no value is assigned to ctxIdxOffset for the corresponding binarization or binarization part in Table J-7 labelled as "na", all bins of the bit strings of the corresponding binarization or of the binarization prefix/suffix part are decoded by invoking the DecodeBypass process as specified in subclause 9.3.3.2.3. In such a case, bypassFlag is set equal to 1, where bypassFlag is used to indicate that for parsing the value of the bin from the bitstream the DecodeBypass process is applied.

- Otherwise, for each possible value of binIdx up to the specified value of maxBinIdxCtx given in Table J-7, a specific value of the variable ctxIdx is further specified in subclause 9.3.3. bypassFlag is set equal to 0.

The possible values of the context index ctxIdx are in the range 1031 to 1052, inclusive. The value assigned to ctxIdxOffset specifies the lower value of the range of ctxIdx assigned to the corresponding binarization or binarization part of a syntax element.

- Table J-7 – Syntax elements and associated types of binarization, maxBinIdxCtx, and ctxIdxOffset

| Syntax element | Type of binarization | maxBinIdxCtx | ctxIdxOffset |
| --- | --- | --- | --- |
| mb_vsskip_flag (P slices only) | FL, cMax=1 | 0 | 1031 |
| mb_vsskip_flag (B slices only) | FL, cMax=1 | 0 | 1034 |
| mb_direct_type_flag (B slices only) | FL, cMax=1 | 0 | 1037 |
| mb_skip_run_type (B slices only) | FL, cMax=1 | 0 | 1040 |
| mb_alc_skip_flag | FL, cMax=1 | 0 | 1041 |
| mb_alc_flag | FL, cMax=1 | 0 | 1044 |
| mb_vsp_flag (P slices only) | FL, cMax=1 | 0 | 1047 |
| mb_vsp_flag (B slices only) | FL, cMax=1 | 0 | 1050 |

      - Decoding process flow

Input to this process is a binarization of the requested syntax element, maxBinIdxCtx, bypassFlag and ctxIdxOffset as specified in subclause Error: Reference source not found.

Output of this process is the value of the syntax element.

This process specifies how each bit of a bit string is parsed for each syntax element.

After parsing each bit, the resulting bit string is compared to all bin strings of the binarization of the syntax element and the following applies.

–	If the bit string is equal to one of the bin strings, the corresponding value of the syntax element is the output.

–	Otherwise (the bit string is not equal to one of the bin strings), the next bit is parsed.

While parsing each bin, the variable binIdx is incremented by 1 starting with binIdx being set equal to 0 for the first bin.

The parsing of each bin is specified by the following two ordered steps:

1.	Given binIdx, maxBinIdxCtx and ctxIdxOffset, ctxIdx is derived as specified in subclause  9.1.3.1.

2.	Given ctxIdx, the value of the bin from the bitstream as specified in subclause Error: Reference source not found is decoded.

        - Derivation process for ctxIdx

Inputs to this process are binIdx, maxBinIdxCtx and ctxIdxOffset.

Output of this process is ctxIdx.

Table J-8 shows the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxOffset values for the syntax mb_vsskip_flag, mb_direct_type_flag, mb_alc_skip_flag, mb_alc_flag and mb_vsp_flag.

The ctxIdx to be used with a specific binIdx is the sum of ctxIdxOffset and ctxIdxInc, which is found in Table J-8. When more than one value is listed in Table J-8 or Error: Reference source not found for a binIdx, the assignment process for ctxIdxInc for that binIdx is further specified in the subclauses given in parenthesis of the corresponding table entry.

All entries in Table J-8 labelled with "na" correspond to values of binIdx that do not occur for the corresponding ctxIdxOffset.

- Table J-8 – Assignment of ctxIdxInc to binIdx for the ctxIdxOffset values related to the syntax elements mb_vsskip_flag, mb_direct_type_flag, mb_alc_skip_flag, mb_alc_flag and mb_vsp_flag

| ctxIdxOffset | binIdx |  |
| --- | --- | --- |
|  | 0 | >= 1 |
| 1031 | 0,1,2 (subclause 9.1.3.2) | na |
| 1034 | 0,1,2 (subclause 9.1.3.2) | na |
| 1037 | 0,1,2 (subclause 9.1.3.3) | na |
| 1041 | 0,1,2 (subclause 9.1.3.4) | na |
| 1044 | 0,1,2 (subclause 9.1.3.5) | na |
| 1047 | 0,1,2 (subclause 9.1.3.6) | na |
| 1050 | 0,1,2 (subclause 9.1.3.6) | na |

        - Derivation process of ctxIdxInc for the syntax element mb_vsskip_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is not available or mb_vsskip_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 0.

–	Otherwise (mbAddrN is available and mb_vsskip_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(J-82)

        - Derivation process of ctxIdxInc for the syntax element mb_direct_type_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If any of the following conditions is true, condTermFlagN is set to 0:

–	If mbAddrN is not available,

–	Both mb_vsskip_flag and mb_direct_type_flag for the macroblock mbAddrN is equal to 0

–	Otherwise, condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(J-83)

        - Derivation process of ctxIdxInc for the syntax element mb_alc_skip_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause 6.4.11.1 is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is available and mb_alc_skip_flag or mb_alc_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 1.

–	Otherwise (mbAddrN is not available or mb_alc_skip_flag and mb_alc_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 0.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		 (J-84)

        - Derivation process of ctxIdxInc for the syntax element mb_alc_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause 6.4.10.1 is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is available and mb_alc_skip_flag or mb_alc_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 1.

–	Otherwise (mbAddrN is not available or mb_alc_skip_flag and mb_alc_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 0.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(J-85)

        - Derivation process of ctxIdxInc for the syntax element mb_vsp_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is not available or mb_vsp_flag for the macroblock mbAddrN is equal to 0, condTermFlagN is set equal to 1.

–	Otherwise (mbAddrN is available and mb_vsp_flag for the macroblock mbAddrN is equal to 1), condTermFlagN is set equal to 0.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(J-86)

  - Profiles and levels

The specifications in Annex I apply. Additional profiles and specific values of profile_idc are specified in the following.

The profiles that are specified in subclause J.10.1 are also referred to as the profiles specified in Annex J.

    - Profiles

All constraints for picture parameter sets that are specified in the following are constraints for picture parameter sets that become the active picture parameter set or an active view picture parameter set inside the bitstream. All constraints for 3D-AVC sequence parameter sets that are specified in the following are constraints for 3D-AVC sequence parameter sets that become the active 3D-AVC sequence parameter set or an active view 3D-AVC sequence parameter set inside the bitstream.

      - Enhanced Multiview Depth High profile

Bitstreams conforming to the Enhanced Multiview Depth High profile shall obey the following constraints:

–	The base view bitstream as specified in subclause I.8.5.3 shall obey all constraints of the High profile specified in subclause Error: Reference source not found and all active sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1,

–	profile_idc is equal to 100.

–	Only I, P, and B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	When the profile_idc is equal to 139 in a sequence parameter set, the frame_mbs_only_flag in the sequence parameter set shall be equal to 1.

–	3D-AVC sequence parameter sets shall have chroma_format_idc equal to 0 only.

–	3D-AVC sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	3D-AVC sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	3D-AVC sequence parameter sets shall have qpprime_y_zero_transform_bypass_flag equal to 0 only.

–	For each access unit, the value of level_idc for all active view 3D-AVC sequence parameter set RBSPs shall be the same as the value of level_idc for the active 3D-AVC sequence parameter set RBSP.

–	The level constraints specified for the Enhanced Multiview Depth High profile in subclause I.10.2 shall be fulfilled.

Conformance of a bitstream to the Enhanced Multiview Depth High profile is indicated by profile_idc being equal to 139.

Decoders conforming to the Enhanced Multiview Depth High profile at a specific level shall be capable of decoding all bitstreams in which both of the following conditions are true:

- All active 3D-AVC sequence parameter sets have one or more of the following conditions fulfilled:

–	profile_idc is equal to 139,

–	profile_idc is equal to 138,

–	profile_idc is equal to 128,

–	profile_idc is equal to 118 and constraint_set5_flag is equal to 1,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

- All active 3D-AVC sequence parameter sets have one or more of the following conditions fulfilled:

–	level_idc or (level_idc and constraint_set3_flag) represent a level less than or equal to the specific level,

–	level_idc[ i ] or (level_idc[ i ] and constraint_set3_flag) represent a level less than or equal to the specific level.

    - Levels

The specification of subclause I.10.2 apply.

      - Level limits common to 3D High profiles

The specification of subclause I.10.2.1 apply.

      - Profile specific level limits

- In bitstreams conforming to the Enhanced Multiview Depth High profile, 3D-AVC sequence parameter sets shall have frame_mbs_only_flag equal to 1 for the levels specified in Error: Reference source not found.

  - Byte stream format

The specifications in Annex Error: Reference source not found apply.

  - 3D-AVC hypothetical reference decoder

The specifications in Annex Error: Reference source not found apply with substituting 3D-AVC sequence parameter set for MVC sequence parameter set.

  - 3D-AVC SEI messages

The specifications in Annex D and clause I.13 together with the extensions and modifications specified in this subclause apply.

    - SEI message syntax

      - Constrained depth parameter set identifier SEI message syntax

| constrained_depth_parameter_set_identifier( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| max_dps_id | 5 | ue(v) |
| max_dps_id_diff | 5 | ue(v) |
| } |  |  |

      - Depth view information SEI message syntax

| depth_view_info ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| residual_depth_flag | 5 | u(1) |
| } |  |  |
| } |  |  |

    - SEI message semantics

      - Constrained depth parameter set identifier SEI message semantics

When present, this message shall be associated with an IDR access unit. The semantics of the message are valid for the current coded video sequence. A constrained depth parameter set identifier SEI message indicates that depth_parameter_set_id and dps_id values present in the coded video sequence are constrained as specified below.

NOTE 1: When a constrained depth parameter set identifier SEI message is present, decoders are able to conclude losses of depth parameter set NAL units.

max_dps_id plus 1 specifies the maximum allowed depth_range_parameter_set_id value.

max_dps_id_diff specifies the value range of depth_range_parameter_set_id values marked as “used”. max_dps_id_diff * 2 shall be less than max_dps_id.

For each coded slice, the following applies.

–	For the first coded slice of an IDR access unit, MaxUsedDpsId is set equal to “no value”, UsedDpsIdSet is an empty set of values, and all depth range parameter set RBSPs included in the bitstream or made available to the decoding process through external means prior to the access unit containing the IDR picture are marked unavailable.

–	When MaxUsedDpsId is not equal to “no value”, the value of dps_id of the slice header is constrained and the variable updateMaxUsedDpsIdFlag is set as follows.

–	If dps_id is equal to 0, the variable updateMaxUsedDpsIdFlag is set equal to 0.

–	Otherwise, the variable zeroBasedDpsId is equal to dps_id – 1 and dps_id is constrained so that zeroBasedDpsId fulfills the following:

–	If MaxUsedDpsId >= max_dps_id_diff and MaxUsedDpsId <= max_dps_id – max_dps_id_diff, zeroBasedDpsId is in the range of MaxUsedDpsId – max_dps_id_diff to MaxUsedDpsId + max_dps_id_diff, inclusive. The variable updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId ).

–	Otherwise, if MaxUsedDpsId < max_dps_id_diff, zeroBasedDpsId is either in the range of 0 to MaxUsedDpsId + max_dps_id_diff, inclusive, or in the range of max_dps_id – ( max_dps_id_diff – MaxUsedDpsId – 1 ) to max_dps_id, inclusive. The updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId  &&  zeroBasedDpsId < max_dps_id – ( max_dps_id_diff – MaxUsedDpsId – 1 ) ).

–	Otherwise (MaxUsedDpsId > max_dps_id – max_dps_id_diff), zeroBasedDpsId is either in the range of 0 to max_dps_id_diff – ( max_dps_id – MaxUsedDpsId ) – 1, inclusive, or in the range of MaxUsedDpsId – max_dps_id_diff to max_dps_id, inclusive. The updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId  ||  zeroBasedDpsId < max_dps_id_diff – ( max_dps_id – MaxUsedDpsId ) ).

–	When updateMaxUsedDpsIdFlag is equal to 1, depth range parameter set RBSPs are marked as unavailable as follows.

prevMinUsedDpsId = MaxUsedDpsId – max_dps_id_diff
if( prevMinUsedDpsId < 0 )
	prevMinUsedDpsId += max_dps_id
minUsedDpsId = dps_id – 1 – max_dps_id_diff
if( minUsedDpsId < 0 )
	minUsedDpsId += max_dps_id		(J-87)
i = prevMinUsedDpsId
do {
	Mark depth range parameter set RBSP with depth_range_parameter_set_id equal to i + 1, 
	if present, as unavailable.
	i = ( i + 1 ) % ( max_dps_id + 1 )
} while( i != minUsedDpsId )

–	When updateMaxUsedDpsIdFlag is equal to 1 or MaxUsedDpsId is equal to “no value”, the following applies.

–	MaxUsedDpsId is set equal to dps_id – 1.

–	If MaxUsedDpsId is greater than or equal to max_dps_id_diff, UsedDpsIdSet is set to the values in the range of MaxUsedDpsId – max_dps_id_diff to MaxUsedDpsId, inclusive.

–	Otherwise (MaxUsedDpsId is smaller than max_dps_id_diff), UsedDpsIdSet is set to the values in the range of 0 to MaxUsedDpsId, inclusive, and in the range of max_dps_id – ( max_dps_id_diff – MaxUsedDpsId ) to max_dps_id, inclusive.

Any depth parameter set RBSP included in the bitstream or made available to the decoding process through external means and having depth_parameter_set_id equal to any value included in UsedDpsIdSet + 1 has the same content as the previous depth parameter set RBSP included in the bitstream or made available to the decoding process through external means having the same depth_range_parameter_id value.

NOTE 2: If a slice header includes a dps_id value marked as unavailable, a decoder should infer an unintentional loss of a depth parameter set with depth_parameter_set_id value equal to the dps_id value of the slice header.

      - Depth view information SEI message semantics

The depth view information SEI message specifies different types of 3D format [Ed. (MH): Unclear what is a “3D format”. At the moment there is only one depth_type specified, so the SEI message specifies only one type of a “3D format” as far as I can see]. The depth view information SEI message, when present, shall be associated with an IDR access unit. The information signaled in the SEI message applies to all the access units in the coded video sequence.

depth_type specifies the type of the depth view information. depth_type equals to 0 specifies the 3D format is Global View and Depth (GVD). The values of depth_type larger than 0 are unspecified [Ed. (MH): I’d think values greater than 0 should rather be reserved for future use].

NOTE: When depth_type is equal to 0, tools for inter-view prediction and depth dependent texture coding should be disabled. [Ed. (MH): The note should rather refer to how to set syntax element values in the SPS or which profile should be used.]

The texture and depth view components of the non-base view contain up to four residual views [Ed. (MH) Term “residual view” should be defined], each represents a difference picture [Ed. (MH) difference between what?] that is horizontally and vertically subsampled [Ed. (MH): Sub-sampling process should be explicitly specified. For example, is there filtering involved in the sub-sampling or just pure decimation? From which positions of the sampling grid is sub-sampling derived?] relative to the corresponding texture or depth base view components.

depth_type equal to 0 indicates that N = num_residual_texture_views_minus1 + 2 input texture views and associated depth views (N=2, 3, 4 and up to 5) are converted into Global texture view (one base texture view and one residual texture view) and Global depth view (one base depth view or one base depth view plus one residual depth view).

Base texture view (B) is the input from central camera (camera ID=3) as shown in Figure J-1. In the case N=2, camera ID=2 and 3 are used. In the case N=3, camera ID=2, 3 and 4 are used. In the case N=4, camera ID=1, 2, 3 and 4 are used. In the case N=5, camera ID=1, 2, 3, 4 and 5 are used. If the number of input texture views is even, base texture view is the first right view [Ed. (MH) phrase “the first right view” is unclear.] from the center (V3). The other N-1 input texture views are converted to N-1 half-size sub-residual texture views (R-x) and packed in the residual texture view from top-left (R-2), to bottom-left (R-4), top-right (R-1) and bottom-right (R-5) order. Their top-left co-ordinates in the residual view of width=W and height=H is shown in Table J-9. The residual texture view represents occluded area or out-of-frame area of the base texture view when it is projected to the N-1 input texture views [Ed. (MH): It is unclear which process “is projected” exactly refers to. Are there normative impacts of this projection into semantics of the SEI message?] The number of sub-residual texture views is signalled by num_residual_texture_views_minus1.

num_residual_texture_views_minus1 plus1 specifies the number of sub-residual texture views packed in the residual texture view. num_residual_texture_views_minus1 is in the range of 0 to 3, inclusive.

1     2     3     4     5

R-1   R-2    B    R-4   R-5

camera ID

GVD view

- Figure J-1 – Relation between camera ID and GVD texture views

- Table J-9 – Top-left corner co-ordinates of GVD residual views packed in a Residual view of width=W and height=H.

| R-1 | (W/2, 0) |
| --- | --- |
| R-2 | (0, 0) |
| R-4 | (0, H/2) |
| R-5 | (W/2, H/2) |

Base depth view (B) is a median [Ed. (MH): “median” should be more clearly specified. Is it a sample-wise median between collocated samples?] of all input depth views and is associated with the base texture view (camera ID=3) as shown in Figure J-2. Residual depth view may exist or may not exist. residual_depth_flag equal to 1specifies the existence of a residual depth view. The other N-1 input depth views are converted to N-1 half-size sub-residual depth views (R-x) [Ed. (MH): the generation of “sub-residual depth view” should be specified] and packed in the residual depth view in the same positions as the residual texture view shown in Table J-9. Each sub-residual depth view associates with its sub-residual texture view [Ed. (MH) the generation of “sub-residual texture view” should be specified].

When residual_depth_flag = 0 and the number of input views is even (2 or 4), base depth view (B) is the center of input views (camera ID=2.5).

Since the input depth views are originally sub-sampled to the half-size of the input texture views, the sub-residual depth views are sub-sampled to the quarter-size of input texture views. The residual depth view represents occluded area or out-of-frame area of the base depth view when it is projected to the N-1 input depth views.

residual_depth_flag equal to 1 specifies that the number of packed depth views in a depth view component of non-base view is equal to the number of packed texture views in a texture view component. residual_depth_flag equal to 0 specifies there is only one depth view.

1     2     3     4     5

(R-1)  (R-2)  B   (R-4)  (R-5)

camera ID

GVD depth

1       2       3       4

2.5

B

camera ID

(even)

GVD depth

wo/res.

- Figure J-2 – Relation between camera ID and GVD depth views

Base depth view is generated by projecting N-input depth views to the base depth view (camera ID=3) and taking the median of them. [Ed. (MH): which process is used in the projection?]

Residual depth view is generated by encoding and decoding the base depth view and then projecting it to the other input depth views (camera ID=1,2, 4 and 5), and then extracting the un-covered area by the projected base depth view to sub-residual depth views and then packing them in a residual depth view.

Residual texture view is generated by projecting the base texture view to the other input texture views (camera ID=1, 2, 4 and 5) by the decoded base depth view, and then extracting the un-covered area in the other input texture views (camera ID=1, 2, 4 and5) by the projected base texture view into sub-residual texture views and then packing them in a residual texture view in the case of residual_depth_flag = 0.

In the case of residual_depth_flag = 1, residual texture view is generated by encoding and decoding the residual depth view, and then projecting the base texture view to the other input texture views (camera ID=1, 2, 4 and 5) by the decoded base depth view and the decoded residual depth views, and then extracting the un-covered area in the other input texture views (camera ID=1, 2, 4 and5) by the projected base texture view into sub-residual texture views and then packing them in a residual texture view.

  - Video usability information

The specifications in Annex I.14 apply.

--End