| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0137 |
| --- | --- |

| Title: | AHG 4: Post Processing in interlace coding mode for MVC+D |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Chieh Lin, Fang-Chu Chen, Chao-Hsiung Hung | Tel: Email: | +886-3-5915758 jacklin@itri.org.tw fcchen@itri.org.tw chhung@itri.org.tw |
| Source: | Industrial Technology Research Institute |  |  |

_____________________________

# Abstract

This document describes the issues and corresponding solutions for the non-mandatory post processing tools in ATM software in interlace coding mode for MVC+D.

# Introduction

Before the fourth JCT-3V meeting, the interlace coding tool had been integrated into ATM and was released as ATM7.2 [1]. The previous efforts on interlace integration were mostly concentrated on the mandatory tools. However, we find that there are anomalies in the two post processing tools in interlace coding mode. In addition, the post dilation filter that is not a normative part of MVC+D [2] specification text is automatically applied in the current ATM. For conformance testing, ATM needs an option to enable/disable the depth dilation filter.

# Issues with ATM8.0r1 and its solutions

### Issues with current post processing tool in interlace coding mode

The current ATM software contains two post processing tools [3]. One is up-sampling and the other is a post dilation filter. The post processing tools are applied to the decoded depth after the normative coding process and generate the output depth (Fig.2.1).

Fig. 2.1 Post processing in the current ATM

In interlace coding mode, the decoded top and bottom fields are interlaced in a decoded depth (Fig.2.2). Current post processing tools use neighbouring pixels to perform up-sampling and dilation filtering. Since the decoded depth is composed of interlaced top and bottom fields, using vertical neighbouring pixels from different fields for post processing leads to design inconsistency in interlace coding mode.

Fig. 2.2 A decoded depth with interlaced top and bottom fields

In the current CTC [4], the depth is down-sampled by a factor of 2. It results in a quarter size of the resolution compared to the coded texture views. After a successful depth decoding progress, the depth is up-sampled when the size of depth is lower than the texture resolution. The up-sampling tool uses vertical adjacent pixels to convert the decoded depth from the coding resolution to the original resolution. For example, the decoded depth with a resolution of 512×384 will be up-sampled to the original spatial resolution of 1024×768. However, the decoded depth contains the interlaced top and bottom fields that are not separated before up-sampling in interlace coding mode. Without proper separating, the up-sampler will use the neighbouring pixels from different fields that leads to design inconsistency. In addition, the post dilation filter takes the neighbouring pixels from different fields to perform filtering in the current design in interlace coding mode. This issue is very similar to the up-sampling one in the post process part; therefore, it will not be elaborated on in the document.

### ITRI’s solution for post processing tool in interlace coding mode

To avoid using the vertical neighbouring pixel from different fields for up-sampling in interlace coding mode, the interlaced decoded depth needs to be extracted to top and bottom fields first. Then, the top and bottom fields of depth are up-sampled separately. After up-sampling, a re-assemble process is required to re-generate the output results with interlaced top and bottom fields. Figure 2.3 gives an illustration of the proposed process. The procedure to resolve the interlace coding issue with choosing the vertical neighbour pixels in the current post dilation filtering design is similar to what’s that used in the up-sampling tool. Before applying the post dilation filter in interlace coding mode, the depth should be separated into top and bottom field parts in advance. After that, the post dilation filter is applied to both the top and bottom field individually. The last step is to re-assemble the depth with the interlaced top and bottom field part. (Figure 2.4)

## Options for post dilation filter

In the fourth meeting, it was agreed to provide an option in configuration files to enable/disable the up-sampling tool. A “NormalizeDepth” option was added in ATM 8.0. However, the post dilation filter is always applied before writing out the results to the output files. Even if the “NormalizeDepth” option is off, the post dilation filter is still applied to the decoded depth without up-sampling which leads to the incorrect results for conformance testing. Since the post dilation filter is not a normative part of MVC+D specification text, ATM should be able to provide an option in the encoding/decoding configuration files to turn it on/off. Therefore, a “PostDilation” option is added into the encoding/decoding configuration files. If “PostDilation” is set to 1, the post dilation filter is enabled. Otherwise, the post dilation filtering is off.

## Issue with inconsistent simulation results in different platform

In previous interlace integration, it reported that the simulation results of Class A sequences between Windows and Linux platform were not match perfectly in interlace coding mode. We observed that the mismatches were caused by incorrect DPB frames assignment that is inherited from old JM. Therefore, we provide patches to resolve the simulation results mismatches among different platforms for interlace coding on Class A sequences. Moreover, we also provide a patch to correct the depth PSNR values in interlace coding mode when the NDR tool is ON.

Fig. 2.3 Proposed up-sampling process in interlace coding mode

Fig. 2.4 Proposed post dilation filtering in interlace coding mode

# The interlace coding configuration for ATM

For interlace coding, four configuration files are provided to the software. Note that the “ViewSynRDO” option needs to be disabled in interlace coding mode. The usage guide of interlace coding is as follows:

The four configuration files designated for interlace coding mode are as follows:

-configs\3dv_enc_25Hz_depth_interlace_HP.cfg

-configs\3dv_enc_25Hz_texture_interlace_HP.cfg

-configs\3dv_enc_30Hz_depth_interlace_HP.cfg

-configs\3dv_enc_30Hz_texture_interlace_HP.cfg

Commands for execution of encoding:

- Example of command line for encoding progressive texture with progressive depth map:

lencod-R.exe -d "..\3dv_enc_30Hz_texture_HP.cfg" -depd "..\3dv_enc_30Hz_depth_HP.cfg" -p OutputFile="test_texture.264" -p 3DVConfigFile="..\3dv_texture_kendo.cfg" -depp 3DVConfigFile="..\3dv_depth_kendo.cfg" -p FramesToBeEncoded="300" -p QPISlice="41" -p QPPSlice="41" -p QPBSlice="41" -depp QPISlice="41" -depp QPPSlice="41" -depp QPBSlice="41" > HPC3S05R1.log

- Example of command line for encoding stereoscopic interlace texture with interlace depth map:

lencod-R.exe -d "..\3dv_enc_30Hz_texture_interlace_HP.cfg" -depd "..\3dv_enc_30Hz_depth_interlace_HP.cfg" -p OutputFile="test_texture.264" -p 3DVConfigFile="..\3dv_texture_kendo.cfg" -depp 3DVConfigFile="..\3dv_depth_kendo.cfg" -p NumberOfViews=2 -p 3DVCodingOrder="T0D0T1D1" -depp NumberOfViews=2 -p FramesToBeEncoded="300" -p QPISlice="41" -p QPPSlice="41" -p QPBSlice="41" -depp QPISlice="41" -depp QPPSlice="41" -depp QPBSlice="41" -depp ViewSynRDO="0" > HPC2S05R1.log

- Example of command line for encoding stereoscopic interlace texture with progressive depth map:

lencod-R.exe -d "..\3dv_enc_30Hz_texture_interlace_HP.cfg" -depd "..\3dv_enc_30Hz_depth_HP.cfg" -p OutputFile="test_texture.264" -p 3DVConfigFile="..\3dv_texture_kendo.cfg" -depp 3DVConfigFile="..\3dv_depth_kendo.cfg" -p NumberOfViews=2 –p 3DVCodingOrder="T0D0T1D1" -depp NumberOfViews=2 -p FramesToBeEncoded="300" -p QPISlice="41" -p QPPSlice="41" -p QPBSlice="41" -depp QPISlice="41" -depp QPPSlice="41" -depp QPBSlice="41" -depp ViewSynRDO="0"> HPC2S05R1.log

# Experiment results

The proposed technique was integrated into the ATM8.0r1. And the current version of ATM 8.0r1 supports the following cases for both encoding and decoding:

•	Case 1: Coding progressive video with progressive depth map for 3D-AVC

•	Case 2: Coding progressive video with progressive depth map for MVC+D

•	Case 3: Coding stereoscopic interlace video with interlace depth map for MVC+D

•	Case 4: Coding stereoscopic interlace video with progressive depth map for MVC+D

The experiment results for the above four cases are shown in Table 1-4. The anchor, which is used for comparison, was produced by ATM 8.0r1. From Table 1 and 2, it shows that the proposed technique does not have any influence on progressive video with progressive depth map for 3D-AVC coding (case 1) and progressive video with progressive depth map coding for MVC+D (case 2). After applying patches to resolve the cross platform mismatch issue with Class A sequences, there are small discrepancies compared to the anchor results in interlace coding mode in Table 3 and Table 4. However, there are no impacts on coding results of Class C sequences for interlace coding.

Table 1. Experiments results of progressive video with progressive depth map coding for 3D-AVC (case 1)

Table 2. Experiments results of progressive video with progressive depth map coding for MVC+D (case 2)

Table 3. Experiments results of stereoscopic interlace video with interlace depth map coding for MVC+D (case 3)

Table 4. Experiments results of stereoscopic interlace video with progressive depth map coding for MVC+D(case 4)

All the simulation results are reported on template excel files and were provided along with this text file.

# Conclusion and recommendations

This proposal proposed a post processing tool to use appropriate neighbouring pixels in interlace coding mode for MVC+D. It is reported that there is no influence on progressive coding mode but only in current interlace coding mode. In addition, the “PostDilation” option is added in the depth encoding/decoding configuration files to enable/disable the post dilation filter. Moreover, the solution of cross platform simulation results issue in interlace coding mode on Class A sequences is provided.

We recommend the adoption of the following:

- Modifying the procedure of applying the up-sampling and post dilation tool to the decoded depth in interlace coding mode

- Adding the “PostDilation” option in the configuration files

- Modifying the DPB frames assignment in interlace coding mode

# Reference

- Ching-Chieh Lin, Fang-Chu Chen, Chao-Hsiung Hung, “AHG4: Interlace coding on ATM 7.2”, 4th JCT-3V meeting, JCT3V-D0073, Incheon, Korea, Apr. 2013

- "Advanced Video Coding", ISO/IEC 14496-10 and ITU-T Rec. H.264

- Dmytro Rusanovskyy, Fang-Chu Chen, Li Zhang, Teruhiko Suzuki, "3D-AVC Test Model 6",4th JCT-3V meeting, JCT3V-D1003, Incheon, Korea, Apr. 2013

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, "Common Test Conditions of 3DV Core Experiments",4th JCT-3V meeting, JCT3V-D1100, Incheon, Korea, Apr. 2013

# Patent rights declaration(s)

Industrial Technology Research Institute may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).