| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0138 |
| --- | --- |

| Title: | AHG 12: Proposal on interlace conformance bitstream for MVC+D |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Chieh Lin, Fang-Chu Chen | Tel: Email: | +886-3-5915758 jacklin@itri.org.tw fcchen@itri.org.tw |
| Source: | Industrial Technology Research Institute |  |  |

_____________________________

# Abstract

This document proposes disabling up-sampling and post dilation filter for generating a conformance test bitstream related to interlace coding mode for MVC+D extension of AVC.

# Introduction

The current 3DV-ATM software contains two post processing tools. One is up-sampling and the other one is a post dilation filter. Before this meeting, a “NormalizeDepth” option was added in ATM 8.0 configuration files to enable/disable the up-sampling tool. However, there is no option in configuration files to turn the post dilation filter on or off. Even if the “NormalizeDepth” option is off, the post dilation filter is still automatically applied before writing out the output depth data.

# Interlace conformance bitstream for MVC+D

For conformance testing, decoded image YUV files or md5 checksum of the decoded files should be provided [1]. Since the up-sampling and the post dilation filter are not normative parts of MVC+D specification text[2], the two tools should be turned off in decoding process in interlace coding mode to generate the decoded YUV files. Therefore, the decoded image produced by normative decoding process would be written to the output YUV files without passing any post processing tools.

# Conclusion and recommendation

We recommend that the AHG to disable both the up-sampling and post dilation tools in interlace coding mode for the conformance test related to the MVC+D extensions of AVC. The software for turning off the post dilation filter is provided in contribution JCT3V-E0137 submitted to this meeting.

# Reference

- Teruhiko Suzuki, Dmytro Rusanovskyy, “Draft 3 of MVC plus depth conformance”, 4th JCT-3V meeting, JCT3V-D1006, Incheon, Korea, Apr. 2013

- "Advanced Video Coding", ISO/IEC 14496-10 and ITU-T Rec. H.264

- Ching-Chieh Lin, Fang-Chu Chen, Chao-Hsiung Hung, “AHG 4: Post Processing in interlace coding mode for MVC+D”, 5th JCT-3V meeting, JCT3V-E0137, Vienna, Austria, 27 July – 2 Aug. 2013

# Patent rights declaration(s)

Industrial Technology Research Institute may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).