| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0140_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Test Model for JCT3V-E0140 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides a proposed text of 3D-HEVC Test Model. The changes are highlighted in green.

- H.8.5.2.12	Derivation process for a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

Outputs of this process are

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

- the reference indices refIdxL0VSP and refIdxL1VSP ,

- the prediction list utilization flags predFlagL0VSP and predFlagL1VSP,

- the motion vectors mvL0VSP and mvL1VSP.

- The variable availableFlagVSP is set equal to 1, the variables predFlagL0VSP and predFlagL1VSP are set equal to 0, the variables refIdxL0VSP and refIdxL1VSP are set equal to −1 and the variable refViewAvailableFlag is set equal to 0.

- For X in the range of 0 to 1, inclusive, the following applies:

  - For i in the range of 0 to NumRefPicsLX – 1, inclusive, the following applies:

    - When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xC ][ yC ], the following applies:

        - refViewAvailableFlag = 1		(Error: Reference source not found-1)
predFlagLXVSP = 1		(Error: Reference source not found-2)
mvLXVSP = MvRefinedDisp[ xC ][ yC ]		(Error: Reference source not found-3)
refIdxLXVSP = i		(Error: Reference source not found-4)
Y = 1 – X		(Error: Reference source not found-5)

When the current slice is a B slice and refViewAvailableFlag is equal to 1, refViewAvailableFlag is set equal to 0 and the following applies:

  - For i in the range of 0 to NumRefPicsLY – 1, inclusive, the following applies.

    - When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListY[ i ] ) is not equal to RefViewIdx[ xC ][ yC ] and ViewIdx( RefPicListY[ i ] ) is not equal to ViewIdx, the following applies:

        - refViewAvailableFlag = 1		(Error: Reference source not found-6)
predFlagLYVSP = 1		(Error: Reference source not found-7)
mvLYVSP = MvRefinedDisp[ xC ][ yC ]		(Error: Reference source not found-8)
refIdxLYVSP = i		(Error: Reference source not found-9)