| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0141_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Test Model for JCT3V-E0141 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides a proposed text of 3D-HEVC Test Model. The changes are highlighted in green.

- H.8.5.4.3	Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit.

- variable nSubBlkW and nSubBlkW specifying the conversion precision of the corresponding depth samples,

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPels be an array of reconstructed depth samples of the depth view component with ViewIdx equal to depthViewIdx. The luma location (xTL, yTL) of top-left luma sample of a block in refDepPels is derived by

  - xTL = xP + ( ( mvDisp[ 0 ] + 2 ) >> 2 )	(Error: Reference source not found-1)
yTL = yP + ( ( mvDisp[ 1 ] + 2 ) >> 2 )	(Error: Reference source not found-2)

The array disparitySamples of size (nPSW)x(nPSH) is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH) –1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW) –1 ), inclusive, the following applies:

    - The variable maxDep is set equal to –1 and modified as specified in the following.

xP0 = Clip3(0, pic_width_in_luma_samples – 1, xTL + sBx * nSubBlkW
			yP0 = Clip3(0, pic_heigh_in_luma_samples – 1, yTL + sBy * nSubBlkH
			xP1 = Clip3(0, pic_width_in_luma_samples – 1, xTL + sBx * nSubBlkW + nSubBlkW – 1)
			yP1 = Clip3(0, pic_height_in_luma_samples – 1, yTL + sBy * nSubBlkH + nSubBlkH – 1)
			maxDep = Max( maxDep, refDepPels[ xP0 ][ yP0 ] )
			maxDep = Max( maxDep, refDepPels[ xP0 ][ yP1 ] )
			maxDep = Max( maxDep, refDepPels[ xP1 ][ yP0 ] )
			maxDep = Max( maxDep, refDepPels[ xP1 ][ yP1 ] )

    - The values of the array depthSamples are derived as specified in the following:

for ( yOff = 0; yOff < nSubBlkH; yOff++ )
				for( xOff = 0; xOff < nSubBlkW; xOff++ ) {
					x = sBx * nSubBlkW + xOff 
					y = sBy * nSubBlkH + yOff 
					disparitySamples[ x ][ y ] = DepthToDisparityB[ refViewIdx ][ maxDep ]
				}