| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0141 |
| --- | --- |

| Title: | 3D-CE2.h related: Clipping in depth-based disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In HTM 7.0r1 software, there is a bug on the depth-based DV (DoNBDV) derivation process. Therefore, this contribution provides a bug-fix to align with the specification of 3D-HEVC. The bug-fix reportedly shows an average of 0.01% BD-rate loss on video 2. In the current specification of 3D-HEVC, when retrieving the corresponding depth block in the depth picture of the reference view to derive a depth-based disparity vector, the corresponding depth block should be located inside of the depth picture even though the corresponding depth block is either outside of depth picture or on the boundary of depth picture. However, if the corresponding depth block can be located outside of the depth picture or on the boundary of the depth picture, like as doing in the motion compensation process, it is expected that more accurate disparity vector can be obtained since allowing this can reflect an actual position of the corresponding block. Therefore, this contribution proposes to allow the corresponding depth block to be located outside of the depth picture or on the boundary of the depth picture by the proposed clipping operation. When applying the proposed method to DoNBDV derivation process, it is reportedly shown that the proposed method provides 0.03% BD-rate gain for video 2. And when applying to the DoNBDV derivation and BVSP process together, the proposed method reportedly shows an average of 0.02% BD-rate gain on synthesis views.

# Introduction

In the current 3D-HEVC [1], there is a bug on the depth-based disparity vector (DoNBDV) derivation process. From the specification of 3D-HEVC, the corresponding depth block used for the depth-based disparity vector derivation should be inside of the depth picture. However, in HTM 7.0r1 software, the corresponding depth block can be located outside of or on the left and above boundary of the depth picture. Thus, this contribution fixes to locate the corresponding depth block inside of the depth picture as described in the current specification. Simulation results of the bug-fix are shown in Table 1. From the results, there is coding loss of 0.01% in video 2.

Table 1. Performance comparison between the bug-fix and the CTC anchor (3-view test case)

As mentioned above, when retrieving the corresponding depth block in the depth picture of the reference view to derive a depth-based disparity vector used for the inter-view candidates and the BVSP modes, the corresponding depth block is located inside of depth picture by the clipping operation even though the corresponding depth block is either outside of depth picture or on the boundary of depth picture as shown in Figure 1.

Figure 1. Example of the clipping process in the depth-based DV derivation (based on specification)

# Proposed Method

However, if the corresponding depth block can be located outside of the depth picture or on the boundary of the depth picture, like as doing in the motion compensation process, it is expected that more accurate disparity vector can be obtained since allowing this can reflect an actual position of the corresponding block.

Therefore, in this contribution, it is proposed to allow the corresponding depth block to be located outside of the depth picture or on the boundary of the depth picture by the proposed clipping operation as shown in Figure 2.

Figure 2. Example of the proposed clipping process in the depth-based DV derivation

# Experimental Results

The simulation was carried out on the common test condition [2] and based on HTM-7.0r1 reference software.

Firstly, when the proposed method is applied in the DoNBDV process, the performance of the proposed method is shown in Table 2. The proposed method provides 0.03% gain for video 2.

Table 2. Performance comparison between the proposed method and the CTC anchor (3-view test case)
(when only applying to DoNBDV process)

Secondly, when the proposed method is applied in both the DoNBDV and BVSP process, the performance of the proposed method is shown in Table 3. The proposed method provides an average of 0.02% gain on synthesis views.

Table 3. Performance comparison between the proposed method and the CTC anchor (3-view test case)

# Conclusion

This contribution proposes to change the clipping operation to allow the corresponding depth block, which is used for the depth-based DV derivation, to be located outside of the boundary of the depth picture and on the boundary of the depth picture. Although the proposed method provides minor gain on synthesis view, the proposed method can provide more accurate refined disparity vector at the boundary of the picture. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Test Model.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 4,” JCT3V-D1005, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-D1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.