| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0142 |
| --- | --- |

| Title: | 3D-CE2.h related: Simplified NBDV and improved disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Byeongdoo Choi Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com b.d.choi@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents three methods for simplifying and improving the current disparity vector derivation. Firstly, to reduce the number of accessing the motion information in the NBDV derivation, it is proposed to only search 2 spatial candidates of left block and above block and 2 temporal candidates of co-located block in the co-located picture and RAP picture whereas the current NBDV derivation needs to access the motion information of 5 spatial candidates and 4 temporal candidates. The first proposed method reportedly shows an average of 0.01% and 0.04% bit-saving on coded and synthesized views for CTC case, and an average of 0.02% and 0.01% bit-saving on coded and synthesized views for BVSP off case. The second proposed method is to extend the depth-based 2-step disparity vector derivation of JCT3V-D0113. It is observed that NBDV is suitable only for ARP which was adopted at the last meeting. Thus, it is proposed to basically use depth-based 2-step disparity vector, and to additionally use the proposed simplified NBDV when the current block is coded with ARP. The second proposed method reportedly shows an average of 0.6% and 0.7% bit-saving on video 1 and video 2, and 0.2% and 0.1% bit-saving on coded and synthesized views for CTC case. The third proposed method is ,when simplified NBDV is unavailable, to use the depth-based 2-step disparity vector for ARP in case of CTC and to use the disparity vector corresponding to the depth value of 128 for ARP in case of BVSP off. The third proposed method reportedly shows an average of 0.5% and 0.7% bit-saving on video 1 and video 2, and 0.2% bit-saving on both coded and synthesized views for CTC case, and 0.3% and 0.4% bit-saving on video 1 and video 2, and 0.1% bit-saving on both coded and synthesized views for BVSP off case.

# Introduction

In the current 3D-HEVC [1], the neighboring block disparity vector (NBDV) derivation needs to access the motion information of 5 spatial candidates (A1, B1, B0, A0, and B2 blocks as shown in Figure 1) and 4 temporal (BR and Col in co-located picture and RAP picture as shown in Figure 1) candidates. However, it looks very complicated to search many candidates, so NBDV derivation needs to be simplified. In addition, in terms of the accuracy of depth-based disparity vector refinement, the depth-based 2-step disparity vector, which was proposed in JCT-D0113 [2], is more accurate than DoNBDV. This proposed 2-step disparity vector can be applied to appropriate coding tools to improve their coding efficiency. On top of that, when NBDV is unavailable, ARP is not performed, however, if an appropriate disparity vector can be used for ARP it would be expected that the coding efficiency can be increased.

Figure 1. Spatial and temporal candidates for NBDV process

The disparity vector derivation can be illustrated as follows.

- A variable availableDV, which indicates whether a disparity vector is found or not, is set to 0.

- The disparity vector NBDV is set to (0, 0)

- The NBDV derivation is performed as follows.

  - For 4 temporal neighbouring blocks (bottom right (BR) and co-located (Col) blocks in co-located picture and RAP picture), if availableDV is equal to 0, find the disparity vector from neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For 5 spatial neighbouring blocks (A1, B1, B0, A0, and B2 blocks), if availableDV is equal to 0, find the disparity vector from a neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For 5 spatial neighbouring blocks (A1, B1, B0, A0, and B2 blocks), if availableDV is equal to 0, perform DV-MCP process to find a disparity vector, and once the disparity vector is found availableDV is set to 1.

- If depth_refinement_flag, which indicates whether refining NBDV from the depth map, is equal to 1, refined NBDV DVref is derived as follows.

  - Find the corresponding depth block of the reference view by using NBDV.

  - Select the representative depth value in the corresponding depth block.

  - Convert the representative depth value to the disparity vector.

# Proposed Methods

## Method 1: Simplified NBDV

In order to reduce the number of accessing the motion information in the NBDV derivation, it is proposed to only search 2 spatial candidates (A1 and B1 blocks as shown in Figure 2) and 2 temporal candidates (Col in co-located picture and RAP picture as shown in Figure 2).

Figure 2. Spatial and temporal candidates in proposed simplified NBDV

The disparity vector derivation including the proposed simplified NBDV can be illustrated as follows.

- A variable availableDV, which indicates whether a disparity vector is found or not, is set to 0.

- The disparity vector is set to (0, 0)

- The NBDV derivation is performed as follows.

  - For 2 temporal neighbouring blocks (co-located (Col) block in co-located picture and RAP picture), if availableDV is equal to 0, find the disparity vector from neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For 2 spatial neighbouring blocks (A1 and B1 blocks), if availableDV is equal to 0, find the disparity vector from a neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For 2 spatial neighbouring blocks (A1 and B1 blocks), if availableDV is equal to 0, perform DV-MCP process to find a disparity vector, and once the disparity vector is found availableDV is set to 1.

- If depth_refinement_flag, which indicates whether refining NBDV from the depth map, is equal to 1, refined NBDV DVref is derived as follows.

  - Find the corresponding depth block of the reference view by using NBDV.

  - Select the representative depth value in the corresponding depth block.

  - Convert the representative depth value to the disparity vector.

## Method 2: Improved DV derivation

In terms of the accuracy of depth-based disparity vector, the depth-based 2-step disparity vector, which was proposed in JCT-D0113 [2], is more accurate than DoNBDV. In addition, it is observed that NBDV is suitable only for ARP which was adopted at the last meeting. Therefore, it is proposed to basically use depth-based 2-step disparity vector, and to additionally use the proposed simplified NBDV when the current block is coded with ARP.

The proposed disparity vector derivation can be illustrated as follows.

- A variable availableDV, which indicates whether a disparity vector is found or not, is set to 0.

- If depth_refinement_flag, which indicates whether refining NBDV from the depth map, is equal to 1, the depth-based disparity vector DVdep is derived as follows.

  - The DVdep is set to the disparity vector corresponding to the depth value 128.

  - Find the corresponding depth block of the reference view by using DVdep.

  - Select the representative depth value in the corresponding depth block.

  - Convert the representative depth value to DVdep.

  - Repeat b) to d) once again.

- If depth_refinement_flag is equal to 0 or the current block mode is coded with ARP, the simplified NBDV DVsimp is derived as follows.

  - For 2 temporal neighbouring blocks (co-located (Col) block in co-located picture and RAP picture), if availableDV is equal to 0, find the disparity vector from neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For 2 spatial neighbouring blocks (A1 and B1 blocks), if availableDV is equal to 0, find the disparity vector from a neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For 2 spatial neighbouring blocks (A1 and B1 blocks), if availableDV is equal to 0, perform DV-MCP process to find a disparity vector, and once the disparity vector is found availableDV is set to 1.

## Method 3: Using unavailable DV for ARP

In the current 3D-HEVC, when NBDV is unavailable, ARP is not performed. However, an appropriate disparity vector can be used to allow ARP. Therefore, it is proposed , when NBDV is unavailable, to use depth-based 2-step disparity vector for ARP in case of CTC and to use the disparity vector corresponding to the depth value of 128 in case of BVSP and DoNBDV off.

The proposed disparity vector derivation can be illustrated as follows.

- A variable availableDV, which indicates whether a disparity vector is found or not, is set to 0.

- If depth_refinement_flag, which indicates whether refining NBDV from the depth map, is equal to 1, the depth-based disparity vector DVdep is derived as follows.

  - The DVdep is set to the disparity vector corresponding to the depth value 128.

  - Find the corresponding depth block of the reference view by using DVdep.

  - Select the representative depth value in the corresponding depth block.

  - Convert the representative depth value to DVdep.

  - Repeat b) to d) once again.

- If depth_refinement_flag is equal to 0 or the current block mode is coded with ARP, the simplified NBDV DVsimp is derived as follows.

  - For 2 temporal neighbouring blocks (co-located (Col) block in co-located picture and RAP picture), if availableDV is equal to 0, find the disparity vector from neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For 2 spatial neighbouring blocks (A1 and B1 blocks), if availableDV is equal to 0, find the disparity vector from a neighbouring block, and once the disparity vector is found availableDV is set to 1.

  - For 2 spatial neighbouring blocks (A1 and B1 blocks), if availableDV is equal to 0, perform DV-MCP process to find a disparity vector, and once the disparity vector is found availableDV is set to 1.

- When availableDV is equal to 0, if depth_refinement_flag is equal to 0, DVsimp is set to the disparity vector corresponding to the depth value of 128; otherwise, DVsimp is set to DVdep.

# Experimental Results

The simulation was carried out on the common test condition [3] and based on HTM-7.0r1 reference software.

## Method 1: Simplified NBDV

In case of CTC, the proposed method 1 provides minor BD-rate gain of 0.01% and 0.04% on coded and synthesized views as shown in Table 1.

Table 1. Performance comparison between the proposed method and the CTC anchor (3-view test case)
(Method 1: Simplified NBDV w/ CTC case)

In case of BVSP and DoNBDV off, the proposed method 1 provides 0.15% and 0.02% bit-saving on video 1 and video 2, and 0.02% and 0.01% bit-saving on coded and synthesized views as shown in Table 2.

Table 2. Performance comparison between the proposed method and the non-CTC anchor (3-view test case)
(Method 1: Simplified NBDV w/ BVSP & DoNBDV off case)

In addition, the complexity analysis was also carried out for the proposed method 1. This contribution analyzes the worst case of the disparity vector derivation in terms of number of operations, such as comparisons, multiplications and additions, and the memory bandwidth for CTC, and BVSP and DoNBDV off cases. The analysis was done based on the current specification of 3D-HEVC. In terms of the memory bandwidth, the motion information of temporal candidates should be fetched from off-chip memory and the motion information of spatial candidates can be fetched from on-chip memory. For the size of each motion information is 10 bytes (4 bytes motion vector for list0 and list1, and 1 byte reference index for list0 and list1), CU-level memory bandwidth can be easily calculated. In addition, in the depth-based derivation, 4 corner depth pixels should be fetched from off-chip memory. Since the size of each depth pixel is 1 byte, 4 bytes are needed for the depth-based derivation. If 4x2 memory pattern should be used in depth picture, 8 bytes would be needed for fetching 1 depth pixel. The complexity analysis results of proposed methods 1 are shown in Table 3. As results of the analysis, the proposed method 1 provides significant reducing both operations and memory bandwidth.

Table 3. Complexity analysis results of proposed method 1 (CU-level)

## Method 2: Improved DV derivation

In case of CTC, the proposed method 2 provides an average of 0.6% and 0.7% bit-saving on video 1 and video 2, and 0.2% and 0.1% bit-saving on coded and synthesized views as shown in Table 4.

Table 4. Performance comparison between the proposed method and the CTC anchor (3-view test case)
(Method 2: Improved DV derivation w/ CTC case)

In case of BVSP and DoNBDV, the proposed method 2 has the same performance with the proposed method 1 since the simplified NBDV is used for this case.

In addition, the complexity analysis was also carried out for the proposed method 2. This contribution analyzes the worst case of the disparity vector derivation in terms of number of operations, such as comparisons, multiplications and additions, and the memory bandwidth for inter-view merge candidates and BVSP modes, and ARP modes. The analysis was done based on the current specification of 3D-HEVC. In terms of the memory bandwidth, the motion information of temporal candidates should be fetched from off-chip memory and the motion information of spatial candidates can be fetched from on-chip memory. For the size of each motion information is 10 bytes (4 bytes motion vector for list0 and list1, and 1 byte reference index for list0 and list1), CU-level memory bandwidth can be easily calculated. In addition, in the depth-based derivation, 4 corner depth pixels should be fetched from off-chip memory. Since the size of each depth pixel is 1 byte, 4 bytes are needed for each depth-based derivation. If 4x2 memory pattern should be used in depth picture, 8 bytes would be needed for fetching 1 depth pixel. The complexity analysis results of proposed methods 1 are shown in Table 5. As results of the analysis, the proposed method 1 provides significant reducing in terms of operations. For the memory bandwidth of ARP case, when depth pixel is fetched in unit of one byte, the proposed method decreases on-chip memory bandwidth by 30 bytes and off-chip memory bandwidth by 16 bytes. And when 4x2 memory pattern should be used in depth picture, the proposed method decreases on-chip memory bandwidth by 30 bytes and increases off-chip memory bandwidth by 12 bytes. However, for the memory bandwidth of inter-view candidates and BVSP modes, when depth pixel is fetched in unit of one byte, the proposed method decreases on-chip memory bandwidth by 50 bytes and off-chip memory bandwidth by 36 bytes. And when 4x2 memory pattern should be used in depth picture, the proposed method decreases on-chip memory bandwidth by 50 bytes and off-chip memory bandwidth by 8 bytes. To sum up, the proposed method can reduce the computational complexity and the required memory bandwidth with 0.2% and 0.1% bit-saving on coded and synthesized views.

Moreover, the depth-based 2-step derivation is always performed in the same row of the current coding unit (CU) since the horizontal component of the disparity vector is always zero, and also the 2-step derivation can be done in parallel with other processing since the proposed method only accesses a depth map.

Table 5. Complexity analysis results of proposed method 2 (CU-level)

## Method 3: Using unavailable DV for ARP

In case of CTC, the proposed method 3 provides an average of 0.5% and 0.7% bit-saving on video 1 and video 2, and 0.2% bit-saving on both coded and synthesized views as shown in Table 6.

Table 6. Performance comparison between the proposed method and the CTC anchor (3-view test case)
(Method 3: Using unavailable DV for ARP w/ CTC case)

In case of BVSP and DoNBDV off, the proposed method 3 provides 0.3% and 0.4% bit-saving on video 1 and video 2, and 0.1% bit-saving on both coded and synthesized views as shown in Table 7.

Table 7. Performance comparison between the proposed method and the non-CTC anchor (3-view test case)
(Method 3: Using unavailable DV for ARP w/ BVSP & DoNBDV off case)

# Conclusion

In this contribution, the three methods are proposed to simplify and to improve the disparity derivation. The proposed methods provide 0.2% bit-saving on coded and synthesized views in case of CTC and 0.1% bit-saving in case of BVSP and DoNBDV off. In addition, the complexity of the proposed methods is comparable or lower compared with the current disparity vector derivation. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Test Model.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 4,” JCT3V-D1005, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.

[2] M. W. Park, J. Y. Lee, J. Yoon, H.-C. Wey, B. Choi, Y. Cho, C. Kim, “3D-CE2.h related: Simplification of disparity vector derivation,” JCT3V-D0113, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.

[3] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-D1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.