| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0143 |
| --- | --- |

| Title: | 3D-CE4.h related: Illumination compensation for BVSP and adaptive IC model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Ho-Cheon Wey Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com hc.wey@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the ic_flag is signalled for every BVSP mode to remove parsing dependency, however, the BVSP mode does not apply the illumination compensation. Therefore, it is proposed to simply apply the illumination compensation to BVSP mode with the offset model. This proposed method reportedly provides 0.1% bit-saving on coded and synthesized views. In addition, it is observed that if IC model adaptively can apply to each component (i.e. luma, chroma, and depth map), more coding gain could be obtained and the complexity also could be decreased. When the offset model is applied to chroma, while other components use the current IC model, the proposed method reportedly provides 0.1% bit-saving on coded and synthesized views. Additionally, when the offset model is applied to depth, while other components use the current IC model, the proposed method reportedly provides no coding loss. When combining all propose methods, 0.2% and 0.1% bit-savings are reportedly obtained for coded and synthesized views, respectively.

# Introduction

In the current 3D-HEVC [1], the ic_flag, which indicates whether enabling the illumination compensation, is signalled for every BVSP mode to remove parsing dependency, however, the BVSP mode does not apply the illumination compensation. In addition it is observed that if IC model adaptively can apply to each component (i.e. luma, chroma, and depth map), more coding gain could be obtained and the complexity also could be decreased.

# Proposed Methods

## Method 1: Applying IC to BVSP mode

It is proposed to apply the illumination compensation to BVSP modes with the offset model. The offset parameter derivation is done in the current block level as shown in Figure 1. Here, the neighboring pixels of the corresponding block are found by the disparity vector used for finding the corresponding depth block to derive disparity vectors for sub-blocks.

Figure 1. Example of neighboring pixels of the current block and the corresponding block

The proposed parameter derivation process can be explained as following equations.

,

,

,

,

where N represents number of available neighbouring pixel, NBcurr(i) represents a the neighbouring sample of the current block, NBref(i) represents a neighbouring sample of the reference block, offset represents the offset parameter, (x, y) represents the sample position in the reference block, Pred’(x,y) represents an illumination compensated sample, Pred(x,y) represents a sample in the reference block.

## Method 2: Offset mode for chroma

It is proposed to simply apply the offset mode to chroma components, while other components use the current linear regression model. The parameter derivation method can be used the same method as described in Method 1.

## Method 3: Offset mode for depth map

It is proposed to simply apply the offset mode to depth map, while other components use the current linear regression model. The parameter derivation method can be used the same method as described in Method 1.

# Experimental Results

The simulation was carried out on the common test condition [3] and based on HTM-7.0r1 reference software.

## Method 1: Applying IC to BVSP mode

The proposed method 1 provides an average of 0.1% and 0.4% bit-saving on video 1 and video 2, and 0.1% bit-saving on coded and synthesized views as shown in Table 1.

Table 1. Performance comparison between the proposed method 1 and the CTC anchor (3-view test case)

## Method 2: Offset mode for chroma

The proposed method 2 provides an average of 0.2% and 0.4% bit-saving on video 1 and video 2, and 0.1% bit-saving on coded and synthesized views as shown in Table 2.

Table 2. Performance comparison between the proposed method 2 and the CTC anchor (3-view test case)

## Method 3: Offset mode for depth map

The proposed method 3 provides no coding loss as shown in Table 3.

Table 3. Performance comparison between the proposed method 3 and the CTC anchor (3-view test case)

## Combining all methods

When combining all proposed methods, the proposed methods provide an average of 0.4% and 0.6% bit-saving on video 1 and video 2, and 0.2% and 0.1% bit-saving on coded and synthesized views as shown in Table 4.

Table 4. Performance comparison between the all proposed methods and the CTC anchor (3-view test case)

# Conclusion

In this contribution, it is proposed to apply the illumination compensation to BVSP mode with the offset model, and to use the offset model for chroma components and depth map. The proposed methods provide 0.2% and 0.1% gains on coded and synthesized views. Therefore, it is recommended to adopt all proposed methods into next 3D-HEVC Test Model.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 4,” JCT3V-D1005, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-D1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.