| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0144_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Test Model for JCT3V-E0144 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Jaewon Yoon Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com jw0413.yoon@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides a proposed text of 3D-HEVC Test Model. The changes are highlighted in green

H.7.3.9.1	General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if ( ic_flag && iv_res_pred_flag[ nuh_layer_id ] && TempRefPicInListsFlag ) |  |
| iv_res_pred_weight_idx | ae(v) |
| } |  |
| else { |  |
| … |  |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if ( ic_flag && merge_flag[x0][y0] && iv_res_pred_flag[ nuh_layer_id ] && TempRefPicInListsFlag ) |  |
| iv_res_pred_weight_idx | ae(v) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |
| } |  |

H.7.4.9.1	General coding unit semantics

…

iv_res_pred_weight_idx specifies the index of the weighting factor used for residual prediction. When not present, the value of iv_res_pred_weight_idx is inferred to be equal to 0.

When DefaultDispFlag[ x0 ][ y0 ] is equal to 1, iv_res_pred_weight_idx shall be equal to 0.

- The variable icEnableFlag is set equal to 0 and when slice_ic_enable_flag is equal to 1 and PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

  - icEnableFlag = ( merge_idx[ x0 ][ y0 ] ! = 0 ) | | !slice_ic_disable_merge_zero_idx_flag 	(Error: Reference source not found-1)

- Otherwise ( merge_flag[ x0 ][ y0 ] is equal to 0 ), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to the view order index of RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ].

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = 	
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_L0 && refViewIdxL1[ x0 ][ y0 ] ! = ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_L1 && refViewIdxL0[ x0 ][ y0 ] ! = ViewIdx)	(Error: Reference source not found-2)

ic_flag equal to 1 specifies illumination compensation is used for the current coding unit. ic_flag equal to 0 specifies illumination compensation is not used for the current coding unit. When not present, ic_flag is inferred to be equal to 0.

…

H.8.5.2.2	Decoding process for inter prediction samples

…

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1.

- If VspModeFlag[ xC + xB ][ yC + yB ] is equal to 0, the following ordered steps apply:

  - Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and predSampleL0Cb, predSampleL1Cb, predSampleL0Cr, and predSampleL1Cr be (nPbW/2)x(nPbH/2) arrays of predicted chroma sample values.

  - For X being replaced by either 0 or 1, the following applies:

    - When predFlagLX is equal to 1, the following applies.

      - The variable resPredFlag is derived as specified in the following:

        - resPredFlag = 	( iv_res_pred_weight_idx != 0 ) &&	
	( PicOrderCnt( RefPicListX[ refIdxLX ] ) != PicOrderCntVal ) 	(Error: Reference source not found-3)

      - If resPredFlag is equal to 1, the bilinear sample interpolation and residual prediction process as specified in subclause  is invoked with the luma locations ( xC, yC ), ( xB, yB ), the size of the current luma coding block nCS, the width and the height of the current luma prediction block nPbW, nPbH, the prediction list indication X, the prediction list utilization flag predFlagLX, the reference index refIdxLX, and the motion vectors mvLX, mvCLX, as the inputs and the outputs are the arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr.

      - Otherwise, ( resPredFlag is equal to 0 ), the following applies:

        - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX as input.

        - If DepthFlag is equal to 0, the arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the fractional sample interpolation process specified in subclause 8.5.2.2.2 with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr given as input.

Otherwise, (DepthFlag is equal to 1), arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the full sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr given as input.

…

H.8.5.1.1.6	Bilinear sample interpolation and residual prediction process

The process is only invoked if res_pred_flag is equal to 1.

Inputs to this process are:

- a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,prediction list utilization flags, predFlagL0 and predFlagL1,

- the prediction list indication X,

- the prediction list utilization flag predFlagLX,

- the reference index refIdxLX,

- the motion vectors mvLX, mvCLX

Output of this process are:

- the (nPbW)x(nPbH) array predSamplesLXL,

- the (nPbW / 2)x(nPbH / 2) arrays predSamplesLXCb and predSamplesLXCr.

The motion vector mvLX is scaled as specified in the following:

  - tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td		(Error: Reference source not found-4)

  - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(Error: Reference source not found-5)

  - mvLX =	Clip3( −32768, 32767, Sign( distScaleFactor * mvLX ) * 
	( ( Abs( distScaleFactor * mvLX ) + 127 ) >> 8 ) )	(Error: Reference source not found-6)

where td and tb are derived as:

  - td = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) )	(Error: Reference source not found-7)

  - tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ 0 ] ) )	(Error: Reference source not found-8)

The motion vector mvCLX is set equal to mvLX.

The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr  are derived as specified in the following:

- The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX equal to 0 as input.

- The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr. as the inputs.

The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr  are derived as specified in the following:

- Let currIvRefPic be the picture with PicOrderCnt( currIvRefPic ) equal to PicOrderCntVal and ViewIdx equal to RefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays currIvRefPicLXL, currIvRefPicLXCb, and currIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture currIvRefPic.

- The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the width nCS and height nCS, the motion vector mvLX equal to MvDisp[ xP ][ yP ] and the motion vector mvCLX equal to MvDisp[ xP ][ yP ], and the reference arrays with currIvRefPicLXL, currIvRefPicLXCb and currIvRefPicLXCr as the inputs.

The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr  are derived as specified in the following:

- Let refIvRefPic be the picture with PicOrderCnt( refIvRefPic ) equal to PicOrderCnt( RefPicListX[ 0 ] ) and ViewIdx equal to RefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays refIvRefPicLXL, refIvRefPicLXCb, and refIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture refIvRefPic.

- The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the width nCS and height nCS, the motion vector mvLX equal to ( mvLX + MvDisp[ xP ][ yP ] ) and the motion vector mvCLX equal to ( mvCLX + MvDisp[ xP ][ yP ] ), and the reference arrays with refIvRefPicLXL, refIvRefPicLXCb and refIvRefPicLXCr as the inputs.

The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

The modified prediction samples predSamplesLXL[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as specified in the following:

  - predSamplesLXL[ x ][ y ] =	 predSamplesLXL[ x ][ y ] + 
	( ( currIvSamplesLXL[ x ][ y ] − refIvSamplesLXL[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-9)

The modified prediction samples predSamplesLXCb[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 )−1 are derived as specified in the following:

  - predSamplesLXCb[ x ][ y ] =	 predSamplesLXCb[ x ][ y ] + 
	 ( currIvSamplesLXCb[ x ][ y ] − refIvSamplesLXCb[ x ][ y ] )	(Error: Reference source not found-10)

The modified prediction samples predSamplesLXCr[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 ) − 1 are derived as specified in the following:

  - predSamplesLXCr[ x ][ y ] =	 predSamplesLXCr[ x ][ y ] + 
	 ( currIvSamplesLXCr[ x ][ y ] − refIvSamplesLXCr[ x ][ y ] )  	(Error: Reference source not found-11)