| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0144 |
| --- | --- |

| Title: | Simplification and improvement of ARP for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Jaewon Yoon Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com jw0413.yoon@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents four methods for simplifying and improving the ARP process. Firstly, it is proposed to apply ARP only to 2Nx2N merge to reduce the complexity with no coding loss. Secondly, it is proposed not to use ARP when IC is enabled. In the current 3D-HEVC, ARP weight is signalled even if the current block is an inter-view prediction block to remove parsing dependency. If IC is enabled, the current block is most likely to be the inter-view prediction mode. Most of these cases do not need to perform ARP. With the second proposed method, unnecessary ARP weight signalling and the encoding complexity can be reduced. Thirdly, it is proposed not to use the half weight for chroma to improve coding performance. That is, even if the weight of the current block is 0.5, the original residual data can be applied as is for chroma. Fourthly, it is proposed to allow a non-zero value for ARP weight in case of the inter-view prediction mode to improve entropy coding efficiency. The proposed methods reportedly provide 0.3% bit-saving for dependent views (video 1 and video 2), and 0.1% bit-saving for both coded and synthesized views with 94.7% encoding time.

# Introduction

In the current 3D-HEVC [1], the advanced residual prediction (ARP) is used to predict the residual data of the current block from the one of the corresponding block in the reference view. However, since ARP has two weighting factors 0.5 and 1, two more mode decisions are needed for every 2Nx2N coding unit (CU). Thus, the mode decision complexity needs to be reduced while maintaining the coding efficiency.

Additionally, in ARP, in order to remove parsing dependency, the ARP weight is also signalled even if the current block is an inter-view prediction block mode. Therefore, in this case, signalling the ARP weight needs to be reduced and ARP weight can be set to an appropriate value to obtain the best coding performance.

# Proposed Methods

## Method 1: Applying ARP only to merge mode

The current ARP is applied to 2Nx2N merge modes and 2Nx2N Inter modes, thus the number of mode decision for ARP in 2Nx2N partition size is six. Therefore, in order to reduce the mode decision complexity, it is proposed to apply ARP only to 2Nx2N merge mode. With the proposed method 1, the number of the mode decision can be reduced from six to three.

## Method 2: Not using ARP in case of enabling IC

In the current 3D-HEVC, in order to remove parsing dependency, ARP weight is signalled even if the current block is an inter-view prediction mode. However, if the illumination compensation (IC) is enabled, the current block is most likely to be an inter-view prediction mode. Most of these cases do not need to perform ARP because ARP can be only applied to a temporal prediction mode. Moreover, if the current block is bi-prediction mode from both temporal and view direction, the combination number of the mode decision to be performed is six since it is needed to decide the weighting factor of ARP (0, 0.5, and 1) and enabling flag of IC (on and off). Therefore, it is proposed not to use when IC is enabled. With this proposed method, unnecessary ARP weight signalling can be removed and the encoding complexity by the mode decision can be reduced.

## Method 3: Not using half weight for chroma

Unlike the luma component, it is observed that the most of the chroma residual data can believe as it is without applying a weight. Therefore, it is proposed not to use the half weight for chroma components to improve coding efficiency. That is, even if the weight of the current block is 0.5, the original residual data can be applied as is for chroma components.

## Method 4: Allowing non-zero weight value for inter-view prediction block

In the current 3D-HEVC, as mentioned above, the ARP weight is signalled even if the current block is an inter-view prediction mode and is always set to zero. However, it does not need to always set ARP weight to zero in this case, since the decision whether ARP is used or not is performed at the decoding process of ARP, in which the prediction direction as well as the value of the ARP weight is additionally checked. Thus, the ARP weight can be set to the value which can result in the best performance in the CABAC coding. Therefore, it is proposed to allow a non-zero value for the ARP weight when the current block mode is an inter-view prediction mode.

# Experimental Results

The simulation was carried out on the common test condition [3] and based on HTM-7.0r1 reference software.

## Method 1: Applying ARP only to merge mode 
(Test 1: Method 1)

The proposed method 1 provides no coding loss with 97% encoding time as shown in Table 1. The results shows that ARP is only appropriate for the merge modes and the proposed method 1 can effectively reduce the mode decision complexity.

Table 1. Performance comparison between the proposed method and the CTC anchor (3-view test case)
(Method 1)

## Method 2: Not using ARP in case of enabling IC 
(Test 2: Methods 1 and 2)

The proposed method 2 was simulated with method 1. The proposed method 1 and 2 provide 0.1% gain on dependent views (video 1 and video 2), and 0.1% gain on synthesized views with 95% encoding time. The simulation results show that the coding performance can be increased by removing unnecessary ARP weight signalling and the proposed method 2 can effectively reduce the mode decision complexity as well.

Table 2. Performance comparison between the proposed method and the CTC anchor (3-view test case)
(Method 1 and 2)

## Method 3: Not using half weight for chroma 
(Test 3: Methods 1, 2, and 3)

The proposed method 3 was simulated with method 1 and 2. The proposed method 1, 2, and 3 provide 0.2% bit-saving on dependent views (video 1 and video 2), and 0.1% bit-saving on video PSNR vs. video bitrate, and synthesis PSNR vs. total bitrate cases.

Table 3. Performance comparison between the proposed method and the CTC anchor (3-view test case)
(Method 1, 2 and 3)

## Method 4: Allowing non-zero weight value for inter-view prediction block 
(Test 4: Methods 1, 2, 3, and 4)

The proposed method 4 was simulated with method 1, 2, and 3. The proposed method 1, 2, 3, and 4 provide 0.3% and 0.2% bit-saving on video 1 and video 2, and 0.1% bit-saving on coded and synthesized views.

Table 4. Performance comparison between the proposed method and the CTC anchor (3-view test case)
(Method 1, 2, 3)

## Test Summary

Table 5 shows the summary of the experiments results. The proposed methods provide 0.1% bit-saving for overall performance with 94.7% encoding time.

Table 5. Summary of experimental results

# Conclusion

This contribution presents four methods for simplifying and improving the current ARP. The proposed methods provide 0.1% bit-saving for overall performance with 94.7% encoding time. Therefore, it is recommended to adopt all proposed methods into next 3D-HEVC Test Model.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 4,” JCT3V-D1005, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-D1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, April 2013, Incheon, Korea.