| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0146 |
| --- | --- |

| Title: | 3D-CE5: DMM simplification and signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Ho-Cheon Wey, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In this proposal, the performance of a depth modelling mode (DMM) is evaluated. For the efficient edge coding, four DMMs, which consist of explicit wedgelet partitioning (DMM1), intra-predicted wedgelet partitioning (DMM2), inter-component wedgelet partitioning (DMM3), and inter-component-predicted contour partitioning (DMM4), were developed. In addition, the region boundary chain coding mode is also used for improving edge regions in depth maps. However, it is observed that only DMM1 is enough to code the edge and the remaining modes are redundant. Hence, the proposed method removes several modes. Results demonstrate that the proposed method reduces the encoding time by about 95% and 75% with the negligible coding loss in CTC and all intra case, respectively. Furthermore, the specification text related to DMMs and the chain mode is significantly cleaned up (more than 10 pages).

# DMM Simplification and Signalling

Four DMMs and one chain mode have been added to the conventional intra modes for the efficient depth edge coding as follows:

- DMM1: Explicit wedgelet signalling;

- DMM2: Intra-predicted wedgelet partitioning;

- DMM3: Inter-component prediction of wedgelet partitions;

- DMM4: Inter-component-predicted contour partitioning;

- Chain mode: Region boundary signalling with chain codes.

Since DMMs and chain mode have been developed with similar concept, they are significantly redundant.

As described in Test2-C1 in JCT3V-D1105, the proposed method removes DMM3 and DMM4, and then signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 00 | - | 0 |
| INTRA_DEP_NONE | 010 | 0 | 10 |
| INTRA_DEP_SDC_DMM_WFULL | 011 | - | - |
| INTRA_DEP_DMM_WFULL | 100 | 10 | - |
| INTRA_DEP_SDC_DC | 101 | - | 11 |
| INTRA_DEP_DMM_WPREDDIR) | 110 | - | - |
| INTRA_DEP_CHAIN | 111 | 11 | - |

Test2-C2 in JCT3V-D1105 removes DMM2, DMM3, and DMM4 and signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 00 | - | 0 |
| INTRA_DEP_NONE | 010 | 0 | 10 |
| INTRA_DEP_SDC_DMM_WFULL | 011 | - | - |
| INTRA_DEP_DMM_WFULL | 100 | 10 | - |
| INTRA_DEP_SDC_DC | 101 | - | 11 |
| INTRA_DEP_CHAIN | 110 | 11 | - |

Test2-D in JCT3V-D1105 removes DMM2, DMM3, DMM4, and the chain mode. The proposed method signals the remaining modes with the following code table:

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 00 | - | 0 |
| INTRA_DEP_NONE | 010 | 0 | 10 |
| INTRA_DEP_SDC_DMM_WFULL | 011 | - | - |
| INTRA_DEP_DMM_WFULL | 100 | 10 | - |
| INTRA_DEP_SDC_DC | 101 | - | 11 |

Through the exhaustive experiments above, it is observed that DMM1 is very dominant in terms of the coding performance. Therefore, this contribution proposes the removal of the several redundant modes for the simplification.

Furthermore, this DMM simplification and signaling makes the specification text much simpler than the current one, as shown in the proposed specification text.

# Test Results

Tables 1 and 2 show the performance in CTC and all intra case, respectively, when DMM3 and DMM4 are removed. (Test2-C1 in JCT3V-D1105)

Table 1. Performance in CTC for Test2-C1 in JCT3V-D1105

Table 2. Performance in all intra case for Test2-C1 in JCT3V-D1105

Tables 3 and 4 show the performance in CTC and all intra case, respectively, when DMM2, DMM3, and DMM4 are removed. (Test2-C2 in JCT3V-D1105)

Table 3. Performance in CTC for Test2-C2 in JCT3V-D1105

Table 4. Performance in all intra case for Test2-C2 in JCT3V-D1105

Tables 5 and 6 illustrate the performance in CTC and all intra case, respectively, when DMM2, DMM3, DMM4, and the chain mode are removed. (Test2-C2 in JCT3V-D1105)

Table 5. Performance in CTC for Test2-D in JCT3V-D1105

Table 6. Performance in all intra case for Test2-D in JCT3V-D1105

# Conclusion

By integrating the proposed methods on 3D-HTM 7.0r1, it significantly reduces the encoding times with the very small coding loss at the same time simplifies the current design and specification. Since the modifications are simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).