| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0147 |
| --- | --- |

| Title: | 3D-CE6 related: Simplified depth coding based on most probable mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Byeongdoo Choi, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Simplified depth coding (SDC) is employed for efficient depth intra coding in 3D-HEVC. SDC consists of Planar, DC, and DMM1 modes and its partition size is always 2Nx2N. Instead of the transform and quantization processes for coding of the residual samples, SDC signals one or two constant values, which are the difference between the DCs of the original depth values and the predicted depth values. In order to reduce the DC calculation process, the proposed method does not signal the residual information by SDC. Therefore, it can remove the SDC-related syntaxes and contexts. In addition, the proposed method uses most probable modes as SDC modes to improve the coding performance, instead of the fixed modes such as Planar, DC, and DMM1 modes. The results demonstrate that the proposed method achieves the gain by -0.3% in terms of the coded PSNR as well as simplifies the current design and specification. As the other proposed method for the simplification, this contribution provides the results when Planar mode is only used as SDC mode.

# SDC Based on Most Proposed Mode

The proposed SDC mode employs the most probable modes to improve the coding performance, instead of the fixed modes such as Planar, DC, and DMM1 modes. When the most probable modes are decided, if the depth modeling modes such as DMM and Chain modes are used, Planar mode is instead used. Also, the proposed method removes the residual signalling parts, so the DC calculation parts are not needed and the syntaxes and contexts related to the SDC residual are simplified.

|  | Current SDC | Proposed SDC |
| --- | --- | --- |
| Modes | Planar, DC, and DMM1 modes | Three most probable modes |
| Residual | Signalling difference between the DCs of the original depth values and the predicted depth values | No Signalling the residual data |
| Syntax and Context | sdc_residual_flag sdc_residual_sign_flag sdc_residual_abs_minus | sdc_residual_flag sdc_residual_sign_flag sdc_residual_abs_minus |

# Only Using Planar Mode for Simplification

The proposed SDC mode only uses Planar mode as SDC mode for the simplification.

|  | Current SDC | Proposed SDC |
| --- | --- | --- |
| Modes | Planar, DC, and DMM1 modes | Planar |
| Residual | Signalling difference between the DCs of the original depth values and the predicted depth values | No Signalling the residual data |
| Syntax and Context | sdc_residual_flag sdc_residual_sign_flag sdc_residual_abs_minus | sdc_residual_flag sdc_residual_sign_flag sdc_residual_abs_minus |

# Test Results

Test results of the proposed method are summarized in Tables 1 to 4. The coding result of the proposed method based on the most probable mode achieves some gain in CTC and all intra case. Especially, the proposed method obtains -0.3% gain in terms of the coded PSNR over the total bitrate. Furthermore, the proposed method removes the DC calculation process and residual-related syntaxes and contexts. As the additional method, when the proposed method only uses Planar mode as SDC, there are some minor loss of -0.2%. However, it also simplifies the current design and specification.

Table 1. Performance of the proposed method based most probable mode in CTC

Table 2. Performance of the proposed method based most probable mode in all intra case

Table 3. Performance of the proposed method only using Planar mode in CTC

Table 4. Performance of the proposed method only using Planar mode in all intra case

# Conclusion

By integrating the proposed methods on 3D-HTM 7.0r1, it improves the coding performance at the same time simplifies the current design and specification. Since the modifications are significantly simple, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).