- H.7.3.9.3 Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 ) { | Descriptor |
| --- | --- |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( depth_intra_mode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ depth_intra_mode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if( depth_intra_mode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WPREDTEX ) |  |
| wedge_predtex_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if( depth_intra_mode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WPREDDIR ) { |  |
| dmm_delta_end_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmm_delta_end_flag[ x0 ][ y0 ] ) { |  |
| dmm_delta_end_abs_minus1[ x0 ][ y0 ] | ae(v) |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( depth_intra_mode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_start_left_flag[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes_minus1[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| } |  |
| if( DmmFlag[ x0 ][ y0 ] ) { |  |
| dmm_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmm_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < 2; i ++ ) { |  |
| dmm_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( dmm_dc_abs[ x0 ][ y0 ][ i ]) |  |
| dmm_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| else if( depth_intra_mode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_dc_flag[ x0 ][ y0 ] | ae(v) |
| if( edge_dc_flag[ x0 ][ y0 ] ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| edge_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if( edge_dc_abs[ x0 ][ y0 ][ i ] != 0 ) |  |
| edge_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- H.7.4.9.3 Depth mode parameter semantics

The variable Log2MaxDmmCbSize is set equal to 5.

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. Table Error: Reference source not found-1 specifies the value for the depth intra mode and the associated names.

The variable SdcFlag[ x0 ][ y0 ] is derived as specified in the following:

  - SdcFlag[ x0 ][ y0 ] =	( depth_intra_mode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_MPM1 )  | |  	(Error: Reference source not found-1)
	( depth_intra_mode[ x0 ][ y0 ]  = =  INTRA_DEP_SDCLMPM2 )  | |  
	( depth_intra_mode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC MPM3 )

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

  - DmmFlag[ x0 ][ y0 ] =	( depth_intra_mode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WFULL )  | |  	(Error: Reference source not found-2)
	( depth_intra_mode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_CPREDTEX )  | |  
	( depth_intra_mode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WPREDTEX )  | |  
	( depth_intra_mode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WPREDDIR )

Table Error: Reference source not found-1 – Specification of depth_intra_mode and associated names

| depth_intra_mode | DepthIntraMode |
| --- | --- |
| 0 | INTRA_DEP_NONE |
| 1 | INTRA_DEP_SDC_MPM1 |
| 2 | INTRA_DEP_SDC_MPM2 |
| 3 | INTRA_DEP_SDC_MPM3 |
| 4 | INTRA_DEP_DMM_WFULL |
| 5 | INTRA_DEP_DMM_WPREDDIR |
| 6 | INTRA_DEP_DMM_WPREDTEX |
| 7 | INTRA_DEP_DMM_CPREDTEX |
| 8 | INTRA_DEP_CHAIN |

  - -

- H.8.4 Decoding process for coding units coded in intra prediction mode

- H.8.4.2 Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PbSize specifying the size of the current luma prediction block.

Table Error: Reference source not found-2 specifies the value for the intra prediction mode and the associated names.

Table Error: Reference source not found-2 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_DC |
| 2..34 | Intra_Angular |
| 35...42 | Intra_DepthPartition (used only for depth) |
| Otherwise (43, 44) | Intra_Chain (used only for depth) |

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

- If

- depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_DMM_WFULL, IntraPredMode[ xB ][ yB ] is set equal to Intra_DepthPartition( 35 + dmm_dc_flag[ xB ][ yB ] ) .

- Otherwise, if depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_DMM_WPREDTEX, IntraPredMode[ xB ][ yB ] is set equal to Intra_DepthPartition( 37 + dmm_dc_flag[ xB ][ yB ] ) .

- Otherwise if depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_DMM_CPREDTEX, IntraPredMode[ xB ][ yB ] is set equal to Intra_DepthPartition( 39 + dmm_dc_flag[ xB ][ yB ] ) .

- Otherwise, if depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_DMM_WPREDDIR, IntraPredMode[ xB ][ yB ] is set equal to Intra_DepthPartition( 41 + dmm_dc_flag[ xB ][ yB ] ) .

- Otherwise if depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_CHAIN, IntraPredMode[ xB ][ yB ] is set equal to Intra_Chain( 43  + edge_dc_flag[ xB ][ yB ] ).

- Otherwise ( depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_NONE, INTRA_DEP_SDC_MPM1, INTRA_DEP_SDC_MPM2, or INTRA_DEP_SDC_MPM3 ), IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

  - The neighbouring locations ( xBA, yBA ) and ( xBB, yBB ) are set equal to ( xB−1, yB ) and ( xB, yB−1 ), respectively.

  - For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

    - The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location ( xCurr, yCurr ) set equal to ( xB, yB ) and the neighbouring location ( xN, yN ) set equal to ( xBN, yBN ) as the input and the output is assigned to availableN.

    - The candidate intra prediction mode candIntraPredModeN is derived as follows.

      - If availableN is equal to FALSE, candIntraPredModeN is set equal to Intra_DC.

      - Otherwise, if PredMode[ xBN ][ yBN ] is not equal to MODE_INTRA, candIntraPredModeN is set equal to Intra_DC,

      - Otherwise, if N is equal to B and yB−1 is less than (( yB >> Log2CtbSizeY ) << Log2CtbSizeY), intraPredModeB is set equal to Intra_DC.

      - Otherwise, if candIntraPredModeN is larger than 34, candIntraPredModeN is set equal to Intra_PLANR.

      - Otherwise, candIntraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ].

  - The candModeList[ x ] with x=0..2 is derived as follows:

    - If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

      - If candIntraPredModeA is less than 2 (either Intra_Planar or Intra_DC), candModeList[ x ] with x=0..2 is derived as:

          - candModeList[0] = Intra_Planar		(Error: Reference source not found-4)
candModeList[1] = Intra_DC		(Error: Reference source not found-5)
candModeList[2] = Intra_Angular (26)	(Error: Reference source not found-6)

      - Otherwise, candModeList[ x ] with x=0..2 is derived as:

          - candModeList[0] = candIntraPredModeA	(Error: Reference source not found-7)
candModeList[1] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(Error: Reference source not found-8)
candModeList[2] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(Error: Reference source not found-9)

    - Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

      - candModeList[0] and candModeList[1] are derived as follows:

          - candModeList[0] = candIntraPredModeA	(Error: Reference source not found-10)
candModeList[1] = candIntraPredModeB	(Error: Reference source not found-11)

      - If none of candModeList[0] and candModeList[1] is equal to Intra_Planar, candModeList[2] is set equal to Intra_Planar,

      - Otherwise, if none of candModeList[0] and candModeList[1] is equal to Intra_DC, candModeList[2] is set equal to Intra_DC,

      - Otherwise, candModeList[2] is set equal to Intra_Angular (26).

  - IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

    - If depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_MPM1, IntraPredMode[ xB ][ yB ] is set equal to candModeList[ 0 ].

    - Otherwise if depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_MPM2, IntraPredMode[ xB ][ yB ] is set equal to candModeList[ 1 ].

    - Otherwise if depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_MPM3, IntraPredMode[ xB ][ yB ] is set equal to candModeList[ 2 ].

    - Otherwise if depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_NONE and  prev_intra_luma_pred_flag[ xB ][ yB ] is equal to 1, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[ mpm_idx ].

    - Otherwise if depth_intra_mode[ xB ][ yB ] is equal to INTRA_DEP_NONE, IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

- The array candModeList[x], x=0..2 is modified as the following ordered steps:

  - When candModeList[0] is greater than candModeList[1], both values are swapped as follows.

( candModeList[0], candModeList[1] ) = Swap( candModeList[0], candModeList[1] )	(Error: Reference source not found-12)

  - When candModeList[0] is greater than candModeList[2], both values are swapped as follows.

( candModeList[0], candModeList[2] ) = Swap( candModeList[0], candModeList[2] )	(Error: Reference source not found-13)

  - When candModeList[1] is greater than candModeList[2], both values are swapped as follows.

( candModeList[1], candModeList[2] ) = Swap( candModeList[1], candModeList[2] )	(Error: Reference source not found-14)

- IntraPredMode[xB][yB] is derived as the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_luma_pred_mode[ xB ][ yB ]

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 2 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

- H.8.4.4.3 Depth value reconstruction process

Inputs to this process are:

- a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable nT specifying the prediction size

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1

- a variable intraPredMode specifying the prediction mode of the current prediction block

Output of this process is:

- reconstructed depth value samples resSamples[ x ][ y ], with x, y = −1..2*nT−1.

the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - resSamples[ x ][ y ] = predSamples[ x ][ y ]		(Error: Reference source not found-15)

      - -
	-
		
		-

-3 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  |  | Table Error: Reference source not found-4 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Table Error: Reference source not found-5 |  |  |  |

- Values of variable initValue for sdc_residual_flag ctxIdx

-s_minus1 ctxIdx

-6 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

|  |  |  |  | Table Error: Reference source not found-4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Table Error: Reference source not found-4 |  |
|  |  |  |  | Table Error: Reference source not found-4 |  |
|  |  |  |  | Table Error: Reference source not found-5 |  |
|  |  |  |  | Table Error: Reference source not found-5 |  |
|  |  |  |  | Table Error: Reference source not found-5 |  |

- H.9.3.2.10 Binarization process for depth_intra_mode

- Input to this process is a request for a binarization for the syntax element depth_intra_mode, a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, and a variable cLog2CbSize specifying the current luma coding block size.

- Output of this process is the binarization of the syntax element.

- The binarization for part_mode is given by Table Error: Reference source not found-7 depending on PartMode[ xC ][ yC ] and cLog2CbSize.

-e

Table Error: Reference source not found-8 –Binarization for depth_intra_mode

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_NONE | 00 | 0 | 0 |
| INTRA_DEP_SDC_MPM1 | 010 | - | 10 |
| INTRA_DEP_SDC_MPM2 | 011 | - | 110 |
| INTRA_DEP_SDC_MPM3 | 100 | - | 111 |
| INTRA_DEP_DMM_WFULL | 101 | 10 | - |
| INTRA_DEP_DMM_WPREDDIR | 110 | - | - |
| INTRA_DEP_DMM_WPREDTEX | 1110 | 110 | - |
| INTRA_DEP_DMM_CPREDTEX | 11110 | - | - |
| INTRA_DEP_CHAIN | 11111 | 111 | - |

  - -

    - -

-s

-s