J.8.x Initialization of disparity vector derivation

Disparity vector defaultDV is set to as follows.

srcIndex = ViewIdTo3DVAcquisitionParamIndex( srcViewId )

refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )

defaultDV[0] = ( NdrInverse[ ((1<<BitDepthY)>>1)] * DisparityScale[ dps_id ][ srcIndex ][ refIndex ] + 
                  ( DisparityOffset[ dps_id ][ srcIndex ][ refIndex ] << BitDepthY ) + 
                  ( 1 << ( log2Div – 1 ) ) ) >> log2Div

defaultDV[1] = 0

J.8.2 3D-AVC inter prediction, inter-view prediction, view synthesis prediction and adaptive luminance compensation

This process is invoked when decoding P and B macroblock types and when nal_unit_type is equal to 21.

Outputs of this process are Inter prediction samples for the current macroblock that are a 16x16 array predL of luma samples and when ChromaArrayType is not equal to 0 two (MbWidthC)x(MbHeightC) arrays predCb and predCr of chroma samples, one for each of the chroma components Cb and Cr.

The variable log2Div is specified to be equal to BitDepthY + 6.

The variable CorDepthFlag is specified as follows.

srcIndex = ViewIdTo3DVAcquisitionParamIndex( srcViewId )

CorDepthFlag = depth_preceding_texture_flag[srcIndex]

The function Disparity( depthSample, srcIndex, refIndex ) is specified to return disparityValue specified as follows.

disparityValue = ( NdrInverse[ depthSample ] * DisparityScale[ dps_id ][ srcIndex ][ refIndex ] + 
	( DisparityOffset[ dps_id ][ srcIndex ][ refIndex ] << BitDepthY ) + 	(J-1)
	( 1 << ( log2Div – 1 ) ) ) >> log2Div

The function DisparityForBlock( depthPic, x1, y1, blWidth, blHeight, srcViewId, refViewId ) is specified to return maxDisp specified with the following ordered steps.

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < blHeight; j += ( blHeight – 1 ) )
	for( i = 0; i < blWidth; i += ( blWidth – 1 ) )		(J-2)
		if( depthPic[ x1 + i, y1 + j ] > maxDepth ) 
			maxDepth = depthPic[ x1 + i, y1 + j ]

- The variable maxDisp is specified as follows:

srcIndex = ViewIdTo3DVAcquisitionParamIndex( srcViewId )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )	(J-3)
maxDisp = Disparity( maxDepth, srcIndex, refIndex)

When DepthFlag is equal to 0 and dmvp_flag is equal to 1 and CorDepthFlag is equal to 1, the variables DepthRefPicList0, DepthRefPicList1 for B slices, and DepthCurrPic are specified as follows. The variable DepthRefPicList0 is specified to consist of the depth view components of the view component pairs for which the texture view components are in RefPicList0 in the order that RefPicList0[ i ] and DepthRefPicList0[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l0_active_minus1. The variable DepthRefPicList1 is specified for B slices to consist of the depth view components of the view component pairs for which the texture view components are in RefPicList1 in the order that RefPicList1[ i ] and DepthRefPicList1[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l1_active_minus1. The variable DepthCurrPic is specified to be the upsampled decoded sample array of the depth view component of the view component pair for which the texture view component is the current texture view component. When DepthFlag is equal to 0 and dmvp_flag is equal to 1 and CorDepthFlag is equal to 0, the variables DepthRefPicList0, DepthRefPicList1 for B slices, and DepthCurrPic are specified to consist of the empty component (empty variable).

…

J.8.2.1.4 Derivation process for the disparity vector and the inter-view reference

Inputs to this process are depth reference view component depthPic, the location of a top-left sample ( dbx1, dby1 ) of a partition and the listSuffixFlag.

Outputs of this process are a picture InterViewPic, an offset vector dV and a variable interViewAvailable.

Set interViewAvailable equal to 0.

The following applies to derive an inter-view reference picture or inter-view only reference picture InterViewPic:

for( cIdx = 0;cIdx<=num_ref_idx_l0_active_minus1  && !interViewAvailable; cIdx ++)
	if ( view order index of RefPicList0[ cIdx ] is equal to 0) {
		InterViewPic = RefPicList0[ cIdx ]		(J-4)
		interViewAvailable = 1
	}

When interViewAvailable is equal to 1, the following steps apply in order.

–	The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 0 as input and with reference indices refIdxCandL0[ i ] and the motion vectors mvCandL0[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

–	The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 1 as input and with reference indices refIdxCandL1[ i ] and the motion vectors mvCandL1[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

–	The variable dV is derived as specified by the following ordered steps:

–	Set DvAvailable[ i ] and mvCand[ i ] with i equal to 0, 1, and 2 corresponding to neighbouring partitions A, B, and C, respectively, as follows

for( i = 0; i < 3; i++ )
	if( view order index of RefPicList0[ refIdxCandL0[ i ] ] is equal to 0 ) {
		DvAvailable[ i ] = 1
		mvCand[ i ] = mvCandL0[ i ]
	} else if( view order index of RefPicList1[ refIdxCandL1[ i ] ] is equal to 0 ) {	(J-5)
		DvAvailable[ i ] = 1
		mvCand[ i ] = mvCandL1[ i ]
	} else
		DvAvailable[ i ] = 0

–	If DvAvailable[ 0 ] + DvAvailable[ 1 ] + DvAvailable[ 2 ] is equal to 1, the following applies:

dV[ 0 ] = mvCand[ i ][ 0 ]
dV[ 1 ] = mvCand[ i ][ 1 ]		(J-6)

–	Otherwise if CorDepthFlag is equal to 1, the following steps apply in order:

–	The variable maxDepth is specified as follows:

maxDepth = INT_MIN 
for( j = 0; j < partHeight; j += ( partHeight – 1 ) )
	for( i = 0; i < partWidth; i += ( partWidth – 1 ) )	(J-7)
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth )
			maxDepth = depthPic[ dbx1 + i, dby1 + j ]

–	The variable dispVector is specified as follows:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the InterViewPic )

dispVector[ 0 ] = Disparity( maxDepth, currIndex, refIndex )	(J-8)
dispVector[ 1 ] = 0

–	For each value of i equal to 0, 1, and 2, when DvAvailable[ i ] is equal to 0, mvCand[ i ] is set to dispVector.

–	Each component of the variable dV is derived as follows:

dV[ 0 ] = Median( mvCand[ 0 ][ 0 ], mvCand[ 1 ][ 0 ], mvCand[ 2 ][ 0 ] )
dV[ 1 ] = Median( mvCand[ 0 ][ 1 ], mvCand[ 1 ][ 1 ], mvCand[ 2 ][ 1 ] )	(J-9)

– Otherwise if CorDepthFlag is equal to 0, the following steps apply in order:

– For each value of i equal to 0, 1, and 2, when DvAvailable[ i ] is equal to 0, mvCand[ i ] is set to defaultDV

– Each component of the variable dv is derived as follows:

dv[ 0 ] = Median( mvCand[ 0 ][ 0 ], mvCand[ 1 ][ 0 ], mvCand[ 2 ][ 0 ] )

dv[ 1 ] = Median( mvCand[ 0 ][ 1 ], mvCand[ 1 ][ 1 ], mvCand[ 2 ][ 1 ] )

J.8.2.1.7.1 Depth-based derivation process for median luma motion vector prediction

Inputs to this process are:

–	the neighbouring partitions mbAddrN\mbPartIdxN\subMbPartIdxN (with N being replaced by A, B, or C),

–	the motion vectors mvLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference indices refIdxLXN (with N being replaced by A, B, or C) of the neighbouring partitions,,

–	the reference index refIdxLX of the current partition.

Output of this process is the motion vector prediction mvpLX.

When either partition mbAddrN\mbPartIdxN\subMbPartIdxN is not available or refIdxLXN is not equal to refIdxLX, mvLXN is derived as specified by the following ordered steps:

1.	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

2.	The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

3.	The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

4.	If CorDepthFlag is equal to 1, the modification process of inter-view motion vector in median luma motion vector prediction as specified in subclause .1 is invoked with depthPic being equal to DepthRefPicList0[ refIdxL0 ], mbx1 being equal to x1 and mby1 being equal to y1 as inputs and the output is assigned to the motion vector mvLXN. Otherwise if CorDepthFlag is equal to 0, defaultDV is assigned to the motion vector mvLXN.

Each component of the motion vector prediction mvpLX is given by the median of the corresponding vector components of the motion vector mvLXA, mvLXB, and mvLXC:

mvpLX[ 0 ] = Median( mvLXA[ 0 ], mvLXB[ 0 ], mvLXC[ 0 ] ) 	(J-10)
mvpLX[ 1 ] = Median( mvLXA[ 1 ], mvLXB[ 1 ], mvLXC[ 1 ] )	(J-11)

If CorDepthFlag is equal to 0, defaultDV is set to as follows.

defaultDV[0] = mvpLX[ 0 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 0 ]

defaultDV[1] = 0

J.8.2.1.7.2.1 Derivation process for the disparity vector and the inter-view reference

Inputs to this process are depth reference view component depthPic, the location of a top-left sample ( mbx1, mby1 ) of the current macroblock and the listSuffixFlag.

Outputs of this process are a picture InterViewPic, an offset vector dV and a variable interViewAvailable

The variable interViewAvailable is set equal to 0.

The variables partHeight and partWidth are derived as follows:

partHeight = 16 >> reduced_resolution_flag		(J-12)
partWidth = 16 >> reduced_resolution_flag		(J-13)

The following applies to derive an inter-view reference picture or inter-view only reference picture, InterViewPic, with X set to 1 when listSuffixFlag is 1 or 0 otherwise:

for( cIdx = 0;cIdx<num_ref_idx_l0_active_minus1 + 1 && !interViewAvailable; cIdx ++)
	if ( view order index of RefPicList0[ cIdx ] is equal to 0) {
		InterViewPic = RefPicList0[ cIdx ]		(J-14)
		interViewAvailable = 1
	}

When interViewAvailable is equal to 1  and CorDepthFlag is equal to 1, the variable refViewId is set equal to view_id of InterViewPic and the variable dV is set equal to the output of function DisparityForBlock( depthPic, ( mbx1 >> reduced_resolution_flag ), ( mby1 >> reduced_resolution_flag ), partWidth, partHeight, view_id, refViewId ). Otherwise if InterViewAvailable is equal to 1 and CorDepthFlag is equal to 0, the variable refViewId is set equal to view_id of InterViewPic and the variable dv is set equal to the defaultDV.