| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0148 |
| --- | --- |

| Title: | 3D-CE7 related: Depth-based motion vector prediction in texture-first coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Byeongdoo Choi, Yongjin Cho, Ho-Cheon Wey, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

The current 3D-AVC supports both texture-first and depth-first coding orders. In the depth-first coding order, 3D-AVC achieves the significantly high coding performance compared to MVC+D. However, it was reported that the performance of 3D-AVC is very small in the texture-first coding. One of the reasons why the current 3D-AVC has the low coding efficiency in the texture-first coding is that the depth-based motion vector prediction (DMVP) is not supported. Since the previously coded depth map for the motion vector prediction of the associated texture image is unavailable, DMVP is turned off. In order to improve the coding performance in the texture-first coding at the same time keep the current 3D-AVC design, this contribution proposes how to employ DMVP when the corresponding depth image is not available. The proposed method introduces four different solutions. At first, when the depth map does not exist, the zero disparity can be used instead of the disparity converted from a max pixel value in the corresponding depth block. As the second solution, the disparity converted from a middle value in the range can be employed in DMVP. The third proposed method uses the zero disparity for the first block and the updated disparity in JCT3V-D0186 [1] for the remaining blocks. The final method is that the disparity converted from the middle value and the updated disparity are used for the first block and the remaining blocks, respectively. Results demonstrate that the proposed method obtains the significantly high coding performance, as compared to MVC+D.

# Proposed DMVP in Texture-First Coding

[Current Design] DMVP provides the significantly high coding performance in the depth-first coding, but it is turned off in the texture-fist coding. Since a texture image is coded earlier than a depth image, the motion information of the corresponding depth map cannot be employed for the motion vector prediction of the texture image. This results in the low coding efficiency of the current 3D-AVC in the texture-first coding order. Fig. 1 (a) illustrates the current motion vector prediction in the dependant texture views. As shown in Fig. 1 (a), the conventional motion MVP is used when the coded depth image is not available, while DMVP is employed when the coded depth map exists.

[Proposed#1] When the coded depth map is not available, the proposed method uses the zero disparity for DMVP, as displayed in Fig. 1 (b).

[Proposed#2] When the coded depth map is not available, the disparity converted from a middle value in the range (ex. 8 bit image = 127) is employed for DMVP, as displayed in Fig. 1 (c).

[Proposed#3] When the coded depth map is not available, the proposed method uses the zero disparity for the first block and the updated disparity introduced in JCT3V-D0186 [1] for the remaining blocks, as shown in Fig. 1 (d).

[Proposed#4] When the coded depth map is not available, the proposed method employs the disparity converted from the middle value and the updated disparity in JCT3V-D0186 [1] for the remaining blocks, as shown in Fig. 1 (d).

(a) Current MVP                            (b) Proposed MVP #1

(c) Proposed MVP #2                            (b) Proposed MVP #3/4

Fig. 1. MVP design for texture-first coding

# Test Results

Test results of the proposed methods are summarized in Tables 1 to 5. Anchor is MVC+D in all the tests. Table 1 illustrates the coding performance of the current 3D-AVC in the texture-first coding. As reported in JCT3V-D0185 [2], the gain is very small. Tables 2 to 5 represent the performance of the proposed methods 1 to 4, respectively. When the proposed method uses the disparity converted from the middle value and the updated disparity introduced in JCT3V-D0186 for the remaining blocks, the maximum gains of about -15% and -13% are achieved on average in terms of the coded and synthesized PSNR, respectively. Since the proposed method does not change the basic processes in DMVP at all, the result of the performance in the depth-first coding is same as that in CTC.

Table 1. Performance of the current MVP in the texture-first coding

Table 2. Performance of the proposed method using the zero disparity in the texture-first coding

Table 3. Performance of the proposed method using the disparity converted from the middle value in the texture-first coding

Table 4. Performance of the proposed method using the zero disparity for the first block and the updated disparity in JCT3V-D0186 [1] for the remaining blocks in the texture-first coding

Table 5. Performance of the proposed method using the disparity converted from the middle value for the first block and the updated disparity in JCT3V-D0186 [1] for the remaining blocks in the texture-first coding

# Conclusion

By integrating the proposed methods on 3D-ATM 8.0r3, it significantly improves the coding performance in the texture-first coding. Since the modifications are simple, it is recommended to adopt the proposed method to the next 3D-ATM.

# Reference

[1] X. Zhao, Y. Chen, L. Zhang, and M. Karczewicz, “CE2.a related: Derived disparity vector in 3D-AVC,” JCT3V-D0186, Apr. 2013, Incheon, Korea.

[2] Y. Chen, J. Kang, L. Zhang, X. Zhao, Y.-K Wang, R. Joshi, and M. Karczewicz, “CE2.a related: MB-level NBDV for 3D-AVC,” JCT3V-D0185, Apr. 2013, Incheon, Korea.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).