| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0161 |
| --- | --- |

| Title: | 3D-CE2.h related: Depth to disparity vector conversion for non-horizontal camera arrangements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Yu-Pao Tsai, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, yupao.tsai, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, only horizontal information of the camera arrangement is coded, and only the horizontal component of a disparity vector (DV) can be converted from depth. The vertical component of the DV is always set equal to 0. Even if the vertical positions of the cameras are different, the vertical component of the DV cannot be converted due to the lack of vertical information of the camera arrangement. A depth to vertical DV conversion scheme along with high-level syntax on vertical camera parameters is proposed to enable the vertical component of the DV to be used under non-horizontal camera arrangements. The experimental results show that the proposed method functions correctly under common test condition and is able to support camera arrangements with different vertical positions.

# Introduction

3D video is represented using the Multiview Video plus Depth (MVD) format, in which a small number of captured views as well as associated depth maps are coded and multiplexed into a 3D video bitstream. After decoding the video and depth data, additional intermediate views suitable for rendering the 3D content on an auto-stereoscopic display can be synthesized using depth-image-based rendering (DIBR) techniques. For the purpose of view synthesis, camera parameters are additionally signaled in bitstreams.

In the current 3D-HEVC working draft (WD) text and reference software 3D-HTM [1], the camera parameters required for converting depth values into disparity vectors (DVs) are included in the bitstream. As only a linearly arranged camera setup is considered, the camera parameters include a scale factor and an offset, as shown in Equ. (1).

dv = ( s * v + o ) >> n,									(1)

In Equ. (1), dv is the converted disparity vector (DV), v is the depth sample value, s is the transmitted scale factor, o is the transmitted offset, and n is a shift parameter that depends on the required accuracy of the DV.

In the current 3D-HEVC, only horizontal information of the camera is coded in the bitstream, and only the horizontal component of the DV can be converted from depth. The vertical component of the DV is always set equal to 0. Even if the vertical positions of the cameras are different, the vertical component of the DV cannot be converted due to the lack of vertical information of the camera arrangement.

To solve this issue, a depth to vertical DV conversion along with high-level syntax (HLS) on vertical camera parameter is proposed to enable the vertical component of the DV to be used under non-horizontal camera arrangements. The camera parameter of the vertical part is also represented by a scale factor and an offset as the horizontal one. Then the vertical component of the DVs can be extracted through depth to vertical DV conversion and used by depth oriented tools such as DoNBDV [3] and VSP [4].

# Proposed scheme

We propose to use Equ. (1) for deriving both the horizontal and the vertical components of the DV.

As shown in Section 3.1, in the proposed depth to vertical DV conversion scheme, the vertical scale factor and vertical offset are transmitted in sequence parameter set (SPS) and slice header along with the horizontal ones. With the vertical scale factor cp_vert_scale, vertical offset cp_vert_off, and other parameters, DepthToVertDisparityB and DepthToVertDisparityF can be derived as described in the working draft changes H.7.4.5.1.1 of Sec. 3.2. Then, depth oriented coding tools such as depth-oriented neighboring block depth vector (DoNBDV) and view synthesis prediction (VSP) can then use DepthToVertDisparityB and DepthToVertDisparityF to extract the vertical component of DV from depth values.

# Working draft changes

The HLS suggestions are demonstrated in this section. The changes are highlighted by turquoise color.

## Syntax tables

- H.7.3.2.2.2 Sequence parameter set extension 2 syntax

| sps_extension2( ) { | Descriptor |
| --- | --- |
| if( !VpsDepthFlag[ nuh_layer_id ] ) { |  |
| cp_precision | ue(v) |
| cp_in_slice_header_flag | u(1) |
| if( !cp_in_slice_header_flag ) { |  |
| for ( i = 0; i < ViewId[ nuh_layer_id ]; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| cp_vert_scale[ i ] | se(v) |
| cp_vert_off[ i ] | se(v) |
| cp_inv_vert_scale_plus_vert_scale[ i ] | se(v) |
| cp_inv_vert_off_plus_vert_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- H.7.3.5.1.1 Slice header extension syntax

| slice_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_header_flag ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| cp_vert_scale[ i ] | se(v) |
| cp_vert_off[ i ] | se(v) |
| cp_inv_vert_scale_plus_vert_scale[ i ] | se(v) |
| cp_inv_vert_off_plus_vert_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

## Semantics

- H.7.4.2.2.1 Sequence parameter set extension 2 semantics

cp_precision specifies the precision of cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ]. The value of cp_precision shall be in the range of 0 to 5, inclusive.

cp_in_slice_header_flag equal to 0 specifies that the syntax elements of cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] are present in the sequence parameter set. A value of camera_parameter_in_slice_header_flag equal to 1 specifies that the cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] syntax elements are present in the slice segment header. When not present, cp_in_slice_header_flag is inferred to be equal to 0.

cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] specify conversion parameters for converting a depth value to a horizontal disparity value.

cp_vert_scale[ i ], cp_vert_off[ i ], cp_inv_vert_scale_plus_ vert_scale[ i ], and cp_inv_vert_off_plus_vert_off[ i ] specify conversion parameters for converting a depth value to a vertical disparity value.

- H.7.4.5.1.1 Slice header extension semantics

cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] specify conversion parameters for converting a depth value to a horizontal disparity value.

cp_vert_scale[ i ], cp_vert_off[ i ], cp_inv_vert_scale_plus_vert_scale[ i ], and cp_inv_vert_off_plus_vert_off[ i ] specify conversion parameters for converting a depth value to a vertical disparity value.

The arrays DepthToDisparityB[ i ][ d ] and DepthToVertDisparityB[ i ][ d ] specifying the horizontal and vertical disparity between the current view and the view with ViewIdx equal i corresponding to the depth value d in the view with ViewIdx equal to i and the arrays DepthToDisparityF[ i ][ d ] and DepthToVertDisparityF[ i ][ d ] specifying the horizontal and vertical disparity between the view with ViewIdx equal i and the current view corresponding to the depth value d in the current view is derived as specified in the following:

- The variable log2Div is set equal to BitDepthY − 1 + cp_precision.

- For d in range of 0 to ( ( 1 << BitDepthY )− 1), inclusive, the following applies:

  - For i in the range of 0 to ViewIdx − 1, the following applies:

    - offset = ( cp_off[ i ] << BitDepthY ) + ( ( 1 << log2Div ) >> 1 )	(Error: Reference source not found-1)

    - scale = cp_scale[ i ]		(Error: Reference source not found-2)

    - DepthToDisparityB[ i ][ d ] = ( scale * d + offset ) >> log2Div	(Error: Reference source not found-3)

    - invOffset = ( ( cp_inv_off_plus_off[ i ] − cp_off[ i ] ) << BitDepthY ) + ( ( 1 << log2Div ) >> 1 )	(Error: Reference source not found-4)

    - invScale = ( cp_inv_scale_plus_scale[ i ] − cp_scale[ i ] )	(Error: Reference source not found-5)

    - DepthToDisparityF[ i ][ d ] = ( invScale * d + invOffset ) >> log2Div 	(Error: Reference source not found-6)

    - vert_offset = ( cp_vert_off[ i ] << BitDepthY ) + ( ( 1 << log2Div ) >> 1 )	(Error: Reference source not found-8)

    - vert_scale = cp_vert_scale[ i ]		(Error: Reference source not found-9)

    - DepthToVertDisparityB[ i ][ d ] = ( vert_scale * d + vert_offset ) >> log2Div	(Error: Reference source not found-10)

    - invVertOffset = ( ( cp_inv_vert_off_plus_vert_off[ i ] − cp_vert_off[ i ] ) << BitDepthY ) + ( ( 1 << log2Div ) >> 1 )	(Error: Reference source not found-11)

    - invVertScale = ( cp_inv_vert_scale_plus_vert_scale[ i ] − cp_vert_scale[ i ] )	(Error: Reference source not found-12)

    - DepthToVertDisparityF[ i ][ d ] = ( invVertScale * d + invVertOffset ) >> log2Div 	(Error: Reference source not found-13)

- H.8.5.4.3 Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit.

- variable nSubBlkW and nSubBlkW specifying the conversion precision of the corresponding depth samples,

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of horizontal disparities values.

- a (nPSW)x(nPSH) array vertDisparitySamples of vertical disparities values.

Let refDepPels be an array of reconstructed depth samples of the depth view component with ViewIdx equal to depthViewIdx. The luma location (xTL, yTL) of top-left luma sample of a block in refDepPels is derived by

  - xTL = Clip3( 0, pic_width_in_luma_samples – nPSW, xP + ( ( mvDisp[ 0 ] + 2 ) >> 2 ) )	(Error: Reference source not found-7)
yTL = Clip3( 0, pic_height_in_luma_samples – nPSH, yP + ( ( mvDisp[ 1 ] + 2 ) >> 2 ) )	(Error: Reference source not found-8)

The array disparitySamples of size (nPSW)x(nPSH) is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH) –1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW) –1 ), inclusive, the following applies:

    - The variable maxDep is set equal to –1 and modified as specified in the following.

x = xTL + sBx * nSubBlkW
			y = yTL + sBy * nSubBlkH
			maxDep = Max( maxDep, refDepPels[ x ][ y ] )
			maxDep = Max( maxDep, refDepPels[ x ][ y + nSubBlkH – 1] )
			maxDep = Max( maxDep, refDepPels[ x + nSubBlkW – 1][ y ] )
			maxDep = Max( maxDep, refDepPels[ x + nSubBlkW – 1][ y + nSubBlkH – 1] )

    - The values of the array depthSamples disparitySamples and vertDisparitySamlpes are derived as specified in the following:

for ( yOff = 0; yOff < nSubBlkH; yOff++ )
				for( xOff = 0; xOff < nSubBlkW; xOff++ ) {
					x = sBx * nSubBlkW + xOff
					y = sBy * nSubBlkH + yOff
					disparitySamples[ x ][ y ] = DepthToDisparityB[ refViewIdx ][ maxDep ]
					vertDisparitySamples[ x ][ y ] = DepthToVertDisparityB[ refViewIdx ][ maxDep ]
				}

- H.8.5.2.2.7 View synthesis prediction process

Inputs to this process are:

- a location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

- a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

- the width and height of this prediction block, nPbW and nPbH,

- the prediction list indicator X

- a motion vector mvLX

- the reference index refIdxLX

Outputs of this process are:

–	an array predSamplesL of luma prediction samples,

–	an array predSamplesCb of chroma prediction samples for the component Cb

–	an array predSamplesCr of chroma prediction samples for the component Cr

The location ( xP, yP ) given in full-sample units of the upper-left luma samples of the current prediction block relative to the upper-left luma sample location of the current picture is derived by:

  - xP = xC + xB		(Error: Reference source not found-9)
yP = yC + yB		(Error: Reference source not found-10)

The reference picture consisting of an ordered two-dimensional array refPicL of luma samples and two ordered two-dimensional arrays refPicCb and refPicCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX as input.

The variable refViewIdx is set equal to the ViewIdx( RefPicListX[ refIdxLX ] ) and the variable depthViewIdx is set equal to RefViewIdx[ xC + xB ][ yC + yB ].

[Ed. (GT) In software refIdxLX is used to derive the depth picture, whereas the reference index of the texture picture is derived similar to the process specified in the derivation process for a view synthesis prediction merge candidate. ]

The derivation process for disparity sample arrays as specified in section  is invoked with the luma location ( xP, yP ), the disparity vector mvDisp being equal to mvLX, the variable refViewIdx, the variable depthViewIdx, the variables nPSW and nPSH, the variable nSubBlkW being equal 4 and the variable nSubBlkW being equal to 4 as the inputs, and the output is the horizontal disparity array disparitySamples and vertical disparity array vertDisparitySamples of size (nPSW)x(nPSH).

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units.

For each luma sample location ( xL = 0..nPbW−1, yL = 0..nPbH−1 ) inside the prediction luma sample array predSamplesL, the corresponding prediction luma sample value predSamplesL[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by

  - xIntL = xP + xL+ disparitySamples[ xL ][ yL ]		(Error: Reference source not found-11)
yIntL = yP + yL+ vertDisparitySamples[ xL ][ yL ]		(Error: Reference source not found-12)

  - xFracL = disparitySamples[ xL ][ yL ] & 3		(Error: Reference source not found-13)
yFracL = 0vertDisparitySamples[ xL ][ yL ] & 3		(Error: Reference source not found-14)

- The prediction luma sample value predSamplesL[ xL][ yL ] is derived by invoking the process specified in subclause 8.5.2.2.2.1 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicL given as input.

[Ed. (GT): As for inter prediction the treatment of colour planes for depth needs to be discussed. In software colour planes are set to 128 in VSP process. (#12)]

Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-eighth sample units.

For each chroma sample location ( xC = 0..nPbW/2−1, yC = 0..nPbH/2−1 ) inside the prediction chroma sample arrays predSamplesCb and predSamplesCr, the corresponding prediction chroma sample values predSampleLXCb[ xC ][ yC ] and predSamplesCr[ xC ][ yC ] are derived as follows:

- The variables xIntC, yIntC, xFracC, and yFracC are derived by

  - xIntC = ( xP / 2 ) + xC + disparitySamples[ xC << 1 ][ yC << 1 ]	(Error: Reference source not found-15)
yIntC = ( yP / 2 ) + yC + vertDisparitySamples[ xC << 1 ][ yC << 1 ]	(Error: Reference source not found-16)

  - xFracC = disparitySamples[ xC << 1][ yC << 1 ] & 7		(Error: Reference source not found-17)
yFracC = 0vertDisparitySamples[ xC << 1][ yC << 1 ] & 7	(Error: Reference source not found-18)

- The prediction sample value predSamplesCb[ xC ][ yC ] is derived by invoking the process specified in subclause 8.5.2.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicCb given as input.

- The prediction sample value predSamplesCr[ xC ][ yC ] is derived by invoking the process specified in subclause 8.5.2.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicCr given as input.

- H.8.5.4 Derivation process for disparity vectors

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

The variable checkParallelMergeFlag is derived as follows:

- If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

  - PredMode[ xC ][ yC ] is equal to MODE_SKIP.

  - PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xC ][ yC ] is equal to 1.

- Otherwise, checkParallelMergeFlag is set equal to 0.

The derivation process for a disparity vector from temporal neighbour block as specified in Error: Reference source not found is invoked with the luma location ( xC, yC ), and the variable nCS as inputs, and the outputs are the flag availableDV, the disparity vector mvDisp and the reference view order index refViewIdx.

            - ……

When availableDV is equal to 0 for each N being A1, B1, B0, A0, and B2, the following applies.

  - When availableDV is equal to 0 and availableFlagIvpMvN is equal to 1, the following applies:

      - mvDisp = ivpMvDispN		(Error: Reference source not found-19)
refViewIdx = refViewIdxN		(Error: Reference source not found-20)
availableDV = 1		(Error: Reference source not found-21)

When availableDV is equal to 0, refViewIdx is set equal to 0, and mvDisp is set equal to ( 0, 0 ).The variable mvRefinedDisp is set equal to mvDisp.

When depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following ordered steps apply:

  - The derivation process for disparity sample array as specified in subclause  is invoked with the luma locations xC, yC, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCS, the variable nPSH equal to nCS, the variable nSubBlkW equal to nCS, and the variable nSubBlkH equal to nCS, as the inputs, and the output is the horizontal disparity array disparitySamples and vertical disparity array vertDisparitySamples of size (nCS)x(nCS).

  - The horizontal component of the disparity vector mvRefinedDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].

  - The vertical component of the disparity vector mvRefinedDisp[ 0 ] is set equal to vertDisparitySamples[ 0 ][ 0 ].

[Ed. (GT): The disparity vector derivation has the problem related to refViewIdx as described in the editor's comment in subclause Error: Reference source not found and needs further discussion. (#5) ]

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xC.. ( xC + nCS − 1 ), y = yC..( yC + nCS− 1 ):

  - MvDisp[ x ][ y ] = ivpMvFlagLX		(Error: Reference source not found-22)
MvRefinedDisp[ x ][ y ] = mvRefinedDisp		(Error: Reference source not found-23)
RefViewIdx[ x ][ y ] = refViewIdx		(Error: Reference source not found-24)
DefaultDispFlag[ x ][ y ] = !availableDV		(Error: Reference source not found-25)

# Experiment results

The proposed depth to vertical DV conversion for both DoNBDV and VSP is integrated into HTM- 7.0r1 [1] and the simulations are run under the common test condition (CTC) [2].

Two tests are carried out: test 1 only applies the depth to vertical DV conversion on the DoNBDV part; test 2 applies the depth to vertical DV conversion on both DoNBDV and VSP.

The results of test 1 and test 2 are illustrated in Table 1 and Table 2, respectively. The anchors of test 1 and test 2 are both HTM-7.0r1 under CTC. The experimental results show that the proposed depth to vertical DV conversion induces no coding loss under horizontal linear camera arrangements. Further experiments with camera arrangements of different vertical positions can be started from this basis.

Table 1.  Results of the proposed depth to vertical DV conversion for DoNBDV (anchor: HTM-7.0r1 under CTC)

Table 2.  Results of the proposed depth to vertical DV conversion for DoNBDV and VSP (anchor: HTM-7.0r1 under CTC)

# Conclusion

In this contribution, a depth to vertical DV conversion along with HLS on vertical camera parameter is proposed to enable the vertical component of the DV to be used under non-horizontal camera arrangements.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1005, April, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.

- Y.-L. Chang, C.-L. Wu, Y.-P. Tsai, S. Lei, “3D-CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0131, January, 2013.

- D. Tian, F. Zou, A. Vetro, “CE1.h: Backward view synthesis prediction using neighbouring blocks,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0152, January, 2013