| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0162 |
| --- | --- |

| Title: | 3D-CE2.h related: Disparity range restriction for NBDV |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Yu-Pao Tsai, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, yupao.tsai, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Unrestricted neighboring block disparity vector (NBDV) may lead to larger cache size for storing motion data and larger memory bandwidth for transmitting interview reference. Two clipping operations for NBDV with different disparity ranges are proposed in this contribution. The disparity ranges are derived from depth ranges. One of the clipping operations for NBDV uses the converted depth range [0, 255], and the other one uses the converted depth range from the depth lookup table (DLT). The experimental results reportedly show that small BD-Rate gains are achieved, and 2~4% decoding time decrease can be observed.

# Introduction

In 3D video coding, the depth information helps to improve the coding efficiency by many different methods. Depth-based motion vector prediction (DMVP) in 3D-AVC uses the coded depth to improve the accuracy of the motion vector prediction. The depth map helps to identify interview candidate for motion parameter prediction. In view synthesis prediction proposed by MERL [1], provided that the texture and depth pair of a first view is coded/ decoded, a second view can be predicted by warping the first view to the second view position. The depth map helps the texture to be warped to correct position. Coding tools such as interview motion parameter prediction [2] further utilize the depth information to improve the prediction of the current block. The neighboring block disparity vector (NBDV) [3] in 3D-HEVC uses the disparity vector (DV) from interview prediction in the neighboring blocks to improve the motion vector prediction. Furthermore, the depth oriented neighboring block disparity vector (DoNBDV) [4] is proposed to improve the accuracy of DV by referring to the coded depth map in other coded reference views. In [5], it is proposed to restrict the vertical component of an interview disparity vector. With the restriction, decoders can start to decode a dependent view picture while decoding the corresponding picture in the reference view (e.g., base view). It can further help to estimate the additional bandwidth for the interview prediction. Fig. 1 shows the NBDV and DoNBDV distribution for each coding unit (displayed along the horizontal axis) in a coded bitstream of the first several pictures by encoding the Balloon sequence set with QP sets to 35. The vertical axis displays the values of the horizontal component of the disparity vectors. It is discovered that the DoNBDVs are confined to a range, but the NBDVs sometimes exceed the range. The NBDV comes from the DV of the neighboring blocks which are determined by disparity estimation between different views. When the DV of the neighboring blocks is not available, the NBDV is set to 0. So the disparity range of NBDV is not confined or fixed, and it leads to larger cache size while doing motion compensation or storing motion data for temporary use. The DoNBDV is converted from the depth values. An intrinsic depth range [0, 255] for the depth values exists, so DoNBDV can be confined. Furthermore, a real depth range depending on the depth map itself might be narrower than the intrinsic one. The depth lookup table (DLT) for simplified depth coding (SDC) [6] scans the first few depth pictures to extract the real depth range and save it in the DLT. A disparity range can be derived from either the intrinsic depth range or the real one.

In this contribution, a method to clip the NBDV by utilizing the intrinsic depth range or coded depth information is proposed. We propose to clip the NBDV by the converted disparity range from either the intrinsic depth range or the real depth range in DLT. Fig. 2 shows that the possible reference area for storing the motion data could be reduced if the NBDV is restricted by a disparity range.

Fig. 1 The NBDV and DoNBDV distribution for each CU of the first several texture pictures in the dependent views

Fig. 2 The possible reference area of the current NBDV scheme and the proposed NBDV clipping scheme

# Algorithm Description

We propose two methods to clip the NBDV in this contribution.

Method 1: Clip the NBDV with intrinsic depth range [0, 255]

- Obtain the NBDV mvDispNBDV and its reference view index refViewIdx by the original method.

- Convert the intrinsic depth range to disparity range:

Set maximal depth value Depthmax=255 and minimal depth value Depthmin=0, calculate the maximal disparity Dispmax and minimal disparity Dispmin by Equ. (1) and Equ. (2):

Dispmax = MAX( DepthToDisparityB[ refViewIdx ][ Depthmax ], DepthToDisparityB[ refViewIdx ][ Depthmin ])						(1)

Dispmin = MIN( DepthToDisparityB[ refViewIdx ][ Depthmax ], DepthToDisparityB[ refViewIdx ][ Depthmin ])						(2)

- Clip the NBDV mvDispNBDV by Equ. (3).

mvDispNBDV = Clip3( Dispmin, Dispmax, mvDispNBDV)						(3)

Method 2: Clip the NBDV with real depth range from DLT if DLT is available

- Obtain the NBDV mvDispNBDV and its reference view index refViewIdx by the original method.

- Obtain the depth range denoted by maximal depth value Depthmax and minimal depth value Depthmin from DLT.

Set maximal depth value Depthmax=255 and minimal depth value Depthmin=0.

Let layerId be the layer Id of the depth reference picture with viewId equals to refViewIdx.

If num_depth_values_in_dlt[ layerId ] != 0, set maximal depth value Depthmax= dlt_depth_value[num_depth_values_in_dlt[ layerId ]-1]  and minimal depth value Depthmin=dlt_depth_value[0]

- Convert the depth range to disparity range:

Calculate the maximal disparity Dispmax and minimal disparity Dispmin by Equ. (1) and Equ. (2) as described in method 1.

- Clip the NBDV mvDispNBDV by Equ. (3) as described in method 1.

In these two methods, a disparity range is derived from the depth range and presented as the maximal and minimal DVs. The clipping operation is done by the derived disparity range.

# Experiment Results

The proposed methods are integrated into HTM-7.0r1. The depth oriented tools, DoNBDV and VSP, are disabled to test the performance of NBDV only, and it also enables the stereo compatibility of the coded bitstream. Four tests are conducted. The NBDV clipping schemes by depth range [0, 255] and depth range from DLT are applied in test 1 and test 2. The NBDV clipping schemes by depth range [0, 255] and depth range from DLT are applied only on the merge mode in test 3 and test 4. Table 1, 2, 3, and 4 show the coding results of test 1, 2, 3, and 4, respectively. The anchor is HTM-7.0r1 with DoNBDV and VSP off. Minor coding gain is achieved in all the 4 tests. And as the depth range from DLT is applied, the disparity range can be reduced more than the depth range [0, 255]. 2~4% of decoding time decrease can be observed.

Table 1. NBDV clipping by the depth range [0, 255] with DoNBDV and VSP off (anchor: HTM-7.0r1 with DoNBDV and VSP off)

Table 2. NBDV clipping by the depth range from DLT with DoNBDV and VSP off (anchor: HTM-7.0r1 with DoNBDV and VSP off)

Table 3. NBDV clipping by the depth range [0, 255] only in merge mode with DoNBDV and VSP off (anchor: HTM-7.0r1 with DoNBDV and VSP off)

Table 4. NBDV clipping by the depth range from DLT only in merge mode with DoNBDV and VSP off (anchor: HTM-7.0r1 with DoNBDV and VSP off)

# Conclusion

In this contribution, two clipping operations for NBDV with different disparity ranges are proposed to reduce the cache size and memory bandwidth while accessing the interview reference data. The disparity ranges are derived from depth ranges. One of the clipping operations for NBDV uses the converted depth range [0, 255], and the other one uses the converted depth range from DLT. The experimental results reportedly show that small BD-Rate gains are achieved when enabling stereo compatibility, and 2~4% decoding time decrease can be observed.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- D. Tian, D. Graziosi, Y. Wang, N. Cheung, A. Vetro, “Mitsubishi Response to MPEG Call for Proposal on 3D Video Coding Technology”, ISO/IEC JTC1/SC29/WG11 document M22663, Geneva, Switzerland, November 2011.

- Heiko Schwarz et al, “Description of 3D Video Technology Proposal by Fraunhofer HHI(HEVC compatible; configuration A)” , ISO/IEC JTC1/SC29/WG11 document M22570, Geneva, Switzerland, November 2011.

- Li Zhang et al, “CE5.h: Disparity vector generation results”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-JCT3V-A0097, July 2012, Stockholm.

- Y.-L. Chang, C.-L. Wu, Y.-P. Tsai, S. Lei, “3D-CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0131, January, 2013.

- O. Nakagami et al, “MV-HEVC: Vertical length restriction of inter-view vector for HEVC simple 3D extension”, Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B0037, Oct., 2012.

- F. Jäger, “3D-CE6.h Results on Simplified Depth Coding with an optional Depth Lookup Table,” Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, Shanghai, China, JCT3V-B0036, 2012.