| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0166 |
| --- | --- |

| Title: | CE7-related / 3D-AVC: indication of depth sampling grid location and depth cropping rectangle and their impact on disparity derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted that the depth sampling grid might have a different location, width and height compared to the texture sampling grid of the same view and that the cropping rectangle for depth views might differ from that of the texture views. The contribution proposes to add support for these cases in 3D-AVC with the following changes:

- A cropping rectangle for depth views is optionally indicated in seq_parameter_set_3davc_extension( ).

- Horizontal and vertical sampling grid offsets between texture and depth view components of the same view are optionally indicated in seq_parameter_set_3davc_extension( ).

- Disparity derivation (the DisparityForBlock function) is modified to take the sampling grid offset into account when converting a texture block location to a depth block location.

- Disparity derivation (the DisparityForBlock function) is modified to clip the depth coordinates to reside within the cropping rectangle of the depth view.

# Introduction

It is asserted that the sampling grid location of a texture view component might have an offset relative to the sampling grid location of the depth view component of the same view and that the sampling grids might have different heights or widths, for example because of the following reasons:

- The depth view might have been captured with a specific ranging sensor that can have a different focal length and field of view compared to those of the color image cameras.

- If the depth views are generated using stereo matching from two input views, the depth view components contain only the part where the two views overlap (i.e. a disparity match is found in the stereo matching).

For the same reasons as above, depth view components might also require the use of a cropping rectangle that differs from that of the texture view components of the same view. If a flexible depth-to-texture resolution is adopted (as proposed in JCT3V-E0035), a depth cropping rectangle would also be needed to indicate which part of depth view components include sample values that are to be used in the disparity derivation process.

# Proposal

The proposal can be summarized as follows:

- A cropping rectangle for depth views is optionally indicated in seq_parameter_set_3davc_extension( ).

- Horizontal and vertical sampling grid offsets between texture and depth view components of the same view are optionally indicated in seq_parameter_set_3davc_extension( ).

- Disparity derivation (the DisparityForBlock function) is modified to take the sampling grid offset into account when converting a texture block location to a depth block location.

- Disparity derivation (the DisparityForBlock function) is modified to clip the depth coordinates to reside within the cropping rectangle of the depth view.

# Proposed specification text

The proposed specification text is edited on top of the proposed specification text in contribution JCT3V-E0035 (“CE7: Removal of texture-to-depth resolution ratio restrictions”). The reason is that both JCT3V-E0035 and this contribution modify the DisparityForBlock function. The changes proposed in JCT3V-E0035 are indicated in red, whereas the changes proposed in this contribution are indicated in green.

| seq_parameter_set_3davc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| reduced_resolution_flag | 0 | u(1) |
| if( reduced_resolution_flag ) { |  |  |
| depth_pic_width_in_mbs_minus1 | 0 | ue(v) |
| depth_pic_height_in_map_units_minus1 | 0 | ue(v) |
| depth_hor_mult_minus1 | 0 | ue(v) |
| depth_ver_mult_minus1 | 0 | ue(v) |
| depth_hor_div | 0 | ue(v) |
| depth_ver_div | 0 | ue(v) |
| } |  |  |
| depth_frame_cropping_flag | 0 | u(1) |
| if( depth_frame_cropping_flag ) { |  |  |
| depth_frame_crop_left_offset | 0 | ue(v) |
| depth_frame_crop_right_offset | 0 | ue(v) |
| depth_frame_crop_top_offset | 0 | ue(v) |
| depth_frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| grid_pos_num_views | 0 | ue(v) |
| for( i = 0; i < grid_pos_num_views; i++ ) { |  |  |
| grid_pos_view_id[ i ] | 0 | ue(v) |
| grid_pos_x[ grid_pos_view_id[ i ] ] | 0 | se(v) |
| grid_pos_y[ grid_pos_view_id[ i ] ] | 0 | se(v) |
| } |  |  |
| … |  |  |

reduced_resolution_flag equal to 1 specifies that the depth view components of a view component pair have a lower spatial resolution than the luma component of the texture view component of the same view component pair. reduced_resolution_flag equal to 0 specifies that when both depth view components and texture view components are present, they have the same spatial resolution.

The values of pic_width_in_mbs_minus1 and pic_height_in_map_units_minus1 are inferred to be equal to depth_pic_width_in_mbs_minus1 and depth_pic_height_in_map_units_minus1 for depth views for which this 3D-AVC sequence parameter set is an active view 3D-AVC sequence parameter set.

depth_hor_mult_minus1, depth_ver_mult_minus1, depth_hor_div and depth_ver_div are used for specifying the function DisparityForBlock.

depth_frame_cropping_flag equal to 0 specifies that the frame cropping offset parameters for depth view components follow next in the sequence parameter set. depth_frame_cropping_flag equal to 0 specifies that the frame cropping offset parameters for depth view components are not present.

When depth_frame_cropping_flag is equal to 0, the values of depth_frame_crop_left_offset, depth_frame_crop_right_offset, depth_frame_crop_top_offset, and depth_frame_crop_bottom_offset shall be inferred to be equal to 0.

The values of frame_crop_left_offset, frame_crop_right_offset, frame_crop_top_offset, frame_crop_bottom_offset are inferred to be equal to depth_frame_crop_left_offset, depth_frame_crop_right_offset, depth_frame_crop_top_offset, and depth_frame_crop_bottom_offset for depth views for which this 3D-AVC sequence parameter set is an active view 3D-AVC sequence parameter set.

Let the variables DepthCropLeftCoord, DepthCropRightCoord, DepthCropTopCoord and DepthCropBottomCoord be derived from the values of PicWidthInSamplesL, CropUnitX, CropUnitY, FrameHeightInMBs that apply to depth view components as follows:

DepthCropLeftCoord = CropUnitX * depth_frame_crop_left_offset
DepthCropRightCoord = PicWidthInSamplesL − ( CropUnitX * depth_frame_crop_right_offset + 1 )
DepthCropTopCoord = CropUnitY * depth_frame_crop_top_offset
DepthCropBottomCoord = ( 16 * FrameHeightInMbs ) − ( CropUnitY * depth_frame_crop_bottom_offset + 1 )

grid_pos_num_views specifies the number of views for which grid_pos_view_id[ i ], grid_pos_x[ grid_pos_view_id[ i ] ] and grid_pos_y[ grid_pos_view_id[ i ] ] are present. grid_pos_num_views shall be in the range of 0 to 1024, inclusive.

grid_pos_view_id[ i ] specifies a view_id value of a texture view.

grid_pos_x[ grid_pos_view_id[ i ] ] specifies a horizontal offset of a depth sampling grid relative to the luma texture sampling grid in texture luma sample units.

grid_pos_y[ grid_pos_view_id[ i ] ] specifies a vertical offset of a depth sampling grid relative to the luma texture sampling grid in texture luma sample units.

When no value of grid_pos_view_id[ i ] is equal to a view_id value of a texture view, grid_pos_x[ view_id ] and grid_pos_y[ view_id ] are inferred to be equal to 0.

grid_pos_x[ grid_pos_view_id[ i ] ] and grid_pos_y[ grid_pos_view_id[ i ] ]  are used for specifying the function DisparityForBlock.

- J.8.2.	3D-AVC inter prediction, inter-view prediction, view synthesis prediction and adaptive luminance compensation

…

The function DisparityForBlock( depthPic, textureXx1, textureYy1, blWidth, blHeight, srcViewId, refViewId ) is specified to return maxDisp specified with the following ordered steps.

- The variables depthX, depthY, blWidth and blHeight are specified as follows:

dHM = depth_hor_mult_minus1 + 1
dHD = depth_hor_mult_minus1 + depth_hor_div + 1
dVM = depth_ver_mult_minus1 + 1
dVD = depth_ver_mult_minus1 + depth_ver_div + 1
depthX = Clip3( DepthCropLeftCoord, DepthCropRightCoord,
	( textureX + grid_pos_x[ srcViewId ] ) * dHM / dHD )
depthY = Clip3( DepthCropTopCoord, DepthCropBottomCoord,
 	( textureY + grid_pos_y[ srcViewId ] ) * dVM / dVD )
depthXN = Clip3( DepthCropLeftCoord, DepthCropRightCoord,
	( textureX + grid_pos_x[ srcViewId ] + 15 ) * dHM / dHD )
depthYN = Clip3( DepthCropTopCoord, DepthCropBottomCoord,
 	( textureY + grid_pos_y[ srcViewId ] + 15 ) * dVM / dVD )
blWidth = depthXN – depthX + 1
blHeight = depthYN – depthY + 1

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < blHeight; j += ( blHeight – 1 ) )
	for( i = 0; i < blWidth; i += ( blWidth – 1 ) )		(J-1)
		if( depthPic[ depthXx1 + i, depthYy1 + j ] > maxDepth ) 
			maxDepth = depthPic[ depthXx1 + i, depthYy1 + j ]

- The variable maxDisp is specified as follows:

srcIndex = ViewIdTo3DVAcquisitionParamIndex( srcViewId )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )	(J-2)
maxDisp = Disparity( maxDepth, srcIndex, refIndex)

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).