- H.8.5.2.2.5 Illumination compensated sample prediction process

Inputs to this process are:

- a location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

- the size of current luma coding block nCS,

- a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

- the width and height of this prediction block, nPbW and nPbH,

- two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

- prediction list utilization flags, predFlagL0 and predFlagL1,

- reference indices, refIdxL0 and refIdxL1,

- motion vector mvL0 and mvL1

- colour component index, cIdx,

Outputs of this process are:

- the (nPbW)x(nPbH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

- The variable shift1 is set equal to 14 − bitDepth and the variable shift2 is set equal to 15 − bitDepth,

- The variable offset1 is derived as follows.

  - If shift1 is greater than 0, offset1 set equal to 1 << ( shift1 − 1 ).

  - Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

- The variable offset2 is set equal to 1 << ( shift2 − 1 ).

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

The derivation process for illumination compensation mode availability and parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of the current luma coding block nCS, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices refIdxL0 and refIdxL1, motion vectors mvL0 and mvL1, the bit depth of samples, bitDepth, a variable cIdx specifying colour component index as the inputs and the outputs are the flags puIcFlagL0 and puIcFlagL1, the variables icWeightL0 and icWeightL1 specifying weights for illumination compensation, the variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation, the variables icShiftL0 and icShiftL1 specifying bit shifts for illumination compensation.

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x ][ y ] with x = 0..(nPbW)−1 and y = 0..(nPbH)−1 are derived as follows.

- If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

    - clipPredVal= Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 ) >> shift1 )	(Error: Reference source not found-1)

    - predSamples[ x ][ y ] = !puIcFlagL0 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL0 ) >> icShiftL0 ) + icOffsetL0 ) 	(Error: Reference source not found-2)

- Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

    - clipPredVal = Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1 )	(Error: Reference source not found-3)

    - predSamples[ x ][ y ] = !puIcFlagL1 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL1 ) >> icShiftL1 ) + icOffsetL1 ) 	(Error: Reference source not found-4)

- Otherwise,

    - predVal0 = !puIcFlagL0 ? predSamplesL0[ x ][ y ] : 
( ( predSamplesL0[ x ][ y ] * icWeightL0 ) >> icShiftL0  ) + ( icOffsetL0 << shift1 ) ) 	(Error: Reference source not found-5)

    - clipPredVal= Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 ) >> shift1 )	(Error: Reference source not found-6)

    - predVal0 = !puIcFlagL0 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL0 ) >> icShiftL0 ) + icOffsetL0 ) 	(Error: Reference source not found-7)

    - predVal1 = !puIcFlagL1 ? predSamplesL1[ x ][ y ] : 
( ( predSamplesL1[ x ][ y ] * icWeightL1 ) >> icShiftL1 ) + ( icOffsetL1 << shift1 ) ) 	(Error: Reference source not found-8)

    - clipPredVal = Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1 )	(Error: Reference source not found-9)

    - predVal1 = !puIcFlagL1 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL1 ) >> icShiftL1 ) + icOffsetL1 ) 	(Error: Reference source not found-10)

- predSamples[ x ][ y ] = 
Clip3( 0, ( 1 << bitDepth ) − 1, ( predVal0 + predVal1 + offset2 ) >> shift2 )	(Error: Reference source not found-11)