| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0171 |  |
| --- | --- | --- |

| Title: | 3D-CE1.h related : Simplified view synthesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1,3,Yi-Wen Chen2, Jian-Liang Lin2, Jicheng An1, Kai Zhang1, Shawmin Lei2, Siwei Ma4, Debin Zhao3, Wen Gao4 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 3 Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 4School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {na.zhang, yiwen.chen, jl.lin, jicheng.an, kai.zhang, shawmin.lei}@mediatek.com {swma, dbzhao, wgao} @jdl.ac.cn |
| Source: | MediaTek Inc, Harbin Institute of Technology, Peking University |  |  |

_____________________________

# Abstract

Two simplifications for view synthesis prediction (VSP) are proposed. First, in HTM-7.0, when constructing the merge candidate list, if a spatial neighbor of the current prediction unit (PU) utilizes VSP mode, disparity vector (DV) information and VSP mode are both inherited from the spatial neighbor. In this case, it may need to access multiple depth blocks in multiple reference views for performing VSP process of the current PU. To reduce the memory bandwidth of the depth data access and unify the depth data accessed by depth oriented neighboring block disparity vector (DoNBDV) and VSP, it is proposed that only the derived NBDV of the current coding unit (CU) can be used to fetch a depth block in the reference view for performing VSP process. Second, currently VSP mode flags need to be kept in a line memory to inform whether a spatial neighbor of the current PU is VSP coded. It is proposed to remove the line memory by disallowing inheritance of the VSP modes from neighbors belonging to the upper coding tree unit (CTU) row. Experimental results show that both of the simplifications introduce no overall BD-rate change compared to HTM-7.0r1.

# Introduction

In HTM-7.0r1 [1], VSP uses the derived NBDV of the current CU to derive a depth block for performing the backward warping operation, so VSP in HTM-7.0r1 is also called backward view synthesis prediction (BVSP). It is noted that, in the current design, when constructing the merge candidate list, if a spatial neighbor of the current PU utilizes VSP mode, DV information and VSP mode are both inherited from the spatial neighbor. Then NBDV of the spatial neighbor will be used to fetch a depth block in the depth image of the reference view for performing VSP process of the current PU. As a result, it may need to access multiple depth blocks in multiple reference views for performing VSP process of the current PU as shown in Fig. 1. In the scheme of DoNBDV, NBDV is used to retrieve the virtual depth in the reference view to derive a refined DV as shown in Fig. 1.

Fig. 1.  The depth data accessed by DoNBDV and VSP in HTM-7.0r1

Moreover, VSP is used as a common disparity compensated prediction (DCP) candidate for the following modules: temporal merging candidate derivation, motion parameter inheritance for depth coding, depth oriented neighboring block disparity vector (DoNBDV), adaptive motion vector prediction (AMVP), and deblocking filter. VSP is only referred as a VSP candidate for spatial merging candidate derivation by accessing BVSP mode flag as shown in Fig. 2(a). To inform whether a spatial neighbour of the current PU is VSP coded, a line memory is needed to store the BVSP mode flags of neighbour blocks belonging to the upper CTU row(also termed LCU: largest coding unit).

(a)                                        (b)

Fig. 2.  Illustration of spatial neighbors coded with BVSP for the current PU in spatial merging candidate derivation: (a) HTM-7.0r1; (b) proposed

In this invention, there are two purposes. One is to unify the depth data accessed by DoNBDV and VSP for memory bandwidth reduction, and the other is to remove the line memory used for BVSP mode flags for reducing memory requirement.

# Proposed schemes

In this contribution, we propose two simplifications for VSP:

- Unify the depth data accessed by DoNBDV and VSP

If a spatial neighbor is coded using VSP mode, the spatial merging candidate derived from the spatial neighbor also uses the derived NBDV of the current CU to fetch a depth block in the reference view. In this way, the depth data accessed by DoNBDV and VSP are unified as shown in Fig. 3, and also the memory bandwidth of accessing depth data is reduced.

Fig. 3.  The depth data accessed by DoNBDV and the proposed VSP

- Reduce memory requirement for BVSP mode flag

A spatial neighbour coded with VSP mode is only referred as a VSP candidate for spatial merging candidate derivation when the spatial neighbor belongs to the current CTU row. If the spatial neighbour coded with VSP mode crosses the CTU row boundary for spatial merging candidate derivation, it will be used as a common DCP candidate as shown in Fig. 2(b).

# Experimental results

The proposed two simplifications have been integrated into HTM-7.0r1 [1], and all the simulations are conducted under common test conditions [2]. The detailed experimental results of the combined results, simplification 1, and simplification 2 are shown in Table 1, Table 2, and Table3, respectively. It can be seen that both of the simplifications introduce no overall BD-rate change compared to HTM-7.0r1.

Table 1.  Experimental results of the two proposed simplifications

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 99.0% | 99.7% |
| Kendo | 0.2% | -0.1% | 0.0% | 0.0% | 0.1% | 99.3% | 97.0% | 99.5% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99.8% | 94.2% | 99.7% |
| GT_Fly | 0.8% | 0.8% | 0.2% | 0.1% | 0.1% | 97.8% | 95.4% | 100.3% |
| Poznan_Hall2 | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 99.1% | 94.1% | 97.4% |
| Poznan_Street | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 96.6% | 102.4% |
| Undo_Dancer | 0.8% | 0.8% | 0.2% | 0.2% | 0.1% | 98.7% | 101.9% | 98.4% |
| 1024x768 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 96.7% | 99.7% |
| 1920x1088 | 0.4% | 0.4% | 0.1% | 0.1% | 0.1% | 98.9% | 97.0% | 99.6% |
| average | 0.3% | 0.2% | 0.1% | 0.0% | 0.0% | 99.2% | 96.9% | 99.6% |

Table 2.  Experimental results of the proposed simplification 1

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 98.4% | 101.1% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 98.8% | 100.2% | 99.8% |
| Newspaper_CC | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 99.7% | 96.1% | 97.4% |
| GT_Fly | 0.3% | 0.1% | 0.0% | 0.0% | 0.0% | 98.7% | 94.8% | 100.6% |
| Poznan_Hall2 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 94.5% | 99.4% |
| Poznan_Street | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 93.6% | 100.5% |
| Undo_Dancer | 0.6% | 0.6% | 0.1% | 0.1% | 0.0% | 100.0% | 107.6% | 99.5% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.4% | 98.2% | 99.4% |
| 1920x1088 | 0.3% | 0.2% | 0.1% | 0.0% | 0.0% | 99.6% | 97.6% | 100.0% |
| average | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 99.5% | 97.9% | 99.8% |

Table 3.  Experimental results of the proposed simplification 2

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 99.5% | 100.6% |
| Kendo | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.3% | 97.1% | 99.3% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 92.8% | 98.6% |
| GT_Fly | 0.8% | 0.4% | 0.1% | 0.1% | 0.1% | 98.5% | 101.3% | 98.4% |
| Poznan_Hall2 | -0.1% | 0.2% | 0.0% | 0.0% | 0.1% | 99.2% | 93.8% | 98.7% |
| Poznan_Street | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 99.0% | 102.2% |
| Undo_Dancer | 0.6% | 0.5% | 0.1% | 0.1% | 0.1% | 99.5% | 104.7% | 98.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 96.5% | 99.5% |
| 1920x1088 | 0.4% | 0.3% | 0.1% | 0.1% | 0.1% | 99.3% | 99.7% | 99.5% |
| average | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 99.4% | 98.3% | 99.5% |

# Conclusion

This contribution proposes two simplifications for VSP. Both of the simplifications introduce no overall BD-rate change compared to HTM-7.0r1 while the memory bandwidth and memory requirement are reduced.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM-7.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.