| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0173 |
| --- | --- |

| Title: | 3D-CE3.h related: Fast encoder decision for texture coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1,3,Yi-Wen Chen2, Jian-Liang Lin2, Jicheng An1, Kai Zhang1, Shawmin Lei2, Siwei Ma4, Debin Zhao3, Wen Gao4 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 3 Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 4School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {na.zhang, yiwen.chen, jl.lin, jicheng.an, kai.zhang, shawmin.lei}@mediatek.com {swma, dbzhao, wgao} @jdl.ac.cn |
| Source: | MediaTek Inc, Harbin Institute of Technology, Peking University |  |  |

_____________________________

# Abstract

This contribution proposes a fast encoding algorithm to encode the texture pictures of dependent views. The proposed fast algorithm uses two strategies to accelerate mode decision by exploiting the inter-view correlation. One is to make an early merge mode decision, and the other one brings an early CU splitting termination. Experimental results show that the proposed algorithm can achieve 47% encoding time reduction with overall 0.1% BD-rate reduction under the common test condition.

# Introduction

In the current HTM, the encoder examines all the prediction modes and the coding unit (CU) splitting structures to find the optimal coding modes according to the rate-distortion (RD) costs when encoding a coding tree unit (CTU). Specifically, the RD costs for 11 prediction modes, i.e., inter/merge/skip 2Nx2N, inter/merge 2NxN, inter/merge Nx2N, inter/merge NxN, inter/merge 2NxnU, inter/merge 2NxnD, inter/merge nLx2N, inter/merge nRx2N, intra 2Nx2N , intra NxN, and intra PCM, are examined. Among them, the motion estimation invoked for inter modes is the most time-consuming part. In addition, splitting the current CU into 4 sub-CUs and examining the RD performance of smaller CUs are also highly time-consuming. In this contribution, we propose a fast encoding algorithm to avoid the two time-consuming parts without affecting the coding performance.

# Proposed schemes

In this contribution, we propose a fast encoding algorithm using early merge mode decision and early CU splitting termination by exploiting the inter-view correlation as depicted in Fig. 1. Since the current 3D-HEVC already performs depth quadtree (QT) prediction from the texture QT to save encoding time and reduce the bit-rates, the proposed fast algorithm is only applied to the textures of dependent views. The details of the proposed fast mode decision algorithm are elaborated as follows.

Fig. 1.  The proposed fast encoding algorithm

## Early merge mode decision

According to the statistics, if the following two conditions are true, all prediction units (PUs) within the current CU will probably be coded as merge modes and only the merge modes need to be examined for all the PUs within the current CU. In other words, we could omit the examination of intra prediction modes and inter modes, in which the most time-consuming part, motion estimation process, could be skipped.

Condition 2.1.1

All the five inter-view neighbouring blocks (the corresponding block of the current CU and the four direct adjacent blocks of the corresponding block in the reference view as shown in Fig. 2) are coded as merge modes.

Condition 2.1.2

The RD performance of skip mode is better than 2Nx2N merge mode for the current CU.

Fig. 2.  The five inter-view neighboring blocks of the current CU

## Early CU splitting termination

If the following two conditions are true, no further CU splitting is needed for the current CU.

Condition 2.2.1

The CU splitting depth of the current CU is equal to or larger than the maximum depth of the CU splitting depths of the five inter-view neighboring blocks.

Condition 2.2.2

Skip mode is selected as the best prediction mode for the current CU after checking all the possible prediction modes.

# Experimental results

The proposed fast encoding algorithm has been integrated into HTM 7.0r1 [1], and all the tests were conducted under the common test conditions [2]. The experimental results of only applying early merge mode decision, only applying early CU splitting termination, and the combination of the two approaches are shown in Table1, Table2, and Table3, respectively. From the simulation results, we can see the proposed schemes of early merge mode decision and early CU splitting termination could reduce the encoding time by 32% and 39% with 0.0% and 0.2% BD-rate reductions, respectively. Besides, combining the two schemes could significantly reduce the encoding time by 47% with 0.1% overall BD-rate reduction.

Table 1.  Experimental results of early merge mode decision

|  | Video 1 | Video 2 | video/Video bitrate | video/total bitrate | synth / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 71.3% | 104.3% | 100.0% |
| Kendo | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 71.2% | 98.2% | 100.7% |
| Newspapercc | -0.2% | 0.0% | 0.0% | 0.1% | 0.0% | 66.3% | 99.3% | 97.6% |
| GhostTownFly | 0.2% | -0.1% | 0.1% | 0.1% | 0.0% | 73.5% | 98.5% | 100.5% |
| PoznanHall2 | 0.3% | 0.6% | 0.5% | 0.5% | 0.1% | 61.8% | 98.5% | 96.5% |
| PoznanStreet | -0.3% | -0.2% | 0.1% | 0.1% | 0.0% | 63.6% | 94.7% | 100.3% |
| UndoDancer | -0.2% | -0.2% | 0.1% | 0.1% | 0.0% | 67.4% | 101.2% | 98.5% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 69.6% | 100.6% | 99.4% |
| 1920x1088 | 0.0% | 0.0% | 0.2% | 0.2% | 0.0% | 66.5% | 98.2% | 99.0% |
| average | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 67.9% | 99.2% | 99.2% |

Table 2.  Experimental results of early CU splitting termination

|  | Video 1 | Video 2 | video/Video bitrate | video/total bitrate | synth / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.3% | 0.2% | 0.2% | -0.1% | 62.3% | 103.3% | 101.3% |
| Kendo | -0.1% | -0.3% | 0.2% | 0.3% | 0.0% | 63.5% | 100.9% | 100.1% |
| Newspapercc | -0.3% | -0.5% | 0.0% | 0.1% | 0.1% | 60.4% | 95.2% | 98.6% |
| GhostTownFly | -1.8% | -2.1% | -0.1% | -0.2% | -0.3% | 64.8% | 96.9% | 99.4% |
| PoznanHall2 | -1.5% | -1.3% | -0.3% | -0.2% | -0.4% | 53.4% | 95.3% | 98.6% |
| PoznanStreet | -0.9% | -1.0% | -0.1% | -0.1% | -0.2% | 57.9% | 104.5% | 100.6% |
| UndoDancer | -0.8% | -1.0% | 0.0% | 0.0% | -0.1% | 62.8% | 105.7% | 99.3% |
| 1024x768 | -0.2% | -0.3% | 0.1% | 0.2% | 0.0% | 62.0% | 99.8% | 100.0% |
| 1920x1088 | -1.2% | -1.3% | -0.1% | -0.1% | -0.3% | 59.7% | 100.6% | 99.5% |
| average | -0.8% | -0.9% | 0.0% | 0.0% | -0.2% | 60.7% | 100.2% | 99.7% |

Table 3.  Experimental results of the combination

|  | Video 1 | Video 2 | video/Video bitrate | video/total bitrate | synth / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.3% | 0.3% | 0.4% | -0.1% | 54.8% | 101.2% | 99.5% |
| Kendo | -0.1% | -0.3% | 0.3% | 0.4% | 0.1% | 56.5% | 97.5% | 100.5% |
| Newspapercc | -0.4% | -0.5% | 0.1% | 0.2% | 0.1% | 51.9% | 93.2% | 100.0% |
| GhostTownFly | -1.8% | -2.2% | -0.1% | -0.1% | -0.3% | 57.7% | 99.1% | 98.4% |
| PoznanHall2 | -1.1% | -1.0% | 0.3% | 0.3% | -0.3% | 45.1% | 91.8% | 97.0% |
| PoznanStreet | -0.9% | -1.0% | 0.0% | 0.1% | -0.2% | 49.0% | 93.9% | 100.0% |
| UndoDancer | -1.0% | -1.2% | 0.1% | 0.1% | -0.1% | 53.8% | 104.4% | 99.8% |
| 1024x768 | -0.2% | -0.4% | 0.2% | 0.3% | 0.0% | 54.4% | 97.3% | 100.0% |
| 1920x1088 | -1.2% | -1.3% | 0.1% | 0.1% | -0.2% | 51.4% | 97.3% | 98.8% |
| average | -0.8% | -0.9% | 0.1% | 0.2% | -0.1% | 52.7% | 97.3% | 99.3% |

# Conclusion

This contribution proposes a fast encoding algorithm including early merge mode decision and early CU splitting termination by exploiting the inter-view correlation. Experimental results show that applying the proposed fast encoding algorithm to the textures of the dependent views can achieve 47% encoding time reduction with 0.1% overall BD-rate saving.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM-7.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.