| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27th Jul – 2nd Aug. 2013 | Document: JCT3V-E0174 |
| --- | --- |

| Title: | 3D-CE3.h related: A disparity derived depth coding method in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a disparity derived depth (DDD) coding method in 3D-HEVC. When the collocated texture block of the current depth block is predicted by disparity-compensated prediction, a DDD candidate will be inserted into the merging candidate list right after the texture candidate. In a DDD candidate, prediction samples are derived from the disparity vector in the collocated texture block. Experimental results show that the proposed method can achieve 0.22% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], samples in the depth component are used to derive disparity vectors for texture component coding. A disparity vector (DVx, 0) can be derived from its corresponding depth value (dep) by a linear relationship as

,

where w and b are two camera parameters. In 3D-HEVC, Equ. (1) is implemented in an integer form thus w and b can be transmitted from the encoder to the decoder as integers.

# Proposed method

A disparity derived depth (DDD) coding method is proposed to improve the coding efficiency for the depth component. It can be seen from Equ. (1) that the conversion between a depth value dep and DVx is reversible. Thus, a depth value can also be derived from its corresponding disparity vector (DVx, DVy) as

.

When the collocated texture block of the current depth block is predicted by disparity-compensated prediction (DCP), a DDD candidate will be inserted into the merging candidate list right after the texture candidate. In a DDD candidate, all prediction samples in the current block are set as dep calculated by (2), which is implemented in an integer form. Fig. 1 shows the procedure to derive a depth value from its corresponding disparity vector. It should be noted that the DDD candidate is disabled for the base view since DCP is applied only on dependent views.

Fig. 1.  A depth value derived from its corresponding disparity vector

# Experimental results

The proposed methods are integrated on HTM-7.0r1 [2]. Table 1 shows the results of the proposed method under common test condition [3]. It can be seen that the proposed method can achieve 0.22% BD-rate saving for the synthesized views.

Table 1.  Performance of the proposed method

# Conclusion

In this proposal, a disparity derived depth coding method is proposed. The proposed method utilizes the disparity vector in the collocated texture block to derive a depth prediction value. Experimental results show that 0.22% BD-rate saving for the synthesized views can be achieved.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1005, April, 2013.

- HTM-7.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.