| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27th Jul – 2nd Aug. 2013 | Document: JCT3V-E0175 |
| --- | --- |

| Title: | 3D-CE3.h related: An adaptive disparity vector derivation method for ARP in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents an adaptive disparity vector derivation (ADVD) method for advanced residual prediction (ARP) in 3D-HEVC. In ADVD, three DV candidates are derived from neighbouring blocks. The encoder can find the best DV candidate used in ARP and signal the index to the decoder.  Experimental results show that the proposed method can achieve 0.33% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], advanced residual prediction (ARP) [2] is adopted to replace the simple inter-view residual prediction for its better coding efficiency. In ARP, the base view is considered as a mirror of the current view. A mirror block is located with an estimated disparity vector (DV) and a mirror reference block is found for the mirror block with the same motion vector (MV) as the current block. Pseudo residues are generated by subtracting the reconstructed samples in the mirror block and in the mirror reference block. Then pseudo residues are used to predict the real residues of the current block. Fig. 1 depicts a sketch of ARP. In ARP, a coding unit (CU) can choose to use pseudo residual prediction or not. If pseudo residual prediction is used, pseudo residues can be weighted by ½ or not. The encoder can find the best ARP choice for each CU according to the rate-distortion optimization (RDO) criterion, and signal the choice in the bit-stream.

It plays a critical role to locate the mirror block in the base-view accurately since pseudo residues are similar to the real residues only if they are generated by similar samples. In ARP, the estimated DV is derived by a neighbouring block disparity vector (NBDV) procedure. The first encountered DV from a temporal or spatial neighbouring block is utilized by ARP as the estimated DV for the current block. Since such an estimated DV is not accurate enough, pseudo residual prediction may be inefficient in some cases.

Fig. 1.  A sketch of ARP, where PR represents pseudo residue

# Proposed method

An adaptive disparity vector derivation (ADVD) method is proposed to improve ARP coding efficiency. In ADVD, three DV candidates are derived from neighbouring blocks. In other words, the first three unique DVs in the NBDV procedure are treated as DV candidates in ADVD. The encoder can find the best DV candidate used in ARP according to RDO criterion, and signal the index to the decoder. Since the accuracy of the estimated DV is improved, the ½ weighting option is removed.

# Experimental results

The proposed method is integrated on HTM-7.0r1 [3]. Table 1 shows the results of the proposed method under common test condition [4]. It can be seen that the proposed method can achieve 0.33% BD-rate saving for the synthesized views in the common test condition.

Table 1.  Performance of the proposed method under common test condition

# Conclusion

In this proposal, an adaptive disparity vector derivation (ADVD) method is proposed to improve ARP coding efficiency. The proposed method improves the accuracy of the estimated DV. Experimental results reveal that 0.33% BD-rate saving for the synthesized views can be achieved in the common test condition.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1005, April, 2013.

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0177, April, 2013.

- HTM-7.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.