| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27th Jul – 2nd Aug. 2013 | Document: JCT3V-E0176 |
| --- | --- |

| Title: | AHG7: An efficient coding method for DLT in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Fabian Jäger3, Jicheng An1, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 3 Institut für Nachrichtentechnik RWTH Aachen University | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { kai.zhang, jicheng.an, shawmin.lei} @mediatek.com Fabian Jäger +49 (0) 241 80 27678 jaeger@ient.rwth-aachen.de |
| Source: | MediaTek Inc. and RWTH Aachen University |  |  |

_____________________________

# Abstract

This contribution presents a new coding method named enhanced range constrained bit-map (ERCBM) to code the depth lookup table (DLT) more efficiently. In ERCBM, the DLT is represented by a range constrained bit-map (RCBM) as proposed in JCT3V-C0142 and JCT3V-D0151. The bit-map can be coded more efficiently by applying differential run-length coding. In addition, the delta-DLT approach proposed in JCT3V-D0054 can be applied on the dependent view, in order to utilize the inter-view correlation without introducing any potential parsing dependency issue. Experimental results show that the average number of bits in DLT is reduced from 557 to 50.

# Introduction

Simplified depth coding (SDC) and a depth lookup table (DLT) are adopted into HTM [1][2]. For each depth coding unit (CU), if SDC is selected, one of three different prediction modes can be selected. After the prediction, the residual is not coded using transform. As the SDC prediction stage always results in one or two depth segments per coded block, a single residual DC depth value is coded for each of these segments. Moreover, the DLT is used to map coded depth values in SDC to valid depth values of the original depth map. The DLT is constructed based on an initial analysis of the input depth map and is then coded in a sequence parameter set (SPS).

DLT is an optional coding tool. In the current HTM, the encoder will not use DLT if more than half the values from 0 to MAX_DEPTH_VALUE (e.g., 255 for 8-bit depth samples) appear in the original depth map at the analysis step. Otherwise, DLT will be coded in SPS. In order to code DLT, the number of valid depth values is coded with Exp-Golomb code first. Then each valid depth value is also coded with an exp-Golomb code as depicted in Table 1.

Table 1.  DLT coding method in the current HTM

| dlt_flag[ layerID ] | u(1) |
| --- | --- |
| if( dlt_flag[ layerID ] ) { |  |
| num_depth_values_in_dlt[ layerID ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ layerID ][ j ] | ue(v) |
| } |  |
| } |  |

Exp-Golomb codes are efficient when shorter codes possess higher probabilities of occurrence. However, that is not the case for valid depth values and the number of valid depth values. As a result, exp-Golomb codes usually generate long code lengths, and thus DLT requires a large amount of bits to represent. In the common test condition, four sequences Balloons, Kendo, Newspapercc, and PoznanHall use DLT, as determined by the pre-analysis, while the other three do not. We further conducted statistic analysis for the four sequences. As depicted in Table 2, DLT requires 557.33 bits on average and accounts for more than 65% bits of SPS in depth components. In other words, the DLT coding makes SPS in depth components much larger than the one without DLT. Under the HTM encoder strategy, the number of bits generated by DLT achieves the maximum when there are 128 values in DLT, which are 128, 129…., 254, 255. In such a case, DLT generates 1937 bits. In the worst case, DLT generates 3365 bits when 0 - 255 are all in DLT.

Table 2.  Number of bits in DLT and SPS

|  |  | DLT Size (bits) | SPS Size (bits) | D/S (%) |
| --- | --- | --- | --- | --- |
| Balloons | V0_depth | 555 | 845 | 65.68% |
|  | V1_depth | 641 | 937 | 68.41% |
|  | V2_depth | 617 | 915 | 67.43% |
| Kendo | V0_depth | 485 | 775 | 62.58% |
|  | V1_depth | 637 | 933 | 68.27% |
|  | V2_depth | 649 | 947 | 68.53% |
| Newspapercc | V0_depth | 606 | 896 | 67.63% |
|  | V1_depth | 621 | 917 | 67.72% |
|  | V2_depth | 696 | 994 | 70.02% |
| PoznanHall2 | V0_depth | 420 | 712 | 58.99% |
|  | V1_depth | 380 | 678 | 56.05% |
|  | V2_depth | 381 | 681 | 55.95% |
| Average |  | 557.33 | 852.50 | 65.38% |

# Proposed method

To code the DLT more efficiently, an enhanced range constrained bit-map (ERCBM) coding method is proposed. With ERCBM, DLT is represented by a range constrained bit-map as proposed in JCT3V-C0142 [3] and JCT3V-D0151 [4], where min_dlt_value and diff_max_dlt_value are coded as unsigned integer to constrain the value range [MinDltValue, MaxDltValue] of DLT. Values out of this range are not in DLT. Values in DLT between (MinDltValue, MaxDltValue) can be represented by a bit-map BM[i] with i from 0 to MaxDltValue – MinDltValue - 2. If BM[i] = 1, then value (i + MinDltValue + 1) belongs to DLT. Otherwise, value (i + MinDltValue + 1) does not belong to DLT.

A differential run-length coding approach is applied to code the bit-map. In order to code these 0-run-lengths efficiently, a minimum 0-run-length is coded firstly, which is smaller or equal to all 0-run-lengths. Then differences between 0-run-lengths and the minimum 0-run-length are coded sequentially. Fig. 1 demonstrates an example of a bit-map, where 0-run-lengths are 3, 2, 2, and 3 sequentially. The minimum 0-run length equal to 2 is coded firstly, and then the differences equal to 1, 0, 0, and 1 are coded in order. The differences are coded by fixed-length coding with n bits. If a difference exceeds 2n-2, the excess will be coded with Exp-Golomb code. n is signaled before differences are coded. Thus the encoder can choose the optimal n which generates the minimum number of bits.

Fig. 1.  An example of 0-run-length in the bit-map

To guarantee a predictable as well as acceptable number of bits in the worst case, the encoder can choose to code the RCBM directly. In this way, a DLT is coded with 265 bits in the worst case for 8-bit depth samples. A conformable encoder should never code a DLT with more than 265 bits.

In addition, the delta-DLT method proposed in JCT3V-D0054 [5] is adopted on the dependent view to utilize the inter-view correlation. As depicted in Fig. 2, DLT0 and DLT1 are DLTs on the base view and on the dependent view respectively. They are both represented in a full bit-map form. A delta-DLT DLT1’ is calculated as DLT1’ = DLT0⊕DLT1. Then DLT1’ instead of DLT1 is passed to the DLT coder. After DLT1’ is decoded, DLT1 is reconstructed as DLT1 = DLT0⊕DLT1’. Since DLT0 and DLT1 are quite similar, the exclusive or operation will result a lot of zeros, which can benefits the 0-run-length coding. It should be noted that the inter-view information is only utilized at the reconstruction step, so delta-DLT approach does not introduce any parsing dependency issue. Therefore, ERCBM can work well no matter DLTs are coded in video parameter set (VPS), SPS, picture parameter set (PPS), or slice header.

Fig. 2.  Delta-DLT coding diagram

# Experimental results

The proposed methods are integrated on HTM-7.0r1 [6].	Table 3 shows the results of the proposed method under common test condition [7]. It can be seen that the coding performance is similar because SPS only takes a small percentage in the entire bit-stream.

In Table 4, the DLT sizes without and with the proposed method are compared. On average, the proposed method generates only 9.4% bits of the anchor to code DLT.

Table 3.  Performance with the efficient DLT coding method in SPS under common test conditions

# Conclusion

In this proposal, an efficient coding method for the depth lookup table (DLT) is proposed. The proposed method can reduce more than 90% of the original DLT size. Experimental results reveal that the proposed method provides almost the same coding performance as the current HTM.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 4.  Comparison between anchor and the proposed method on DLT sizes

|  |  | DLT Size (bits) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Anchor | Proposed | P/A (%) |
| Balloons | V0_depth | 555 | 70 | 12.61% |
|  | V1_depth | 641 | 40 | 6.24% |
|  | V2_depth | 617 | 38 | 6.16% |
| Kendo | V0_depth | 485 | 65 | 13.40% |
|  | V1_depth | 637 | 50 | 7.85% |
|  | V2_depth | 649 | 51 | 7.86% |
| Newspapercc | V0_depth | 606 | 101 | 16.67% |
|  | V1_depth | 621 | 16 | 2.58% |
|  | V2_depth | 696 | 31 | 4.45% |
| PoznanHall2 | V0_depth | 420 | 80 | 19.05% |
|  | V1_depth | 380 | 40 | 10.53% |
|  | V2_depth | 381 | 21 | 5.51% |
| Average |  | 557.33 | 50.25 | 9.41% |

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1005, April, 2013.

- F. Jager, “3D-CE6.h: Simplified depth coding with an optional depth lookup table,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B0036, October, 2012.

- K. Zhang, J. An, S. Lei, “3D-CE6.h related: An efficient coding method for DLT in 3DVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0142, January, 2013.

- K. Zhang, J. An, S. Lei, “3D-CE6.h results on an efficient coding method for DLT in 3DVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0151, April, 2013.

- F. Jager, “CE6.H related: Results on updating mechanism for coding of depth lookup table (delta-DLT),” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0054, April, 2013.

- HTM-7.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.