| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0177 |
| --- | --- |

| Title: | 3D-AVC HLS: constant depth SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

When using 3D-AVC with constant-valued depth view components as side information to enhance the RD performance of texture views, -13.5% Bjontegaard delta bitrate compared to MVC is reportedly obtained. This contribution proposes an SEI message to indicate the use of constant-valued depth view components. It is asserted that decoders can make use of the SEI message for obtaining lower computational complexity in decoding of depth view components and in depth-based disparity derivation as well as for error resilience.

# Introduction

Contribution JCT3V-E0035 studied the impact of different depth resolutions on the RD performance. One of the use cases studied in JCT3V-E0035 was to consider coding of depth maps only as side information to enhance the RD performance of texture views.

Among other cases, JCT3V-E0035 reported results of texture coding using only one value per each depth view component called Global Depth Value (GDV). In other words, depth maps contain samples with a constant sample value equal to GDV and are coded conventionally according to CTC. Depth view components with a global depth value could be used for example to enhance the RD performance of texture views when the encoder-side has no resources to perform full-blown depth estimation.

In JCT3V-E0035, the PSNR of coded texture views in the proposed GDV scheme along with bitrates of both the texture views and the associated depth views were compared to only texture coding statistics (without including depth maps bitrates), i.e. texture coding conforming to MVC. In other words, the anchor bitstreams were formed by extracting the texture views from the HP anchor of the CTC.

The GDV coding results of JCT3V-E0035 are reprinted in Table 1 for convenience. It can be observed that 3D-AVC using constant-valued depth view components provides a remarkable compression gain compared to MVC (on average, -13.5% in Bjontegaard delta bitrate).

Table 1. Compression efficiency of the GDV used as depth map Vs. Anchor (HP configuration)

If a decoder had knowledge that constant-valued depth view components are included in the bitstream, it could make use of the knowledge in the following ways:

- The decoder could omit the normal decoding process of the depth view component and instead populate the decoded depth view component with the constant sample value. Hence, lower complexity of the decoding process could be reached.

- The decoder could omit the normal macroblock-based disparity derivation from the depth view component and instead derive disparity only once per depth view component based on the constant sample value. Again, lower complexity of the decoding process could be reached.

- Subject to slice losses of the depth view component during transmission, the decoder could recover the lost slices in the decoded depth view component.

An SEI message to indicate the GDV for depth view components is therefore proposed in this contribution.

# Proposed Specification Text

| constant_depth( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| cd_num_views_minus1 | 5 | ue(v) |
| for( i = 0; i <= cd_num_views_minus1; i++ ) { |  |  |
| cd_view_id[ i ] | 5 | ue(v) |
| const_depth_value[ i ] | 5 | u(8) |
| } |  |  |
| } |  |  |

A constant depth SEI message indicates the views for which all the samples within a decoded depth view component have the same value. Moreover, the message indicates the sample value for each such decoded depth view component. This message may be present in any access unit and the semantics of the message are valid for the access unit containing the message.

NOTE: Encoders may use a constant sample value for encoding depth view components, when depth view components are primarily used for assisting multiview texture coding. Decoders may omit the conventional decoding process of the indicated depth view components and instead populate decoded depth view components with the constant values indicated in this SEI message. Moreover, as response to this SEI message, decoders may derive disparity from a constant depth sample value once per depth view component rather than on macroblock basis.

cd_num_views_minus1 + 1 specifies the number of depth view components in this access unit in which all samples have the same value. cd_num_views_minus1 shall be in the range of 0 to NumDepthViews – 1, inclusive.

cd_view_id[ i ] specifies the view_id value of a depth view component in which all samples have the same value.

const_depth_value[ i ] specifies the sample value for all the samples in the depth view component with view_id equal to cd_view_id[ i ] in this access unit.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).