| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27th Jul – 2nd Aug. 2013 | Document: JCT3V-E0178 |
| --- | --- |

| Title: | 3D-CE2.h related: A simplified DV derivation method in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a simplified DV derivation method in 3D-HEVC. In the current 3D-HEVC, a DV-MCP procedure is applied when no DV can be found from neighboring blocks. It is proposed to simplify this procedure in two aspects. First, the checking on DV-MCP block is omitted. Second, only two neighboring blocks may be accessed instead of five. Experimental results show that this simplification does not cause any BD-rate loss in the common test condition.

# Introduction

In the current 3D-HEVC [1], a DV-MCP approach [2] is applied in the DV derivation process when no DV can be found from any temporal or spatial neighboring blocks. In the DV-MCP procedure, five spatial neighboring blocks are checked one by one to identify if a neighboring block is a DV-MCP block. DV-MCP block is defined as a block which is coded in merge mode using inter-view motion vector prediction. If a neighboring block is a DV-MCP block, its derived DV, which is used to locate its inter-view motion vector prediction, will be output as the derived DV of the current block. Fig. 1 depicts a flowchart of the DV-MCP procedure and Fig. 2 shows neighboring blocks used in this procedure. It can be seen from Fig.1 and Fig.2 that a flag to indicate whether a block is a DV-MCP block and its derived DV must be stored since they will be used in the DV-MCP procedure of the following blocks.

To simplify the DV derivation process, it is proposed to remove DV-MCP by introducing a global variable: derived DV (DDV) [3]. DDV is output when no DV can be found from any neighboring blocks, and is updated after coding/decoding each PU. Since DDV is the only global updating variable introduced into reconstruction, it will break the traditional neighboring-based reconstruction rule and cause troubles to some applications. For example, parallelization supported by tiles or wavefront parallel processing (WPP) [4] is hampered by DDV. As another example, insertion of intra-blocks can no longer isolate error since one block does not depend on its neighboring blocks only due to DDV.

Fig. 1 A flowchart of the DV-MCP procedure.

Fig. 2 Neighboring blocks used in the DV-MCP approach.

# Proposed methods

It is proposed to simplify the DV-MCP procedure and keep its neighboring-based feature as well. DV-MCP is simplified in two aspects.

First, the checking on DV-MCP block is omitted. In the current HTM [1], DV is derived for each CU before it is coded/decoded [5]. This derived DV is stored for the CU without identifying whether it is a DV-MCP block. Thus the storage of the DV-MCP flag is required no more. A neighboring block is available when it is non-intra coded and in the same LCU-row with the current CU.

Second, only two neighboring blocks, i.e., L and A, may be accessed instead of five blocks. Actually, for most blocks which are not at the left boundary, only the left neighboring block needs to be accessed. Fig. 3 demonstrates a flowchart of the proposed simplified DV-MCP procedure.

Fig. 3 A flowchart of the simplified DV-MCP procedure.

# Experimental results

The proposed methods are integrated on HTM-7.0r1 [6].	Table 1 shows the results of the proposed method under CTC [7]. It can be seen that the simplification does not affect BD-rate performance obviously.

Table 1. Performance of the proposed method (CTC)

# Conclusion

In this proposal, a simplified DV derivation method in 3D-HEVC is proposed. The proposed method can reduce the complexity of DV-MCP significantly while keeping its neighboring-based feature. In addition, this simplification does not cause any BD-rate loss in the common test condition.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1005, April, 2013.

- J. Sung, M. Koo, S. Yea, “3D-CE5.h: Simplification of disparity vector derivation for HEVC-based 3D video coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-A0126, July, 2012.

- L. Zhang, Y. Chen, M. Karczewicz, “CE2.h related: Derived disparity vector for 3D-HEVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0194, April, 2013.

- C. Chi, etc. “Parallel scalability and efficiency of HEVC parallelization approaches,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, no. 12, pp. 1827-1838, December, 2012.

- J. Kang, Y. Chen, L. Zhang, M. Karczewicz, “CE2.h related: CU-based Disparity Vector Derivation in 3D-HEVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0181, April, 2013.

- HTM-7.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.