| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27th Jul – 2nd Aug. 2013 | Document: JCT3V-E0179 |
| --- | --- |

| Title: | 3D-CE5.h related: A simplified DMM prediction method in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a simplified depth map modeling (DMM) prediction method in 3D-HEVC. In the current 3D-HEVC, DMM prediction is conducted differently in different cases. Several judgments are involved to determine a specific case. In the proposed method, two of these cases are unified. Experimental results show that this simplification can achieve 0.1% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], DMM prediction is conducted differently in different cases [2]. Several judgments are involved to determine a specific case. As depicted in Fig. 1 copied from JCTVC-D0183 [2], DMM prediction cases are described as follows.

Set bT = (bPattern0,0 != bPatternN-1,0); set bL = (bPattern0,0 != bPattern0,N-1)

Case 1:  bT == bL && bL == 1

- DCX = (p-1,0 + p0,-1)>>1, DC1-X=(p-1,N-1 + pN-1,-1)>>1;

Case 2:  bT == bL && bL == 0

- DCX = (p-1,0 + p0,-1)>>1, DC1-X=2B-1;

Case 3:  bT != bL && bL == 1

- DCX = p(N-1)>>1,-1, DC1-X = p-1,N-1;

Case 4:  bT != bL && bL == 0

- DCX =p-1, (N-1)>>1, DC1-X = pN-1,-1;

(a) Case 1                        (b) Case 2

(c) Case 3                        (d) Case 4

Fig. 1.  DMM prediction cases

# Proposed method

It is proposed to unify case 1 and case 2 as a single case. The proposed prediction cases are as follows.

Case 1’:  bT == bL

- DCX = (p-1,0 + p0,-1)>>1, DC1-X=(p-1,2N-1 + p2N-1,-1)>>1;

Case 2’:  bT != bL && bL == 1

- DCX =p(N-1)>>1,-1, DC1-X = p-1,N-1;

Case 3’:  bT != bL && bL == 0

- DCX =p-1, (N-1)>>1, DC1-X = pN-1,-1;

It can be seen that case 1 and case 2 in HTM are unified to case 1’ in the proposed method. As shown in Fig. 2, (X+Y)>>1 is used to replace (S+T)>>1 or 128 in the current HTM.

Fig. 2.  Neighboring samples in the proposed method

# Experimental results

The proposed methods are integrated on HTM-7.0r1 [3].	Table 1 and Table 2 show the results of the proposed method under the common test condition [4] and the all-intra test condition respectively. The simplification causes no coding efficiency loss and can even achieve 0.1% BD-rate saving for the synthesized views.

Table 1.  Performance of the proposed method under common test condition

Table 2.  Performance of the proposed method under all-intra condition

# Conclusion

In this proposal, a simplified DMM prediction method in 3D-HEVC is proposed. The proposed method can unify two prediction cases. In addition, this simplification can achieve 0.1% BD-rate saving for the synthesized views.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1005, April, 2013.

- X. Zhao, L. Zhang, Y. Chen, M. Karczewicz, “CE6.h related: Simplified DC predictor for depth intra modes,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0183, April, 2013.

- HTM-7.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.