| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27th Jul – 2nd Aug. 2013 | Document: JCT3V-E0180 |
| --- | --- |

| Title: | 3D-CE5.h related: A simplified method on DMM mode coding in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a simplification on depth map modeling (DMM) mode coding in 3D-HEVC. In the current 3D-HEVC, the table index in DMM1 is binarizied as a fixed-length code, and all bins share one context model. It is proposed to code these bins in the bypass mode without any context model. Similarly, the table index in DMM3 and delta_end_abs_minus1 in DMM2 are also coded in the bypass mode. As a result, two context models can be removed. Experimental results show that the simplification causes no coding efficiency loss.

# Introduction

In the current 3D-HEVC [1], the table index in DMM1 is binarizied as a fixed-length code, and all bins share one context model, as an example shown in Fig.1. Since bins in the fixed-length code can hardly obey the same distribution, it may be inefficient to code them according to a single context model. In addition, the table index in DMM3 and delta_end_abs_minus1 in DMM2 are coded in the similar way.

Fig. 1.  An example of coding the table index in DMM1

# Proposed method

It is proposed to code these bins of the table index in DMM1/DMM3 and delta_end_abs_minus1 in DMM 2 in the bypass mode. Two context models can be removed since delta_end_abs_minus1 shares the same context model with dmm_delta_end_flag.

# Experimental results

The proposed methods are integrated on HTM-7.0r1 [2]. Table 1 and Table 2 show the results of the proposed method under the common test condition [3] and the all-intra test condition respectively. The simplification causes no coding efficiency loss for the synthesized views.

Table 1.  Performance of the proposed method under common test condition

Table 2.  Performance of the proposed method under all-intra condition.

# Conclusion

In this proposal, a reduction of context models for DMM coding is proposed. The proposed method can remove two context models as well as cause no coding efficiency loss for the synthesized views.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1005, April, 2013.

- HTM-7.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.