| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27th Jul – 2nd Aug. 2013 | Document: JCT3V-E0183 |
| --- | --- |

| Title: | 3D-CE3.h related: An additional texture merging candidate in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In 3D-HEVC depth map coding, a current texture merging candidate inherits motion parameters from the center position of the collocated texture prediction unit (PU). This contribution proposes an additional texture merging candidate, which inherits motion parameters from the right bottom position of the collocated texture PU. Experimental results show that the proposed method can achieve 0.1% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], an inter-view merging candidate is adopted into the merging candidate list at position 0 in texture coding, which inherits motion parameters from the center position of the collocated PU in the reference inter-view picture. Similarly a texture merging candidate is adopted into the merging candidate list at position 0 in depth coding, which inherits motion parameters from the center position of the collocated PU in the reference texture picture as depicted in Fig. 1.

Fig. 1.  The center position in the collocated PU

# Proposed method

JCT3V-D0109 [2] proposes an additional inter-view merging candidate in texture coding, which inherits motion parameters from the right bottom position of the collocated PU in the reference view picture. The additional candidate is inserted into the merging candidate list before the temporal motion vector prediction (TMVP) if it is not pruned.

Similarly, this contribution proposes an additional texture merging candidate in depth coding, which inherits motion parameters from the right bottom position of the collocated PU in the reference texture picture as depicted in Fig. 2. The proposed additional texture merging candidate is also inserted into the merging candidate list before TMVP, if it is not identical to the merging candidate at position 0.

Fig. 2.  The right bottom position in the collocated PU

# Experimental results

The proposed methods are integrated on HTM-7.0r1 [3]. Table 1 shows the results of the proposed method under the common test condition (CTC) [4]. The proposed method can achieve 0.1% BD-rate saving for the synthesized views.

Table 1.  Performance of the proposed method

# Conclusion

In this proposal, an additional texture merging candidate is proposed for depth coding. The proposed candidate inherits motion parameters from the right bottom position of the collocated texture PU. Experimental results show that 0.1% BD-rate saving for the synthesized views can be achieved.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1005, April, 2013.

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei, “3D-CE5.h related: Additional inter-view merging candidate,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0109, April, 2013.

- HTM-7.0.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0.1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.