    - Decoding process for prediction units in inter prediction mode

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma prediction block relative to the top left sample of the current luma coding block,

–	a variable nCS specifying the size of the current luma coding block,

–	a variable nPbW specifying the width of the current luma prediction block,

–	a variable nPbH specifying the width of the current luma prediction block,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma prediction samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1.

The decoding process for prediction units in inter prediction mode consists of the following ordered steps:

- The derivation process for motion vector components and reference indices as specified in subclause H.Error: Reference source not found is invoked with the luma coding block location ( xC, yC ), the luma prediction block location ( xB, yB ), the luma coding block size block nCS, the luma prediction block width and height, nPbW and nPbH, and the prediction unit index partIdx specifying as inputs and the luma motion vectors mvL0 and mvL1, the chroma motion vectors mvCL0 and mvCL1, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1, the luma motion vector arrays mvL0IvMC and mvL1IvMC, the chroma motion vector arrays mvCL0IvMC and mvCL1IvMC, the reference index arrays refIdxL0IvMC and refIdxL1IvMC, and the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC, and the flag ivMCFlag as outputs.

- When the ivMCFlag is equal to 0, the following applies:

  - The decoding process for inter sample prediction as specified in subclause H.Error: Reference source not found is invoked with the luma coding block location ( xC, yC ), the luma prediction block location ( xB, yB ), the luma coding block size block nCS, the luma prediction block width height, nPbW and nPbH, the luma motion vectors mvL0 and mvL1, the chroma motion vectors mvCL0 and mvCL1, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as inputs and the inter prediction samples (predSamples) which are a (nCSL)x(nCSL) array predSamplesL of prediction luma samples and two (nCSC)x(nCSC) arrays predSamplesCr, and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr as outputs.

  - For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xB..xB + nPbW − 1, y = yB..yB + nPbH− 1:

MvL0[ x ][ y ] = mvL0		(8-1)
MvL1[ x ][ y ] = mvL1		(8-2)

RefIdxL0[ x ][ y ] = refIdxL0		(8-3)
RefIdxL1[ x ][ y ] = refIdxL1		(8-4)

PredFlagL0[ x ][ y ] = predFlagL0		(8-5)
PredFlagL1[ x ][ y ] = predFlagL1		(8-6)

- When the ivMCFlag is equal to 1, For each block (blkX=0..nPbW/4-1, blkY=0..nPbH/4-1), the following applies:

  - The decoding process for inter sample prediction as specified in subclause H.Error: Reference source not found is invoked with the luma coding block location ( xC, yC ), the luma prediction block location ( xB+blkX*4, yB+blkY*4 ), the luma coding block size block 4, the luma prediction block width height, 4 and 4, the luma motion vectors mvL0IvMC[blkX][blkY] and mvL1IvMC[blkX][blkY], the chroma motion vectors mvCL0IvMC[blkX][blkY]  and mvCL1IvMC[blkX][blkY], the reference indices refIdxL0IvMC[blkX][blkY]  and refIdxL1IvMC[blkX][blkY], and the prediction list utilization flags predFlagL0IvMC[blkX][blkY]  and predFlagL1IvMC[blkX][blkY]  as inputs and the inter prediction samples (predSamples) which are a 4x4 array spPredSamplesL of prediction luma samples and two 2x2 arrays spPredSamplesCr, and spPredSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr as outputs

  - For each luma sample (x=0..3, y=0..3), the following applies:

PredSamplesL[blkX*4+x][blkY*4+y] = spPredSamplesL[x][y]

  - For each chroma sample (x=0..1, y=0..1), the following applies:

PredSamplesCb[blkX*2+x][blkY*2+y] = spPredSamplesCb[x][y]

PredSamplesCr[blkX*2+x][blkY*2+y] = spPredSamplesCr[x][y]

  - For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x=xB..xB+3, y=yB..yB+3:

MvL0[ blkX*4+x ][ blkY*4+y ] = mvL0IvMC[blkX][blkY]  		(8-7)
MvL1[ blkX*4+x ][ blkY*4+y ] = mvL1IvMC[blkX][blkY]  		(8-8)

RefIdxL0[ blkX*4+x ][ blkY*4+y ] = refIdxL0IvMC[blkX][blkY]  		(8-9)
RefIdxL1[ blkX*4+x ][ blkY*4+y ] = refIdxL1IvMC[blkX][blkY]  		(8-10)

PredFlagL0[ blkX*4+x ][ blkY*4+y ] = predFlagL0IvMC[blkX][blkY]  		(8-11)
PredFlagL1[ blkX*4+x ][ blkY*4+y ] = predFlagL1IvMC[blkX][blkY]  		(8-12)

      - Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1

- chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

- the luma motion vector arrays mvL0IvMC and mvL1IvMC,

- the chroma motion vector arrays mvCL0IvMC and mvCL1IvMC,

- the reference index arrays refIdxL0IvMC and refIdxL1IvMC,

- the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC,

- and the flag ivMCFlag

Let ( xP, yP ) specify the top-left sample location of the current luma prediction block relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

Let the variable currPic and ListX be the current picture and RefPicListX (with X being 0 or 1) of the current picture, respectively.

The function LongTermRefPic( picX, refIdx, LX ), with X being either 0 or 1, is defined as follows. If the picture with index refIdx from reference picture list LX of the picture picX was marked as "used for long term reference" at the time when picX was the current picture, LongTermRefPic( picX, refIdx, LX ) is equal to 1; otherwise LongTermRefPic( picX, refIdx, LX ) is equal to 0.

The variables vspModeFlag, ivpMvFlagL0 and ivpMvFlagL1, ivMCFlag are set equal to 0.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as prefFlagL0 and prefFlagL1, the following applies.

- If PredMode[ xC ][ yC ] is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause H.8.5.2.1.1 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH and the partition index partIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the flag vspModeFlag, the luma motion vector arrays mvL0IvMC and mvL1IvMC, the reference index arrays refIdxL0IvMC and refIdxL1IvMC, and the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC, and the flag ivMCFlag .

- Otherwise, if PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause H.8.5.2.1.1 is invoked with the luma location ( xC, yC ), luma location ( xP, yP ), variables nCS, nPbW and nPbH and the partition index partIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the flag vspModeFlag, the luma motion vector arrays mvL0IvMC and mvL1IvMC, the reference index arrays refIdxL0IvMC and refIdxL1IvMC, and the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC, and the flag ivMCFlag.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and MvdLX, the following applies.

    - The variables refIdxLX and predFlagLX are derived as follows.

      - If inter_pred_idc[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

          - refIdxLX = ref_idx_lX[ xP ][ yP ]		(Error: Reference source not found-13)
predFlagLX = 1		(Error: Reference source not found-14)

      - Otherwise, the variables refIdxLX and predFlagLX are specified by

          - refIdxLX = −1		(Error: Reference source not found-15)
predFlagLX = 0		(Error: Reference source not found-16)

    - The variable mvdLX is derived as follows.

        - mvdLX[ 0 ] = MvdLX[ xP ][ yP ][ 0 ]		(Error: Reference source not found-17)
mvdLX[ 1 ] = MvdLX[ xP ][ yP ][ 1 ]		(Error: Reference source not found-18)

    - When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), variables nPbW and nPbH, refIdxLX, and the partition index partIdx as the inputs and the output being mvpLX.

    - When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

        - uLX[ 0 ] = (mvpLX[ 0 ] + mvdLX[ 0 ] + 216) % 216	(Error: Reference source not found-19)
mvLX[ 0 ] = ( uLX[ 0 ] >= 215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(Error: Reference source not found-20)
uLX[ 1 ] = (mvpLX[ 1 ] + mvdLX[ 1 ] + 216) % 216	(Error: Reference source not found-21)
mvLX[ 1 ] = ( uLX[ 1 ] >= 215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(Error: Reference source not found-22)

NOTE 1 – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1.

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the following applies:

      - when the flag ivMCFlag is equal to 0, the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

      - When the flag ivMCFlag is equal to 1, for each block (blkX=0..nPbW/4-1, blkY=0..nPbH/4-1), the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLXIvMC[blkX][blkY] and refIdxLXIvMC[blkX][blkY] as inputs and the output being mvCLXIvMC[blkX][blkY],

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xP.. ( xP + nPbW − 1 ), y = yP..( yP + nPbH− 1 ) (with X being either 0 or 1):

  - IvpMvFlagLX[ x ][ y ] = ivpMvFlagLX		(Error: Reference source not found-23)
VspModeFlag[ x ][ y ] = vspModeFlag		(Error: Reference source not found-24)

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode[ xC ][ yC ] is equal to MODE_SKIP or PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- the luma motion vectors mvL0 and mvL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1,

- the flag vspModeFlag, specifying, whether the current PU is coded using view synthesis prediction,

- the luma motion vector arrays mvL0IvMC and mvL1IvMC,

- the reference index arrays refIdxL0IvMC and refIdxL1IvMC,

- the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC,

- the flag ivMCFlag

The variables singleMCLFlag is derived as follows.

  - If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

  - Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPbW and nPbH are set equal to nCS.

NOTE 2  – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

- The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1 and the flag vspModeFlag, the luma motion vector arrays mvL0IvMC and mvL1IvMC, the reference index arrays refIdxL0IvMC and refIdxL1IvMC, the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC, the flag ivMCFlag are derived as specified by the following ordered steps:

  - The derivation process for spatial merge candidates as specified in subclause G.8.5.2.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

  - The reference index for temporal merging candidate refIdxLX (with X being 0 or 1) is set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, refIdxLX, and mergeFlag being equal to 1 as the inputs and with the output being the availability flag availableFlagLXCol, the temporal motion vector mvLXCol, and the reference index refIdxLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

      - availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(Error: Reference source not found-25) 
predFlagLXCol = availableFlagLXCol		(Error: Reference source not found-26)

  - Depending on iv_mv_pred_flag[ nuh_layer_id ], the following applies.

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0, the flags availableFlagIvMC and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause H.8.5.2.1.2 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, as the inputs and the output is assigned to the availability flags availableFlagIvMC and availableFlagIvDC, the reference indices refIdxLXIvMC and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC, and the motion vectors mvLXIvMC and mvLXIvDC, the luma motion vector arrays mvL0IvMC and mvL1IvMC, the reference index arrays refIdxL0IvMC and refIdxL1IvMC, the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC (with X being 0 or 1, respectively)..

  - Depending on view_synthesis_pred_flag[ nuh_layer_id ], the following applies.

    - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0, the flag availableFlagVSP is set equal to 0.

    - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma locations ( xC, yC ) as input and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

  - Depending on DepthFlag, the following applies.

    - If DepthFlag is equal to 0, the variable availableFlagT is set equal to 0.

    - Otherwise ( DepthFlag is equal to 1), the derivation process for the texture merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH as the inputs and the outputs are the flag availableFlagT, the prediction utilization flags predFlagL0T and predFlagL1T, the reference indices refIdxL0T and refIdxL1T, and the motion vectors mvL0T and mvL1T.

  - The merge candidate lists mergeCandList and mergeCandIsVspFlag are constructed as specified by the following ordered steps:

  - The variable numMergeCand is set equal to 0.

  - When availableFlagT is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to T, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1, the following applies:

      - When the following condition is true,

        - availableFlagT = = 0 && availableFlagIvMC  = = 0,

        - or one or more of the following conditions are true, with N being replaced by T and IvMC:

        - availableFlagN = = 1 && predFlagLXN ! = predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && mvLXN ! = mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && refIdxLXN ! = refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, the following applies:

      - When the following condition is true ,

        - availableFlagT = = 0 && availableFlagIvMC = = 0,

or one or more of the following conditions is true, with N being replaced by T and IvMC:

        - availableFlagN = = 1 && predFlagLXN ! = predFlagLXB1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && mvLXN ! = mvLXB1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && refIdxLXN ! = refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP + nPbW − 1 ][ yP − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP + nPbW ][ yP − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

      - availableFlagA1 = = 0,

      - predFlagLXA1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXA1 ! = mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXA1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

      - availableFlagB1 = = 0,

      - predFlagLXB1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXB1 ! = mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXB1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagVSP is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to VSP, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal 1 and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH ] and the variable numMergeCand is increased by 1..

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to B2, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP − 1 ][ yP − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to Col, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - The variable numOrigMergeCand is set equal to numMergeCand.

  - When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, mergeCandIsVspFlag, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + k ] is set equal to 0.

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.2.1.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + numCombMergeCand + m ] is set equal to 0.

  - The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

- If N is not equal to IvMC, the following applies:

      - mvLX[ 0 ] = mvLXN[ 0 ]		(Error: Reference source not found-27)

      - mvLX[ 1 ] = mvLXN[ 1 ]		(Error: Reference source not found-28)

      - refIdxLX = refIdxLXN		(Error: Reference source not found-29)

      - predFlagLX = predFlagLXN		(Error: Reference source not found-30)

      - ivMCFlag = 0

- otherwise,

- ivMCFlag = 1

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nPbW + nPbH ) is equal to 12, the following applies.

      - refIdxL1 = −1		(Error: Reference source not found-31)

      - predFlagL1 = 0		(Error: Reference source not found-32)

  - The variable vspModeFlag is set equal to mergeCandIsVspFlag[ merge_idx[ xP][ yP ] ]..

  - When vspModeFlag is equal to 1, the following applies:

    - If RefPicList0[ 0 ] is a long-term reference picture, refIdxL0 is set equal to 0.

    - Otherwise ( RefPicList0[ 0 ] is a short-term reference picture ), refIdxL0 is set equal to AltRefIdxL0.

  - The disparity availability flag ivpMvFlagLX is derived as follows (with X being replace by 0 or 1).

    - If all of the following conditions are true, ivpMvFlagLX is set equal to 1

- [Ed. (GT) There is some redundancy in draft and software since disparities equal for both lists.(#7) ]

      - availableFlagIvMC = = 1

      - merge_idx[ xP][ yP ] = = 0

      - predFlagLXIvMC = = 1

[Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

    - Otherwise, ivpMvFlagLX is set equal to 0.

        - Derivation process for inter-view merge candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagIvMC and availableFlagIvDC specifying whether the inter-view merge candidates are available,

- the reference indices refIdxLXIvMC and refIdxLXIvDC,

- the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC,

- the motion vectors mvLXIvMC and mvLXIvDC,

- the luma motion vector arrays mvL0IvMC and mvL1IvMC,

- the reference index arrays refIdxL0IvMC and refIdxL1IvMC,

- the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC

The temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause H.8.5.2.1.10 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the prediction list indication X , the view order index RefViewIdx[ xP ][ yP ], the disparity vector MvRefinedDisp[ xP ][ yP ], the flag mergeFlag being equal to 1, and the reference index refIdxLX being equal to −1, and as the inputs and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC, the luma motion vector arrays mvL0IvMC and mvL1IvMC, the reference index arrays refIdxL0IvMC and refIdxL1IvMC, the prediction list utilization flag arrays predFlagL0IvMC and predFlagL1IvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

      - availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(Error: Reference source not found-33)
predFlagL0IvMC = availableFlagL0IvMC		(Error: Reference source not found-34)
predFlagL1IvMC = availableFlagL1IvMC		(Error: Reference source not found-35)

The disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for the disparity inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the view order index RefViewIdx[ xP ][ yP ], the disparity vector MvRefinedDisp[ xP ][ yP ], the prediction list indication X, the flag mergeFlag being equal to 1, and the reference index refIdxLX being equal to −1, as the inputs and the outputs are the flag availableFlagLXIvDC, the motion vector mvLXIvDC, and the reference index refIdxLXIvDC.

  - The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

      - availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC 	(Error: Reference source not found-36)
predFlagL0IvDC = availableFlagL0IvDC		(Error: Reference source not found-37)
predFlagL1IvDC = availableFlagL1IvDC		(Error: Reference source not found-38)

        - Derivation process for a temporal inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a reference view index refViewIdx.

- a disparity vector mvDisp,

- a flag mergeFlag specifying whether a merge candidate is derived,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the temporal inter-view motion vector candidate is available,

- a temporal inter-view motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1),

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX,

- the inter-view motion vector arrays spMvLXInterView,

- the reference index arrays spRefIdxLX,

- the prediction list utilization flag arrays spPredFlagLX.

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

For each block location (blkX = 0..nPSW/4-1, blkY=0..nPSH/4-1), the following applies:

- The spRefIdxLX[blkX][blkY] is set equal to -1

- The flag availableFlagLXInterView is set to 0. The flag spPredFlagLX[blkX][blkY] is set to 0.

- The reference layer luma location ( xRef, yRef ) is derived by

    - xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + blkX*4+2 + ( ( mvDisp[ 0 ] + 2 )  >>  2 ) )	(Error: Reference source not found-39)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + blkY*4+2 + ( ( mvDisp[ 1 ] + 2 )  >>  2 ) )	(Error: Reference source not found-40)

- Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx.

- [Ed. (GT:) In software refCU is the coding unit that covers the luma location ( xRef, yRef ) in the view component that closest (in terms of ViewIdx) to the current view component and that is included in one of the reference lists of the current view component.(#6)]

- [Ed. (CY): What is implemented in the software has no impact on the final results in common test condition, so it is preferred to have the current text and software changes may be desirable. ]

- When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply, for Y in the range of X to (1 – X), inclusive:

  - The variable refPredFlagLY is set equal to the prediction utilization flag predFlagLY of the prediction unit refPU.

  - The variable refRefIdxLY, is set equal to the reference index refIdxLY of  the prediction unit refPU.

  - The variable refMvLY is set equal to the motion vector mvLY of the prediction unit refPU.

  - The variable refRefPicListLY, is set equal to the reference picture list RefPicListLY of the prediction unit refCU.

  - When refPredFlagLY is equal to 1, the following applies for each i from ( mergeFlag ? 0 : refIdxLX ) to ( mergeFlag ? num_ref_idx_lX_active_minus1 : refIdxLX), inclusive:

    - When availableFlagLXInterView is equal to 0, and the picture order count of the picture refRefPicListLY[ refRefIdxLY ] is equal to the picture order count of the picture RefPicListLX[ i ], the flag availableFlagLXInterView is set equal to 1, the flag spPredFlagLX[blkX][blkY] is set equal to 1 and the following applies.

      - The motion vector spMvLXInterView[blkX][blkY] is derived by:

      - spMvLXInterView[blkX][blkY] [ 0 ] = refMvLY[ 0 ]		(Error: Reference source not found-41)
spMvLXInterView[blkX][blkY] [ 1 ] = refMvLY[ 1 ]		(Error: Reference source not found-42)

      - When mergeFlag is equal to 1, the reference index spRefIdxLX[blkX][blkY] is derived by:

      - spRefIdxLX[blkX][blkY] = i		(Error: Reference source not found-43)

    - if (spRefIdxL0[blkX][blkY]!=-1 || spRefIdxL1[blkX][blkY]!=-1)

      - if (availableFlagL0InterView == -1 && availableFlagL1InterView == -1)

        - For each block location (i = 0..blkX-1, j=0..blkY-1), the following applies

          - spMvLXInterView[i][j][0] = spMvLXInterView[blkX][blkY][0]

          - spMvLXInterView[i][j][1] = spMvLXInterView[blkX][blkY][1]

          - spRefIdxLX[i][j] = spRefIdxLX[blkX][blkY]

          - spPredFlagLX[i][j] = spPredFlagLX[blkX][blkY]

        - mvLXInterView[0] = spMvLXInterView[blkX][blkY][0]

        - mvLXInterView[1] = spMvLXInterView[blkX][blkY][1]

        - refIdxLX = spRefIdxLX[blkX][blkY]

        - availableFlagLXInterView = refIdxLX!=-1

      - tempMvLX[0] = spMvLXInterView[blkX][blkY][0]

      - tempMvLX[1] = spMvLXInterView[blkX][blkY][1]

      - tempRefIdxLX = spRefIdxLX[blkX][blkY]

      - tempPredFlagLX = spPredFlagLX[blkX][blkY]

    - else  if (tempRefIdxL0!=-1 || tempRefIdxL1!=-1)

      - spMvLXInterView[blkX][blkY][0] = tempMvLX[0]

      - spMvLXInterView[blkX][blkY][1] = tempMvLX[1]

      - spRefIdxLX[blkX][blkY] = tempRefIdxLX

      - spPredFlagLX[blkX][blkY] = tempPredFlagLX