      - Decoding process for inter prediction samples

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCS specifying the size of the current luma coding block,

–	variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

–	luma motion vectors mvL0 and mvL1,

–	chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma residual samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1.

- If VspModeFlag[ xC + xB ][ yC + yB ] is equal to 0, the following ordered steps apply:

  - Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and predSampleL0Cb, predSampleL1Cb, predSampleL0Cr, and predSampleL1Cr be (nPbW/2)x(nPbH/2) arrays of predicted chroma sample values.

  - For X being replaced by either 0 or 1, the following applies:

    - When predFlagLX is equal to 1, the following applies.

      - The variable resPredFlag is derived as specified in the following:

        - resPredFlag = 	( iv_res_pred_weight_idx != 0 )	
	(Error: Reference source not found-1)

      - If resPredFlag is equal to 1, the bilinear sample interpolation and residual prediction process as specified in subclause H.8.5.2.2.1 is invoked with the luma locations ( xC, yC ), ( xB, yB ), the size of the current luma coding block nCS, the width and the height of the current luma prediction block nPbW, nPbH, the prediction list indication X, the prediction list utilization flag predFlagLX, the reference index refIdxLX, and the motion vectors mvLX, mvCLX, as the inputs and the outputs are the arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr.

        - Bilinear sample interpolation and residual prediction process

The process is only invoked if res_pred_flag is equal to 1.

Inputs to this process are:

- a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,prediction list utilization flags, predFlagL0 and predFlagL1,

- the prediction list indication X,

- the prediction list utilization flag predFlagLX,

- the reference index refIdxLX,

- the motion vectors mvLX, mvCLX

Output of this process are:

- the (nPbW)x(nPbH) array predSamplesLXL,

- the (nPbW / 2)x(nPbH / 2) arrays predSamplesLXCb and predSamplesLXCr.

- If PicOrderCnt(RefPicListX [ refIdxLX ]) is equal to PicOrderCntVal, the following applies:

- tempARP = 1

- DV = mvLX

- DVC = mvCLX

- refViewIdx = ViewIdx of RefPicListX [ refIdxLX ]

- The derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), the variables nPbW and nPbH, the prediction list indication X , the view order index refViewIdx, the disparity vector DV, the flag mergeFlag being equal to 1, and the reference index refIdxLX being equal to −1 as the inputs and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

- If the availableFlagLXIvMC is equal to 0, the following applies;

  - mvLX[0] = 0

  - mvLX[1] = 0

  - for each reference picture with i=0.. num_ref_idx_lX_active_minus1, the following applies:

    - if the PicOrderCntVal of the picture RefPicListX[i] is not equal to the PicOrderCntVal of current picture and availableFlagLXIvMC is equal to 0, the refIdxLX is set equal to i.

- otherwise

  - mvLX[0] = mvLXIvMC[0]

  - mvLX[1] = mvLXIvMC[1]

  - refIdxLX = refIdxLXIvMC

  - the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

- Otherwise, the following applies:

- tempARP = 0

- DV = MvDisp[ xP ][ yP ]

- DVC = MvDisp[ xP ][ yP ]

The motion vector mvLX is scaled as specified in the following:

  - tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td		(Error: Reference source not found-2)

  - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(Error: Reference source not found-3)

  - mvLX =	Clip3( −32768, 32767, Sign( distScaleFactor * mvLX ) * 
	( ( Abs( distScaleFactor * mvLX ) + 127 ) >> 8 ) )	(Error: Reference source not found-4)

where td and tb are derived as:

  - td = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) )	(Error: Reference source not found-5)

  - tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ 0 ] ) )	(Error: Reference source not found-6)

The motion vector mvCLX is set equal to mvLX.

The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr  are derived as specified in the following:

- The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX equal to 0 as input.

- The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr. as the inputs.

The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr  are derived as specified in the following:

- Let currIvRefPic be the picture with PicOrderCnt( currIvRefPic ) equal to PicOrderCntVal and ViewIdx equal to RefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays currIvRefPicLXL, currIvRefPicLXCb, and currIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture currIvRefPic.

- The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the width nCS and height nCS, the motion vector mvLX equal to DV and the motion vector mvCLX equal to DVC, and the reference arrays with currIvRefPicLXL, currIvRefPicLXCb and currIvRefPicLXCr as the inputs.

The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr  are derived as specified in the following:

- Let refIvRefPic be the picture with PicOrderCnt( refIvRefPic ) equal to PicOrderCnt( RefPicListX[ 0 ] ) and ViewIdx equal to RefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays refIvRefPicLXL, refIvRefPicLXCb, and refIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture refIvRefPic.

- The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the width nCS and height nCS, the motion vector mvLX equal to ( mvLX + DV ) and the motion vector mvCLX equal to ( mvCLX + DVC ), and the reference arrays with refIvRefPicLXL, refIvRefPicLXCb and refIvRefPicLXCr as the inputs.

The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

The modified prediction samples predSamplesLXL[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as specified in the following:

  - predSamplesLXL[ x ][ y ] =	 tempARP ? currIvSamplesLXL[ x ][ y ] + ( ( predSamplesLXL [ x ][ y ] − refIvSamplesLXL[ x ][ y ] )  >>  shiftVal ): predSamplesLXL[ x ][ y ] + 
	( ( currIvSamplesLXL[ x ][ y ] − refIvSamplesLXL[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-7)

The modified prediction samples predSamplesLXCb[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 )−1 are derived as specified in the following:

  - predSamplesLXCb[ x ][ y ] =	 tempARP ? currIvSamplesLXCb[ x ][ y ] + ( ( predSamplesLXCb [ x ][ y ] – refIvSamplesLXCb[ x ][ y ] )  >>  shiftVal ): predSamplesLXCb[ x ][ y ] + 
	( ( currIvSamplesLXCb[ x ][ y ] − refIvSamplesLXCb[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-8)

The modified prediction samples predSamplesLXCr[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 ) − 1 are derived as specified in the following:

predSamplesLXCr[ x ][ y ] =	 tempARP ? currIvSamplesLXCr[ x ][ y ] + ( ( predSamplesLXCr [ x ][ y ] – refIvSamplesLXCr[ x ][ y ] )  >>  shiftVal ): predSamplesLXCr[ x ][ y ] + 
	( ( currIvSamplesLXCr[ x ][ y ] − refIvSamplesLXCr[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-9)