| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0185 |
| --- | --- |

| Title: | 3D-CE3.h related: Advanced temporal residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an advanced temporal residual prediction method to make the current advanced residual prediction method more complete. The experimental results show that the proposed method can achieve 0.8% BD-rate saving for video 1 and video 2, and 0.2% saving for synthesized views with no run time changes.

# Introduction

The advanced residual prediction (ARP) [1] was adopted in the last JCT-3V meeting. In ARP, to ensure high correlation between residues of two views, motion parameters of the current prediction unit (PU) in the current view picture are applied to the corresponding block in a reference view picture to generate pseudo residual in the reference view to be used for inter-view residual prediction. The corresponding block in the reference view picture is located by a derived disparity vector (DDV) [2]. Fig. 1 shows the concept of the current ARP.

Fig. 1.  The concept of the current ARP

However, the current ARP is not complete. It is only applied to motion compensated prediction but not to disparity compensated prediction. Therefore, the current ARP should be renamed as advanced inter-view residual prediction (AIVRP). In this proposal, it is proposed to further apply the ARP to disparity compensated prediction.

# Proposed advanced temporal residual prediction

An advanced temporal residual prediction (ATRP) method is proposed for the disparity compensated prediction. In ATRP, the disparity parameters of the current PU are applied to the corresponding block in a temporal reference picture in the same view to generate the reference residual in temporal direction. The corresponding block in the temporal reference picture is located by a derived motion vector (DMV), which is the motion vector (MV) of the block which the current DV is pointing to in the reference view. The concept of proposed ATRP is illustrated in Fig. 2, and the derivation of DMV is illustrated in Fig. 3.

Fig. 2.  The concept of the proposed ATRP

Fig. 3.  Derivation of DMV

# Experimental results

The proposed ATRP has been implemented on top of HTM7.0r1, and the experiments are conducted following the common test condition [3]. The coding performance is shown in Table 1.

Table 1.  Coding performance of the proposed ATRP

# Conclusion

This contribution proposes an advanced temporal residual prediction method. The experimental results show that the proposed method can achieve 0.8% BD-rate saving for video 1 and video 2, and 0.2% saving for synthesized views with no runtime changes. It is suggested to adopt this method to the next version of HTM.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0177, April, 2013.

- L. Zhang, Y. Chen, M. Karczewicz, “CE5.h: Disparity vector generation results,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-A0097, July, 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1100, April, 2013.