| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0187 |
| --- | --- |

| Title: | 3D-CE3.h Related: Improvement on Merge Candidate List Construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taesup, Kim Junghak, Nam Sehoon, Yea #221, Yangjae-dong, Seocho-gu, Seoul, Korea | Tel: Email: | +82-10-2573-6393 taesup.kim@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current 3D-HEVC (HTM7.0r1) [1], the merge candidate list is constructed by inheriting neighboring blocks’ motion parameters and introducing new motion parameters as default candidates, which are DCP and VSP mode with CU-level derived disparity vectors (NBDV and DoNBDV). In this contribution, it is proposed that the default VSP candidate has to use DoNBDV rather than NBDV to reduce redundancy in the merge candidate list without any additional complexity. Moreover, the pruning process of the merge candidate list construction is modified for the further reduction of redundancy. Both proposed approaches for improvement on the merge candidate list construction are combined and report that bit-rate gains are respectively 0.25% and 0.37% for the dependent views 1, 2, and the overall total bit-rate gain is 0.12%.

# Introduction

The NBDV is a derived disparity vector (inter-view motion parameters) that is directly referred from the neighboring blocks, and the DoNBDV is the refined NBDV by using the reference picture’s coded depth map. Both of them are derived on each CU, and belonging PUs share them for inter-view prediction tools. Especially, the merge candidate list uses DoNBDV for the default DCP candidate and NBDV for the default VSP candidate. The default VSP candidate with NBDV could be repeated by inheriting neighboring blocks’ motion parameters that increase redundancy in the merge candidate list. Moreover, the pruning process of the merge candidate list construction, in the current 3D-HEVC (HTM7.0r1), is not covering the default VSP candidate and only compares motion parameters without VSP flag, which is used to distinguish between DCP and VSP modes, that it could also degrade reducing the redundancy.

# Proposed Method

To insert the default VSP candidate without potential redundancy in the merge candidate list, it is proposed to use DoNBDV, instead of NBDV, for it as shown in Figure 1-(a). This could be done without any additional complexity that each PU use CU-level derived disparity vectors (NBDV and DoNBDV). Moreover, the pruning process is modified to cover the default VSP candidate and distinguish between DCP and VSP mode for the further reduction of redundancy on the merge candidate list construction as shown in Figure 1-(b).

Figure 1. It is proposed to (a) use DoNBDV instead of NBDV for the default VSP candidate, and (b) modify the pruning process for the merge candidate list construction.

# Experimental Results

Table 1. The result of using DoNBDV for the default VSP candidate on the merge candidate list. 
(Anchor: HTM 7.0 r1 CTC, Full frame )

Table 2. The result of using modification on the pruning process. 
(Anchor: HTM 7.0 r1 CTC, Full frame )

Table 3. The result of (a) using DoNBDV for the default VSP candidate in the merge candidate list with (b) modification on the pruning process. (Anchor: HTM 7.0 r1 CTC, Full frame )

# Conclusion

In this contribution, two approaches are proposed to improve the merge candidate list construction by reducing potential redundancy. The combined approach provides an overall bit rate gain of 0.12%. It is recommended to adopt the proposed modification into the current HTM and the common test conditions.

# References

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”,  JCT3V-D1100, Incheon, KR, April 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).