| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Vienna, AT, 27 July–2 Apr. 2013 | Document: JCT3V-E0189 |
| --- | --- |

| Title: | CE3.h related : AMVP candidate list construction for DCP blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Taesup Kim Jung Hak Nam Sehoon Yea 19 Yangjae-daero 11gil, Seocho-gu Seoul South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

At the last meeting, a list construction process for AMVP candidates was changed identically as one for simulcast HEVC, which means the AMVP process does not consider any inter-view prediction benefit. However, the AMVP process can be improved when the reference picture is an inter-view reference picture. Therefore, the AMVP list construction process for DCP blocks is proposed. For the proposed method, the refined disparity of neighboring blocks, which has been used for merge process, is inserted into the AMVP candidate list. As a result, it achieved approximately 0.1% of coding gain from texture video.

# Introduction

In 3D-HEVC, there are two prediction modes for inter prediction, a merge and a motion vector prediction mode (AMVP). For the merge mode, an inter-view motion vector predictor (IVMP), which is predicted from the neighboring view, is considered as one of merge candidates. Additionally, a refined disparity vector from a neighboring block is also added as one of merge candidates. These inter-view predictors have guaranteed more efficient coding performance. However, for the AMVP any inter-view prediction has not been taken into account since the last meeting. At the last meeting, a performance evaluation is presented that the IVMP for an AMVP candidate list construction brought only negligible coding efficiency, and it was adopted for the IVMP to be removed from the candidate list construction. But we still have a chance to improve the AMVP for inter-view prediction. If the currently referenced picture has different view ID, which means an inter-view reference picture, the motion vector predictor implies a disparity vector. In this document, an improvement for DCP blocks of AMVP are proposed.

# Proposed method

When the inter-view picture is current reference picture in the AMVP process, there are less amount of usable candidates compared to the temporal reference picture, since temporal mvp can be scaled and used if the mvps from the neighboring PUs have different reference indices. In order to increase the number of usable candidates for the DCP AMVP, reusing the refined disparity from a neighboring block, DoNBDV, as a supplement of AMVP candidates is proposed.

Fig. 1 An example for the proposed method

The proposed method is to fill the empty candidate with the DoNBDV instead of the zero vector, as shown in Fig. 1. According to the current process, if the number of candidates is smaller than the maximum number of candidates, it is filled with zero motion vector. Instead, in the proposed method if the candidate list is not filled maximally, then the DoNBDV, which is already calculated for the merge mode, is reused as the candidate. If there is no candidate for the current PU, the list is then filled with DoNBDV and NBDV for the first candidate and the second candidate, respectively.

# Experimental results

The performance evaluation for the proposed method has been conducted under CTC, according to JCT3V-D1100[1]. The experimental result for the proposed method is shown in Table 1. For the result of video 1 and video 2, we can see coding gains for most sequences and achieved 0.14% and 0.19% of coding gains in average, respectively.

Table 1 The experimental result against HTM-7.0r1 CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 101.4% | 100.0% | 101.3% |
| Kendo | 0.0% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 103.7% | 96.3% | 103.6% |
| Newspaper_CC | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | 103.4% | 99.8% | 103.3% |
| GT_Fly | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 101.0% | 98.7% | 105.6% |
| Poznan_Hall2 | 0.0% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 102.0% | 99.8% | 103.7% |
| Poznan_Street | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 101.5% | 99.1% | 106.6% |
| Undo_Dancer | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% | 0.3% | 100.8% | 105.2% | 106.1% |
| 1024x768 | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 102.8% | 98.7% | 102.7% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | 101.3% | 100.7% | 105.5% |
| average | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% | 102.0% | 99.8% | 104.3% |

# Conclusion

The proposed method is to reuse DoNBDV as one of AMVP candidates if the reference picture is the inter-view reference picture, when the number of PU-level motion vectors in neighbor blocks are not sufficient. The coding result presents it is beneficial to most of test sequences. Therefore, we recommend it is adopted for the next version of 3D-HEVC.

# Reference

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1100, April 2013, Incheon, KR.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).