| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0193 |
| --- | --- |

| Title: | 3D-CE5.h related: new intra coding structure for depth map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Eunyong Son, Junghak Nam, Sehoon Yea | Email: | jin78.heo@lge.com, eunyong.son@lge.com, junghak.nam@lge.com, sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents a new intra coding method for depth map. The proposed method substitutes the depth intra prediction tools, including depth modeling modes (DMM), region boundary chain (RBC) mode, and simplified depth coding (SDC) mode with non-conventional intra prediction method. Although the proposed method still uses the concept of SDC, it is only applied to the non-conventional intra prediction. The proposed method leads to bit rate increase of 1.1% for random access and 3.8% for all-intra configuration. However, the proposed method reduces encoding time by 37% and 93% for random access and all-intra configuration, respectively.

# Introduction

In order to enhance the BD-rate of synthesized views, three sets of depth intra prediction modes are used: depth modeling modes (DMM), region boundary chain (RBC) mode, and simplified depth coding (SDC) mode in the current 3DV-HTM. They are designed based on the characteristics of depth map and can improve coding performance. However, it takes long time to find the best prediction mode and uses additional syntax elements to signal its prediction mode becase of the addition of depth intra prediction modes. Moreover, the direciton of depth intra prediction modes can be overlapped with those of conventional intra prediction modes as well as the depth intra prediction mode and consequently it causes signaling overhead.

This contribution proposes an alternative intra coding path which is a non-conventional intra prediction on top of the conventional intra coding path. The non-conventional intra prediction reuses 35 prediction modes of the conventional intra prediction. If the non-conventional intra prediction mode is selected as the best prediction mode, the residual data is coded based on the SDC approach. That means that the differential index by a depth lookup table (DLT) is directly coded without transform and quantization steps. While, the convetnional intra prediction still uses transform and quantization processes.

The proposed design has the following advantages:

- Intra prediction Alignment: 35 conventional intra prediction modes are reused in the non-conventional intra prediction. Therefore, the non-conventional intra prediction can be aligned with the conventional intra prediction.

- Complexity reduction: DMM1 searches all possible wedgelet partition patterns to find the best matching partition. Such a full search requries a substantial amount of processing. The proposed method deteremines the best prediction modes in the totally 70 prediction modes (35 conventional and 35 non-conventional intra prediction modes).

- Dependency removal: DMM3 and DMM4 has dependency between texture and depth. However, since they are not used in the non-conventional intra prediction any more, the proposed method can remove dependency of texture and depth.

# Proposed Method

## New Intra Coding Structure

New intra coding structure has two coding paths; conventional intra coding and non-conventional intra coding for depth intra coding, as shown in Fig 1. For a conventional intra coding block, after the conventional intra prediction is used, the resulting residual is coded with transform and quantization. Finally, entropy coding is performed. While, if a block is coded with non-conventional intra prediction, a differential index generated by DLT is signaled without transform and quantization.

Figure 1.  New intra coding structure

## Non-conventional Intra Prediction

Non-conventional intra prediction reuses 35 directional prediction modes used in conventional intra prediction. It consists of the four steps.

- Reference pixels selection: The reference pixels are determined within the left and above adjacent pixels according to the prediction direction.

- Maximum slope point decision: It selects the boundary edge using the maximum slope point in the reference pixels.

- Block partition: The block is separated into two constant valued regions based on the direction of each intra prediction mode at the determined maximum slope point. Here, it employs the same directional modes defined in the conventional intra prediction, that is, 35 prediction modes.

- Block prediction: The average values of reference pixels for each region are used as the predictors. Each separated region is filled with each calculated predictor.

Figure 2.  Example of non-conventional intra prediction

## Signaling

New intra coding method, including conventional and non-conventional intra predictions are signaled by one syntax element. After signaling this syntax element, all other signaling processes relative to the non-conventional intra prediction are exactly the same as the conventional intra prediction, e.g. prev_intra_luma_pred_flag, mpm_idx, and rem_intra_luma_pred_mode.

# Experimental Results

According to the CE6.H description [1] and the common test conditions [2], three tests are conducted under random access (CTC) and intra-only configuration. First test is the result of the proposed method, and second test is the comparison result for 3DV-HTM7.0r1 and 3DV-HTM7.0r1 without depth intra coding tools, including DMM, RBC, and SDC. The last test is the comparison result for the proposed method and 3DV-HTM7.0r1 without depth intra coding tools.

## Test 1: Results of the proposed method

Table 1 Result for 3-view scenario under CTC

Table 2 Result for 3-view scenario under intra-only configuration

## Test 2: Comparison results for 3DV-HTM7.0r1 without depth intra coding tools and 3DV-HTM7.0r1

Table 3 Result for 3-view scenario under CTC

Table 4 Result for 3-view scenario under intra-only configuration

## Test 3: Comparison results for 3DV-HTM7.0r1 without depth intra coding tools and the proposed method

Table 5 Result for 3-view scenario under CTC

Table 6 Result for 3-view scenario under intra-only configuration

# Conclusion

This contribution proposes an alternative intra coding for depth map. The proposed non-conventional intra prediction removes the depth intra coding tools, including depth modeling modes (DMM), region boundary chain (RBC) mode, and simplified depth coding (SDC) mode and adds the non-conventional intra prediction method. The proposed method uses two different coding paths; conventional intra coding path with transform and quantization process and non-conventional intra coding path with SDC like coding. The proposed method significantly reduces encoding time, while provides 1.1% and 3.8% coding losses under common test conditions and all-intra configuration, respectively.

# Reference

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-A1100, Stockholm, Sweden, July 2012.

- Philipp Merkle, “Description of Core Experiment 6 (CE6) on Depth Intra Coding,” JCT3V-A1106, Stockholm, July 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).