| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 02 Aug. 2013 | Document: JCT3V-E0204 |
| --- | --- |

| Title: | CE5 related: Results on Simplified Binarization for depth_intra_mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Lu and Lu Yu Zhejiang University Zheda Road 38 Hangzhou, Zhejiang, China | Tel: Email: | +86 15968882677 yiqiu8932@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This proposal simplifies the binarization of the syntax element depth_intra_mode. By using truncated unary code, a new method of binarization is avoided. Moreover, this proposal achieves 0.1% and 0.00% BD-rate gains compared to 3D-HTM7.0_r1 under the common test condition (CTC) and all intra test condition with similar encoding/decoding time.

# Problem Statement

In 4th JCT-3V  Meeting, a unified intra mode singling design was proposed in JCT3V-D0195 [1] and adopted. JCT3V-D0195 significantly simplifies the syntax design of depth intra modes. However, a new syntax element depth_intra_mode was introduced and a new method of binarization was used for this syntax element, as shown in Table 1-1. Note that when (cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N) || (cLog2CbSize > 3 && cLog2CbSize < 6), a new method of binarization is used, which is not appropriate in the consideration of hardware implementation.

Table 1-1. Binarization for depth_intra_mode

| Name of depth_intra_mode | Bin String |  |  |
| --- | --- | --- | --- |
|  | ( cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( cLog2CbSize > 3 && cLog2CbSize < 6 ) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| INTRA_DEP_SDC_PLANAR | 00 | - | 0 |
| INTRA_DEP_NONE | 010 | 0 | 10 |
| INTRA_DEP_SDC_DMM_WFULL | 011 | - | - |
| INTRA_DEP_DMM_WFULL | 100 | 10 | - |
| INTRA_DEP_DMM_CPREDTEX) | 101 | - | - |
| INTRA_DEP_DMM_WPREDTEX) | 110 | 110 | - |
| INTRA_DEP_SDC_DC | 1110 | - | 11 |
| INTRA_DEP_DMM_WPREDDIR) | 11111 | - | - |
| INTRA_DEP_CHAIN | 11110 | 111 | - |

# Proposed Method

To simplify the binarization and avoid new method of binarization, we propose to use truncated unary code to binarize depth_intra_mode. However, in the consideration of coding efficiency, we use depth_intra_mode_offset_flag to indicate the offset of DepthIntraMode. Therefore, the DepthIntraMode can be derived form the depth_intra_mode_offset_flag and depth_intra_mode. The depth_intra_mode_offset_flag is coded in bypass mode. Therefore, the number of contexts is not increased.

The modified depth modelling table syntax and the modified binarization for depth_intra_mode are shown in Table 2-1 and Table 2-2, respectively. Changes of the specification by this proposal are indicated with yellow highlighting base on [2]. More detail can be found in the attachment of this proposal.

Table 2-1. Depth modelling table syntax

| depth_modeling_table( x0 , y0 ) { | Descriptor |
| --- | --- |
| if ((cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N) ¦¦ (cLog2CbSize > 3 && cLog2CbSize < 6)) |  |
| depth_intra_mode_offset_flag[ x0 ][ y0 ] | u(1) |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| …… |  |
| } |  |

Table 2-2. Binarization for depth_intra_mode

| Name of depth_intra_mode | Bin String |  |  |  |
| --- | --- | --- | --- | --- |
|  | (cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N) ¦¦ (cLog2CbSize > 3 && cLog2CbSize < 6) |  | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
|  | !depth_intra_mode_offset_flag | depth_intra_mode_offset_flag |  |  |
| INTRA_DEP_SDC_PLANAR |  | 0 | - | 0 |
| INTRA_DEP_NONE |  | 10 | 0 | 1 0 |
| INTRA_DEP_DMM_WPREDDIR) |  | 110 | - | - |
| INTRA_DEP_CHAIN |  | 111 | 1 1 1 | - |
| INTRA_DEP_SDC_DC | 0 |  | - | - |
| INTRA_DEP_SDC_DMM_WFULL | 10 |  | - | - |
| INTRA_DEP_DMM_CPREDTEX) | 110 |  | - | - |
| INTRA_DEP_DMM_WPREDTEX) | 1110 |  | 1 1 0 | - |
| INTRA_DEP_DMM_WFULL | 1111 |  | 1 0 | 1 1 |

# Experimental Results

Proposed method is implemented into HTM 7.0_r1 software, and proposed method is compared with HTM 7.0_r1. The test condition is common test condition (CTC)and All Intra Configuration.

Experimental results show that the proposed method achieves 0.1% and 0.00% BD-rate gains compared to 3D-HTM 7.0_r1 under CTC and all intra test condition with the similar encoding/decoding time.

## Results of CTC

Table 3-1: BD rate results for proposed method (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.0% |
| Newspaper_CC | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% |
| GT_Fly | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Poznan_Hall2 | 0.0% | -0.4% | 0.7% | 0.0% | 0.0% | -0.1% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| 1920x1088 | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |

Table 3-2: Complexity results for proposed method (CTC)

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 100.5% | 100.9% |
| Kendo | 101.1% | 102.6% |
| Newspaper_CC | 96.1% | 99.6% |
| GT_Fly | 97.8% | 98.0% |
| Poznan_Hall2 | 100.8% | 100.9% |
| Poznan_Street | 100.3% | 100.2% |
| Undo_Dancer | 97.3% | 97.5% |
| 1024x768 | 99.2% | 101.0% |
| 1920x1088 | 99.0% | 99.2% |
| average | 99.1% | 100.0% |

## Results of All Intra Configuration

Table 3-3: BD rate results for proposed method (All Intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.10% | -0.02% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.16% | 0.03% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.09% | -0.05% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.00% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | -0.01% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.02% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | 0.05% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.12% | -0.01% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.01% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.07% | 0.00% |

Table 3-4: Complexity results for proposed method (All Intra)

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 99.5% | 99.7% |
| Kendo | 94.9% | 98.0% |
| Newspaper_CC | 92.7% | 101.3% |
| GT_Fly | 95.9% | 101.6% |
| Poznan_Hall2 | 96.1% | 101.5% |
| Poznan_Street | 98.0% | 98.1% |
| Undo_Dancer | 97.7% | 98.5% |
| 1024x768 | 95.7% | 99.7% |
| 1920x1088 | 96.9% | 99.9% |
| average | 96.4% | 99.8% |

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[1] Q. Yu, L. Zhang, Y. Chen, S. Ma and M. Karczewicz, “CE6.h related: Unification of new intra modes in 3D-HEVC,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D0195, Apr. 2013.

[2] G. Tech, K. Wegner, Y. Chen and S. Yea “3D-HEVC Test Model 4,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1005, Apr. 2013.