| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Incheon, AU, 28 Jul. –02 Aug. 2013 | Document: JCT3V-E0205 |
| --- | --- |

| Title: | CE1.h: Forward Block-based View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang Lu Yu | Tel: Email: | +86 571 87953107 felixzyc@gmail.com yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution describes the forward block-based view synthesis prediction (FVSP) design for 3D-HEVC. FVSP estimates a reference block in the base view by using the base view depth map. Then pixels in the reference block are warped to a target block in current coding view to be the prediction block of current coding block. The overall bitrate savings on the coded and synthesized views are -0.2% for XGA sequences and -0.4% for HD sequences compared to HTM-7.0r1 and the decoding time is 99.6% for all sequences on average. The maximum bitrate savings of texture on dependent left and right views are -5.8% and -7.2%, respectively.

# Introduction

Backward block-based view synthesis prediction (BVSP) for 3D-HEVC was proposed at the 3rd JCT-3V meeting in Geneva [1]. BVSP in the current 3D-HEVC uses the neighboring block disparity vector (NBDV) to derive a depth block from a decoded view to perform the backward warping operation.

In this contribution, a forward-warping block-based view synthesis method is proposed.

# Forward Block-based View Synthesis Prediction (FVSP)

The forward block-based view synthesis prediction (FVSP) was proposed in JCT3V-D0168 [2] in the last meeting. It is applied on both texture and depth coding.

FVSP estimates a reference blocks in based view which is to be warped to current coding view to generate the view synthesis prediction block (called the target block). The reference blocks estimation procedure (described in 2.1) only dependents on depth map of the decoded view. Then reference block is warped to the target block by a low complexity forward-warping method (described in 2.2).

## Reference block estimation

Since the width and location of reference block in base view is not known when coding the current block. So firstly, the reference block is determined by four estimated correspondence of the four corner pixels of current block.

Fig 1. Corner pixels of current PU and the reference block

For a corner pixel (CP) CPj (x, y) (as shown in Fig 1(a)) of current block (of size width*height), 3 correspondence candidates (CC) are estimated:

CC1 = (x – disp(128), y)

CC2 = (x – disp(CC1), y)

CC3 = (x – disp(CC2), y)

where disp(X) is the disparity value of depth of X.

- For correspondence Rj (R0 and R1) of each corner pixel (CP0 and CP1) along LEFT boundary of current block:

Rj=argmaxCCi(CCi|CCi+disp(CCi)≤x)

Then left boundary (LB) of reference block is determine by the LEFT most correspondences:

LB=min(R0,R1)

- For correspondence Rj (R2 and R3) of each corner pixel (CP2 and CP3) along RIGHT boundary of current block:

Rj=argminCCi(CCi|CCi+disp(CCi)≥x)

Right boundary (RB) of reference block is determined by the RIGHT most correspondences:

RB=max(R2,R3)

Then reference block determined as a block lies between LB and RB with same height as current block (as shown in Fig 1(b)).

## Reference block warping

A low complexity forward-warping method [3] is used in this contribution:

- No z-buffering is required during the warping procedure;

- The target block has the same size as current block and the warping area in current coding view is restricted within the target block, i.e., pixels whose warping position is outside of target block are not warped to the current view;

- Interpolation operations are conducted only at most one time for every warped pixel in reference block. This operation is also used in BVSP;

- Interpolation operations are disabled for depth warping (i.e., integer-pixel warping precision).

# Constrained FVSP (CFVSP)

- Disparity used for estimating first correspondence

In the previous described FVSP, the disparity used for the first correspondence estimation in subclause 2.1 is a default disparity vector corresponding to the medium depth value, i.e., 128. In the proposed CFVSP, the first correspondence of every corner pixel is estimated by NBDV to achieve higher precision and efficiency of correspondence estimation.

- Limitation on depth access range

Furthermore, the range of depth map that used for correspondence estimation is constrained adaptively based on the size of prediction unit (PU) to reduce memory bandwidth required for FVSP.

For a MxN PU (M and N are width and height of the PU, respectively):

The motion/disparity compensated prediction (MCP/DCP) in current 3D-HEVC requires a MxN depth block (to retrieve the DoNBDV) and a (M+7)x(N+7) texture block (for vertical and horizontal interpolation);

The 4x4 VSP compensation in current 3D-HEVC requires a MxN depth block (located by NBDV and used for disparity conversion) and (M/4)*(N/4) texture blocks whose size is (4+7)x4;

The proposed CFVSP tries to achieve equivalent needs on number of texture-plus-depth samples compared to MCP/DCP in current 3D-HEVC.

In the proposed CFVSP, the range of depth maps is limited to rectangle whose size is (M+beta)xN, where beta is a variable corresponding to the PU size (listed in Table 1). The center of the rectangle is located by NBDV which is also utilized by current BVSP in 3D-HEVC.

Table 1. Beta value for PU with different size

| M(Width) | N(Height) | beta | beta+M |
| --- | --- | --- | --- |
| 4 | 8 | 5 | 9 |
| 8 | 4 | 13 | 21 |
| 8 | 8 | 7 | 15 |
| 8 | 16 | 4 | 12 |
| 16 | 8 | 10 | 26 |
| 16 | 16 | 5 | 21 |
| 16 | 32 | 3 | 19 |
| 32 | 16 | 9 | 41 |
| 32 | 32 | 5 | 37 |
| 32 | 64 | 2 | 34 |
| 64 | 32 | 8 | 72 |
| 64 | 64 | 4 | 68 |

For every correspondence estimated in subclause 2.1 (including CC1, CC2 and CC3), the horizontal coordinate is clipped by the left and right boundaries of the rectangle.

With the proposed limitation on depth access range, the maximal width of the reference block would be M+beta which results in equivalent requirement on total number of texture-plus-depth samples with MCP/DCP in current 3D-HEVC (as shown in Table 2). The proposed CFVSP requires a (M+beta)xN depth block and a (M+beta+7)xN texture block.

Table 3 shows the comparison result on texture-plus-depth samples requiremtn between 4x4 VSP and CFVSP. The result shows that the proposed CFVSP requires less or equal number of pixels when PU is larger than 8x8.

Table 2. Comparison on texture-plus-depth samples requirement between MCP/DCP and CFVSP

|  |  | MCP / DCP |  |  | 1x1 CFVSP |  |  |  |  | CFVSP vs MCP/DCP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | N | Texture | Depth | T+D |  |  | Texture | Depth | T+D |  |
|  |  | (M+7)x(N+7) | MxN | Total | beta | beta+M | (M+beta+7)xN | (M+beta)xN | Total |  |
| 4 | 8 | 165 | 32 | 197 | 5 | 9 | 128 | 72 | 200 | 101.52% |
| 8 | 4 | 165 | 32 | 197 | 13 | 21 | 112 | 84 | 196 | 99.49% |
| 8 | 8 | 225 | 64 | 289 | 7 | 15 | 176 | 120 | 296 | 102.42% |
| 8 | 16 | 345 | 128 | 473 | 4 | 12 | 304 | 192 | 496 | 104.86% |
| 16 | 8 | 345 | 128 | 473 | 10 | 26 | 264 | 208 | 472 | 99.79% |
| 16 | 16 | 529 | 256 | 785 | 5 | 21 | 448 | 336 | 784 | 99.87% |
| 16 | 32 | 897 | 512 | 1409 | 3 | 19 | 832 | 608 | 1440 | 102.20% |
| 32 | 16 | 897 | 512 | 1409 | 9 | 41 | 768 | 656 | 1424 | 101.06% |
| 32 | 32 | 1521 | 1024 | 2545 | 5 | 37 | 1408 | 1184 | 2592 | 101.85% |
| 32 | 64 | 2769 | 2048 | 4817 | 2 | 34 | 2624 | 2176 | 4800 | 99.65% |
| 64 | 32 | 2769 | 2048 | 4817 | 8 | 72 | 2528 | 2304 | 4832 | 100.31% |
| 64 | 64 | 5041 | 4096 | 9137 | 4 | 68 | 4800 | 4352 | 9152 | 100.16% |
|  |  |  |  |  |  |  |  |  | Ave. | 101.10% |

Table 3. Comparison on texture-plus-depth samples requirement between 4x4 VSP and CFVSP

|  |  | 4x4 VSP |  |  | 1x1 CFVSP |  |  |  |  | CFVSP vs 4x4 VSP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | N | Texture | Depth | T+D |  |  | Texture | Depth | T+D |  |
|  |  | (4+7)*4 *(M*N)/16 | MxN | Total | beta | beta+M | (M+beta+7)xN | (M+beta)xN | Total |  |
| 4 | 8 | 88 | 32 | 120 | 5 | 9 | 128 | 72 | 200 | 166.67% |
| 8 | 4 | 88 | 32 | 120 | 13 | 21 | 112 | 84 | 196 | 163.33% |
| 8 | 8 | 176 | 64 | 240 | 7 | 15 | 176 | 120 | 296 | 123.33% |
| 8 | 16 | 352 | 128 | 480 | 4 | 12 | 304 | 192 | 496 | 103.33% |
| 16 | 8 | 352 | 128 | 480 | 10 | 26 | 264 | 208 | 472 | 98.33% |
| 16 | 16 | 704 | 256 | 960 | 5 | 21 | 448 | 336 | 784 | 81.67% |
| 16 | 32 | 1408 | 512 | 1920 | 3 | 19 | 832 | 608 | 1440 | 75.00% |
| 32 | 16 | 1408 | 512 | 1920 | 9 | 41 | 768 | 656 | 1424 | 74.17% |
| 32 | 32 | 2816 | 1024 | 3840 | 5 | 37 | 1408 | 1184 | 2592 | 67.50% |
| 32 | 64 | 5632 | 2048 | 7680 | 2 | 34 | 2624 | 2176 | 4800 | 62.50% |
| 64 | 32 | 5632 | 2048 | 7680 | 8 | 72 | 2528 | 2304 | 4832 | 62.92% |
| 64 | 64 | 11264 | 4096 | 15360 | 4 | 68 | 4800 | 4352 | 9152 | 59.58% |
|  |  |  |  |  |  |  |  |  | Ave. | 94.86% |

# Simulation results and discussions

The proposed method was implemented into HTM 7.0r1. Simulations are conducted according to the common test conditions as defined in [4].

Table 3 shows the performance of the FVSP against HTM7.0r1. The total BD-rate gains are:

- 0.4% for video PSNR vs. video bitrate

- 0.4% for video PSNR vs. total bitrate

- 0.3% for synthesis PSNR vs. total bitrate

The maximum BD-rate gains for synthesis PSNR vs. total bitrate is 1.1%.

The decoding complexity of FVSP is very close to the anchor.

Table 3. FVSP performance compared to HTM7.0r1 anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  | 100.7% |
| Kendo | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% |  | 99.5% |
| Newspaper_CC | 0.0% | -0.3% | -0.5% | -0.1% | -0.1% | -0.4% |  | 100.2% |
| GT_Fly | 0.0% | -1.1% | -1.4% | -0.3% | -0.3% | -0.3% |  | 98.1% |
| Poznan_Hall2 | 0.0% | -0.4% | -0.5% | -0.2% | -0.1% | -0.3% |  | 100.0% |
| Poznan_Street | 0.0% | -0.6% | -0.4% | -0.1% | -0.1% | -0.1% |  | 99.0% |
| Undo_Dancer | 0.0% | -5.8% | -7.2% | -1.8% | -1.6% | -1.1% |  | 99.8% |
| 1024x768 | 0.0% | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% |  | 100.2% |
| 1920x1088 | 0.0% | -2.0% | -2.4% | -0.6% | -0.6% | -0.4% |  | 99.2% |
| average | 0.0% | -1.2% | -1.5% | -0.4% | -0.4% | -0.3% |  | 99.6% |

Table 4 shows the performance of the proposed CFVSP against HTM7.0r1. The total BD-rate gains are:

- 0.2% for video PSNR vs. video bitrate

- 0.2% for video PSNR vs. total bitrate

- 0.2% for synthesis PSNR vs. total bitrate

The maximum BD-rate gains for synthesis PSNR vs. total bitrate is 0.5%.

The decoding complexity of FVSP is very close to the anchor.

Table 3. FVSP performance compared to HTM7.0r1 anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% |  | 98.0% |
| Kendo | 0.0% | 0.0% | -0.4% | -0.1% | 0.0% | 0.0% |  | 99.4% |
| Newspaper_CC | 0.0% | -0.1% | -0.2% | -0.1% | 0.0% | -0.3% |  | 100.7% |
| GT_Fly | 0.0% | -0.2% | -0.4% | -0.1% | -0.1% | -0.2% |  | 96.4% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% |  | 98.1% |
| Poznan_Street | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% |  | 97.6% |
| Undo_Dancer | 0.0% | -2.4% | -3.1% | -0.8% | -0.8% | -0.5% |  | 96.8% |
| 1024x768 | 0.0% | -0.2% | -0.2% | -0.1% | 0.0% | -0.1% |  | 99.4% |
| 1920x1088 | 0.0% | -0.7% | -0.9% | -0.3% | -0.2% | -0.2% |  | 97.2% |
| average | 0.0% | -0.5% | -0.6% | -0.2% | -0.2% | -0.2% |  | 98.2% |

- Conclusions

In this contribution, a constrained forward-warping block-based view synthesis method is proposed. Better block-based synthesis images are generated by forward warping method. It achieves -0.2% gain for synthesis PSNR vs. total bitrates. The proposed CFVSP brings higher coding performance with similar decoding complexity compared the BVSP in anchor.

# References

- D. Tian, F. Zou, A. Vetro, “CE1.h: Backward View Synthesis Prediction Using Neibouring Blocks “, JCT3V-C0152, Geneva, CH, Jan. 2013.

- Y. Zhang, Y. Zhao, L. Yu, “CE1.h related: Forward Block-based View Synthesis Prediction”, JCT3V-D0168, Incheon, KR, Apr. 2013.

- S. Shimizu, S. Sugimoto and H. Kimata, “CE1.a related results on the modification of in-loop view synthesis”, JCT3V-A0023, Stockhom, SE, Jul. 2012.

- D. Rusanovskyy, K. Muller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-D1100, Incheon, KR, Apr. 2013.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).