| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0207r1 |
| --- | --- |

| Title: | 3D-CE1.h: Adaptive block partitioning for VSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

View synthesis prediction in 3D-HTM consists of two-step disparity compensated prediction (DCP). The first DCP is performed to derive a virtual depth map at PU level, and prediction signals are generated by the second DCP which is operated for each 4x4 pixels. Since no 4x4 inter prediction is supported in the HEVC specification, this contribution proposes to use either 4x8 or 8x4 block size in VSP. In this contribution, it is proposed that the block size is adaptively decided by analyzing virtual depth map generated by the first DCP. Experiments reportedly show that the average bitrate increases are 0.04% and 0.03% for coded and synthesis views, respectively, with reducing decoder complexity and removing 4x4 DCP.

# Introduction

The current test model of 3D-HEVC support view synthesis prediction (VSP) [1]. VSP is performed two-step disparity compensated prediction (DCP) where virtual depth map is first generated by DCP with NBDV (neighbouring disparity vector) and prediction signals are generated by DCP with disparity vectors derived from the virtual depth map.

The second DCP is performed for each 4x4 pixels in the current design although 4x4 inter prediction is prohibited in the base specification of HEVC. In order to exclude 4x4 inter prediction, some adaptive block partitioning methods have been proposed at the Incheon meeting. However, through the decoder complexity analysis during the meeting, it was realized that no proposal could reduce decoder complexity in the worst case.

In this contribution, it is proposed to decide either 4x8 or 8x4 partition for each 8x8 subblock in VSP using virtual depth map with less computational operations.

# Proposals

The proposed method is to decide block partition in 2nd DCP of VSP using virtual depth map. In the proposed method, the block partition is decided for each 8x8 block in VSP predicted block. For each 8x8 block, either 4x8 or 8x4 partition is selected by analyzing depth values at 4 corners of 8x8 block.

The block partition is decided by the following rule.

if vdepthTL<vdepthBR?0:1vdepthTR<vdepthBL?0:1 use 4x8partitionelse use 8x4partition

, where TL, TR, BL, and BR represent top-left, top-right, bottom-left, and bottom-right corner pixels, respectively, and ^ represents the exclusive OR (XOR) operator. Note that this check can be implemented by two comparison operations.

# Experiments

The proposed methods are built on the top of the reference software 3DV-HTM v7.0r1. And the experiments are conducted according to the common test condition [2].

TABLE 1 shows the simulation results by the proposed adaptive block partitioning. It can be seen that there are minor coding loss for video 1 and video 2 but the overall coding loss is negligible.

TABLE 1: HTM v7.0r1 vs. proposed adaptive partitioning for 2nd DCP in VSP

TABLE 2 shows the summary of the complexity analysis. The details can be found in the attached EXCEL sheet. The analysis shows that the complexity is decreased even in the worst case with complete removal of 4x4 inter predictions. Note that the data storage increase is inevitable since the proposed method always uses a larger block size for the 2nd disparity compensated predictions than HTM v7.0r1.

TABLE 2: Complexity analysis

# Conclusion

In this document, VSP with either 4x8 or 8x4 DCP is proposed. In the proposed method, block partition of 2nd DCP in VSP is decided by analyzing depth values at four corner samples of PU. The overall performance losses are negligible, but decoder complexity is decreased. And the proposed method can remove 4x4 inter predictions, which is better aligned to the HEVC design. We therefore recommend adopting the proposed method.

# Patent rights declaration

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “3D-HEVC Test Model 4”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-D1005, June 2013, Incheon, Korea.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-D1100, April 2013, Incheon, Korea.