| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0212 |
| --- | --- |

| Title: | AHG7 Related: Inter-view predictive coding method for DLT in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongwei Li Ming Li Ping Wu Guoqiang SHANG Yutang XIE No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | +86-025-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In HTM7.0, the depth loop-up table (DLT) is signaled in the corresponding sequence parameter set (SPS) for depth components in each view. Due to the fact that there is a strong correlation between the DLTs among different views, a method is proposed to further remove the redundancy in DLT information among views. Experimental results demonstrated that compared with HTM7.0 anchors, an average of about 55% DLT bits are saved with almost no impact on overall BD-BitRate change.

# Introduction

In 3D-HEVC, DLT is optionally used in simplified depth coding (SDC) method for depth intra coding. When DLT is enabled for SDC, each depth value is mapped to its corresponding index in the list of valid depth maps, and intra prediction is performed using the mapped index values. The residual of index values are transmitted, and the reconstructed index values are then mapped back to the corresponding valid depth values. The advantage of using DLT as described in Error: Reference source not found is the reduced bit depth of the residual index for sequences with reduced depth range.

To construct the look-up tables for mappings between depth values and indexes, all valid depth values are explicitly signaled in VPS as specified in H.7.3.2.1.2 [2] shown in Table 1.

- Table 1. Video parameter set extension syntax in H.7.3.2.1.2

| vps_extension2( ) { | Descriptor |
| --- | --- |
| … |  |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

Under the common test conditions (CTC) for HTM 7.0, the coding bits of DLT are counted and presented in Table 2.

Table 2. Number of bits used for signaling DLT in HTM-7.0

|  |  | DLT Size (bits) |
| --- | --- | --- |
| Balloons | View 0 | 556 |
|  | View 1 | 642 |
|  | View 2 | 618 |
| Kendo | View 0 | 486 |
|  | View 1 | 638 |
|  | View 2 | 650 |
| Newspaper | View 0 | 607 |
|  | View 1 | 622 |
|  | View 2 | 697 |
| PoznanHall | View 0 | 421 |
|  | View 1 | 381 |
|  | View 2 | 382 |
| Average | View 0 | 517.5 |
|  | View 1 | 570.8 |
|  | View 2 | 586.8 |
|  | All Views | 558.3 |

# Proposed method

To reduce the coding bits for DLT, an inter-view DLT prediction method is proposed. The syntax for this method on HTM7.0 is presented in Table 3 and Table 4.

Table 3.  Proposed syntax for inter-view DLT prediction in VPS extension on HTM7.0

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| dlt_valid_flag | u(1) |
| if(dlt_valid_flag){ |  |
| num_depth_values_in_unified_dlt | ue(v) |
| for ( j = 0; j < num_depth_values_in_ unified_dlt ; j++) |  |
| unified_dlt_depth_value [ j ] | ue(v) |
| } |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| VpsDepthFlag[ layerId ] = layerId % 2 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V DepthLayerFlag[ layerId ] should be derived from dimension_id to be aligned with basic idea of MV-HEVC Draft 3. ] |  |
| ViewId[ layerId ] = layerId >> 1 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V ViewId[ layerId ] should be derived as specified in MV-HEVC Draft 3. ] |  |
| view_id[ layerId ] [ Ed. (GT): This syntax element is different from ViewId[ layerId ]. See comment in MV-HEVC Draft 3. Renaming/alignment is required. ] | u(8) |
| if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| } |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| vps_dmm_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

The semantics for the proposed syntax are presented below.

dlt_valid_flag equal to 1 specifies that depth lookup table is to be used. dlt_valid_flag equal to 0 specifies that depth lookup table is not used and dlt_flag[ i ] shall be equal to 0 for j from 0 to vps_max_layers_minus1.

num_depth_values_in_unified_dlt specifies the number of elements in the unified depth lookup table for depth view components.

Table 4.  Proposed syntax for inter-view DLT prediction in SPS on HTM7.0

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| if ( DepthFlag ) { |  |
| dlt_flag | u(1) |
| if (dlt_flag) { |  |
| num_depth_values_in_dlt | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value [ j ] | ue(v) |
| num_removed_depth_values_from_unified_dlt | ue(v) |
| for ( j = 0; j < num_depth_diff_values_in_subdlt ; j++) |  |
| removed_depth_value_from_unified_dlt [ j ] | ue(v) |
| } |  |
| } |  |
| …… |  |
| } |  |

The semantics for the proposed syntax are presented below.

num_removed_depth_values_from_unified_dlt specifies the the number of depth values that are removed from unified depth lookup table in the construction of the depth lookup table for a depth component. The value of num_depth_values_in_dlt is derived as num_depth_values_in_dlt = num_depth_values_in_unified_dlt - num_removed_depth_values_from_unified_dlt.

removed_depth_value_from_unified_dlt[ j ] specifies the j-th depth value that is removed from unified depth lookup table in the construction of the depth lookup table for a depth component. The value of removed_depth_value_from_unified_dlt[ j ] shall be larger than the value of removed_depth_value_from_unified_dlt[ j – 1] for j from 1 to num_removed_depth_values_from_unified_dlt. The depth lookup table dlt_depth_value [ k ] for the depth component is derived as follow:

j = 0;

k = 0;

for (i=0; i<num_depth_values_in_unified_dlt; i++)

{

if ( unified_dlt_depth_value [ i ] == removed_depth_value_from_unified_dlt [ j ] )

{

if ( j < (num_removed_depth_values_from_unified_dlt - 1) )

j ++;

continue;

}

if( (unified_dlt_depth_value[ i ] < removed_depth_value_from_unified_dlt [ j ]) ||

(unified_dlt_depth_value[ i ] >

removed_depth_value_from_unified_dlt [num_removed_depth_values_from_unified_dlt - 1]) )

{

dlt_depth_value [layerId] [k] = unified_dlt_depth_value[ i ];

k ++;

}

}

When DLT is used in coding at least one depth component of the “multiview video + depth” signal, encoder sets dlt_valid_flag to 1. Then according to the proposed syntax and semantics, DLT is encoded in the following steps:

- Step 1: Merging the depth values in the DLTs for depth components into a unified DLT array, where the elements are arranged in ascending order.

- Step 2: Encoding the size of unified DLT and the depth values in it.

- Step 3: Encoding the number of depth values that are not used for the depth component of a view but exist in unified DLT.

- Step 4: Arranging the depth values mentioned in Step 3 in ascending order and encoding them in bit-stream.

# Experimental results

The proposed method was implemented on top of HTM7.0. The comparisons of sum DLT coding bits between the HTM7.0 anchor and the proposed method are listed in Table 5, where only the four sequences, i.e. Balloons, Kendo, Newspaper and PoznanHall2, are relevant and hence tested. As observed, when using the proposed method, the coding bits for DLT are reduced to 45.31% (Proposed/anchor).

Table 5. Number of bits used for signaling DLT in HTM-7.0

| Sequence | Proposed |  | HTM7.0 Anchor |  | Proposed/Anchor |
| --- | --- | --- | --- | --- | --- |
|  | DLT Size (bits) |  | DLT Size (bits) |  |  |
| Balloons | unified DLT | 651 | View 0 | 556 |  |
|  | DLT bits for View 0 | 103 |  |  |  |
|  | DLT bits for View 1 | 13 | View 1 | 642 |  |
|  | DLT bits for View 2 | 39 | View 2 | 618 |  |
|  | Total bits | 806 | Total bits | 1816 | 44.38% |
| Kendo | unified DLT | 650 | View 0 | 486 |  |
|  | DLT bits for View 0 | 172 |  |  |  |
|  | DLT bits for View 1 | 16 | View 1 | 638 |  |
|  | DLT bits for View 2 | 2 | View 2 | 650 |  |
|  | Total bits | 840 | Total bits | 1774 | 47.35% |
| Newspaper | unified DLT | 697 | View 0 | 607 |  |
|  | DLT bits for View 0 | 96 |  |  |  |
|  | DLT bits for View 1 | 81 | View 1 | 622 |  |
|  | DLT bits for View 2 | 2 | View 2 | 697 |  |
|  | Total bits | 876 | Total bits | 1926 | 45.48% |
| PoznanHall | unified DLT | 421 | View 0 | 427 |  |
|  | DLT bits for View 0 | 2 |  |  |  |
|  | DLT bits for View 1 | 46 | View 1 | 387 |  |
|  | DLT bits for View 2 | 45 | View 2 | 388 |  |
|  | Total bits | 514 | Total bits | 1184 | 43.41% |
| Total bits for All DLTs |  | 3036 |  | 6700 | 45.31% |

The performance evaluation following HTM7.0 CTC is shown in the Table 6 and 7. With the proposed method, the coding performance is almost not changed.

Table 6.  Performance comparisons between HTM7.0 CTC anchor and proposed (3 view test case)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.2% | 99.2% | 99.8.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.4% | 100.9% | 100.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.8% | 100.1% | 102.3% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 99.90% | 100.9% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 100.9% | 100.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.6% | 100.1% | 100.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.1% | 98.9% | 99.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.8% | 100.1% | 101.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.2% | 100.0% | 100.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 100.0% | 100.5% |

Table 7.  Performance comparisons between HTM7.0 CTC anchor and proposed (2 view case)

|  | video 0 | video 1 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | -0.02% | -0.02% |
| Kendo | 0.0% | 0.0% | 0.00% | -0.02% | -0.02% |
| Newspaper_CC | 0.0% | 0.0% | 0.00% | -0.02% | -0.02% |
| GT_Fly | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.00% | -0.03% | -0.03% |
| Poznan_Street | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% |
| Undo_Dancer | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% |
| 1024x768 | 0.0% | 0.0% | 0.00% | -0.02% | -0.02% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | -0.01% | -0.01% |
| average | 0.0% | 0.0% | 0.00% | -0.01% | -0.01% |

# Conclusions

In this document, an inter views’ DLT prediction method is proposed, which reduces the DLT coding bits by an average of 55%. Additionally, the test results while following CTC show that the proposed method provides almost the same coding performance as that of HTM7.0 anchor. Since the proposed method reduces the total DLT related parameter set bits significantly (note that such a working direction has been agreed with consensus in past two JCT-3V meetings), therefore, it is recommended to adopt this method into 3D-HEVC.

# References

- Fabian Jager. 3D-CE6.h: Simplified depth coding with an optional depth lookup table, JCT-3V document JCT3V-B0036, October, 2012.

- Gerhard Tech, Krzysztof Wegner, Ying Chen, Sehoon Yea. 3D-HEVC Test Model Description, draft specification, JCT-3V document JCT3V-C1005, January, 2013.

# Modified working draft

The modifications on the working draft are detailed in this section. The proposed inter-view DLT prediction method is integrated into VPS extension syntax and marked in yellow in Table 8.

Table 8.  Modified syntax for inter-view DLT prediction in VPS extension

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| dlt_valid_flag | u(1) |
| if(dlt_valid_flag){ |  |
| num_depth_values_in_unified_dlt | ue(v) |
| for ( j = 0; j < num_depth_values_in_ unified_dlt ; j++) |  |
| unified_dlt_depth_value [ j ] | ue(v) |
| } |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| VpsDepthFlag[ layerId ] = layerId % 2 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V DepthLayerFlag[ layerId ] should be derived from dimension_id to be aligned with basic idea of MV-HEVC Draft 3. ] |  |
| ViewId[ layerId ] = layerId >> 1 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V ViewId[ layerId ] should be derived as specified in MV-HEVC Draft 3. ] |  |
| view_id[ layerId ] [ Ed. (GT): This syntax element is different from ViewId[ layerId ]. See comment in MV-HEVC Draft 3. Renaming/alignment is required. ] | u(8) |
| if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| } |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| vps_dmm_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| num_removed_depth_values_from_unified_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_diff_values_in_subdlt[ layerId ] ; j++) |  |
| removed_depth_value_from_unified_dlt [ layerId ] [ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

The semantics for the proposed syntax are presented below.

dlt_valid_flag equal to 1 specifies that depth lookup table is to be used. dlt_valid_flag equal to 0 specifies that depth lookup table is not used and dlt_flag[ i ] shall be equal to 0 for j from 0 to vps_max_layers_minus1.

num_depth_values_in_unified_dlt specifies the number of elements in the unified depth lookup table for depth view components.

num_removed_depth_values_from_unified_dlt[ layerId ] specifies the the number of depth values that are removed from unified depth lookup table in the construction of the depth lookup table for the depth component with layer_id equal to layerId. The value of num_depth_values_in_dlt[ layerId ] is derived as num_depth_values_in_dlt[ layerId ] = num_depth_values_in_unified_dlt - num_removed_depth_values_from_unified_dlt [ layerId ].

removed_depth_value_from_unified_dlt[ layerId ][ j ] specifies the j-th depth value that is removed from unified depth lookup table in the construction of the depth lookup table for the depth component with layer_id equal to layerId. The value of removed_depth_value_from_unified_dlt[ layerId ][ j ] shall be larger than the value of removed_depth_value_from_unified_dlt[ layerId ][ j – 1] for j from 1 to num_removed_depth_values_from_unified_dlt [ layerId ]. The depth lookup table dlt_depth_value [ layerId ][ k ] for the depth component with layer_id equal to layerId is derived as follow:

j = 0;

k = 0;

for (i=0; i<num_depth_values_in_unified_dlt; i++)

{

if ( unified_dlt_depth_value [ i ] == removed_depth_value_from_unified_dlt [ layerId ][ j ] )

{

if ( j < (num_removed_depth_values_from_unified_dlt [ layerId ] - 1) )

j ++;

continue;

}

if( (unified_dlt_depth_value[ i ] < removed_depth_value_from_unified_dlt [ layerId ] [ j ]) ||

(unified_dlt_depth_value[ i ] >

removed_depth_value_from_unified_dlt [layerId] [num_removed_depth_values_from_unified_dlt - 1]) )

{

dlt_depth_value [layerId] [k] = unified_dlt_depth_value[ i ];

k ++;

}

}

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).